- 3GPP TSG-SA5 Meeting #136e	S5-212052rev1

- 01 March to 09 March 2021, E-meeting

- Source:	Intel

- Title:	pCR Update and evaluate the solution for jitter analysis

- Document for:	Approval

- Agenda Item:	6.5.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.809 “Study on enhancement of Management Data Analytics (MDA)”

# 3	Rationale

The UC, potential requirements and possible solution for jitter analysis have been studied in TR 28.809 [1], however the evaluation is missing.

This pCR is to update and evaluate the solution for jitter analysis.

# 4	Detailed proposal

1st Modified Section

### 6.3.7	Jitter analysis

#### 6.3.7.1	Use case

In addition to E2E latency, some new network services have extra new demands for SLA parameters. In extreme business scenarios such a V2X and telemedicine, network jitter also needs to be satisfied. Jitter assurance with high certainty becomes a new dimension of end-to-end delay-sensitive service quality.

The analysis of jitter does not cover the entire network, but the analysis should consider the whole process of data frame and also the definition of different network layers. It could be impacted by the configuration of clock synchronization, traffic forwarding, bandwidth reservation and latency etc. In some circumstances, it will also be affected by the external environment, which can only be resolved non-automatically.

The performance measurements and measurements related to data forwarding mechanisms can be used by the MDAS producer to generate a jitter analysis report. For example, in some network node, one particular data flow can have bandwidth competition with normal data flow which causes high network jitter. The MDAS will first analyse the bandwidth resource and provide bandwidth reservation modification so that it can satisfy network performance requirement of this particular data flow. Jitter is also related to E2E latency parameter, the jitter analysis will then need to be cross-domain, e.g. RAN domain, core network domain transport network domain. The jitter analysis from the multiple domains may be combined to calculate the end-to-end jitter, which can be checked against end-to-end SLA requirements. The jitter analysis will reference the E2E analysis result to support SLS assurance. In case of excessive end-to-end jitter, the jitter analysis from each domain may be used to identify the likely source of problems.

#### 6.3.7.2	Potential requirements

REQ-JITTER_ASS-CON-1: MDAS producer should have the capability to provide analytics report describing the jitter problem.

REQ-JITTER_ASS-CON-2: The analytics report describing the jitter problem should include the following information:

-	The identifier of the jitter issue;

-	Indication of jitter issue type;

-	The start time and end time of the jitter issue;

-	The geographical area and location where the jitter issue exists;

-	 Root cause of the jitter issue;

-	The objects affected by the jitter issue;

-	The severity level of the jitter issue;

-      The concrete parameters of jitter value;

-	The recommended actions to solve the jitter issue.

#### 6.3.7.3	Possible Solutions

##### 6.3.7.3.1	Solution description

The solution requires MDAS producer to collect various data and provide jitter analytics report. To support jitter assurance in order to satisfy the customer’s requirements, the MDAS producer is required to provide the analytics report as defined in 6.3.7.3.3. This procedure may be triggered by the request or periodically

##### 6.3.7.3.2 	Data required

The management data required to analyze the jitter are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | “S-NSSAI” as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information according to S-NSSAI |
| Performance measurement | Measurement data | Packet delay: “packet delay” measurement as defined in clause 5.1.1.1, clause 5.1.3.3, TS 28.552 [8]; Round-trip GTP Data Packet Delay: “Round-trip GTP Data Packet Delay” as defined in clause 5.4.1.9, TS 28.552 [8]; GTP packets delay in UPF: “GTP packets delay in UPF” as defined in clause 5.4.5, TS 28.552 [8]; Round-trip GTP Data Packet Delay on N9 interface: “Round-trip GTP Data Packet Delay on N9 interface” as defined in clause 5.4.4.1, TS 28.552 [8]; One way packet delay between NG-RAN and PSA UPF as defined in clause 5.4.7, TS 28.552 [8]; Round-trip packet delay between PSA UPF and NG-RAN as defined in clause 5.4.8, TS 28.552 [8]; One way packet delay between PSA UPF and UE as defined in clause 5.4.9, TS 28.552 [8]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; End-to-end Latency of 5G Network: “End-to-end Latency of 5G Network” as defined in clause 6.3.1, TS 28.554 [7]; Throughput for network slice: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; New measurements on UL and DL jitter for network slice in NG-RAN, on N3/N9 interface, between PSA UPF and UE, between PSA UPF and NG-RAN. |
| QoE Data | Measurement data | The details information of QoE data required by this case is FFS. |
| Service experience data | Network analytics data | The analysis result regarding observed service experience from the NWDAF defined in TS 23.288 [18]. |

Note: The above parameters may not be the complete list.

##### 6.3.7.3.3	Analytics report

The jitter analytics report contains the following information.

| Jitter Analytics Report | Attribute Name | Description |
| --- | --- | --- |
|  | Jitter Identifier | Identification of the jitter issue. |
|  | Jitter Issue Type | Indication of jitter issue type, e.g. RAN jitter issue, CN jitter issue. |
|  | Start Time | The start time of the jitter issue. |
|  | End Time | The end time of the jitter issue. |
|  | Location Information | The geographical area and location where the jitter issue exists. |
|  | Root cause | Root cause of the jitter issue. |
|  | Affected objects | The MOIs of cells, subnetwork or network slices affected by the jitter |
|  | Severity level | The severity level (e.g., critical, medium, not important) of the jitter issue. |
|  | Jitter Value | The average network latency with delay variation range |
|  | Recommended actions | The recommended actions to solve the jitter issue. |

#### 6.3.7.4	Evaluation

The solution described in clause 6.3.7.3 requires the analytics inputs as described in in clause 6.3.7.3.2, wherein

-	the S-NSSAI can be provided along with the performance measurements, NRM and planning data.

-	the delay related performance measurements and KPIs are available in TS 28.552 [8] and TS 28.554 [7], and the jitter related measurements can be defined in TS 28.552 [8].

-	QoE data as defined in TS 26.247 [29] and TS 26.114 [30] can be acquired through the procedures defined in TS 28.405 [31].

-	The analysis results regarding observed service experience from the NWDAF are defined in TS 23.288 [18].

With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.3.7.3.3 can be derived.

Therefore, this solution is a feasible candidate for jitter analysis.

End of Modified Sections