3GPP TSG SA WG5 Meeting 136-e		S5-212241

- electronic meeting, online, 1st - 10th March 2021 					                     revision for S5-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	use case – granular access control by operator

- Document for:	Approval

Agenda Item:

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TR 28.817: "Management and orchestration; Study on access control for management service"

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

3GPP management system provides MnSs to the MnSs’ consumer to provision and monitor 3GPP network including network slice. Based on operator’s policy and contract with the customer, the MnSs exposed to different consumers could be different. The operator could configure fine grain access control policies to restrict which MnSs are visible from which consumers

This pCR is to study granular access control on MnSs of 3GPP management system by mobile network operator

# 4	Detailed proposal

Start of 1st Change

## 5.x	use case – granular access control by operator

### 5.x.1	Description

As described in use case 5.1-5.4, there’re different MnS consumers of 3GPP management system. According to operator’s security policy, regulation and SLA between the operator and its customer, the access permission assigned to different consumer could be different. E.g. based on operator security policies, modification of a network slice may be allowed by high privileged MnS consumer while other MnS consumer can only read the network slice. Another example, based on operator security policies, reading associated network slice subnetwork of a network slice and corresponding managed functions could be available to an internal MnS consumer, but not visible to external MnS consumer. Additional example, according SLA signed between the operator and its customers, one MnS consumer may receive critical and major alarms, as well as KPI of a network slice allocated to the consumer, while another MnS consumer may only receive critical alarms of a network slice allocated to the consumer.

The operator needs to configure fine grain access control policies to differentiate the exposure of MnSs for the consumers.

Note: Exposure of 3GPP MnS does not mandate BSS bypassing for the cases the MnS consumer acts on behalf of the operator’s customer (cf. use cases in clauses 5.1 and 5.2). BSS needs to be involved (cf. sit between externa MnS consumer and OSS of the operator in some case) especially for accounting, auditing, etc.. In any case, fine grain access control will be enforced on OSS as only OSS has detail knowledge of the MnSs.

5.x.2	Issue and gaps

#### Potential security issues:

Without granular access control, MnSs of 3GPP management system could be exposed to unauthorized consumers after the consumers being authenticated. e.g. an authenticated vertical customer of an operator could delete a network slice allocated to the customer but shared by multiple consumers, or an authenticated vertical customer could read network functions (and attributes) which compose the network slice allocated to the customer, however, the operator does not want to expose this kind of detail network topology or NFs to the customer, or an authenticated vertical customer could receive all alarms and KPI related to a network slice allocated to the customer, that is unexpected by the operator without any SLA or NDA in place.

#### Gap:

MnS and MnS components A, B, C were defined in Management and orchestration Architecture framework (see [2]), and Management Capability Exposure Governance (MCEG) was also introduced in the spec (see 4.4. of [2]) to provide exposure governance on MnS components A, B, C.

However, how does MCEG control the access/exposure of MnS based on permissions granted to a MnS consumer after the consumer being authenticated, as well as how to associate the access permission/policy of the MnS consumer (which derived from security policies of the operator, regulation, and SLA signed between the operator and customer) to the MnS component A, B, C to realize MCEG, are not investigated or solved in existing specification or study report.

End of Change

Start of 2nd Change

- 6	Potential requirements for access control for MnS

- The 3GPP management system could have the capability to support operator to configure fine grain/granular access policies for a MnS consumer based on MnS Component Type A/operations

Note: e.g. In case of provisioning a MOI, expose only read MOI operation to the MnS consumer instead of all CRUD operations.

- The 3GPP management system could have the capability to support operator to configure fine grain/granular access policies for a MnS consumer based on MnS Component Type B/managed object

Note: e.g. expose some MOIs to the MnS consumer but hide others, or expose some attributes of a MOI to the MnS consumer but hide other attributes

- The 3GPP management system could have the capability to support operator to configure fine grain/granular access policies for a MnS consumer based on MnS Component Type C/managed data

Note: e.g. expose critical alarms of a MOI to the MnS consumer but hide all other alarms and performance data of the MOI.

- The 3GPP management system could have the capability to support operator to configure fine grain/granular access policies for a MnS consumer based on any combination of MnS Component Type A, B and C.

End of Change