| 3GPP TR 28.809 V1.3.0 (2021-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on enhancement of Management Data Analytics (MDA) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	12

4	Concepts and overview	12

4.1	Overview	12

4.2	MDA functionality and service framework	12

5	MDA process and role	13

5.1	MDA role in management loop	13

5.2	Management interaction with NWDAF and gNB	14

5.3	MDA process	16

6	Use cases, potential requirements and possible solutions	17

6.1	Coverage related issues	17

6.1.1	Coverage issue analysis	17

6.1.1.1	Use case	17

6.1.1.2	Potential requirements	18

6.1.1.3	Possible solutions	18

6.1.1.3.1	Solution description	18

6.1.1.3.2	Data required for coverage issue analysis	18

6.1.1.3.3	Analytics report for coverage issue	19

6.1.1.4	Evaluation	19

6.1.2	Slice coverage optimization	20

6.1.2.1	Use case	20

6.1.2.2	Potential requirements	20

6.1.2.3	Possible solutions	21

6.1.2.3.1	 Solution description	21

6.1.2.3.2	 Data required	21

6.1.2.3.3	 Analytics report	22

6.1.2.3.3.1	 TA optimization	22

6.1.2.3.3.2	 gNB optimization	23

6.1.2.4	Evaluation	23

6.1.3	 Paging optimization	24

6.1.3.1	Use case	24

6.1.3.2	Potential Requirements	24

6.1.3.3	Possible Solutions	25

6.1.3.3.1	Solution description	25

6.1.3.3.2 	Data required	25

6.1.3.3.3	Analytics report	25

6.1.3.4	Evaluation	25

6.2	Resource related issues	26

6.2.1	RAN user plane congestion analysis	26

6.2.1.1	Use case	26

6.2.1.2	Potential requirements	26

6.2.1.3	Possible Solutions	26

6.2.1.3.1	Solution description	26

6.2.1.3.2	Data required for RAN user plane congestion problem	27

6.2.1.3.3	Analytics report for RAN user data congestion	27

6.2.1.4	Evaluation	28

6.2.2	Resource utilization analytics	28

6.2.2.1	Use case	28

6.2.2.2	Potential requirements	29

6.2.2.3	Possible solutions	29

6.2.2.3.1	Solution description	29

6.2.2.3.2	Required data for resource utilization analysis for RAN domain	29

6.2.2.3.3	Required data for resource utilization analysis for CN domain	30

6.2.2.3.4	Analytics report for resource utilization analysis	31

6.2.2.4	Evaluation	31

6.2.3	Cross-slice resource optimization	31

6.2.3.1	use case	31

6.2.3.2	Potential requirements	32

6.2.3.3	Possible Solution	32

6.2.3.3.1	Solution description	32

6.2.3.3.2	Required data for cross-slice resource optimization analysis	32

6.2.3.3.3	Analytics report for cross-slice resource optimization	32

6.2.3.4	Evaluation	32

6.2.4	NAS level congestion control optimization	33

6.2.4.1	Use case	33

6.2.4.2	Potential requirements	33

6.2.4.3	Possible solutions	33

6.2.4.3.1	Solution Description	33

6.2.4.3.2	Data required for NAS level congestion control analysis	33

6.2.4.3.3	Analytics report	33

6.2.4.4	Evaluation	34

6.3	SLS analysis related issues	34

6.3.1	E2E latency analysis	34

6.3.1.1	Use case	34

6.3.1.2	Potential requirements	35

6.3.1.3	Possible solutions	35

6.3.1.3.1	Solution description	35

6.3.1.3.2	Required data for latency analysis for RAN domain	35

6.3.1.3.3	Required data for latency analysis for CN domain	36

6.3.1.3.4	Required data for E2E latency analysis for cross domain	36

6.3.1.3.5	Analytics report for latency analysis	37

6.3.1.4	Evaluation	37

6.3.2	Network slice load analysis	38

6.3.2.1	Use case	38

6.3.2.2	Potential requirements	38

6.3.2.3	Possible solutions	39

6.3.2.3.1	Solution description	39

6.3.2.3.2	Required data for network slice load analysis for cross domain	39

6.3.2.3.3	Required data for network slice load for RAN domain	39

6.3.2.3.4	Required data for network slice load for CN domain	40

6.3.2.3.5	Analytics report for network slice load analysis	40

6.3.2.4	 Evaluation	41

6.3.3	Service experience analysis	41

6.3.3.1	Use case	41

6.3.3.2	Potential requirements	42

6.3.3.3	Possible solutions	42

6.3.3.3.1	Solution description	42

6.3.3.3.2	Data required for service experience analysis	42

6.3.3.3.3	Analytics report for service experience analysis	42

6.3.3.4	Evaluation	43

6.3.4	Network slice throughput analysis	43

6.3.4.1	Use case	43

6.3.4.2	Potential requirements	43

6.3.4.3	Possible solutions	44

6.3.4.3.1	Solution description	44

6.3.4.3.2	Data required for network slice throughput statistics/predictions for RAN domain	44

6.3.4.3.3	Data required for network slice throughput statistics/predictions for CN domain	44

6.3.4.3.4	Data required for network slice throughput statistics/predictions for cross domain	45

6.3.4.3.5	Analytics report for network slice throughput analysis	45

6.3.4.4	 Evaluation	45

6.3.5	Uplink/downlink throughput per UE in network slice analysis	45

6.3.5.1	Use case	45

6.3.5.2	Potential requirements	46

6.3.5.3	Possible solutions	46

6.3.5.3.1	Solution description	46

6.3.5.3.2	Required data for UE uplink/downlink throughput analysis	46

6.3.5.3.3	Analytics report for UE uplink/downlink throughput analysis	47

6.3.5.4	Evaluation	47

6.3.6	KPI anomaly analysis	48

6.3.6.1	Use case	48

6.3.6.2	Potential requirements	48

6.3.6.3	Possible solutions	48

6.3.6.3.1	Solution description	48

6.3.6.3.2	Data required for KPI anomaly analysis for RAN domain	49

6.3.6.3.3	Data required for KPI anomaly analysis for CN domain	49

6.3.6.3.4	Data required for KPI anomaly analysis for cross domain	49

6.3.6.3.5	Analytics report for KPI anomaly analysis	50

6.3.6.3.6	Analytics report for network situation analysis	50

6.3.6.4	Evaluation	50

6.3.7	Jitter analysis	51

6.3.7.1	Use case	51

6.3.7.2	Potential requirements	51

6.3.7.3	Possible Solutions	51

6.3.7.3.1	Solution description	51

6.3.7.3.2 	Data required	51

6.3.7.3.3	Analytics report	52

6.3.7.4	Evaluation	53

6.3.8	Network slice traffic projection	53

6.3.8.1	Use case	53

6.3.8.2	Potential Requirements	53

6.3.8.3	Possible Solutions	54

6.3.8.3.1 	Solution description	54

6.3.8.3.2 	Data required	54

6.3.8.3.3 	Analytics report	54

6.3.8.4	Evaluation	54

6.4	Fault management related issues	55

6.4.1	Alarm incident analysis	55

6.4.1.1	Use case	55

6.4.1.2	Potential requirements	55

6.4.1.3	Possible Solutions	55

6.4.1.3.1	Solution description	55

6.4.1.3.2	Data required for alarm incident analysis for RAN domain	56

6.4.1.3.3	Data required for alarm incident analysis for CN domain	56

6.4.1.3.4	Analytics report for alarm incident analysis	57

6.4.1.4	Evaluation	58

6.4.2	Fault prediction analysis	58

6.4.2.1	Use case	58

6.4.2.2	Potential requirements	59

6.4.2.3	Possible solutions	59

6.4.2.3.1	Solution description	59

6.4.2.3.2	Data required for fault prediction analysis for RAN domain	59

6.4.2.3.3	Data required for fault prediction analysis for CN domain	59

6.4.2.3.4	Analytics report for fault prediction analysis	60

6.4.2.4	Evaluation	60

6.4.3	Alarm malfunction analytics	60

6.4.3.1	Use case	60

6.4.3.2	Potential requirements	61

6.4.3.3	Possible solutions	61

6.4.3.3.1	 Solution description	61

6.4.3.3.2	 Data required	61

6.4.3.3.3	 Analytics report	62

6.4.3.4	Evaluation	62

6.5	Mobility management related issues	63

6.5.1	Handover optimization	63

6.5.1.1	Use case	63

6.5.1.2	Potential requirements	63

6.5.1.3	Possible solutions	64

6.5.1.3.1	 Solution description	64

6.5.1.3.2	 Data required	64

6.5.1.3.3	 Analytics report on gNB resource consumption	65

6.5.1.4 	Evaluation	66

6.5.2	Inter-gNB Beam Selection Optimization	67

6.5.2.1	Use case	67

6.5.2.2	Potential requirements	67

6.5.2.3	Possible solutions	68

6.5.2.3.1	 Solution description	68

6.5.2.3.2	 Data required	68

6.5.2.3.3	 Analytics report	68

6.5.2.4 	Evaluation	68

6.5.3	Load Balancing optimization	69

6.5.3.1	Use case	69

6.5.3.2	Potential requirements	69

6.5.3.3	Possible solutions	70

6.5.3.3.1	 Solution description	70

6.5.3.3.2	 Data required	70

6.5.3.3.3	 Analytics report	71

6.5.3.4	Evaluation	72

6.5.4	Mobility performance analysis	72

6.5.4.1	Use case	72

6.5.4.2	Potential requirements	73

6.5.4.3	Possible solutions	73

6.5.4.3.1	Solution description	73

6.5.4.3.2	Data required for mobility performance analysis	73

6.5.4.3.3	Analytics report for mobility performance analysis	74

6.5.4.4	Evaluation	74

6.5.5	Handover optimization based on UE trajectory	75

6.5.5.1	Use case	75

6.5.5.2	Potential requirements	75

6.5.5.3	Possible solutions	75

6.5.5.3.1	 Solution description	75

6.5.5.3.2	 Data required	75

6.5.5.3.3	 Analytics report on user trajectory-based handover optimization	75

6.5.5.4 	Evaluation	76

6.5.6	Handover optimization based on UE Load	76

6.5.6.1	Use case	76

6.5.6.2	Potential requirements	76

6.5.6.3	Possible solutions	76

6.5.6.3.1	 Solution description	76

6.5.6.3.2	 Data required	76

6.5.6.3.3	 Analytics report on UE Load	77

6.5.6.4	Evaluation	77

6.6	Energy efficiency related issues	77

6.7	Software management related issues	82

6.7.1	 RAN Node Software Upgrade	82

6.7.1.1	 Use case	82

6.7.1.2	 Potential Requirements	82

6.7.1.3	 Possible Solutions	83

6.7.1.3.1	Solution description	83

6.7.1.3.2	Data required	83

6.7.1.3.3	Analytics report	83

6.7.1.4	 Evaluation	83

6.8	MDA assisted SON coordination	84

6.8.1	SON conflict prevention and resolution	84

6.8.1.1	Use case	84

6.8.1.2	Potential requirements	85

6.8.1.3	Possible solutions	85

6.8.1.3.1	Solution description	85

6.8.1.3.2	Data required for SON conflict analysis	85

6.8.1.3.3	Analytics report SON conflict prevention and resolution	86

6.8.1.4 	Evaluation	86

6.9	Security related issues	87

6.9.1	Security risk assessment	87

6.9.1.1	Use case	87

6.9.1.2	Potential requirements	87

6.9.1.3	Possible solutions	87

6.9.1.3.1	Solution description	87

6.9.1.3.2	Data required	87

6.9.1.3.3	Analytics report	88

6.9.1.4	Evaluation	89

6.10	MDA management aspects	89

6.10.1	ML model training for MDA	89

6.10.1.1	Use case	89

6.10.1.2	Potential requirements	89

6.10.1.3	Possible solutions	90

6.10.1.3.1	Solution description	90

6.10.1.3.2	Data required for ML model training for MDA	90

6.10.1.3.3	ML model training report	90

6.10.1.4	Evaluation	91

6.10.2	Requesting and reporting of Management Data Analytics Reports	91

6.10.2.1	Use case	91

6.10.2.2	Potential requirements	91

6.10.2.3	Possible solutions	91

6.10.2.4	Evaluation	93

6.10.3	Retrieval of historical Management Data Analytics Reports	93

6.10.3.1	Use case	93

6.10.3.2	Potential requirements	93

6.10.3.3	Possible solutions	93

6.10.3.4	Evaluation	93

6.10.4	Confidence indicator in analysis results	93

7	Conclusions and recommendations	94

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies the enhancements of MDA (Management Data Analytics).

In this TR, the MDA use cases and the corresponding potential requirements are identified and documented, and their possible solutions with analytics input and output (report) are developed and evaluated.

The study also captures the MDA functionality and service framework, MDA process, MDA role in management loop and management aspects of MDA.

Moreover, the TR provides recommendations for the normative specifications work in full alignment with SA5 5G Services Based Management Architecture (SBMA).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[20]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[21]	3GPP TS 38.304 NR: "User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[22]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[23]	3GPP TS 28.813: "Study on new aspects of Energy Efficiency (EE) for 5G".

[24]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[25]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[26]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[27]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[28]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[29]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[30]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[31]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[32]	GSMA NG.116: "Generic Network Slice Template".

[33]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[34]	3GPP TS 28.628: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AI	Artificial Intelligence

MDA	Management Data Analytics

MDAS	Management Data Analytics Service

ML	Machine Learning

# 4	Concepts and overview

## 4.1	Overview

The Management Data Analytics is defined in TS 28.550 [2] and has also been mentioned by various other technical specifications and reports including for example TS 28.533 [3], TS 28.530 [4], TR 28.861 [5] and TR 28.805 [6].

The MDA provides a capability of processing and analysing the raw data related to network and service events and status (e.g., performance measurements, Trace/MDT/RLF/RCEF reports, QoE reports, alarms, configuration data, network analytics data, and service experience data from AFs, etc.) to provide analytics report (including recommended actions) to enable the necessary actions for network and service operations.

The MDA, in conjunction with Artificial Intelligence (AI) and Machine Learning (ML) techniques, brings intelligence and automation to the network service management & orchestration.

MDA can help to perform management tasks in preparation, commissioning, operation as well as in the termination phases. For example, MDA can support service provisioning by preparing service catalogues, evaluating network requirements for a new service and carrying out feasibility check. During operation phase, the MDA can identify ongoing issues impacting the performance of the network and service, and discover in advance potential issues that would cause potential failure and/or performance degradation. The MDA can also assist to predict the network and service demand to enable the timely resource provisioning and deployments which would allow fast time-to-market network and service deployment.

The MDAS can be consumed by various consumers, for instance the MFs (i.e., MnS service producers/consumers for network and service management), NFs (e.g., NWDAF), SON functions, network and service optimization tools/functions, SLS assurance functions, human operators, and AFs, etc.

The MDA is an enabler for the automation and cognition of the network and service management & orchestration.

## 4.2	MDA functionality and service framework

Figure 4.2-1 illustrates the MDA functionality and service framework. Depending on the scenario, MDA producer may collect data for analysis by acting as an MnS Consumer, and/or as an NWDAF subscriber, and/or as a consumer of another MDAS producer. After analysis, the MDAS Producer exposes the analysis results to MDAS Consumer(s).

Management

Data Analytics

Analysis

MDAS

MDAS

MnS

Nnwdaf

Other MDAS Producer

MnS Producer

NWDAF

MDAS Consumer

MDAS Producer

MDAS Consumer

MnS Consumer

NWDAF Subscriber

Figure 4.2-1: Functional overview and service framework of MDA

# 5	MDA process and role

## 5.1	MDA role in management loop

The MDA forms a part of the management loop (which can be open loop or closed loop, see TS 32.500 [10]), and it brings intelligence and generates value by processing and analysis of management and network data, where the AI and ML techniques may be utilized.

The MDA plays the role of Analytics in the management loop illustrated in figure 5.1-1 below.

Figure 5.1-1: Analytics in management loop

Observation: The observation of the managed networks and services. The observation involves monitoring and collection of events, status and performance of the managed networks and services, and providing the observed/collected data (e.g., performance measurements, Trace/MDT/RLF/RCEF reports, network analytics reports, QoE reports, alarms, etc).

Analytics: The data analytics for the managed networks and services. The MDA described in this TR plays the role of Analytics in the management loop. The MDA prepares, processes and analyses the data related to the managed networks and services, and provides the analytics reports for root cause analysis of ongoing issues, prevention of potential issues and prediction of network or service demands. The analytics report contains the description of the issues or predictions with optionally a degree of confidence indicator, the possible causes for the issue and the recommended actions. Techniques such as AI and ML (e.g., ML model) may be utilized by MDA with the input data including not only the observed data of the managed networks and services, but also the execution reports of actions (taken by the execution step). The MDA classifies and correlates the input data (current and historical data), learns and recognizes the data patterns, and makes analysis to derive inference, insight and predictions.

Decision: The decision making for the management actions for the managed networks and services. The management actions are decided based on the analytics reports (provided by MDA) and other management data (e.g., historical decisions made previously) if necessary. The decision may be made by the consumer of MDAS (in the closed management loop), or a human operator (in the open management loop). The decision includes what actions to take, and when to take the actions.

Execution: The execution of the management actions according to the decisions. During the execution step, the actions are carried out to the managed networks and services, and the reports (e.g., notifications, logs) of the executed actions are provided.

## 5.2	Management interaction with NWDAF and gNB

There are two types of data analytics services, one is the network data analytics service provided by NWDAF, another is the MDAS provided by 3GPP management system. The MDAS producer provides the analytics data for management purposes based on the data related to different types of NFs or entities in the network, e.g., data reported from gNB and other core network functions. Depending on the scenario and when needed, the MDAS producer may use the analytics results of NWDAF as input.

MDAS Producer may be deployed as 3GPP domain-specific (e.g., RAN or CN) or as 3GPP cross-domain. Figure 5.2-1 shows an example of the coordination between NWDAF, gNB and MDAS producer(s) for data analytics purpose.

1.	The NWDAF may consume the MDAS for identified scenarios and provide analytics service for 5GC NF for control purpose.

2.	The CN Domain MDAS producer may consume the service provided by NWDAF and other 5GC NFs and provide analytics data for management purpose.

3.  The gNB many consume the MDAS for identified scenarios for RAN control purpose.

4.	The RAN Domain MDAS producer may consume the service provided by gNB and provide analytics data for management purpose.

5.  The 3GPP cross domain MDAS Producer may consume (acting as Domain MDAS consumer) MDAS provided by domain-specific (RAN and/or CN) MDAS producer(s), and produce MDAS that may be consumed by 3GPP cross-domain MDAS consumer(s).

Figure 5.2-1: Example of coordination between NWDAF, gNB and MDAS producer for data analytics

Figure 5.2-2 shows another example of the coordination between NWDAF and MDAS producer for data analytics purpose.

1.	The NWDAF may consume the MDAS for identified scenarios and provide analytics service for 5GC NF for control purpose.

2.	The gNB may consume the MDAS for identified scenarios for RAN control purpose.

3.	The Domain MDAS producer may consume the service provided by NWDAF, other 5GC NFs and gNB, provide analytic data for management purpose.

Figure 5.2-2: Example of coordination between NWDAF, gNB and MDAS producer for data analytics

## 5.3	MDA process

This clause illustrates an example of MDA process scenario where the ML model and the management data analysis module are residing in the MDAS producer, other scenarios have not been addressed by the present document.

The MDA may rely on ML technologies, which may need the consumer to be involved to optimize the accuracy of the MDA results.

The MDA process in terms of the interaction with the consumer, when utilizing ML technologies, is described in the figure below.

Figure 5.3-1: Example of MDA process

There are two kinds of processes for MDA, the process for ML model training and the process for management data analysis. In the process for ML model training, the MDA producer, trains the ML model and provides the ML training report. The process for ML model training may also get the consumer involved, i.e., allowing the consumer to provide input for ML model training. The ML model training may be performed on an un-trained ML model or a trained ML model. In the process for management data analysis, the MDA producer analyses the data by the trained ML model, and provides the analytics report to the consumer. The MDAS consumer may validate the training report and analytics report and provide a report validation feedback to the MDAS producer.

For each received report the MDAS consumer may provide a feedback towards the MDAS producer, which may be used to optimize ML model.

Data classification: The data input to the MDA producer could be used for ML model training or for the actual management data analysis. The MDA producer classifies the input data and passes the classified data along to corresponding step for further processing.

ML model training: The MDAS producer trains the ML model, i.e., to train the algorithm of the ML model to be able to provide the expected training output by analysis of the training input. The data for ML model training may be the training data (including the training input and the expected output) and/or the report validation  feedback provided by the consumer. After the ML model training, the MDAS producer provides an ML model training report.

Management data analysis: The trained ML model analyses the classified data and generates the management data analytics report(s). Analytics reports were presented in clause 5.1.

Report validation: The consumer may validate the report provided by the MDAS producer. The  report to be validated may be the analytics report and/or the ML model training report as described above. The consumer may provide a feedback to the MDAS producer.

As a result of validation, the consumer: (i) may also provide training data and request to train the ML model and/or (ii) provide feedback indicating the scope of inaccuracy, e.g. time, geographical area, etc.

# 6	Use cases, potential requirements and possible solutions

## 6.1	Coverage related issues

### 6.1.1	Coverage issue analysis

#### 6.1.1.1	Use case

The coverage issue may cause various UE and network failures and degrade the network performance offered the UEs.

The coverage issue could be a weak coverage, a coverage hole, a pilot pollution, an overshoot coverage, or a DL and UL channel coverage mismatch as described in clause 5.1.1, 3GPP TS 37.816 [11]. The weak coverage may result in low success rate of random access, paging, RRC connection establishment and handover, low data throughput, more abnormal releases of RRC connection, DRB retainability, QoS flow and/or PDU session resources, and dissatisfied QoE. The coverage hole is a more severe problem and would further lead to the UE out of service in the area.

The 5G related coverage issue may exist only in 5G (i.e., 5G issue only with good coverage provided by other RATs) or exist in all RATs (i.e., no RAT provides good coverage in the area).

Coverage performance should be assured to guarantee user service experience. It is desirable that the coverage issue can be detected by MDA from the various symptoms, together with the geographical and terrain data and the configuration parameters of the RAN.

Once the coverage issue is detected, the MDAS producer provides the analytics report that precisely describes the coverage issue, and the analytics report needs to contain sufficient information to enable the MDAS consumer (e.g., SON CCO function) to take the remedy actions. The MDAS producer may also provide the recommended actions to solve the identified coverage issue in the analytics report, so that the MDAS consumer can execute the actions accordingly or by taking the recommended actions into account.

The MDAS producer is informed when the actions are taken by the MDAS consumer to solve the coverage issue described in the analytics report, so that the MDAS producer can start evaluating the result of the executed actions.

The MDAS producer gets the execution reports describing the actions taken by the MDAS consumer, and takes the execution reports into account to fine-tune the accuracy of the future (new or updated) analytics report.

The MDAS producer also provide update(s) of the analytics report to indicate the status change (e.g., solved, mitigated or deteriorated) of the coverage issue.

#### 6.1.1.2	Potential requirements

REQ-COV_ANA-CON-1	The MDAS producer should have a capability to provide the analytics report describing the coverage issue.

REQ-COV_ANA-CON-2	The analytics report describing the coverage issue should contain the following information:

-	The identifier of the coverage issue described in the analytics report;

-	Indication of whether the coverage issue is weak coverage or coverage hole, a pilot pollution, an overshoot coverage, or a DL and UL channel coverage mismatch;

-	The start time and end time of the coverage issue;

-	The geographical area and location where the coverage issue exists;

-	Root cause of the coverage issue;

-	Whether the coverage issue exists in 5G only or in all RATs;

-	The cells affected by the coverage issue;

-	The severity level (e.g., critical, medium, or cleared) of the coverage issue;

-	The recommended actions to solve the coverage issue.

#### 6.1.1.3	Possible solutions

##### 6.1.1.3.1	Solution description

The MDAS producer correlates, processes and analyses the data described in the following subclause within a time period on a regular basis or trigged by events (e.g., the RLF reports) to identify the coverage issue, and provide the analytics reports to describe the identified coverage issues (which could be new issues or the updates of existing issues).

##### 6.1.1.3.2	Data required for coverage issue analysis

The following table describes the data required for coverage issue analysis:

| Data category | Required data |
| --- | --- |
| Performance measurements | - Average/distribution of UE reported RSRPs/RSRQs/SINRs of the serving cell when the TA (Timing Advance) or UE rx-tx applied to the UEs is in a specific range; - Average/distribution of UE reported RSRPs/RSRQs/SINRs of each neighbour cell when the UE reported RSRPs/RSROs of the serving cell is in a specific range, measured per NCR (neighbour cell relation), per SSB index and per CSI-RS index of each NCR; - Number of abnormal releases of DRBs, QoS flows, PDU sessions, and UE contexts in the serving cell measured per SSB index and per CSI-RS index |
| MDT reports | MDT reports containing RSRPs and RSRQs of the serving cell and neighbour cells reported by each UE with anonymous id (e.g., C-RNTI) and location information. |
| RLF reports | RLF reports containing RSRP(s) and RSRQ(s) of the serving cell and neighbour cells reported by each UE with anonymous id (e.g., C-RNTI) and location information. |
| RECF reports | RCEF reports containing RSRP(s) and RSRQ(s) of the serving cell and neighbour cells reported by each UE with anonymous id (e.g., C-RNTI) and location information. |
| UE location reports | UE location information provided by the LCS which can be used to correlate with the MDT/RLF/RCEF reports. |
| Geographical data and terrain data of the RAN | - The geographical information (longitude, latitude, altitude) of the deployed RAN (gNBs and eNodeBs, antennas, sector carrier equipment, etc.). - The terrain data for the area of the deployed RAN. |
| Configuration data | - The current NRMs containing the attributes affecting the RAN coverage, such as maximum transmission power of the cell, directions and tilts of the antennas or beams, etc. - The NRM update reports (notifications or logs) containing the creations or changes of the MOIs (Managed Object Instance) affecting the RAN coverage. |

##### 6.1.1.3.3	Analytics report for coverage issue

The analytics report describing the coverage issue contains the following information:

-	Coverage issue identifier: The identifier of the coverage issue;

-	Coverage issue type indication: Indication that the coverage issue is weak coverage or coverage hole, pilot pollution, overshoot coverage, or DL and UL channel coverage mismatch;

-	Start time: The start time of the coverage issue;

-	Stop time: The stop time of the coverage issue;

-	Location: The geographical area and location where the coverage issue exists;

-	Root cause: Root cause of the coverage issue, e.g., weak transmission power, blocked by constructions, restricted by terrain, etc;

-	RAT indication: Indication that the coverage issue exists in 5G only or in all RATs;

-	Affected objects: The MOIs of the cells affected by the coverage issue;

-	Severity level: The severity level (e.g., critical, medium, cleared) of the coverage issue;

-	Recommended actions: The recommended actions to solve the coverage issue. The recommended action could be re-configurations of coverage related attributes, creation of new cells or beams, or manual operations to add or change the physical units.

#### 6.1.1.4	Evaluation

The solution described in clause 6.1.1.3 requires the analytics inputs as described in in clause 6.1.1.3.2, wherein

-	the performance measurements for E-UTRAN are available in TS 32.425 [26], the similar performance measurements can be defined for NG-RAN in TS 28.552 [8].

-	the MDT reports, RLF reports, RECF reports for E-UTRAN and NG-RAN are available in TS 32.422 [25].-	the UE location reports can be accessed by consuming the LCS via service-based interfaces as defined in TS 23.273 [27].

-	the geographical data and terrain data of the RAN can be modelled in the NRMs.

-	the configuration data for NG-RAN have been defined in TS 28.541 [20].

The coverage issue analysis may require full set or subset of the above-mentioned input based different deployment scenarios (e.g., NSA, SA, and Multi-RAT). With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.1.1.3.3 can be derived.

Therefore, this solution is a feasible candidate for coverage issue analysis.

### 6.1.2	Slice coverage optimization

#### 6.1.2.1	Use case

In providing a network slice, a 3rd party (i.e. slice tenant) issues a slice request indicating the desired SLA, which includes among other parameters as indicated in GSMA NG.116 [32], the slice coverage (also referred to as coverage area of the network slice or area of service as per GSMA NG.116 [32]). The main challenge of providing the slice coverage is to map the desired geographical coverage area with the available radio coverage, which depends on the base station planning and deployment. In 5G the notion of coverage is represented by a set of one or more Tracking Areas (TAs), which are contained in a Registration Area (RA), which is assigned to a UE once it registers to the network.

In mapping the desired slice coverage into a geographical coverage area there are two main challenges:

-	The coverage area in service profile need to be mapped to TA list assigned to cells which are selected to support the slice coverage.

-	It is not guaranteed that the desired slice coverage can be mapped perfectly to the “coverage footprint” of a cell or set of cells (that belong to a TA) or that the desired slice coverage is even available since radio coverage may not be available in certain areas.

Currently to resolve these problems more TAs and consequently cells may be allocated to a slice to enhance the coverage, if such an option is available, or more capacity resources can be provisioned in the allocated cells. Each cell can only be associated to one TA at a time and an S-NSSAIList configured in all cells that form a TA should be the same. This approach may prove to be inefficient since it wastes network resources.

The distribution of users, the expected radio resource availability and mobility patterns may govern the configuration parameters of each TA and cell (e.g. antenna downtilt, SSB beamforming patterns determining the coverage of a cell, handover parameters, etc.), which can be allocated per slice. Cell configuration parameters may help to adjust the coverage.

To translate the business slice coverage to the actual radio deployment, without overprovisioning while leveraging the benefits of flexible gNB radio features adjustment, MDA can be used. MDA can enable an MDAS consumer to optimize the slice coverage and load distribution on the slice instantiation and runtime considering (i) slice-aware statistics, e.g., slice-UE distributions and mobility patterns, (ii) slice SLA and (iii) access node capabilities.

Depending on the MDAS producer output, TA and RA planning, i.e. grouping cells to form a TA and then TAs to an RA, can be optimized and the RAN parameters can be adjusted to shape the cell edges and load distribution. The main objective is to fulfill a given slice SLA involving as few cells as possible by leveraging the benefits of adjusting cell configurations for satisfying the desired coverage.

#### 6.1.2.2	Potential requirements

REQ-NS_COV_THR_OPT-1		The MDAS producer should have a capability to provide the analytics report describing the slice coverage, slice availability and slice prediction information to authorized consumers, e.g., gNB.

REQ-NS_COV_THR_OPT-2	The analytics report should contain the following information:

-	the identifier that indicates the gNB configuration issue with respect to a slice request;

-	the identifier that indicates the TA and RA optimization issue with respect to a slice request;

-	the geographical location and the affected TA and/or cell configuration attributes;

- 	the start and stop time in where the report is valid;

-	the originator issue, that may be a coverage problem due to inefficient TA and/or RAN configuration;

-	the recommended actions that involve options to reconfigure TA and/or RAN attributes including HO parameters, cell reselection parameters, beam configuration, computing resource and slice support in a cell.

#### 6.1.2.3	Possible solutions

##### 6.1.2.3.1		Solution description

The solution considers slice coverage assessment with respect to RAN deployment. The MDAS producer analyses performance measurements, slice availability and mobility patterns to identify the RAN attribute configuration options for adjusting selected cells in order to fulfill the SLA of a slice request in terms of the coverage.

##### 6.1.2.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Service Data | S-NSSAI as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information. |
| Performance Measurements | Radio resource utilization – usage of physical radio resource utilization of the network as per clause 5.1.1.3 of TS 28.552 [8]. Virtual resource usage of NF: The resource usage of virtual network functions, see clause 5.7.1 of TS 28.552 [8]. |
| Beam Report per Slice | Beam statistics per slice usage in the serving cell: - Beam IDs (SSB IDs or CSI-RS IDs) which have been serving users of a specific network slice - Temporal Information of these Beams serving a specific Slice, (i.e., when, e.g., morning or evening, and how long) - Number of slice users served by a specific Beam ID Beam statistics of a neighboring cell measured by specific slice users: - Relevant neighboring Beam IDs (SSB IDs or CSI-RS IDs) which have been detected by users of a specific network slice |
| Capacity Planning Data | Capacity management as per clause 5.4.15 of TS 28.530 [4] and clause 7.15 of TS 28.531 [33]. |
| TA Measurements | - TA load: Load utilization of an indicated TA - TA usage: Number of active/passive UEs in TA per cell – Indicates the potential of re-configuring a TA - Intra/inter TA handover: Number of handovers among cells with a TA and among cells between neighboring TAs |
| Slice Unavailability Report | Slice unavailability in serving cell: - Statistics about the rejected or remapped PDU sessions belonging to a specific slice - How long the UEs stayed in a cell not supporting specific slices, e.g., short stay with Slice Unavailability - Temporal Information of the UEs stayed in a cell not supporting specific slices, e.g., when these events occurred - Number of UEs stayed in a cell not supporting specific slices - Cell IDs to which the UEs reconnect after staying in a cell not supporting specific slices, e.g., after short stay with Slice Unavailability. |
| MDT Reports | UE measurements related to RSRPs, RSRQs, RLF, RECF of the cells in slice and UE location information |
| NWDAF Mobility Analytics | Group UE mobility analytics as per TS 23.288 [18] All UE mobility analytics within Area of Interest defined via slice coverage as per TS 23.288 [18] |
| Configuration Data | NRM attributes affecting the radio configuration and virtual NF resource allocation and configuration |
| Network Topology | Topology of the network |

##### 6.1.2.3.3		Analytics report

##### 6.1.2.3.3.1		TA optimization

The cross domain MDAS producer output report contains the following information:

| Information | Description |
| --- | --- |
| TA Incident Identifier | Identifier that indicates the TA configuration case for slice coverage enhancement (or slice unavailability) |
| Type of Analytics | Statistics or Prediction |
| Recommended Actions | List of mapping: - < NSSP, [ < subArea, [TAI] > ] > ] [ ]: represent a list TAI: tracking area identifier which is comprise of PLMNId + TAC subArea: sub geography area represented by geographic zones, or scope of geo coordinates (latitude, longitude and elevation), etc. < subArea, [TAI] > : map between sub geo area and tracking area, one subArea can be translated to a TAI list < NSSP, [ < subArea, [TAI] > ] >: map between on NSSP and sub geo area. One NSSP can support a list of sub geo areas, each sub geo area can be translated to a list of TAIs. |

The RAN domain MDAS producer output report contains the following information:

| Information | Description |
| --- | --- |
| TA Incident Identifier | Identifier that indicates the TA configuration case for slice coverage enhancement (or slice unavailability) |
| Type of Analytics | Statistics or Prediction |
| Recommended Actions | List of Mappings: [ < subArea, [TAI] > ] [ ]: represent a list TAI: tracking area identifier which is comprise of PLMNId + TAC subArea: sub geography area represented by geographic zones, or scope of geo coordinates (latitude, longitude and elevation), etc. < subArea, [TAI] > : map between sub geo area and tracking area, one subArea List of Cells: [ cell, TAC, [<attribute, value>] ] [ ]: represent a list [<attribute, value>]: list of attributes and corresponding values for the attributes which were proposed to be reconfigured. [ cell, TAC, [<attribute, value>] ]: list of cells with assigned TAC and (re)configured attributes for the cells which were proposed to be created or modified. |

##### 6.1.2.3.3.2		gNB optimization

The network congestion analytics report contains the following information.

| Information | Description |
| --- | --- |
| gNB Incident Identifier | Identifier that indicates the gNB configuration case for slice coverage enhancement (or slice unavailability) |
| Type of Analytics | Statistics or Prediction |
| Location | Geographical location affected by the gNB incident |
| Affected Object Attributes | Cell Configurations: Antenna Tilt, HO parameters, cell reselection parameters, beam configuration, compute resources, etc. |
| Start/Stop Time | Starts/stop time of the incident |
| Root Cause | The originator of the issue, e.g. user mobility, load peak, user distribution, beam configuration, etc. |
| Severity Level | The severity level (e.g. critical, medium, not important) |
| Recommended Actions | Recommendation actions to resolve the issue: - Antenna Tilt configuration options - HO parameters configuration options - Cell reselection configuration options - Beam configuration options - Compute resource configuration options - Enable slice support in determined Cell(s) |

#### 6.1.2.4	Evaluation

The solution described in clause 6.1.2.3.1 requires the analytics inputs as described in clause 6.1.2.3.2, wherein:

-	Input PM on RAN resource utilization, Virtual resource usage of NF are specified in TS 28.552 [8].

-	Input PM that need to be specified include:

-	Beam statistics per slice usage.

-	Slice unavailability related measurements.

-	MDT data is specified in TS 37.320 [12].

-	NWDAF mobility analytics are specified in TS 23.288 [18].

-	Configuration Data and network topology data, by the NRMs defined in TS 28.541 [20].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.1.2.3.3 can be derived.

Therefore, this solution is a feasible candidate for slice coverage optimization.

### 6.1.3		Paging optimization

#### 6.1.3.1	Use case

As per the current procedures, if the UE goes out-of-coverage (OOC) the paging which was initiated by the network Access and Mobility Management Function (AMF) fails. The re-attempts continue to fail until UE comes in the coverage and reacts to the paging attempts. This repetitive paging attempts result in the wastage of network resources. As an example, the use case includes a user or a group of users getting into an area, with no cellular coverage on a regular basis for a considerably long duration, for e.g., the user gets into a shielded room for some testing purpose every day for a defined period. The Network initiated paging for such users will fail until they are back in the area with cellular coverage. This would result in in-efficient network resource usage.

It is desirable to use MDAS (Management data analytic service) to optimize the current paging procedures in 5G networks. MDAS producer provides an analytics report containing the user(s) paging analytics indicating the time window at which the user is OOC on a regular basis at the particular location and hence will not be able to respond on a network-initiated paging. Based on the report MDAS consumer (e.g., AMF, gNB) decides on whether, when and where to initiate or not to initiate the paging procedures, thereby ensuring the efficient paging procedures and optimal network resource utilization, as paging can be initiated only when there are more chances for it to be successful.

#### 6.1.3.2	Potential Requirements

REQ-PA_OPT_CON-1: The MDAS producer should have a capability allowing the authorized consumer to get the paging analytics report describing paging results for a particular user or a group of users.

REQ-PA_OPT_CON-2: The MDAS producer should have a capability to provide the paging analytics report describing the paging results based on successful and un-successful paging attempts at a particular time and duration.

REQ-PA_OPT_CON-3: The paging analytics report describing the paging results should contain the following information:

-	User Identification: Identification of the user or a group of users.

-	Daily-OOC-Duration: Identifying the time window during which UE is out-of-coverage every day.

-	Daily-OOC-Location: Identifying the last known location before UE going out-of-coverage every day.

-	Recommended Action: The recommendation may suggest stopping paging the UE for Daily-OOC-Duration at Daily-OOC-Location.

#### 6.1.3.3	Possible Solutions

##### 6.1.3.3.1	Solution description

The solution requires MDAS producer to collect various data and provide the paging analytics report. Daily-OOC-Duration and Daily-OOC-Location will be included in the Paging Analytics Report mentioning the time window during which UE is out-of-coverage every day at a particular location. The paging is not initiated for the UE during the period provided as Daily-OOC-Duration if the last know UE location is the location identified by Daily-OOC-Location.

##### 6.1.3.3.2 	Data required

The consumer (e.g. AMF) can subscribe to obtain the paging analytics report for a user or a group of the user from MDAS producer. The subscription request may include identification for the target user or a group of the user, reporting interval etc. The MDAS producer collects the following per UE per day data from various sources.

| Data category | Required data |
| --- | --- |
| UE Paging Measurements | Number of successful paging attempt. |
|  | Successful Timestamp: The timestamp for each successful paging attempt. |
|  | Successful Location: Last known location of UE. |
|  | Number of un-successful paging attempt: Total number of un-successful paging attempt. |
|  | Un-Successful Timestamp: The timestamp for each un-successful paging attempt. |
|  | Un-Successful Location: Last known location of UE. |

##### 6.1.3.3.3	Analytics report

The paging analytics report contains the following information

| Paging Analytics Report | Attribute Name | Description |
| --- | --- | --- |
|  | User Identification | Identification of the user or a group of users. |
|  | Daily-OOC-Duration | Identifying the time window during which UE is out-of-coverage every day. This will be provided per UE. |
|  | Daily-OOC-Location | Identifying the last known location before UE going out-of-coverage every day. This will be provided per UE. |
|  | Recommended Action | The recommendation may suggest stopping paging the UE for Daily-OOC-Duration at Daily-OOC-Location. This will be provided per UE. |

Based on the report and the recommendation, the consumer decides whether to change the paging strategy for a particular UE or a group of UE. If the paging policy needs to be changed, the AMF may decide whether, when and where to page the UE. The AMF may not page the UE during the period provided as Daily-OOC-Duration if the last know UE location is the location identified by Daily-OOC-Location.

NOTE:  This use case may require MDAS to know UE Identifier. How that will be done is not addressed in the present document.

#### 6.1.3.4	Evaluation

The solution described in clause 6.1.3.3 requires the analytics inputs as described in in clause 6.1.3.3.2, wherein

-	The UE location reports can be accessed by consuming the LCS via service-based interfaces as defined in TS 23.273 [27].

-	The required UE paging data can be/should be defined as part of MDT data.

With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.1.3.3 can be derived.

Therefore, this solution is a feasible candidate for paging optimization.

## 6.2	Resource related issues

### 6.2.1	RAN user plane congestion analysis

#### 6.2.1.1	Use case

With the development of diverse communication services and the increasing number of connections, user data volume demanded by end users grows rapidly which may not be satisfied by the current deployed 5G network.

In clause 3.1 of TS 22.101 [9], the RAN user plane congestion is defined as the situation where the demand for RAN resources to transfer user data exceeds the available RAN capacity to deliver the user data for a significant period of time in the order of few seconds or longer. The case where a short-duration burst of user plane traffic is not identified as RAN congestion.

Due to the complexity of 5G network and wireless environment, multiple types of performance deteriorate are related with RAN user plane congestion, e.g., high drop rate of PDCP PDU, the full PRB utilisation, inappropriate mobility parameters configuration or inefficient usage of radio resources. The root causes should be analysed and identified to help to resolve the RAN user plane congestion and improve the end users' experience, e.g., the issue of lack of physical or virtual resources or unsuitable resources allocation, unsuitable mobility parameters. The recommended actions may also be provided, e.g., recommended policies of physical and virtual resources allocation, possible means to improve the radio condition, load balancing mechanisms etc.

The producer of MDAS is able to, from the perspective of the management aspects, provide the user plane data congestion analytics report related to a specific cell, specific network slicing instance or subnetwork. This analytics report can be considered as an input to support SLS assurance to perform further evaluation.

#### 6.2.1.2	Potential requirements

REQ-CONG_ANA-CON-1: The MDAS producer should be able to provide the analytics report describing the RAN user plane congestion problem.

REQ-CONG_ANA-CON-2: The analytics report describing the RAN user plane congestion problem should contain the following information:

-	The identifier of the RAN user plane congestion;

-	Indication of the RAN user plane congestion type;

-	The start time and end time of the RAN user plane congestion;

-	The geographical area and location where the RAN user plane congestion affects;

-	 Root cause of the RAN user plane congestion;

-	The objects affected by the RAN user plane congestion;

-	The severity level of the RAN user plane congestion;

-	The recommended actions to solve the RAN user plane congestion problem.

#### 6.2.1.3	Possible Solutions

##### 6.2.1.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to identify the RAN user plane congestion problems. As the table in clause 6.2.1.3.3 shows, the analytics report is able to be provided by the MDAS producer to describe the root causes and recommendations of identified RAN user plane problem. This procedure may be triggered by the request or periodically.

##### 6.2.1.3.2	Data required for RAN user plane congestion problem

The following table shows the potential data required to analyse the RAN user plane congestion problem.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [8]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552 [8]; PDCP Data Volume: The transmitted PDCP data volume, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [8]; TB related measurements: The TB transmitted in a cell, see clause 5.1.1.7 of TS 28.552 [8]; CQI related measurements: the distribution of Wideband CQI (Channel Quality Indicator) reported by UEs in the cell, see clause 5.1.1.11 of TS 28.552 [8]; MCS related Measurements: the distribution of the MCS scheduled for PDSCH RB by NG-RAN, the distribution of the MCS scheduled for PUSCH RB by NG-RAN, see clause 5.1.1.12 in TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR and UE location information. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs related with RAN user plane congestion. |

Note: 	The above parameters may not be the complete list.

##### 6.2.1.3.3	Analytics report for RAN user data congestion

The following table shows the potential information carried in the analytics report of RAN user plane congestion.

| Analytics Report of RAN user plane congestion | Attribute Name | Description |
| --- | --- | --- |
|  | Resource issue identifier | The identifier of the RAN user plane congestion |
|  | RAN user plane congestion type indication | Indicator of the root cause of the RAN user plane congestion, e.g., PRB resources shortage, unsuitable resource allocation, inappropriate mobility parameters configuration, inefficient usage of radio resources |
|  | Start time | The start time of the RAN user plane congestion problem |
|  | Stop time | The end time of the RAN user plane congestion problem |
|  | Location | The geographical area or the cells where the RAN user plane congestion exists |
|  | Root cause | The root cause of the UP congestion issue, e.g. poor radio condition, inappropriate radio resource allocation, bad handover parameters, etc. |
|  | Affected objects | The MOIs of the cells or subnetworks or network slices affected by the RAN user plane congestion problem |
|  | Severity level | The severity level (e.g., critical, medium, not important) of the RAN user plane congestion |
|  | Recommended actions | The recommend actions to solve the RAN user plan congestion problem. The recommended actions could be to update the policies of physical and virtual resources allocation, improve the radio condition quality, or optimize load balancing mechanisms. |

#### 6.2.1.4	Evaluation

The solution described in clause 6.2.1.3 requires the analytics inputs as described in in clause 6.2.1.3.2, wherein

-	the performance data (measurements and KPIs) for are available in TS 28.552 [8] and TS 28.554 [7].

-	the MDT data are available in TS 32.422 [25].

-	QoE data as defined in TS 26.247 [29] and TS 26.114 [30] can be acquired through the procedures defined in TS 28.405 [31].

However, these data cannot provide a clear indication of RAN user plane congestion. Therefore, the data that can clearly indicate the RAN user plane congestion are needed so that this solution can be feasible for RAN user plane congestion analysis.

### 6.2.2	Resource utilization analytics

#### 6.2.2.1	Use case

The network is a resource limited system, it is therefore quite imperative to ensure an optimum resource utilization for the network so that the required resources can be efficiently allocated while ensuring no wastage or under allocation of resources to cause additional CapEx and OpEX.

The resources usage for a network, a portion of network or a network slice could be higher or lower during different time periods depending on the traffic patterns. The traffic patterns could also vary in different geographical locations (e.g., business area, entertainment area and residential area) of the network, and could vary for different network slices. It could happen that at some point in some areas or network slices the resources are in shortage status while in some other areas or network slices there are abundant resources. This may result in that the users cannot be satisfactorily served in some areas or network slices due to lack of resources even though the overall maximum capacity is sufficient. Resource shortage may affect the QoS and potentially impact user quality of service experience, e.g., lowering down the user data throughput, prolong the user data delay, raise the rejections and failures for establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increase the drops of the existing connections, sessions and resources and deteriorates user quality of service experience. Especially in shared scenarios, it is usually that network slices may share the same RAN resources, TN resources and/or some CN network resources. When resources become limited, the suddenly high increase in resource usage by one network slice may potentially affect the performance of other network slices. If resources are not limited but one network slice is in relatively higher resource usage and the others are in lower resource usage, the proportion of resource distribution may need to be reallocated without provisioning additional spare resources in order to achieve efficient and optimum network resource usage, which can effectively reduce the CapEx and OpEx.

Therefore, it would be desirable that the spare resource of the under-utilized usage areas or network slices can be re-allocated to areas or network slices that requires more resources within the same period of time to prevent the resource shortage from happening.

The MDAS producer can analyse the current and historical performance data related to resource usage, network traffic and user quality of service experience for the network or network slices, and identify the ongoing issues on resource utilization and predict potential issues.

The MDAS producer may provide analytics report on capacity planning, resource requirements, resource utilization, resource availability and resource reservation proposals to assist the feasibility check by network and network slice management system before the provisioning of the communication service.

The MDAS producer provides analytics report describing the ongoing and/or potential resource utilization issues to the authorized consumers. The issues need to be described precisely, including (but not limited to) the information about which part of the network or network slice has encountered or is going to encounter the resource utilization issue, it is resource shortage or resource excess, over which time period, etc. The MDAS producer may also provide the recommendations to solve the resource utilization issues identified by the analytics report. The recommended actions may be for example to schedule the "scale-in" and "scale out" of VNFs to dynamically (re-) allocate the virtualized resources to where they are needed, or to create/update the resource allocation policy for different network slices to allow the network slices getting different percentage of resources in different time periods according the traffic patterns and user quality of service experience. In case of sharing scenario, the MDAS producer should also weighs the resource usages of the shared network resources which belong to different network slices. If reallocating the already allocated network resource can solve the resource utilization issues, no other newly allocated network resource is needed.

The MDAS producer gets the execution reports of the actions taken by the MDAS consumer to solve the resource utilization issue, and takes the execution reports into account in the following analysis.

The MDAS producer continue analysing the reported issue and provides update(s) if there is any status change (e.g., solved, mitigated or deteriorated) till it is solved.

#### 6.2.2.2	Potential requirements

REQ-RES-ANA-1	The MDAS producer should have a capability to provide the analytics report describing the resource utilization issue.

REQ-RES-ANA-2	The analytics report describing the resource utilization issue should contain the following information:

-	The identifier of the resource utilization issue;

-	Indication that it is an ongoing issue or potential issue;

-	The time period(s) during which the resource utilization issue has happened or is potentially going to happen;

- 	Indication that resource issue is shortage or excess over each time period;

-	Percentage of resource shortage or excess in each time period;

-	The network entities involved in the resource utilization issue;

-	The network slices involved in the resource utilization issue;

-	The recommended actions to solve the resource utilization issue.

REQ-RES-ANA-3	The MDAS producer should have a capability to weigh the allocation of shared network resource when providing the analytics report describing the resource utilization issue.

#### 6.2.2.3	Possible solutions

##### 6.2.2.3.1	Solution description

The MDAS producer correlates and analyses the ongoing and/or potential resource utilization issues based on the current and historical performance data related to resource usage and network traffic for the network or network slices. The required data can be from RAN domain or CN domain or both. Based on the analysis above, the MDAS producer is able to provide the domain specific or cross domain analytics report as defined in clause 6.2.2.3.4 related with resource utilization analytics triggered by event or periodically.

To assist the feasibility check, the MDAS producer may consider the following information: e.g., capacity planning of the network slice instance and/or network slice subnet instance, existing active or non-active network slice instance and/or network slice subnet instance resource information, slice provisioning requirements, etc.

##### 6.2.2.3.2	Required data for resource utilization analysis for RAN domain

The management data required to analyse the RAN related resource utilization are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Identifier | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for resource utilization analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552 [8]; |
| MDT Data | Measurement data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12] and TS 32.422 [25]. |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS 28.530 [5] and clause 7.15 of TS 28.531 [33]. |
| Network topology | Network topology data | The topology of the network for resource utilization analytics. The topology can be reflected by the NG-RAN NRMs as defined in TS 28.541 [20]. |
| User service experience | Analysis data | User service experience relevant attributes and/or analytics analysis obtained from NWDAF or AF, see clause 6.4 of TS 23.288 [18]. |

##### 6.2.2.3.3	Required data for resource utilization analysis for CN domain

The management data required to analyse the CN related resource utilization are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Identifier | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for resource utilization analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | User subscription data by performance measurement for AMF as defined in clause 5.2 of TS 28.552 [8]; Performance measurement data for SMF as defined in clause 5.3 of TS 28.552 [8]; Performance measurement data for UPF as defined in clause 5.4 of TS 28.552 [8]; Performance measurement data for NF as defined in clause 5.7 of TS 28.552 [8]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; PDU sessions for network slice instance: number of PDU sessions and PDU session establishment time of network slice instance, see clause 6.4.1 and clause 6.4.3 of TS 28.554 [7]; Virtualised resource utilization: Virtualised resource utilization of Network Slice Instance, see clause 6.4.2 of TS 28.554 [7]; |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS 28.530 [5] and clause 7.15 of TS 28.531 [33]. |
| Network topology | Network topology data | The topology of the network for resource utilization analytics. The topology can be reflected by the 5GC NRMs as defined in TS 28.541 [20]. |

##### 6.2.2.3.4	Analytics report for resource utilization analysis

The following table provides the potential contents of the domain specific or cross domain analytics report of resource utilization analysis based on the required data received as described in 6.2.2.3.2 and 6.2.2.3.3.

| Analytics Report of resource utilization analysis | Attribute Name | Description |
| --- | --- | --- |
|  | Resource utilization issue Identifier | The identifier indicates the resource utilization issue |
|  | Indication of resource utilization issue type | Indicates the type of the resource utilization issue, e.g., ongoing or potential resource utilization issue |
|  | Time period | Describes the time period(s) during which the resource utilization issue has happened or is going to happen |
|  | Indication of resource usage demand | Indicates that resource issue is shortage or excess in each time period |
|  | Percentage of resource usage demand | Describes percentage of resource shortage or excess in each time period |
|  | List of network entities | Lists the network entities involved in the resource utilization issue |
|  | A List of network slices | List of the network slices involved in the resource utilization issue |
|  | Recommended actions | Describes the recommended actions to solve the resource utilization issue |

#### 6.2.2.4	Evaluation

The solution described in clause 6.2.2.3 requires the analytics inputs as described in in clause 6.2.2.3.2 and 6.2.2.3.3, wherein

-	the S-NSSAI can be provided along with the performance measurements, NRM and planning data.

-	the performance measurements are available in TS 28.552 [8].

-	the capacity planning data can be accessed according to TS 28.531 [33].

-	the network topology can be reflected by the NRMs defined in TS 28.541 [20].

-	the user service experience data can be accessed from NWDAF or AF according to TS 23.288 [18].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.2.2.3.4 can be derived.

Therefore, this solution is a feasible candidate for resource utilization analytics for RAN domain, CN domain and network slices.

### 6.2.3	Cross-slice resource optimization

#### 6.2.3.1	use case

In TR 28.861 [5], SON use case of cross-slice network resource optimizations described. The resource allocated for each network slice instance may vary considering the network resource utilization, traffic patterns, network bandwidth and the demand of resources for each NSI, the priorities of NSI, etc.

The MDAS producer is expected to have the capability to provide analytics report of resource optimization for multiple network slices. Resource optimizations for multiple network slices may be dependent on each other. Resource scale out / in of one slice may impact resource allocation strategy of the other network slices considering the overall resource availability constraints. Aspects to be considered by MDAS include network slice type, priority, resource availability, traffic load, QoS requirements and QoS flow related measurements etc. The recommendations of the most optimal resource allocation for each network slice and the predictions of the resource demand for each NSI may also be provided. This analytics report can be consumed by SON function to help the optimization of cross-slice resource optimization.

#### 6.2.3.2	Potential requirements

REQ-Multi-SLICE-Resource_MDA-1: MDAS producer should have the capability to provide analytics report describing resource optimization across multiple network slices.

REQ-Multi-SLICE-Resource_MDA-2: The analytics report describing the resource optimization across multiple network slices should include the following information:

-	Resource optimization threshold for each network slice;

-	Recommendations of resource allocation for each NSI;

-	The predictions of resource demand for each NSI

#### 6.2.3.3	Possible Solution

##### 6.2.3.3.1	Solution description

The MDAS producer correlates and analyses the resource optimization strategies across multiple network slices based on the current and historical resource utilization measurements and network traffic for each network slice instances. Based on the analysis above, the MDAS producer is able to provide the analytics report as defined in 6.2.3.3.3 related with cross slice resource optimization analytics triggered by event or periodically.

##### 6.2.3.3.2	Required data for cross-slice resource optimization analysis

The input data in clause 6.2.2.3.2 and 6.2.2.3.3 can be reused as the required management data to analyse the resource optimization across multiple network slices.

##### 6.2.3.3.3	Analytics report for cross-slice resource optimization

The following table provides the potential contents of the analytics report of cross-slice resource optimization.

| Analytics Report of cross-slice resource optimization analysis | Attribute Name | Description |
| --- | --- | --- |
|  | Cross-slice resource optimization Identifier | The identifier indicates the Cross-slice resource optimization |
|  | Resource optimization threshold | Indicates the thresholds to trigger network resource optimizations for each network slice; |
|  | Resource allocation recommendation | Describes the recommendations of resource allocation (e.g. increase or decrease NSI capacities like storage, computing, network bandwidth and radio resources) for each NSI; |
|  | Resource demand prediction | Describes the predictions of resource demand for each NSI |

#### 6.2.3.4	Evaluation

The solution described in clause 6.2.3.3 requires the analytics inputs as described in in clause 6.2.3.3.2, wherein

-	the S-NSSAI can be provided along with the performance measurements, NRM and planning data.

-	the performance measurements are available in TS 28.552 [8].

-	the capacity planning data can be accessed according to TS 28.531 [33].

-	the network topology can be reflected by the NRMs defined in TS 28.541 [20].

-	the user service experience data can be accessed from NWDAF or AF according to TS 23.288 [18].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.2.3.3.4 can be derived.

Therefore, this solution is a feasible candidate for cross-slice resource optimization.

### 6.2.4	NAS level congestion control optimization

#### 6.2.4.1	Use case

The current NAS level congestion control as described in TS 23.501 [13] uses back-off timer to avoid AMF receiving large amounts of NAS messages from UEs including Mobility Registration Update request. The AMF will accept or rejects the request depending on various aspects. In virtualized environment, the Mobility Registration Update request may be rejected due to inadequacy of available resources with the target AMF. The resource may include virtual resource (e.g., compute, memory and disk). If the Mobility Registration Update request is rejected by the AMF, UE will receive the reject message with a Mobility Management back off time and send a new Mobility Registration Update request after the timer. It is possible that the new Mobility Registration Update request will be rejected again because of the load of the AMF. This mechanism results in wastage of UE and network resources. It also brings inefficiency in network procedures.

It is desirable to use MDAS (Management data analytic service) to assist congestion control in order to avoid too many rejections of NAS messages at the AMF. MDAS producer provides analytics report containing the current and future/predicted resource consumption status for the target AMF. The analytics report also provides recommended actions to optimize the target AMF for congestion control. Based on the report MDAS consumer adjusts (e.g., scale-out/up the virtual resource, set the suitable timer) the resources before continuing processing the received messages.

#### 6.2.4.2	Potential requirements

REQ-NAS_OPT_CON-1		MDAS producer should have the capability to provide analytics report describing the NAS level congestion issue at AMF.

REQ-NAS_OPT_CON-2		The analytics report describing the NAS level congestion issue should contain the following information:

-	The identifier of the NAS level congestion issue described in the analytics report;

-	The start time and end time of the NAS level congestion issue;

-	Affected AMF;

-	Root cause of the NAS level congestion issue;

-	The recommended actions to solve NAS level congestion issue.

#### 6.2.4.3	Possible solutions

##### 6.2.4.3.1	Solution Description

The MDAS producer correlates and analyses the management data described in the following subclause to provide AMF resource consumption statistics/predictions, identification of NAS level congestion issues and the root cause as the table in clause 6.2.4.3.3 shows. This procedure may be triggered by the request or periodically

##### 6.2.4.3.2	Data required for NAS level congestion control analysis

The management data required to analyse the NAS level congestion issue are defined as the following table.

| Data category | Required data |
| --- | --- |
| Performance Measurements | Performance measurement for AMF: registration and service related measurement for AMF as defined in clause 5.2 of TS 28.552 [8]. Virtual resource usage measurement for AMF as defined in clause 5.7 of TS 28.552 [8]. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |

##### 6.2.4.3.3	Analytics report

The NAS level congestion issue analytics report contains the following information.

| Analytics Report of NAS level congestion issue | Attribute Name | Description |
| --- | --- | --- |
|  | Issue identifier | The identifier of the NAS level congestion. |
|  | Start Time | The start time of the issue happened. |
|  | Stop Time | The end time of the issue has been solved. |
|  | Affected AMF | The MOI of the AMF that affected by the NAS level congestion issue. |
|  | Root cause | The root cause of the NAS level congestion issue, e.g. increased resource consumption. |
|  | Recommended actions | Recommendation for AMF in order to make it optimal for NAS level congestion control e.g. scale-out AMF, Increase or adjust back-off time to avoid large amounts of UEs initiate deferred requests simultaneously. |

#### 6.2.4.4	Evaluation

The solution described in clause 6.2.4.3 requires the analytics inputs as described in in clause 6.2.4.3.2, wherein

-	the performance measurements for AMF are available in TS 28.552 [8].

-	the configuration data are available in TS 28.541 [20].

With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.2.4.3.3 can be derived.

Therefore, this solution is a feasible candidate for NAS level congestion control analysis.

## 6.3	SLS analysis related issues

### 6.3.1	E2E latency analysis

#### 6.3.1.1	Use case

Latency is one of the SLA parameters for URLLC services. User data packets should be successfully delivered within certain time constraints to satisfy the end users requirements.

Latency could be impacted by the network capability and network configurations, e.g. configuration of service priority, RAN capacity, network load, number of re-transmissions, Wireless channel environment and the processing time of the network functions, etc. These factors may be the root cause if the latency requirements cannot be achieved. Packet transmission latency may dynamically change if one or multiple of these factors change. The latency requirement should be assured even if some of the network conditions may degrade. There are some mechanisms to assure latency, e.g. to upgrade the service priority, allocate or reserve more network resource, prepare backups.

With regard to latency analysis for URLLC services, the performance data and fault data are required to be collected, reported and analysed in near real time. Distributed MDAS deployment architecture should be applied for this scenario. The domain MDAS producers located in the edge network provides latency analysis or predictions for local services in near real time. E.g. for latency and other related QoS evaluation or prediction for V2X application, user location and user trajectory may need to be analysed in near-real time. The analytics report can be consumed by edge AFs to perform actions in time. The centralized MDAS producers analyses integrate latency performance for cross domain. It may provide more comprehensive analytics report to the centralized AF. AI/ML models or analytics data may need to be exchanged between the neighbouring domain MDAS producers, or between domain MDAS producers and the centralized MDAS producers.

From the management perspective, resource configuration and allocation algorithms or policies should support latency assurance. E2E latency is the latency across multiple domains, e.g. RAN domain, core network domain and transport network domain, each domain should ensure its own latency requirement to achieve the total E2E latency goal. The domain specific MDAS can be utilized to provide the domain specific latency analysis, and together with the cross-domain MDAS to provide E2E latency analysis to support SLS assurance.

#### 6.3.1.2	Potential requirements

REQ-LATENCY_ASS-CON-1: MDAS producer should have the capability to provide analytics report describing the latency problem.

REQ-LATENCY_ASS-CON-2: The MDAS producer should have the capability to provide the latency analytics report in time for URLLC services according to the corresponding latency requirement.

REQ-LATENCY_ASS-CON-3: Distributed MDAS deployment and exchange of analytics data between MDAS producers should be supported.

REQ-LATENCY_ASS-CON-4: The analytics report describing the latency problem should include the following information:

-	The identifier of the latency issue;

-	Indication of latency issue type;

-	The start time and end time of the latency issue;

-	The geographical area and location where the latency issue exists;

-	 Root cause of the latency issue;

-	The objects affected by the latency issue;

-	The severity level of the latency issue;

-	The recommended actions to solve the latency issue.

#### 6.3.1.3	Possible solutions

##### 6.3.1.3.1	Solution description

The performance measurements, e.g., network latency, UE throughput, network resource utilization and packet loss can be utilized for E2E latency analysis. To support E2E latency assurance in order to satisfy the latency requirements from the vertical users, MDAS producer is able to provide the analytics report as defined in clause 6.3.1.3.5 related with E2E latency analytics triggered by event or periodically.

##### 6.3.1.3.2	Required data for latency analysis for RAN domain

The management data required to analyse the latency are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information according to S-NSSAI |
| Performance measurement | Measurement data | Packet delay: "packet delay" measurement as defined in clause 5.1.1.1, clause 5.1.3.3, TS 28.552 [8]; IP Latency measurements: "IP Latency measurements" as defined in clause 5.1.3.4, TS 28.552 [8]; UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552 [8]; CQI related measurements: the distribution of Wideband CQI (Channel Quality Indicator) reported by UEs in the cell, see clause 5.1.1.11 of TS 28.552 [8]; MCS related Measurements: the distribution of the MCS scheduled for PDSCH RB by NG-RAN, the distribution of the MCS scheduled for PUSCH RB by NG-RAN, see clause 5.1.1.12 in TS 28.552 [8]; |
| MDT Data | Measurement data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |
| QoE Data | Measurement data | The details information of QoE data required by this case is FFS. |

Note: 	The above parameters may not be the complete list.

##### 6.3.1.3.3	Required data for latency analysis for CN domain

The management data required to analyse the latency are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information according to S-NSSAI |
| Performance measurement | Measurement data | Round-trip GTP Data Packet Delay: "Round-trip GTP Data Packet Delay" as defined in clause 5.4.1.9, TS 28.552 [8]; GTP packets delay in UPF: "GTP packets delay in UPF" as defined in clause 5.4.5, TS 28.552 [8]; Round-trip GTP Data Packet Delay on N9 interface: "Round-trip GTP Data Packet Delay on N9 interface" as defined in clause 5.4.4.1, TS 28.552 [8]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; |

Note: 	The above parameters may not be the complete list.

##### 6.3.1.3.4	Required data for E2E latency analysis for cross domain

For cross domain analysis, the RAN and CN domain required data as described in clauses 6.3.1.3.2 and 6.3.1.3.3 may be also needed, as well as the potential data described in the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information according to S-NSSAI |
| Performance measurement | Measurement data | End-to-end Latency of 5G Network: "End-to-end Latency of 5G Network" as defined in clause 6.3.1, TS 28.554 [7]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; |
| QoE Data | Measurement data | The details information of QoE data required by this case is FFS. |
| "Required data for latency analysis for RAN domain" or RAN domain "Analytics report for latency analysis" | Measurement data and/or analytics data | Raw data in "6.3.1.3.2 Required data for latency analysis for RAN domain", or analytics data of RAN domain in "6.3.1.3.5 Analytics report for latency analysis" |
| "Required data for latency analysis for CN domain" or CN domain "Analytics report for latency analysis" | Measurement data and/or analytics data | Raw data in "6.3.1.3.3 Required data for latency analysis for CN domain", or analytics data of CN domain in "6.3.1.3.5 Analytics report for latency analysis" |

Note: 	The above parameters may not be the complete list.

##### 6.3.1.3.5	Analytics report for latency analysis

The following table shows the potential information of the domain specific or cross domain E2E analytics report for latency analysis based on the required data received as described in clauses 6.3.1.3.2, 6.3.1.3.3 and 6.3.1.3.4.

| Analytics Report of E2E latency analysis | Attribute Name | Description |
| --- | --- | --- |
|  | SLS assurance issue Identifier | The identifier indicates the SLS assurance issue |
|  | Indication of SLS assurance issue type | Indicates the type of the SLS assurance issue, e.g., RAN latency issue, CN latency issue, TN latency issue. |
|  | Location | The geographical area and location where the latency issue exists |
|  | Start time | The start time of the latency issue |
|  | Stop time | The stop time of the latency issue |
|  | Root cause | The root cause of the E2E latency degradation, e.g. coverage issue, load issue in RAN, load issue in CN, low throughput in RAN, low throughput in CN etc. |
|  | Affected objects | The MOIs of cells, subnetwork or network slices affected by the latency issue; |
|  | Severity level | The severity level (e.g., critical, medium, not important) of the latency issue; |
|  | Recommended actions | The recommended actions to solve the latency issue, e.g., resource radio resources re-allocation in gNB, scaling the VNF. |

#### 6.3.1.4	Evaluation

Based on the potential solutions, the required inputs are listed in clause 6.3.1.3.2, clause 6.3.1.3.3 and clause 6.3.1.3.4, where:

-	The S-NSSAI as defined in clause 5.15.2, TS 23.501 [2] and TS 28.531 [33], can be acquired along with the performance measurements in TS 28.552 [8], TS 28.554 [7] and configuration data in TS 28.531 [33].

-	The performance measurements and KPIs is defined in TS 28.552 [8] and TS 28.554 [7].

-	The MDT Data defined in TS 37.320 [12] can be acquired in procedures defined in TS 32.422 [25].

-	QoE data as defined in TS 26.247 [29] and TS 26.114 [30] can be acquired through the procedures defined in TS 28.405 [31].

Therefore, MDA is feasible to support the analyses and predictions of E2E latency analysis related factors for cross domain MDAS, RAN domain and CN domain respectively.

### 6.3.2	Network slice load analysis

#### 6.3.2.1	Use case

Network slice load may vary over time. Therefore, network resources allocated initially could not always satisfy the traffic requirements, for example, the network slice may be overloaded or underutilized. Various factors may impact the network slice load, e.g. number of UEs accessing the network, number of PDU sessions, service types and the end users distribution. Overload of signalling in control plane and/or user data congestion in user plane will lead underperforming network. Besides, allocating excessive resources for network slice with light load will decrease resource efficiency.

The analysis of network slice load should consider the load of services with different characteristics (e.g., QoS information, service priority), load distribution to derive the corresponding resource requirements. Load level and detailed load distribution analytic result may be provided, e.g. load distribution for different applications (probably coming from respective AFs), different frequency layers, different RATs, different locations and/or time periods, different NFs etc. Proportion of high loaded/light loaded cells, NSSI, NFs etc can also be provided.

The analysis of network slice load should consider the network slice capacity, so that the number of users and the numbers of PDU sessions setup can be further supported by certain network slice could be analysed and predicted. "maxNumberofUEs" and "maxNumberofConns" are two capacity related requirements in the ServiceProfile in [20]. In addition, from end to end network slice perspective, the capacity related requirements and the actual network performance measurements also help to reflect the slice load and slice capacity headroom.

Traffics and resources related performance measurements and UE measurements can be utilized by MDAS producer to identify degradation of the performance measurements and KPI documented in an SLS due to load issues and provide the analytic reports of load analysis. For example, the analytic report for a RAN NSSI, slice load related information may include load of DRBs and SRBs, radio resource utilization and availability, virtual resources of RAN CU, etc. MDAS may provide statistic or predictive results of the above load related information and their correlations for multiple cells in certain coverage areas and time periods. MDAS may further provide recommendations of slice RRM Policy ratio for the analysed cells. In addition, MDAS may utilize the analytics data of slice load analysis (as appropriate), NF load analysis, UE mobility statistics/predictions and UE communication statistics/predictions from NWDAF [18] to assist its network slice load analysis. The producer of MDAS is able to, from the perspective of the management aspects, provide the network slice load analytics report related to an NSI, a specific RAN NSSI, a specific CN NSSI. This analytics report can be considered as an input to support SLA assurance to perform further evaluation.

#### 6.3.2.2	Potential requirements

REQ-NS_LOAD_ANA-1: MDAS producer should have the capability to provide analytics report describing the network slice load problem.

REQ-NS_LOAD_ANA-2: MDAS producer should have the capability to analyse and predict the network slice load based on the slice capacity including the number of users and the numbers of PDU sessions setup supported by certain network slice.

REQ-NS_LOAD_ANA-3: The analytics report describing the network slide load problem should include the following information:

-	The identifier of the network slice load problem;

-	Indication of the network slice load problem;

-	The start time and end time of the network slice load problem;

-	The type of network slice load problem (overloaded or underutilized);

-	The geographical area, location (UE, gNB, or UPF), domain (RAN, CN) where the network slice load problem exists;

-	The cause(es) of network slice problem (e.g., number of UEs accessing the network, number of PDU sessions, service types and the end user's distribution);

-	 Root cause of the network slice load problem;

-	The applications affected by the network slide load problem

-	The severity level of the network slice load problem;

-	The recommended actions to solve the network slice load problem.

#### 6.3.2.3	Possible solutions

##### 6.3.2.3.1	Solution description

The MDAS producer correlates and analyses the ongoing and/or potential network slice load problem based on the current and historical performance data related to resource usage and network traffic for the target network slice. The slice load is a comprehensive result. Various factors may impact the network slice load, e.g. number of UEs accessing the network, number of QoS flows, the resource utilizations of different NFs which is related with the network slice instance. The analysis of network slice load should consider the load of services with different characteristics (e.g., QoS information, service priority), load distribution to derive the corresponding resource requirements.  To identify degradation of the KPIs documented in an SLS due to load issues, traffics and resources related performance measurements and UE measurements can be utilized. Note that this solution covers both domain specific and cross-domain load problem analysis scenarios. Based on the analysis above, the MDAS producer is able to provide the analytics report as defined in clause 6.3.2.3.5 related with the network slice load analytics triggered by event or periodically.

##### 6.3.2.3.2	Required data for network slice load analysis for cross domain

For cross domain analysis, the RAN and CN domain required data as described in clauses 6.3.2.3.3 and 6.3.2.3.4 may be also needed, as well as the potential data described in the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for network slice load analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | End-to-end Latency of 5G Network: "End-to-end Latency of 5G Network" as defined in clause 6.3.1, TS 28.554 [7]; UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [8]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Virtualised resource utilization of Network Slice Instance, see clause 6.4.2 of TS 28.554 [7] |
| MDT Data | Measurement data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS 28.530 [5] and clause 7.15 of TS 28.531 [33]. |
| Network topology | Network topology data | The topology of the network for network slice load analytics. |

Note:	The above parameters may not be the complete list.

##### 6.3.2.3.3	Required data for network slice load for RAN domain

The management data required to analyse the network slice load for RAN domain are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for network slice load analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | RAN domain measurement data: RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552 [8]; Performance measurement data for gNB as defined in clause 5.1 of TS 28.552 [8]; |
| MDT Data | Measurement data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS 28.530 [5] and clause 7.15 of TS 28.531 [33]. |
| Network topology | Network topology data | The topology of the network for network slice load analytics. |

Note: 	The above parameters may not be the complete list.

##### 6.3.2.3.4	Required data for network slice load for CN domain

The management data required to analyse the network slice load for CN domain are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for network slice load analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | CN domain measurement data User subscription data by performance measurement for AMF as defined in clause 5.2 of TS 28.552 [8]; Performance measurement data for SMF as defined in clause 5.3 of TS 28.552 [8]; Performance measurement data for UPF as defined in clause 5.4 of TS 28.552 [8]; Performance measurement data for NF as defined in clause 5.7 of TS 28.552 [8]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS 28.530 [5] and clause 7.15 of TS 28.531 [33]. |
| Network topology | Network topology data | The topology of the network for network slice load analytics. |

Note: 	The above parameters may not be the complete list.

##### 6.3.2.3.5	Analytics report for network slice load analysis

The following table provides the potential information of the domain specific or cross domain analytics report for network slice load analysis based on the required data received as described in clauses 6.3.2.3.2, 6.3.2.3.3 and 6.3.2.3.4.

| Analytics Report of resource utilization analysis | Attribute Name | Description |
| --- | --- | --- |
|  | Network slice load problem identifier | The identifier indicates the network slice load problem |
|  | Type of network slice load problem | Indicates the type of the network slice load problem, e.g., ongoing/potential or overload/underutilized network slice load problem |
|  | Time period | Describes the time period(s) during which the network slice load problem has happened or is going to happen |
|  | Slice load level and distribution | Describes the detailed load distribution e.g., load distribution for different applications (probably coming from respective AFs), different frequency layers, different RATs, different locations and/or time periods, different NFs etc. |
|  | Slice load information | Describes the load of the following resources in the network slice: DRBs and SRBs, radio resource utilization and availability, virtual resources of RAN CU, etc |
|  | Slice load statistics | Describes the load statistics of the above resources and their correlated results in terms of coverage and time periods |
|  | Slice load predictive results | Describes the load predictive results of the above resources and their correlations in terms of coverage and time periods |
|  | Indication of network slice load demand | "Indicates, for each time period, that the network slice is overutilized (load is too high) or underutilized (load is too low) |
|  | Percentage of network slice load demand | Describes how much of the slice load capacity is either overloaded or underutilized in each time period |
|  | List of network entities | Lists the network entities involved in the network slice load problem |
|  | Recommended actions | Describes the recommended actions to solve the network slice load problem |

#### 6.3.2.4		Evaluation

Based on the potential solutions, the required inputs are listed in clause 6.3.2.3.2, clause 6.3.2.3.3 and clause 6.3.2.3.4, where:

-	The S-NSSAI as defined in clause 5.15.2, TS 23.501 [2] and TS 28.531 [33], can be acquired along with the performance measurements in TS 28.552 [8], TS 28.554 [7] and configuration data in TS 28.531 [33].

-	The performance measurements and KPIs is defined in TS 28.552 [8] and TS 28.554 [7].

-	The MDT Data defined in TS 37.320 [12] can be acquired in procedures defined in TS 32.422 [25].

-	The capacity planning data is available as the configuration data defined in TS 28.541 [20].

-	The topology data is available as the NRM defined in TS 28.541 [20].

-	User service experience data can be acquired from NWDAF or AF as defined in clause 6.4 of TS 23.288 [18].

Therefore, MDA is feasible to support the analyses and predictions of load related factors for cross domain MDAS, RAN domain and CN domain respectively.

### 6.3.3	Service experience analysis

#### 6.3.3.1	Use case

The variety of the services provided to the verticals is an important feature. The network requirements which support different services may vary a lot. For example, the users who require URLLC service have high requirements on the network latency and reliability, while the users who require NB-IoT service considers the maximum access UE numbers as the first priority for network requirement. Furthermore, for services supported by different QoS flows, the requirements of different QCIs/5QIs should be assured to satisfy the end user experience, e.g., the voice service which is critical on the users’ experience. The analysis of service experience of different vertical consumers and services types is important to support the SLA assurance from the network aspect.

The UE level's experience analysis based on the information from AF provided by NWDAF, e.g., the service MOS may be utilized by MDAS producer as one of the service performance inputs. The network performance measurements from RAN, e.g., the UE IP throughput, coverage, latency, successful handover ratio, etc. and network performance measurements from core network could also be used by MDAS producer for the analysis of service fulfilment by the network. Resource utilization efficiency may impact the user quality of service experience as described in clause 6.2.2. The analytics information from combining the user quality of service experience and, network performance and network resource utilisation analysis could be made available to operators or verticals for further action and analysis. For example, root cause for service deteriorating and recommendation actions may be provided by MDAS producer if some of the SLA requirements cannot be achieved, operators could perform the network optimization actions when needed. The MDAS producer may also coordinate with other management services producer to perform close loop SLA assurance of the network under the operator configured policies.

#### 6.3.3.2	Potential requirements

REQ-Ser_Exp_CON-1		The MDAS producer should have a capability to provide the analytics report on service experience analysis

REQ-Ser_Exp_CON-2	The analytics report describing the service experience should contain the following information describing the service experience aspects and potentially future prediction:

-	The predictive service experience or observed service experience statistics, may split into subcounters in different levels, e.g. per S-NSSAI, per 5QI, per UE, etc.

-	Service experience indication and root cause analysis.

#### 6.3.3.3	Possible solutions

##### 6.3.3.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to provide service experience analysis and identify the root cause. As the table in clause 6.3.3.3.3 shows, the analytics report is able to be provided by the MDAS producer to describe the service experience issue, root causes and recommendations. This procedure may be triggered by the request or periodically.

##### 6.3.3.3.2	Data required for service experience analysis

The following table shows the potential data required to analyse the service experience analysis.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Packet delay: The average and distribution of DL/UL packet delay between UPF and UE, see clause 5.4.9 of TS 28.552 [8] UE throughput: Average and distribution of DL/UL UE throughput in gNB, see clause 5.1.1.3 of TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]. Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR, packet delay and UE location information. |
| QoE Data | The details information of QoE data required by this case is FFS. The App data which shows the end user experience. |
| Analytics Data | NWDAF analytics data: Slice QoE, see clause 6.3 of TS 23.288 [18] NWDAF analytics data: Observed Service Experience and/or observed Service MoS, see clause 6.4 of TS 23.288 [18] |

Note: 	The above parameters may not be the complete list.

##### 6.3.3.3.3	Analytics report for service experience analysis

The following table shows the potential information of the domain specific or cross domain analytics report for service experience analysis based on the required data received as described in clause 6.3.3.3.2.

| Analytics Report of service experience | Attribute Name | Description |
| --- | --- | --- |
|  | Service experience identifier | The identifier indicates the analytics report is related with service experience analysis. |
|  | Managed Objects of service experience | The object instances where the service experience is applicable, e.g., SubNetwork Instance, NetworkSlice Instance, S-NSSAI. |
|  | Service experience level | The level of service experience, e.g. there are five levels which are represented by 1, 2, 3, 4, 5 where level 1 represents the users are endures bad experience while level 5 represents the users’ requirements are perfectly satisfied. |
|  | Info of service experience | Statistics or predictions of the service experience, may be analysed in different levels, e.g. per S-NSSAI, per network slice, per network slice subnet, per 5QI, per UE etc. |
|  | Root cause | The root cause of the service experience issues, e.g., unstable handover successful rate, high latency, low QoS retainability etc. |

#### 6.3.3.4	Evaluation

Based on the potential solutions, the required inputs are listed in clause 6.3.3.3.2, where:

-	The performance measurements and KPIs is defined in TS 28.552 [8] and TS 28.554 [7].

-	The MDT Data defined in TS 37.320 [12] can be acquired in procedures defined in TS 32.422 [25].

-	QoE data as defined in TS 26.247 [29] and TS 26.114 [30] can be acquired through the procedures defined in TS 28.405 [31].

-	User service experience data can be acquired from NWDAF or AF as defined in clause 6.4 of TS 23.288 [18].

Therefore, MDA is feasible to support the analyses and predictions of service experience.

### 6.3.4	Network slice throughput analysis

#### 6.3.4.1	Use case

Throughput is of great importance which represents the end users' experiences and also reflects the network problems, e.g., low UE throughput may be caused by the resource shortage.

In order to satisfy the requirements of dL/ulThptPerSlice in the ServiceProfile, MDAS may be utilized for throughput related analysis/predictions for, network slice instance. The related performance measurements in 28.552 [2] and KPIs in 28.554 [8] are utilized as the input data for analysis.

MDAS producer should have the capability to detect potential throughput issues and identify the root cause to assist throughput assurance. Network slice throughput analysis can be for a specific domain or for cross-domain. It is expected that 3GPP Cross Domain MDAS producer provides the federated throughput analytics report and Domain MDAS producer provides the domain specific throughput analysis. These two levels of MDAS producers worked in a coordinated way to assure the throughput performance.

The CSC concerns the statistics or predictions of subscribers' DL/UL throughput for the network slice, e.g. average percentage of users, for which the required SLS throughput is met or the average percentage of time, during which the required SLS throughput is, or could be, met. The 3GPP Cross Domain MDAS producer may indicate whether the problem is caused by NG-RAN or CN. In general, NG-RAN is the bottleneck because of the limited radio resources and complicated wireless radio conditions. In this case, resource reconfigurations may be needed to resolve the throughput degradation issue. To identify the root causes, RAN Domain MDAS Producer may collect radio resource related configurations and measurements, radio condition related measurements and throughput related measurements.

The producer of MDAS is able to, from the perspective of the management aspects, provide the network slice throughput analytics report. This analytics report can be considered as an input to support SLS assurance to perform further evaluation.

#### 6.3.4.2	Potential requirements

REQ-THP_CON-1		The MDAS producer should have a capability to provide the analytics report on network slice throughput.

REQ-THP_CON-2		The analytics report of the network slice throughput should contain the following information:

-	Network slice throughput statistics/predictions;

-	Root cause analysis of network slice throughput degradation;

#### 6.3.4.3	Possible solutions

##### 6.3.4.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to provide network slice throughput statistics/predictions, identification of throughput degradation issues and the root cause as the table in clause 6.3.4.3.5 shows. This procedure may be triggered by the request or periodically.

##### 6.3.4.3.2	Data required for network slice throughput statistics/predictions for RAN domain

The following table shows the potential data required to analyse the network slice throughput for RAN domain.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Packet Delay, see clause 5.1.1.1.1 in TS 28.552 [8]; UE throughput, see clause 5.1.1.3 of TS 28.554 [7]; RAN UE throughput, KPI shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput at N3 interface, Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Radio resource utilization, the usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552 [8]. |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR and UE location information. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. The data comes from the provisioning MnS producer, e.g., the configured ServiceProfile. |

Note: 	The above parameters may not be the complete list.

##### 6.3.4.3.3	Data required for network slice throughput statistics/predictions for CN domain

The following table shows the potential data required to analyse the network slice throughput for CN domain.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Throughput at N3 interface, Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Number of incoming/outgoing GTP data packets on the N3 interface, see clauses 5.4.1.1 and 5.4.1.2 of TS 28.552 [8]. Number of octets of incoming/outgoing GTP data packets on the N3 interface, see clauses 5.4.1.3 and 5.4.1.4 of TS 28.552 [8]. Virtualised resource usage measurement, see clause 6.2 of TS 28.552 [8]. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. The data comes from the provisioning MnS producer, e.g., the configured ServiceProfile. |

Note: 	The above parameters may not be the complete list.

##### 6.3.4.3.4	Data required for network slice throughput statistics/predictions for cross domain

For cross domain analysis, the RAN and CN domain required data as described in 6.3.4.3.2 and 6.3.4.3.3 may be also needed, as well as the potential data described in the following table.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Latency of 5G Network, see clause 6.3.1 of TS 28.554 [7]; Throughput for network slice instance, see clause 6.3.2 of TS 28.554 [7]; Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.3 of TS 28.554 [7]. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. The data comes from the provisioning MnS producer, e.g., the configured ServiceProfile. |

Note: 	The above parameters may not be the complete list.

##### 6.3.4.3.5	Analytics report for network slice throughput analysis

The following table shows the potential information of the domain specific or cross domain analytics report for network slice throughput analysis based on the required data received as described in clauses 6.3.4.3.2, 6.3.4.3.3 and 6.3.4.3.4.

| Analytics Report of network slice throughput | Attribute Name | Description |
| --- | --- | --- |
|  | Network slice throughput issue identifier | The identifier of the network slice throughput issues; |
|  | Network slice throughput statistics/predictions | Average percentage of users, for which the required SLS throughput is, or could be, met. Average percentage of time, during which the required SLS throughput is, or could be, met. |
|  | Root cause | The root cause of the network slice throughput degradation issues, e.g. PRB resource shortage; |
|  | Recommended actions | The recommend actions to solve the network slice throughput degradation issues. |

#### 6.3.4.4		Evaluation

Based on the potential solutions, the required inputs are listed in clause 6.3.4.3.2, clause 6.3.4.3.3 and clause 6.3.4.3.4, where:

-	The performance measurements and KPIs is defined in TS 28.552 [8] and TS 28.554 [7].

-	The MDT Data defined in TS 37.320 [12] can be acquired in procedures defined in TS 32.422 [25].

-	The configuration data is available as the NRM defined in TS 28.541 [20].

-	QoE data as defined in TS 26.247 [29] and TS 26.114 [30] can be acquired through the procedures defined in TS 28.405 [31].

Therefore, MDA is feasible to support the analyses and predictions of network slice throughput related factors for cross domain MDAS, RAN domain and CN domain respectively.

### 6.3.5	Uplink/downlink throughput per UE in network slice analysis

#### 6.3.5.1	Use case

Uplink/downlink throughput per UE in network slice is one of the SLA parameters. Data rate in uplink and downlink should be guaranteed to assure quality experience of UE using specific network slice.

UE uplink/downlink throughput could be impacted by the network capability and network configurations, e.g. configuration of service priority, RAN capacity, network load, number of UE in one TA, Wireless channel environment and the processing time of the network functions, etc. These factors may be the root cause if the UE uplink/downlink throughput requirements cannot be achieved. There are some mechanisms to assure UE uplink/downlink throughput, e.g. to upgrade the service priority, allocate or reserve more network resource.

From the management perspective, resource configuration and allocation algorithms or policies should support UE uplink/downlink throughput assurance. MDAS can be utilized to provide UE uplink/downlink throughput analysis to support SLS assurance.

#### 6.3.5.2	Potential requirements

REQ-UETHROUGHPUT_ASS-CON-1: MDAS producer should have the capability to provide analytics report describing the UE uplink/downlink throughput problem.

REQ-UETHROUGHPUT_ASS-CON-2: The analytics report describing the UE uplink/downlink throughput problem should include the following information:

-	The identifier of the UE uplink/downlink throughput issue;

-	Indication of UE uplink/downlink throughput issue type;

-	The start time and end time of the UE uplink/downlink throughput issue;

-	The geographical area and location where the UE uplink/downlink throughput issue exists;

-	 Root cause of the UE uplink/downlink throughput issue;

-	The objects affected by the UE uplink/downlink throughput issue;

-	The severity level of the UE uplink/downlink throughput issue;

-	The recommended actions to solve the UE uplink/downlink throughput issue.

#### 6.3.5.3	Possible solutions

##### 6.3.5.3.1	Solution description

The performance measurements, e.g., UE throughput, network resource utilization can be utilized for UE uplink/downlink throughput analysis. To support UE uplink/downlink throughput assurance in order to satisfy the throughput per UE requirements from the vertical users, MDAS producer is able to provide the analytics report as defined in 6.3.5.3.3 related with UE uplink/downlink throughput analytics triggered by event or periodically.

##### 6.3.5.3.2	Required data for UE uplink/downlink throughput analysis

The management data required to analyse the UE uplink/downlink throughput are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | "S-NSSAI" as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information according to S-NSSAI |
| Performance measurement | Measurement data | UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552 [8]; CQI related measurements: the distribution of Wideband CQI (Channel Quality Indicator) reported by UEs in the cell, see clause 5.1.1.11 of TS 28.552 [8]; MCS related Measurements: the distribution of the MCS scheduled for PDSCH RB by NG-RAN, the distribution of the MCS scheduled for PUSCH RB by NG-RAN, see clause 5.1.1.12 in TS 28.552 [8]; |
| MDT Data | Measurement data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |

Note: 	The above parameters may not be the complete list.

##### 6.3.5.3.3	Analytics report for UE uplink/downlink throughput analysis

The following table provides the potential contents of the analytics report of UE uplink/downlink throughput analysis.

| Analytics Report of UE uplink/downlink throughput analysis | Attribute Name | Description |
| --- | --- | --- |
|  | SLS assurance issue Identifier | The identifier indicates the SLS assurance issue |
|  | Indication of SLS assurance issue type | Indicates the type of the SLS assurance issue, e.g., |
|  | Location | The geographical area and location where the UE uplink/downlink throughput issue exists |
|  | Start time | The start time of the UE uplink/downlink throughput issue |
|  | Stop time | The stop time of the UE uplink/downlink throughput issue |
|  | Root cause | The root cause of the UE uplink/downlink throughput degradation, e.g. coverage issue, load issue in RAN, load issue in CN, low throughput in RAN, low throughput in CN etc. |
|  | Affected objects | The MOIs of cells, subnetwork or network slices affected by the UE uplink/downlink throughput issue; |
|  | Severity level | The severity level (e.g., critical, medium, not important) of the UE uplink/downlink throughput issue; |
|  | Recommended actions | The recommended actions to solve the UE uplink/downlink throughput issue, e.g., |

#### 6.3.5.4	Evaluation

The solution described in clause 6.3.5.3 requires the analytics inputs as described in in clause 6.3.5.3.2, wherein

-	the S-NSSAI can be provided along with the performance measurements, NRM and planning data.

-	the performance data (measurements and KPIs) are available in TS 28.552 [8] and TS 28.554 [7].

-	the MDT data are available in TS 32.422 [25].

With these analytics inputs which are already defined, the analytics output as described in 6.3.5.3.3 can be derived. Therefore, this solution is a feasible candidate for analysis of Uplink/downlink throughput per UE in network slice.

### 6.3.6	KPI anomaly analysis

#### 6.3.6.1	Use case

KPI(s) are of great importance for network operators to monitor the key performance of the network. For 5G and beyond, a large amount of KPIs are defined in TS 28.554 [7]. The correlations between different KPIs are complicated and it is hard to monitor the KPI(s) manually. Also, how to assign each KPI threshold is a big challenge for network operators, since each KPI threshold should not be a fixed value considering many factors such as network capacity, service types, end user's experiences, etc. In addition, the criteria to determine whether a KPI is anomalous also depends on a variety of requirements.

MDAS is expected to have the capability to analyse KPI(s) for both cross domain and single domain. 3GPP Cross Domain MDAS producers may coordinate with domain MDAS producer to identify the anomalous KPI(s) and the corresponding root cause(s). The KPI anomaly analysis should consider both a single KPI and KPI correlations of different domains. For cross domain KPI anomaly analysis, KPIs or KPI analytics report from each domain should be collected. The MDAS producer should also be able to identify anomalous network situations. A network situation can be characterized as a combination of KPIs with respect to a predefined context that includes time, location, amount of traffic, user characteristics, etc.

MDAS is also expected to have the capability to detect and predict the anomalous KPI(s) and anomalous network situations in different levels e.g. per S-NSSAI, per NSI or per NSSI, etc. The detection and prediction of single KPI anomaly and in relation with multiple correlated KPIs anomaly may be involved. The detection and prediction may also involve a single or multiple KPIs in relation with a network situation. By utilizing Machine learning technology, historical KPI data and performance data may also be used as the input to perform the ML model training. Besides, along with the KPI or network situation anomaly identification and root cause analysis, MDAS may also recommend more appropriate network configurations to optimize and resolve the KPI anomaly issue and improve the slice QoE.

The KPI anomaly analysis is also expected to have the capability to analyse the correlations between SLS and KPIs or network situations according to service model and identify the most relevant anomalous KPIs and network situations, which cause the SLS degradation, this corresponding analytics report can be considered as an input to support further SLS assurance.

#### 6.3.6.2	Potential requirements

REQ-KPI_ANOMALY_CON-1		The MDAS producer should have a capability to provide the analytics report on KPIs anomaly analysis.

REQ- KPI_ANOMALY_CON-2		The analytics report describing the KPIs anomaly should contain the following information describing the KPI anomaly aspects and potentially future prediction:

-	The predictive anomaly KPI(s) or observed anomaly KPI(s), may split into subcounters in different levels, e.g. per S-NSSAI, per NSI, per NSSI, per 5QI, per UE etc;

-	KPI anomaly indication and root cause analysis;

-	Cross domain and domain KPI anomaly analysis;

-	The recommendations for the configurations of network resource and KPI threshold.

REQ- KPI_ANOMALY_CON-3		The MDAS producer should have a capability to provide the analytics report that identifies a possible anomalous KPIs with respect to network situations for a particular network location and network.

#### 6.3.6.3	Possible solutions

##### 6.3.6.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to provide KPI anomaly analysis and identify the root cause. The required data can be from RAN domain or CN domain or both. As the table in 6.3.6.3.5 shows, the analytics report is able to be provided by the MDAS producer to describe the KPI anomaly issue, root causes and recommendations. This procedure may be triggered upon request or periodically.

##### 6.3.6.3.2	Data required for KPI anomaly analysis for RAN domain

The following table shows the potential data required to analyse the RAN domain KPI anomaly.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552 [8]; Performance Measurements for gNB: for example, for RRC connection related KPI anomaly analysis, see clause 5.1, TS 28.552 [8], e.g., RRC connection number, RRC connection establishment, RRC connection re-establishment, RRC connection resuming; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]. |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR and UE location information. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration data | The execution data including the changes or the configuration of the MOIs. |
| Context data | The information on the conditions applicable to the data considered for analytics, e.g. time of day, season or event in relation to location. |

Note:	The above parameters may not be the complete list.

##### 6.3.6.3.3	Data required for KPI anomaly analysis for CN domain

The following table shows the potential data required to analyse the CN domain KPI anomaly.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Performance Measurements for AMF: for example, for number of registered subscribers related KPI anomaly analysis, see clause 5.2.1, TS 28.552 [8]; Performance Measurements for SMF: for example, for PDU session management related KPI anomaly analysis, see clause 5.3.1, TS 28.552 [8]; Throughput at N3 interface: KPI related to Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; |
| QoE Data | The details information of QoE data required by this case is FFS. |

Note: 	The above parameters may not be the complete list.

##### 6.3.6.3.4	Data required for KPI anomaly analysis for cross domain

For cross domain analysis, the RAN domain and CN domain required data as described in clauses 6.3.6.3.2 and 6.3.6.3.3 may be needed, as well as the potential data described in the following table.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; NWDAF analytics data: Slice QoE, see clause 6.4 of TS 23.288 [18]. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration data | The execution data including the changes or the configuration of the MOIs. |
| Context data | The information on the conditions applicable to the data considered for analytics, e.g. time of day, season or event in relation to location. |

Note: 	The above parameters may not be the complete list.

##### 6.3.6.3.5	Analytics report for KPI anomaly analysis

The following table shows the potential information of the domain specific or cross domain analytics report for KPI anomaly analysis based on the required data received as described in clauses 6.3.6.3.2, 6.3.6.3.3 and 6.3.6.3.4.

| Analytics Report of network slice KPI anomaly | Attribute Name | Description |
| --- | --- | --- |
|  | KPI anomaly identifier | The identifier of the KPI anomaly; |
|  | Anomalous KPI Name | The name of the KPI(s) which is identified or predicted as anomalous, the KPI name refers to bullet a) in TS28.554 (7); |
|  | Managed Objects of anomalous KPI | The object instances where the KPI is applicable, e.g., SubNetwork Instance, NetworkSlice Instance |
|  | Info of KPI anomaly | Statistics or predictions of the anomalous KPIs, may concern single KPI or multiple correlated KPIs, and may split into subcounters at different levels, e.g. per S-NSSAI, per NSI, per NSSI, per 5QI, per UE etc. |
|  | Root cause | The root cause of the network slice KPI anomaly issues, e.g., unstable handover successful rate, low PRB utilization, low QoS retainability. |

##### 6.3.6.3.6	Analytics report for network situation analysis

The following table shows the potential information of the domain specific or cross domain analytics report based on the required data received as described in clauses 6.3.6.3.2, 6.3.6.3.3, and 6.3.6.3.4.

| Analytics Report on network situations | Attribute Name | Description |
| --- | --- | --- |
|  | Network situation identifier | The identifier of the possible network situations. |
|  | Network situation description | The network performance, QoE and UE location and context data that describe a network situation. |
|  | Network situations anomaly status | The status of a network situation. This may be a simple binary function that indicates a normal or anomalous status. |
|  | Severity level | Describes the degree of anomaly (low, medium, high). |
|  | Location | Geographical location of anomalous network situations |
|  | Managed Objects of network situation | The object instances, e.g., cell, carrier. |
|  | Type of analytics | Statistics or predictions of the anomalous situation. |

#### 6.3.6.4	Evaluation

The solution described in clause 6.3.6.3.1 requires the analytics inputs as described in clause 6.3.6.3.2, 6.3.6.3.3 and 6.3.6.3.4 wherein:

-	Input PM related to RAN utilization, gNB measurements, AMF and SMF are specified in TS 28.552 [8].

-	Input KPIs related to RAN UE throughput and throughput N3 are specified in TS 28.554 [7].

-	Input MDT data are specified in TS 37.320 [12].

-	Input related to service experience specified in TS 37.320 [12].

-	QoE data is optional.

-	Configuration data is available.

-	Context data that relates to information on the conditions applicable to the data considered for analytics need to be specified as NRM.

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.3.6.3.5 and 6.3.6.3.6 can be derived for RAN domain, CN domain and Cross domain respectively.

Therefore, the solution is feasible.

### 6.3.7	Jitter analysis

#### 6.3.7.1	Use case

In addition to E2E latency, some new network services have extra new demands for SLA parameters. In extreme business scenarios such a V2X and telemedicine, network jitter also needs to be satisfied. Jitter assurance with high certainty becomes a new dimension of end-to-end delay-sensitive service quality.

The analysis of jitter does not cover the entire network, but the analysis should consider the whole process of data frame and also the definition of different network layers. It could be impacted by the configuration of clock synchronization, traffic forwarding, bandwidth reservation and latency etc. In some circumstances, it will also be affected by the external environment, which can only be resolved non-automatically.

The performance measurements and measurements related to data forwarding mechanisms can be used by the MDAS producer to generate a jitter analysis report. For example, in some network node, one particular data flow can have bandwidth competition with normal data flow which causes high network jitter. The MDAS will first analyse the bandwidth resource and provide bandwidth reservation modification so that it can satisfy network performance requirement of this particular data flow. Jitter is also related to E2E latency parameter, the jitter analysis will then need to be cross-domain, e.g. RAN domain, core network domain transport network domain. The jitter analysis from the multiple domains may be combined to calculate the end-to-end jitter, which can be checked against end-to-end SLA requirements. The jitter analysis will reference the E2E analysis result to support SLS assurance. In case of excessive end-to-end jitter, the jitter analysis from each domain may be used to identify the likely source of problems.

#### 6.3.7.2	Potential requirements

REQ-JITTER_ASS-CON-1: MDAS producer should have the capability to provide analytics report describing the jitter problem.

REQ-JITTER_ASS-CON-2: The analytics report describing the jitter problem should include the following information:

-	The identifier of the jitter issue;

-	Indication of jitter issue type;

-	The start time and end time of the jitter issue;

-	The geographical area and location where the jitter issue exists;

-	 Root cause of the jitter issue;

-	The objects affected by the jitter issue;

-	The severity level of the jitter issue;

-      The concrete parameters of jitter value;

-	The recommended actions to solve the jitter issue.

#### 6.3.7.3	Possible Solutions

##### 6.3.7.3.1	Solution description

The solution requires MDAS producer to collect various data and provide jitter analytics report. To support jitter assurance in order to satisfy the customer’s requirements, the MDAS producer is required to provide the analytics report as defined in 6.3.7.3.3. This procedure may be triggered by the request or periodically

##### 6.3.7.3.2 	Data required

The management data required to analyze the jitter are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | “S-NSSAI” as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information according to S-NSSAI |
| Performance measurement | Measurement data | Packet delay: “packet delay” measurement as defined in clause 5.1.1.1, clause 5.1.3.3, TS 28.552 [8]; Round-trip GTP Data Packet Delay: “Round-trip GTP Data Packet Delay” as defined in clause 5.4.1.9, TS 28.552 [8]; GTP packets delay in UPF: “GTP packets delay in UPF” as defined in clause 5.4.5, TS 28.552 [8]; Round-trip GTP Data Packet Delay on N9 interface: “Round-trip GTP Data Packet Delay on N9 interface” as defined in clause 5.4.4.1, TS 28.552 [8]; One way packet delay between NG-RAN and PSA UPF as defined in clause 5.4.7, TS 28.552 [8]; Round-trip packet delay between PSA UPF and NG-RAN as defined in clause 5.4.8, TS 28.552 [8]; One way packet delay between PSA UPF and UE as defined in clause 5.4.9, TS 28.552 [8]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; End-to-end Latency of 5G Network: “End-to-end Latency of 5G Network” as defined in clause 6.3.1, TS 28.554 [7]; Throughput for network slice: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; New measurements on UL and DL jitter for network slice in NG-RAN, on N3/N9 interface, between PSA UPF and UE, between PSA UPF and NG-RAN. |
| QoE Data | Measurement data | The details information of QoE data required by this case is FFS. |
| Service experience data | Network analytics data | The analysis result regarding observed service experience from the NWDAF defined in TS 23.288 [18]. |

Note: The above parameters may not be the complete list.

##### 6.3.7.3.3	Analytics report

The jitter analytics report contains the following information.

| Jitter Analytics Report | Attribute Name | Description |
| --- | --- | --- |
|  | Jitter Identifier | Identification of the jitter issue. |
|  | Jitter Issue Type | Indication of jitter issue type, e.g. RAN jitter issue, CN jitter issue. |
|  | Start Time | The start time of the jitter issue. |
|  | End Time | The end time of the jitter issue. |
|  | Location Information | The geographical area and location where the jitter issue exists. |
|  | Root cause | Root cause of the jitter issue. |
|  | Affected objects | The MOIs of cells, subnetwork or network slices affected by the jitter |
|  | Severity level | The severity level (e.g., critical, medium, not important) of the jitter issue. |
|  | Jitter Value | The average network latency with delay variation range |
|  | Recommended actions | The recommended actions to solve the jitter issue. |

#### 6.3.7.4	Evaluation

The solution described in clause 6.3.7.3 requires the analytics inputs as described in in clause 6.3.7.3.2, wherein

-	the S-NSSAI can be provided along with the performance measurements, NRM and planning data.

-	the delay related performance measurements and KPIs are available in TS 28.552 [8] and TS 28.554 [7], and the jitter related measurements can be defined in TS 28.552 [8].

-	QoE data as defined in TS 26.247 [29] and TS 26.114 [30] can be acquired through the procedures defined in TS 28.405 [31].

-	The analysis results regarding observed service experience from the NWDAF are defined in TS 23.288 [18].

With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.3.7.3.3 can be derived.

Therefore, this solution is a feasible candidate for jitter analysis.

### 6.3.8	Network slice traffic projection

#### 6.3.8.1	Use case

Some of the requirements captured in SliceProfile need to be translated into configurable parameter for various network entities including entities in 5G Core Network (5GC), Radio Access Network (RAN) and Transport Network. One of the example would be: the GST attribute Downlink throughput per slice (dLThptPerSlice) can be translated into maximum downlink throughput per slice (maxDlThptPerSlice) as a configuration parameter, for UPF. However, as one slice may have multiple UPF instances, dividing the total quota available among each UPF instance is critical. The given requirements can be divided among all the targeted NF instance based on the slice traffic analytics. The value for a particular configurable parameter (translated from a particular ServiceProfile attribute) for a particular NF is crucial, especially if multiple instance of that NFs is available in the network slice instance.

It is desirable to use uses MDAS to get the network slice traffic projections including individual traffic projections on each of the constituent network functions instances present in the slice. The individual traffic projections can be used to divide total available quota among the constituent network functions instances which can then be configured for network function(s), as required. For example, MDAS can provide total number of projected terminal or subscription for each AMF instance in the slice. Based on the projections the total available quota can be divided among the multiple AMF instances in the slice. The AMF instance serving more users or required to serve more users in future will have more quota then other AMF instances in the slice.

#### 6.3.8.2	Potential Requirements

REQ-TRA_CON_CON-1: The MDAS producer should have a capability allowing the authorized consumer to request the slice traffic analytics report describing traffic projection of the slice including its constituent network functions.

REQ-TRA_CON_CON-2: The MDAS producer should have a capability to provide the slice traffic analytics report describing the traffic projections for each constituent network function instance in the slice.

REQ-TRA_CON_CON-3: The slice traffic analytics report providing traffic projection for the slice may include the following information:

-	Projected uplink and downlink throughput on each User Plane Function instance (UPF) present in the slice.

-	Projected number of Packet Data Unit (PDU) session for each Session Management Function (SMF) instance present in the slice.

-	Projected number of UE or Registered subscriptions for each AMF instance present in the slice.

-	Projected maximum packet size for each UPF instance present in the slice.

-	Projected UE uplink and downlink throughput on each gNodeB (gNB) instance present in the slice.

-	Projected number of UE for each gNB/NR cell instance present in the slice.

#### 6.3.8.3	Possible Solutions

##### 6.3.8.3.1 	Solution description

The solution requires MDAS producer to produce slice traffic analytics report providing traffic projections on NFs involved in the slice. Based on the report, configuration for the NFs can be decided.

##### 6.3.8.3.2 	Data required

The following table shows the data required to generate the traffic analytics report.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements/KPIs |  |
|  | From each UPF instance in the slice - Current Uplink throughput. See 5.4.1.3 in TS 28.552. - Current Downlink throughput. See 5.4.1.4 TS 28.552 - Current Maximum packet size |
|  | From each gNB instance in the slice - Current Uplink UE throughput. See 5.1.1.3 in TS 28.552. - Current Downlink UE throughput. See 5.1.1.3 in TS 28.552. |
|  | From each SMF instance in the slice - Current Number of PDU session. See 5.2.1 in TS 28.552. |
|  | From each AMF instance in the slice - Current Number of registered subscriptions. See 5.2.1 in TS 28.552. |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |

##### 6.3.8.3.3 	Analytics report

The slice traffic analytics report contains the following:

| Analytics Report of slice traffic projection | Attribute Name | Description |
| --- | --- | --- |
|  | Slice Identifier | Identifier of the slice for which the report is provided |
|  | Projection Timestamp | Provide a particular time stamp for which the projections are provided |
|  | Projection Duration | Provides a time duration during which the average projections are provided |
|  | For each UPF in the slice | Projected Uplink throughput |
|  |  | Projected Downlink throughput |
|  |  | Projected Maximum packet size |
|  | For each gNB in the slice | Projected Uplink UE throughput. |
|  |  | Projected Downlink UE throughput |
|  |  | Projected Number of UE |
|  | For each SMF in the slice | Projected Number of PDU session |
|  | For each AMF in the slice | Projected Number of registered subscriptions |

#### 6.3.8.4	Evaluation

Based on the potential solutions, the required inputs are listed in clause 6.3.8.3.2, where:

-	The performance measurements and KPIs are defined in TS 28.552 [8] and TS 28.554 [7].

-	The MDT Data defined in TS 37.320 [12] can be acquired in procedures in TS 32.422 [25].

With these analytics inputs which are already defined, the analytics output as described in 6.3.8.3.3 can be derived.

Therefore, MDA is feasible to support the analyses and predictions of traffic projection.

## 6.4	Fault management related issues

### 6.4.1	Alarm incident analysis

#### 6.4.1.1	Use case

In 5G network, millions of alarms are generated due to the network complexity. Since the topological relations between different network elements and logical relations between different generated alarms, a series of alarms caused by a same root cause should be correlated with each other. In addition, the same root causes may give rise to the network performance deterioration. For example, an alarm in a lower layer of the communication protocol stack or an alarm related to virtualized resource in the virtualization deployment environment may cause multiple alarms in higher layers. Sequence of alarms may be generated in multiple domains along a communication link if one fault in the source domain occurs.

Large amount of alarms brings difficulties in network operation and maintenance. Therefore, the alarms and deteriorated performance measurements of same root cause should be correlated and analysed. Some ML models and algorithms may be used to group or filter the correlated alarms and indicate the root cause. Also, the historical alarms, performance measurements and network topology data can be utilized as the foreknowledge.

To improve the efficiency of network operation and maintenance, the MDAS producer is able to provide the analytics result including the root alarm or root cause by correlate and group the related alarms and performance measurements into an alarm incident.

#### 6.4.1.2	Potential requirements

REQ-ALARM_MDA-01:	The MDAS producer should have a capability to provide the analytics report describing the alarm incident analysis.

REQ-ALARM_MDA-02:	The analytics report describing the alarm incident should include the following information:

-	Alarm incident Identifier

-	List of Correlated Alarms, performance measurements

-	The start time and end time of the Alarm incident

-	The root cause or root alarm of the Alarm incident

-	Severity level

-	Affected objects and object relationships

-	Recommended actions

#### 6.4.1.3	Possible Solutions

##### 6.4.1.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to identify the alarm incident. The required data can be from RAN domain or CN domain or both. As the table in clause 6.4.1.3.4 shows, the analytics report is able to be provided by the MDAS producer to describe the root causes and recommendations of identified alarm incident. It can be a domain specific or cross domain analytics report. This procedure may be triggered by the request or periodically.

##### 6.4.1.3.2	Data required for alarm incident analysis for RAN domain

Alarm data and performance measurements from correlated logical and physical resources are able to be utilized to perform the alarm incident analysis. The data listed in following table are the potential management data for RAN domain used to construct the alarm incident.

| Data Category | Required Data |
| --- | --- |
| Alarm Data | The alarm information, e.g., the alarm of NG-U, the alarm of radio frequency unit, the alarm of cell outage, as specified in TS 28.532 [28]. |
| Performance measurements | The deteriorated performance or the abnormal performance measurements based on certain performance monitoring threshold. Following performance measurements may be used. UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [8]; PDCP Data Volume: The transmitted PDCP data volume, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [8]; TB related measurements: The TB transmitted in a cell, see clause 5.1.1.7 of TS 28.552 [8]; CQI related measurements: the distribution of Wideband CQI (Channel Quality Indicator) reported by UEs in the cell, see clause 5.1.1.11 of TS 28.552 [8]; MCS related Measurements: the distribution of the MCS scheduled for PDSCH RB by NG-RAN, the distribution of the MCS scheduled for PUSCH RB by NG-RAN, see clause 5.1.1.12 in TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Inter-gNB handovers: Number of requested/successful handover/failed handover preparations/resource allocations/executions, see clause 5.1.1.6.1 of TS 28.552 [8]. Intra-gNB handovers: Number of requested/successful handover executions, see clause 5.1.1.6.2 of TS 28.552 [8]. RRC connection establishment related measurements: Attempted/ successful RRC connection establishments, see clause 5.1.1.15 of TS 28.552 [8]. RRC connection Re-establishment and RRC connection Resuming related measurement, see clause 5.1.1.17 and 5.1.1.18 of TS 28.552 [8]. Packet loss rate, Packet drop rate and Packet delay including the average delay and distribution of delay in CU-UP, F1-U, gNB-DU, CU-UP both of downlink and uplink. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. The topology can be reflected by the NG-RAN NRMs as defined in TS 28.541 [20]. |

##### 6.4.1.3.3	Data required for alarm incident analysis for CN domain

Alarm data and performance measurements from correlated logical and physical resources are able to be utilized to perform the alarm incident analysis. The data listed in following table are the potential management data for CN domain used to construct the alarm incident.

| Data Category | Required Data |
| --- | --- |
| Alarm Data | The alarm information, e.g., the alarm of AMF, the alarm of UPF, as specified in TS 28.532 [28]. |
| Performance measurements | The deteriorated performance or the abnormal performance measurements based on certain performance monitoring threshold. Following performance measurements may be used. Mobility related measurements for AMF: Number of requested/failed PDU sessions/QoS flows for inter-AMF handovers and number of attempted/successful/failed handover between 5GS and EPC, see clause 5.2.5.1, 5.2.5.3 and 5.2.5.4 of TS 28.552 [8]. Session related measurements for SMF: Number of successful/failed PDU session creation and number of requested/successful/failed PDU session modifications, see clause 5.3.1 of TS 28.552 [8]. QoS flow monitoring for SMF: Number of requested/successful/failed QoS flows to create/modify, see clause 5.3.2 of TS 28.552 [8]. Session related measurements for UPF: Number of requested/failed N4 session establishments, see clause 5.4.3.1 of TS 28.552 [8]. Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Virtualised resource usage measurement, see clause 6.2 of TS 28.552 [8]. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. The topology can be reflected by the 5GC NRMs as defined in TS 28.541 [20]. |

##### 6.4.1.3.4	Analytics report for alarm incident analysis

The following table provides the potential information of the domain specific or cross domain analytics report for alarm incident analysis based on the required data received as described in clauses 6.4.1.3.2 and 6.4.1.3.3.

| Analytics Report of alarm incident | Information | Description |
| --- | --- | --- |
|  | Alarm Incident Identifier | The alarm incident id or name for correlated alarms and performance measurements, e.g., NgU transmission alarm incident, Xn transmission alarm incident. |
|  | List of Correlated AlarmInfo | List of Alarm name or alarm ID, e.g., alarm of NgU setup, alarm of user plane link failure, alarm of user plane failure, alarm of cell outage. |
|  | List of Correlated performance measurements info | Performance measurements and the corresponding value, e.g., NgU handover failure rate, NgU setup failure rate. |
|  | Location | The geographical area or the cells where the alarm incident exists |
|  | Start Time | The start time of alarm incident |
|  | Stop Time | The end time of alarm incident |
|  | Affected objects and object relations | The affected objects (e.g., the MOIs of cells or subnetworks or network slices affected by the alarm incident) and the (list of) relationship(s) between the objects, e.g., if two affected cells are connected with each other through Xn interface, the relationship between these two objects may be noted as neighbouring. |
|  | Root cause or Root alarm | Root alarm identified or predicted by root cause decision model, e.g. alarm of NgU setup, alarm of virtualized resource failure. |
|  | Severity level | The severity level (e.g., critical, medium, not important) of the alarm incident |
|  | Recommended actions | The recommend actions to clear the alarm incident. The recommended actions could be to replace the hardware unit, reconfigure the protocol, e.g., Xn application protocol |

#### 6.4.1.4	Evaluation

It is important to identify the root cause and resolve network malfunctions for the network to remain healthy.

The solution described in clause 6.4.1.3.4 requires the analytics inputs as described in in clause 6.4.1.3.2 and 6.4.1.3.3, wherein

-	the alarm data are available in TS 28.532 [28].

-	the performance measurements are available in TS 28.552 [8].

-	the network topology can be reflected by the NRMs defined in TS 28.541 [20].

-	the configuration data for NG-RAN and 5GC have been defined in TS 28.541 [20].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.4.1.3.4 can be derived.

Therefore, this solution is a feasible candidate for Alarm incident analysis for RAN domain, CN domain and cross domain network.

### 6.4.2	Fault prediction analysis

#### 6.4.2.1	Use case

In 5G network, millions of alarms are generated every day. The causes of these alarms are usually the faults or abnormal states of the network. These alarms could be caused by for example the multiple managed entities (MEs) such as NSI (Network slice instance), NSSI (network slice subnet instance), NFs (network functions) (that could be run over Virtual NFs (VNFs) or as Physical NFs (PNFs)) that may be subject to risks e.g. inaccurate programming, unforeseen software errors, incorrect or incompatible version of software or hardware being used, vulnerabilities in design, unforeseen protocol errors, security errors and so on.  The current treatment method is generally based on the alarm information analysis, to find out the cause of the faults or abnormal states, and then determine the fault repair method and solve the problems, so as to eliminate the alarms. Because the amount of alarms is so large, it is a big challenge to deal with these alarms in a timely and efficient manner.

One the other hand, if the network can be maintained very well so that it has fewer faults and abnormal states, then there will be fewer alarms. This requires the system to be able to predict the potential faults or abnormal states before they occur, and to recommend appropriate handling actions to prevent the fault or abnormal state really occur. For example, often long before failures manged entities (particularly software entities) tend to exhibit strange outlying behaviour in comparison with their previous operation (example: sudden increase in time taken to process requests, high compute or memory use, slower response to heartbeat and so forth). In 5G network, MDAS is adopted, which is in conjunction with AI and ML techniques. The MDAS producer may train the ML model of the MDAS by using the historical alarms, performance measurements, configuration data and configuration notifications, and network topology information to obtain the basic health maintenance knowledges (e.g. the relationship between the faults or potential faults and the related maintenance actions).

The MDAS producer monitors and analyses the performance measurements and KPIs continuously, and provides the analytics report which includes the predictive information of potential faults or abnormal states and corresponding recommendation of maintenance actions to prevent the fault or abnormal state really occur, so that the MDAS consumer can execute the recommended actions accordingly or by taking the recommended actions into account.

The MDAS producer is informed when the recommended actions are taken by the MDAS consumer to maintain the network health, so that the MDAS producer can evaluate the result of the executed actions and update its basic health maintenance knowledges.

The MDAS producer also periodically does the ML training based on the new collected alarms, performance measurements and KPIs, configuration data and configuration notifications, and updates its basic health maintenance knowledges.

#### 6.4.2.2	Potential requirements

REQ-HEALTH_MDA-01:	 The MDAS producer should have a capability to provide the analytics report describing the fault prediction analysis results.

REQ-HEALTH_MDA-02:	 The analytics report describing the results from the fault prediction analysis should include the detailed advice on how to eliminate the cause of potential fault(s).

Note: 	The detailed advice described in the fault prediction analytics report may include but not limited the follows:

-	List of potential faults and severity levels

-	Affected objects’ identifiers

-	Timestamp: Time at which the report is generated

-	Relevant performance measures

-	The start time and end time of the Outlying ME analysis

-	Recommended actions

#### 6.4.2.3	Possible solutions

##### 6.4.2.3.1	Solution description

The MDAS producer analyses the management data described in the following subclauses to identify the potential faults or abnormal states and corresponding recommended actions. The required data can be from RAN domain or CN domain or both.

As the table in clause 6.4.2.3.4 shows, the analytics report is able to be provided by the MDAS producer to describe the analytics result and recommendations of network health maintenance. It can be a domain specific or cross domain analytics report. This procedure may be triggered by the request or periodically.

##### 6.4.2.3.2	Data required for fault prediction analysis for RAN domain

The following table shows the potential data required to perform the fault prediction analysis for RAN domain.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | RAN related performance measurements and KPIs, see TS 28.552 [8] and TS 28.554 [7]; The detailed types of performance measurements and KPIs are FFS |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. |
| Versioning and vendor information | The versioning and vendor information of the managed entities |

Note: 	The above parameters may not be the complete list.

##### 6.4.2.3.3	Data required for fault prediction analysis for CN domain

The following table shows the potential data required to perform the fault prediction analysis for CN domain.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | CN related performance measurements and KPIs, see TS 28.552 [8] and TS 28.554 [7]; The detailed types of performance measurements and KPIs are FFS |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. |
| Versioning and vendor information | The versioning and vendor information of the managed entities |

Note: 	The above parameters may not be the complete list.

##### 6.4.2.3.4	Analytics report for fault prediction analysis

The following table shows the potential information carried in the analytics report of fault prediction analysis.

| Analytics Report of fault prediction | Attribute Name | Description |
| --- | --- | --- |
|  | Fault prediction analytics report identifier | The identifier of the fault prediction analytics report |
|  | List of potential faults | List of potential faults, including: Fault type Location Severity level Affected objects |
|  | Root cause or Root alarm | Root alarm identified or predicted by root cause decision model, e.g. incompatible software versions |
|  | Recommended actions | The recommend actions to eliminate the causes of the potential faults |

Note: 	The above parameters carried in the analytics report may not be the complete list.

#### 6.4.2.4	Evaluation

The solution described in clause 6.4.2.3 requires the analytics inputs as described in clause 6.4.2.3.2 and 6.4.2.3.3, wherein

-	the performance measurements and KPIs are available in TS 28.552 [8] and TS 28.554 [7].

-	the configuration data for NG-RAN have been defined in TS 28.541 [20].

-	the network topology can be reflected by the NRMs defined in TS 28.541 [20].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.4.2.3.4 can be derived.

Therefore, this solution is a feasible candidate for fault prediction analytics.

### 6.4.3	Alarm malfunction analytics

#### 6.4.3.1	Use case

Fault management involves a large amount of alarms at different levels, which are inter-related based on topology and logical paradigms. Hence, a single cause may trigger several alarms across multiple domains. To identify the root cause, alarm correlation is adopted leveraging also the benefits of historical data and performance measurements.

However, a major assumption in alarm correlation is the fact that the source(s) of the corresponding alarm(s) are not malfunctioning, i.e. producing false alarms. In fact, there are cases where alarms are triggered in situations where there is no need, (i.e. false positive alarms), as well as cases where no alarm is triggered although it should have been triggered, i.e. false negative alarms. Also, there are cases where an alarm malfunction is caused, e.g. by operational errors, device malfunctions, manmade interruptions, etc., and in different stages, e.g. during proceed alarm subscription, generate alarm, generate alarm notification, report alarm, proceed alarm acknowledge or clear, etc.

To resolve the problem of alarm malfunction, the MDAS producer can assess the risk of a faulty alarm, recommending methods to suppress other related alarms if a faulty alarm is identified in order to avoid large amounts of alarms, which may overload the network. The MDAS producer may correlate alarms and performance measurements in order to check or predict the consistency, e.g. an alarm of abnormal handover failures towards a specific cell need to be consistent with the number of newly residing users within a certain time window. In addition, certain performance measurements need to be consistent, e.g. reports on high mobility towards a specific cell need to be consistent with the user density of that cell. Otherwise, a malfunction risk exists, which needs to be analysed via the corresponding MDAS producer. Such a consistency check can also take place across different automation levels, e.g. considering NE level and domain level or domain level and cross-domain level.

The MDAS producer shall analyse the malfunction issue to identify the root case and recover erroneous data based on historical data in conjunction with “neighboring” data, using e.g. missing value estimation mechanisms. In addition, to track down malfunction across different automation levels, the source of alarms needs to be visible across different automation levels, e.g. via the means of meta-data, to enable backtracking.

#### 6.4.3.2	Potential requirements

REQ-ALR_MUL -1	The MDAS producer should have a capability to provide the analytics report describing alarm malfunction analytics information to authorized consumers.

REQ-ALR_MUL -2	The analytics report describing alarm malfunction information should contain the following information:

-  	the alarm malfunction incident ID;

- 	the location related to the alarm malfunction or faulty performance measurement(s);

-	the affected object(s) related to malfunction alarm or faulty performance measurement(s);

-	the time period(s) related to malfunction alarm or faulty performance measurement(s);

-	the root cause that identifies the source malfunction alarm or the source faulty performance measurement(s);

-	recommended actions involving malfunction alarm isolation and potential data recovery based on history data and “neighboring” data.

#### 6.4.3.3	Possible solutions

##### 6.4.3.3.1		Solution description

For this Solution, MDAS producer provides information related to the assessment of alarm malfunctions to identify the type of malfunction, location, management objects and recommendation actions, e.g. to supress false alarms and recover erroneous data.

##### 6.4.3.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Alarm Data | Fault supervision data report services as defined in TS 28.545 [22]. |
| Service Data | S-NSSAI as defined in clause 5.15.2, TS 23.501 [13]. |
| Performance Measurements | Performance Measurements for gNB (TS 28.552 [8]) - Radio resource utilization (UL/DL), total/available/used PRBs - RRC mean/max number of connections - PDU session management, mean/max request/successful/failed to set up - Inter/intra-gNB mobility, number of request/successful /failed preparations, resource availability, executions, failed - TB/DRB/CQI/QoS flow related measurements Performance Measurements for 5GC (TS 28.552 [8]) - Number of UEs/periodic registration updates (AMF) - Number of service requests (AMF) - Mobility related measurements (AMF) - Session management, number of PDU sessions/modifications/QoS flows (request, successful, failed) (SMF) - Performance measurements N4 (SMF) - N4/N6/N9 measurements traffic volume, link usage, packet loss (UPF) - QoS flow related measurements (UPF) E2E KPIs (TS 28.554 [7]) - Registered subscribers (AMF) - Upstream/downstream throughput for network and NSI - QoS flow/DRB retainability |
| Configuration Data | NRM attributes affecting the NF resource allocation and configuration NRM update reports (notification and log) containing the creation or changes of the MOIs affecting the NFs |
| Network Topology | Topology of the network |

##### 6.4.3.3.3		Analytics report

The network congestion analytics report contains the following information.

| Information | Description |
| --- | --- |
| Incident Identifier | Identifier that indicates the incident e.g. alarm malfunction, faulty performance measurement, etc. |
| Location | Affected geographical location related to e.g. malfunction alarm or faulty performance measurement, etc. |
| Affected Objects | NF, MnS, PDU session, QoS Flow, Slice, etc. |
| Start/Stop Time | Starts/stop time of the malfunction issue |
| Root Cause | The source alarm or performance measurements related to the identified malfunction. |
| Severity Level | The severity level (e.g. critical, medium, not important) of the malfunction |
| Recommended Actions | Recommendation actions to resolve malfunction issues: - Isolate/terminate malfunction e.g. alarm or performance measurement, etc. - Suppress alarms triggered by a faulty alarm. - Recover erroneous data based on “neighbor” data. |

#### 6.4.3.4	Evaluation

The solution described in clause 6.4.3.3.1 requires the analytics inputs as described in clause 6.4.3.3.2, wherein

-	the alarm data and S-NSSAI can be provided along with PMs and NRM data.

-	input PM for gNB and 5C are specified in TS 28.552 [8].

-	input E2E KPIs are specified in TS 28.554 [7].

-	input network topology, provision operations and configuration data can be reflected by the NRMs defined in TS 28.541 [20].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.4.3.3.3 can be derived.

Therefore, this solution is a feasible candidate for alarm malfunction analytics.

## 6.5	Mobility management related issues

### 6.5.1	Handover optimization

#### 6.5.1.1	Use case

Current handover procedures are mainly based on radio conditions for selecting the target gNB upon a handover. The target gNB accepts or rejects the handover (HO) request depending on various conditions. In virtualized environment, the HO may be rejected due to inadequate available resources within the target gNB. The notion of resources may include virtual resources (e.g., compute, memory) and/or radio resources (e.g., PRB, RRC connected users). If the HO request is rejected, a UE will try to connect to a different gNB until the request is successfully accepted. Several target gNBs can be tried until the request is successfully accepted. This process can result in wastage of UE and network resources, while it may also introduce service disruption due to increased latency and radio link failures (RLFs). It also introduces inefficiency in the HO or other network procedures.

To address this handover optimization issue, it is desirable to use MDAS (Management data analytic service) to provision and/or select a particular target gNB for handover in order to reduce or even avoid HO rejections. The MDAS producer provides a HO optimization analytics report containing the current and future/predicted resource consumption, resources capabilities and other KPIs' status for the available target gNB(s). The analytics report also provides recommended actions to optimize the target gNB for handover. This may include resource re-configuration or the updated selection criteria for target gNB. Based on the report, the MDAS consumer adjusts (e.g., scale-out/up the virtual resource, re-schedule/optimize radio resource) the resources before continuing with the handover and/or adjusts the selection criteria of the target gNB by also considering the overlapping coverages of inter-frequency and inter-RAT deployments.

#### 6.5.1.2	Potential requirements

REQ-HO_OPT_CON-1		The MDAS producer should have a capability to provide the analytics report describing the resource consumption to authorized consumers based on the current and future virtual resource consumption of gNB.

REQ-HO_OPT_CON-2		The MDAS producer should have a capability to provide the analytics report describing the resource consumption to authorized consumers based on the current and future radio resource consumption of gNB.

REQ-HO_OPT_CON-3		The analytics report describing the resource consumption should contain the following information describing the current and future resource consumption:

-	Assigned virtual, radio, and transport resources for target gNB.

-	Consumed virtual, radio, and transport resources for target gNB.

-	Projected virtual, radio and transport resource usage in near future for target gNB.

-	Indication on whether the target gNB is optimal for handover.

-	Recommended action to optimize the target gNB and/or the selection of the target gNB for handover.

REQ-HO_OPT_CON-4		The MDAS producer should have a capability to provide an analytics report indicating a selection priority for the target cell, among a set of candidate inter-frequency cells.

REQ-HO_OPT_CON-5		The MDAS producer should have a capability to provide an analytics report indicating a list of target cells to spare, i.e. avoid, a handover for an indicated time period.

REQ-HO_OPT_CON-6		The analytics report describing inter-frequency target cell selection for handover may provide information for provisioning or selecting a target gNB with respect to a specific service or slice, if the same Network Slice Instance (NSI) is available in both the current and target gNB.

REQ-HO_OPT_CON-7		The analytics report describing inter-frequency target cell selection for handover should provide indication of current and expected QoE (for the UE) at the current and target gNB.

#### 6.5.1.3	Possible solutions

##### 6.5.1.3.1		Solution description

The solution considers resource consumption both in terms of virtual and radio resource for the target gNB. The current resource consumption is analysed with the future/predicative resource consumption to decide if the target gNB is optimal for handover or not.

The MDAS producer can correlate and analyse the ongoing and/or potential handover optimization issues based on the current and historical performance data related to handover performance considering intra-gNB and inter-gNB handover measurements as well as other performance measurements including network load, E2E latency, retainability and radio conditions, UE measurements including MDT, location and QoE for the network or network slices. The MDAS producer can provide the analytics report as defined in clause 6.5.1.3.3 related with resource utilization analytics triggered by an event or periodically.

##### 6.5.1.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Performance Measurements | Average/distribution of UE reported RSRPs/RSRQs/SINRs of each neighbour cell; Packet delay related to neighbour cells as defined in clauses 5.1.1.1/5.1.3.3, TS 28.552 [8]; IP Latency to neighbour cells as defined in clause 5.1.3.4, TS 28.552 [8]; Round-trip GTP Data Packet Delay to neighbour cells as defined in clause 5.4.1.9, TS 28.552 [8]; End-to-end Latency of 5G Network to neighbour cells as defined in clause 6.3.1, TS 28.554 [7]; CQI related measurements: The distribution of Wideband CQI reported by UEs, clause 5.1.1.11 of TS 28.552 [8]; Intra-gNB handovers: Number of failed handovers in terms of handover preparation/resource allocation/execution and the mean time of handover execution, clause 5.1.1.6 of TS 28.552 [8]; Inter-gNB handovers: Number of failed handovers in terms of handover preparation/resource preparation clause 5.1.1.6.2, TS 28.552 [8]; Frequency Priority Information (i.e., based on deployment) set by the MNOs: Absolute priorities for different NR frequencies or inter-RAT frequencies, clause 5.2.4.1, TS 38.304 [21]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, clause 6.3.4/6.3.5 of TS 28.554 [7]; Data packet loss: Data volume of outgoing GTP data packets per QoS level on the N3 interface, from UPF to (R)AN and via versa clause 5.4.1.6 TS 28.552 [8] |
| Allocated Virtual Resource | Allocated Compute: This describes the number of vCPUs allocated to the virtual machine on which the gNB VNF is hosted. Allocated Memory: This describes the number of vMemory allocated to the virtual machine on which the gNB VNF is hosted. Allocated Storage: This describes the number of vStorage allocated to the virtual machine on which the gNB VNF is hosted. |
| Consumed Virtual Resource | Consumed Compute: This describes the number of total aggregated compute resource consumption at a particular point of time. Consumed Memory: This describes the number of total aggregated memory consumption at a particular point of time. Consumed Storage: This describes the number of total aggregated storage consumption at a particular point of time. |
| Consumed Radio Resource | Radio resource utilization: The physical radio resource utilization of the target gNB, see clause 5.1.1.2 of TS 28.552 [8]; |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR (serving cell and neighbour cells) and UE location information, TS 37.320 [12]. |
| UE location reports | UE location information provided by the LCS with the anonymous ID, which can be used to correlate with MDT reports. |
| QoE Data | Detailed measurements are FFS. |
| S-NSSAI | S-NSSAI as defined in clause 5.15.2, TS 23.501 [13]. MDAS uses this information to identify target gNBs or inter-RAT cells associated with a network slice performing handover optimization and may derive resource utilization and network performance analytics. |
| Configuration Data | Resource configuration data including RAN and virtualized NFs. The current policy configured in the RAN related to the handover optimization. |

##### 6.5.1.3.3		Analytics report on gNB resource consumption

The resource analytics report contains the following information per gNB.

| Analytics Report of gNB resource consumption | Attribute Name | Description |
| --- | --- | --- |
|  | Allocated Virtual Resource | Allocated Compute: This describes the number of vCPUs allocated to the virtual machine on which the gNB VNF is hosted. Allocated Memory: This describes the number of virtual vMemory allocated to the virtual machine on which the gNB VNF is hosted. Allocated Storage: This describes the number of vStorage allocated to the virtual machine on which the gNB VNF is hosted. |
|  | Consumed Virtual Resource | Consumed Compute: This describes the number of total aggregated compute resource consumption at a particular point of time. Consumed Memory: This describes the number of total aggregated memory consumption at a particular point of time. Consumed Storage: This describes the number of total aggregated storage consumption at a particular point of time. |
|  | Projected Virtual Resource consumption | Projected Compute: This describes the number of total projected compute resource consumption at a particular point of time. Projected Memory: This describes the number of total projected memory consumption at a particular point of time. Projected Storage: This describes the number of total projected storage consumption at a particular point of time. Timestamp: Time for which the projection is made. |
|  | Assigned radio resources | The physical radio resource assignment to the target gNB. |
|  | Consumed radio resource | The physical radio resource utilization of the target gNB. |
|  | Projected radio resource | The physical radio resource projected utilization of the target gNB. |
|  | isOptimal | Indication on whether the target gNB is optimal for handover. This will include: isOptimal: TRUE/FALSE indication if it is optimal. Network slice Identifier: Indication of the target slice (or same slice type) at the target gNB. |
|  | isFutureOptimal | Indication on whether the target gNB is optimal for handover at a future point of time (Timestamp). This will include: isFutureOptimal: TRUE/FALSE indication if it is optimal. TimeStamp: Indicating the timestamp at which the target gNB will be optimal Network slice Identifier: Indication of the target slice (or same slice type) at the target gNB. |
|  | Priority | Priority of the target gNB for optimal HO, in case of multiple targets. |
|  | Remedial Action | Recommendation for gNB modification in order to make it optimal for handover e.g., scale-out gNB, increase radio resource. |

#### 6.5.1.4 	Evaluation

The solution described in clause 6.5.1.3.1 requires the analytics inputs as described in clause 6.5.1.3.2 wherein

-	Input PMs on Packet delay, IP latency, Round-trip GTP data packet delay, CQI measurements, Intra-gNB handover, Inter-gNB handover, data packet loss, virtual resource usage and RAN utilization are specified in TS 28.552[8].

-	Input KPIs on E2E latency of 5G and throughput at N3 are specified in TS 28.554 [7].

-	Input MDT data is specified in TS 37.320 [12].

-	Frequency Priority Information is specified in TS 38.304 [21].

-	Input QoE can be available from service experience analytics from NWDAF TS 23.288 [18].

-	UE location reports can be accessed by consuming the LCS via service-based interfaces as defined in TS 23.273 [27].

-	Input Configuration data and S-NSSAI are available and have been defined in TS 28.541[20].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.5.1.3.3 can be derived.

Therefore, this solution is a feasible candidate for handover optimization.

### 6.5.2	Inter-gNB Beam Selection Optimization

#### 6.5.2.1	Use case

The handover procedure specified in 5G is triggered based on UE measurements, specifically considering the cell level radio quality of the source and target cell(s). In case of beamformed access, one cell can make use of several beams for serving residing users (SSB or CSI-RS) with each user served by a single beam at a time. The cell level quality can be represented as an aggregated metric over one or more beams. So, although handover is performed between two 5G cells, the granularity of handover can be further broken down to beam level.

The target cell provides RACH resources to the UE, which are linked with specific beams. Currently, the target cell can do this, based on beam level measurements performed and reported by the UE. Picking the wrong beam for performing RACH on the target cell could easily result in RLF for the UE and should be avoided.

To address this beam level handover optimization issue, it is desirable to use MDAS to prioritize and/or select a beam in case of handover for a specific target cell, in order to minimize or even avoid RLF.

The MDAS producer provides a beam level HO optimization analytics report considering information about the handover performance of different beam combinations between specific source and target cell pairs. In particular, the MDAS producer considers the beam of the current cell and the selected beam (out of several available beams) of the target cell to keep statistics regarding the handover performance in order to avoid RLF. Beams of the target cell with a successful handover are preferred in the selection.

In other words, the analytics report also provides recommended actions to optimize the beam selection at the target gNB as a function of serving beam on the source cell side. Based on the recommended actions, the MDAS consumer adjusts the priorities for the beam selection at HO, i.e. the beam combinations that are likely to succeed are prioritized, less optimal beam combinations are down prioritized. The analytics report may also be used to aid the target cell to allocate RACH resources in a way that ensures HO success.

#### 6.5.2.2	Potential requirements

REQ-HO_BEAM_OPT_CON-1 		The MDAS producer should have a capability to provide the analytics report describing the handover performance of beam combinations between cell pairs to authorized consumers based on previously recorded HOs.

REQ-HO_BEAM_OPT_CON-2 	The analytics report describing the performance of beam combinations between cell pairs should contain the following information:

-	Number of successful HOs between a given beam pair

-	Number of failed HOs between a given beam pair

-	Indication if a beam pair is to be prioritized or down prioritized

#### 6.5.2.3	Possible solutions

##### 6.5.2.3.1		Solution description

The MDAS producer can analyse the ongoing and/or potential beam handover optimization based on the current and historical beam selection success/failure rate for an inter-gNB handover. The MDAS producer can provide the analytics report as defined in clause 6.5.2.3.3 triggered by an event or periodically.

##### 6.5.2.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Performance Measurements | Inter-gNB handover: Number of success/failed handovers in terms of handover preparation/resource preparation; Beam level measurements: CSI-RS, SSB beam related measurements clause 5.1.1.28, TS 28.552 [8]; Success/failure handover rate per beam pair, i.e. serving beam Id in source gNB and selected beam Id in target gNB. |
| MDT Data | UE measurements related to radio conditions and UE location information. |
| Beam selection policy | The current policy configured in the RAN related to beam selection for inter-gNB handover optimization. |

##### 6.5.2.3.3		Analytics report

The gNB resource analytics report contains the following information.

| Analytics Report of gNB resource consumption | Attribute Name | Description |
| --- | --- | --- |
|  | Beam level Inter-gNB Handover performance | Handover success rate per beam ID pair. This can be quantified as high, medium or low success rate. |
|  | Time period | Time period, in the future, for the handover success rate per beam ID pair |
|  | List of gNBs | Objects involved: gNB(s) and cells of gNBs, |

#### 6.5.2.4 	Evaluation

The solution described in clause 6.5.2.3.1 requires the analytics inputs as described in clause 6.5.2.3.2, wherein

-	Input PMs on Inter-gNB handover and beam level measurements are specified in TS 28.552 [8].

-	Input MDT data is specified in TS 37.320 [12].

-	Input PM that need to be specified:

a.   success/failure handover rate per beam pair, i.e. serving beam Id in source gNB and selected beam Id in target gNB;

b.   current configuration related to beam selection for inter-gNB handover optimization.

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.5.2.3.3 can be derived.

Therefore, this solution is a feasible candidate for Inter-gNB Beam Selection Optimization provided that Input PM related to success/failure handover rate per beam pair and current configuration related to beam selection are specified.

### 6.5.3	Load Balancing optimization

#### 6.5.3.1	Use case

The rapid traffic growth and multiple frequency bands utilized in a commercial network make it challenging to steer the traffic in a balanced distribution. To address the problem, load balancing had been proposed. The objective of load sharing and load balancing is to distribute cell load evenly among cells or to transfer part of the traffic from congested cell, or to offload users from one cell or carrier or RAT to improve the network resource utilization efficiency and achieve network energy saving. This can be done by means of optimization of cell reselection/handover parameters and handover actions.

To ensure the service performance and user experience, the load balancing action based on handovers highly depends on the measurement report (MR) from the UE. For example, the inter-frequency scenarios with the deployment of multiple different frequency bands, the MR configuration and UE MR reports may cause amount of signalling overhead over Uu interface. The frequent inter-frequency measurement will cause huge UE power consumption and severely impact on running service by the data interruption for inter-frequency measurement gap, e.g. the gap time in LTE is number of frequency*60ms per 480ms period and the gap time in NR also depends on SMTC period. The gap assistant inter-frequency measurements mechanism will bring delay of the measurement and decrease the data transmission rate. Solutions are desired to improve the effectiveness of the MR configuration and report, which may help to greatly reduce the MR signalling overhead, UE power consumption and data interruption of running service, and improve the convergence speed of the load balancing.

The MDA can help to predict the measurement results of cell on neighboring frequencies for each UE without the GAP assisted measurement. Via analysing the historical intra-frequency and inter-frequency measurement from both the serving cell and the neighbour cell, the MDA can construct the network "radio finger print", which characterize the network intra-frequency and inter-frequency coverage quality. The "radio finger print" information is composed of multiple virtual grid. The grid index is to identify a specific virtual grid and this index consists of cell ID and corresponding coverage quality, e.g., RSRP, of at least three intra-frequency cells. The attributes of the grid are used to describe the wireless characteristics of the grid, such as coverage of inter-frequency neighbor cells, including RSRP, reference signal receiving quality (RSRQ), received signal strength indication (RSSI), channel quality indicator (CQI), modulation and coding scheme (MCS), beam ID, etc.

The MDAS producer provides the analytics report on "radio finger print" information to the gNB, gNB can directly predict the measurement values of cells on neighboring frequencies for each UE based on the well-constructed "radio finger print" and the real-time intra-frequency measurement. In this case, the GAP assisted inter-frequency measurement is avoided, and the gNB can make proper load balancing actions based on the predictions, which helps to reduce the data interruption of running services and improve the load balancing speed. Moreover, the MDA producer can provide authorised consumers with radio configuration options to perform MLB based handover according to the Service Experience Type, e.g. on voice, video, other, associated with the offloaded UE from one cell to another to ensure the desired service experience.

The MDAS producer may also provide the traffic load prediction report to the authorized consumers, e.g., gNB, to enable the proactive load balancing actions. This would help to prevent the user experience degradation in advance compared to the reactive optimizations based on the delayed load information measurement and exchange.

The Centralized SON can have a bigger picture of the Network Congestion, by looking at a Cluster of cells. This can complement the load balancing at Distributed SON and make it possible to completely mitigate the risks of service degradations due to network congestions. Moreover, predictive analytics can make it possible to take anticipatory steps towards mitigating network congestions. The MDAS producer can predict the cells that will be congested in a near future, in a cluster of NR Cells, by performing data analytics on Cell Level Performance Measurements: DL PRB

Utilization, Latency related Performance Measurements, Average RRC Connected Users, DL Packet Drop Rate, DL PDCP SDU Drop Rate, Average Delay in DL in CU-UP, Average Delay on F1-U, IP Latency. Network Congestions can be predicted at Cell level as well as group of neighbouring cells. Based on this, a list of congested cells can be provided to the MDAS Consumer. Also, a list of cells for which it is predicted that the congestion will ease out, can be provided to the MDAS Consumer.

#### 6.5.3.2	Potential requirements

REQ-MLB_OPT_CON-1		The MDAS producer should have a capability to provide the analytics report describing the radio measurement information to authorized consumers, e.g., gNB.

REQ-MLB_OPT_CON-2		The analytics report describing the radio measurement information should contain the following information:

- 	the applied cell ID;

- 	the time period(s) of the original data used for deriving the analytics report;

- 	the serving cell and its inter-frequency/intra-frequency neighboring cell's cell ID and corresponding radio measurement information, e.g., CSI-RSRP, SS-RSRP, etc;

- 	Indication on whether the gNB is suitable to be selected as the target gNB for the MLB based handover based on the radio signal qualities.

REQ-MLB_OPT_CON-3		The analytics report describing the predicted resource utilization status of gNB should contain the following information:

-	predicted virtual, radio, and transport resources utilizations for potential MLB source and target gNBs in the near future;

-	Indication on whether the gNB is needed to activate the MLB operation;

-	Indication on whether the gNB is suitable to be selected as the target gNB for the MLB based handover.

REQ-MLB_OPT_CON-4 		The MDAS producer should have the capability to provide authorized consumers, e.g. gNB, with the analytics report describing the service specific radio configuration options needed to perform MLB based handover.

The solution considers resource consumption both in terms of virtual and radio resource for the target gNB. The current resource consumption is analysed with the future/predicative resource consumption to decide if the target gNB is optimal for handover or not.

#### 6.5.3.3	Possible solutions

##### 6.5.3.3.1		Solution description

The solution considers resource consumption both in terms of virtual and radio resource for the target gNB. The current resource consumption is analysed with the future/predicative resource consumption to decide if the target gNB is optimal for handover or not.

For this solution, MDAS Consumer shall be C-SON. The MDAS producer can collect Performance Measurement data as described in Clause 6.5.3.3.2. Performance Measurement data shall be captured for a cluster of NR cells in Operator’s network. The MDAS Consumer can provide a mechanism for selecting a cluster of cells, depending upon geographic region, or, based on the network density. The periodicity of the Performance Measurement collections and the storage duration can be configurable. The prediction is required for a time period in advance, defined as Prediction Horizon, which can be a configurable parameter. The MDAS Producer can collect historic Performance Measurements in order to arrive at the congestion predictions.

Prediction shall be made well in advance, in order to compensate for possible latencies in collecting Performance Measurements as well as latencies in applying the mitigation actions to the network. The MDAS consumer can prevent Network congestion by applying cell reselection/handover parameter changes in advance, based on the Prediction Report from the MDAS producer.

##### 6.5.3.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Virtual Resource Measurements | For every Cell in the selected Cluster, Common performance measurements for NFs including: Virtualised resource usage, Virtual memory usage, Virtual disk usage, clause 5.7, TS 28.552 [8] |
| Resource Utilization Measurements | Radio resource utilization: The physical radio resource utilization of the source and target gNB, see clause 5.1.1.2 of TS 28.552 [8]; For every Cell in the selected Cluster, DL Total PRB Usage, clause 5.1.1.2.1, TS 28.552 [8] |
| Average RRC Connection Measurements | For every Cell in the selected Cluster, Mean number of RRC Connections, clause 5.1.1.4.1, TS 28.552 [8] |
| Packet Drop Measurements | For every Cell in the selected Cluster, DL Packet Drop Rate in gNB-DU, clause 5.1.3.2.2, TS 28.552 [8] For every Cell in the selected Cluster, DL PDCP SDU Drop rate in gNB-CU-UP, clause 5.1.3.2.1, TS 28.552 [8] |
| Delay Measurements | For every Cell in the selected Cluster, Average delay DL in CU-UP, clause 5.1.3.3.1, TS 28.552 [8] For every Cell in the selected Cluster, Average delay on F1-U, clause 5.1.3.3.2, TS 28.552 [8] |
| MDT data | UE measurements related to RSRP, RSRQ, SINR as specified in TS 37.320 [12] |
| Service experience data analytics | Service experience statistics and service in use predictions provided by NWDAF per UE (source/target cell) |
| NR Cell ID | Unique NR Cell ID For every Cell in the selected Cluster |
| Timestamp | Timestamp for each Performance Measurement reported |

##### 6.5.3.3.3		Analytics report

The gNB resource analytics report contains the following information for the source and target gNBs.

| Analytics Report of gNB resource consumption | Attribute Name | Description |
| --- | --- | --- |
|  | Predicted Virtual Resource consumption | Predicted Compute: This describes the average predicted compute resource consumption. Predicted Memory: This describes the average predicted memory consumption. Predicted Storage: This describes the average predicted storage consumption. Timestamp: Time for which the prediction is made. |
|  | Predicted radio resource | The physical radio resource predicted utilization of the target gNB. |
|  | Prediction Horizon | Prediction is performed for a time window in advance. |
|  | isMLBNeeded | Indication on whether the target gNB needs to activate the MLB operation. |
|  | List of Predicted Congested Cells | List of cells or list of cells in the selected cluster, for which it is predicted that those will be congested at the Prediction Horizon. |
|  | List of possible target cells | Indication on whether the target gNB is suitable to be selected as the target gNB for the MLB based handover. List of cells in the selected cluster, that are suitable for MLB based handover. |
|  | List of Predicted Congestion Easing out Cells | List of cells or list of cells in the selected cluster, for which it is predicted that the congestion shall ease out at the Prediction Horizon. |
|  | Prediction Confidence | The Prediction Confidence shall define the measured prediction accuracy. |
|  | Service list | List of supported service classes or corresponding slice IDs at the List of possible target cells. |

#### 6.5.3.4	Evaluation

The solution described in clause 6.5.3.3.1 requires the analytics inputs as described in clause 6.5.3.3.2, wherein:

-	Input PM data: Virtual Resource measurements, RAN utilization, Average RRC Connection, Packet Drop Measurements and Delay Measurements are specified in TS 28.552 [8].

-	MDT data is specified in TS 37.320 [12].

-	Service experience analytics is specified in TS 23.288 [18].

-	NR Cell ID and timestamp are available.

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.5.3.3.3 can be derived.

Therefore, this solution is a feasible candidate for load balancing optimization.

### 6.5.4	Mobility performance analysis

#### 6.5.4.1	Use case

The mobility performance related problems may be resulted from different causes, e.g., too long mobility interruption time for latency sensitive services, low handover successful rate due to poor coverage of the cell-edge, handover failure due to lack of handover resources, too-early/too-late/ping-pong handovers due to inappropriate handover parameters.

In addition, there are different handover mechanisms, e.g. Dual Active Protocol Stack (DAPS) to reduce service interruption time during handover, Conditional Handover (CHO) to improve handover robustness, RACH-less handover to reduce handover latency etc. SON MRO solutions can handle multiple handover robustness issues such as too early handover, too late handover, handover to a wrong cell etc. Furthermore, handover mechanisms are also related with NSA and SA deployment architecture. In different scenarios, handover solutions will have different impacts on the mobility performance. The analytics report to identify the most optimal handover mechanism may be provided by MDAS producer. For example, to satisfy the requirements of 0ms mobility interruption time for some URLLC services, MDAS may propose to prioritize the usage of DAPS. If handover successful rate is low due to coverage issue in the cell edge, MDAS may recommend CHO instead of gNB triggered handover mechanisms. To provide optimal handover mechanisms and the corresponding handover related parameters, MDAS may consider multiple factors, e.g. radio conditions, cell load, service requirements, handover successful rate, history performance data around handover, UE RF finger prints etc. MDAS may compare performance of different handover mechanisms and propose optimal handover mechanisms, e.g. the prioritization of different handover mechanisms in different conditions.

MDAS can be used to analyse service experience and network performance during handover period in different mobility scenarios. It may also be able to provide the recommendations of optimal handover parameters, resource configurations and mobility related policies. Mobility performance analysis should cover the following aspects:

-	Mobility scenarios in NSA and SA deployment architectures;

-	Optimal handover mechanisms, e.g. DAPS, CHO, RACH-less handover etc;

-	Optimal handover parameter and resource configurations;

-	Coordination with SON MRO mechanisms to improve handover robustness;

-	Mechanisms for fast handover failure recovery.

The MDAS producer is able to, from the perspective of the management aspects, provide the mobility performance analytics report. This analytics report can be considered as an input to support SLS assurance to perform further evaluation.

#### 6.5.4.2	Potential requirements

REQ-MOB_PMF_CON-1		The MDAS producer should have a capability to provide the analytics report of mobility performance.

REQ-MOB_PMF_CON-2		The analytics report describing the mobility performance should contain the following information describing the mobility related performance aspects:

-	Optimal handover mechanism and the corresponding parameters for DAPS, CHO, RACH-less handover and NR SON MRO scenarios etc.

#### 6.5.4.3	Possible solutions

##### 6.5.4.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to provide optimal handover mechanisms and the corresponding configurations regarding parameters and resources. As the table in 6.5.4.3.3 shows, the analytics report is able to be provided by the MDAS producer to describe the optimal handover mechanisms. This procedure may be triggered by the request or periodically.

##### 6.5.4.3.2	Data required for mobility performance analysis

The following table shows the potential data required to analyse the mobility performance.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Inter-gNB handovers: see clause 5.1.1.6.1 of TS 28.552 [8]; Intra-gNB handovers: see clause 5.1.1.6.2 of TS 28.552 [8]; Handovers between 5GS and EPS: see clause 5.1.1.6.3 of TS 28.552 [8]; RRC Connection Re-establishment: see clause 5.1.1.17 of TS 28.552 [8]; Inter-AMF handovers: see clause 5.2.5.1 of TS 28.552 [8]; Handovers from 5GS to EPS: see clause 5.2.5.3 of TS 28.552 [8]; Handovers from EPS to 5GS: see clause 5.2.5.4 of TS 28.552 [8]; Number of handover events, Number of HO failures, Number of too early HO failures, Number of too late HO failures, Number of HO failures to wrong cell, Number of unnecessary HOs to another RAT: see clause 4.3.5 of TS 28.628 [34]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; NWDAF analytics data: UE mobility analytics, UE Communication Analytics; see clauses 6.7.2 and 6.7.3 of TS 23.288 [18]; Measurements related to DAPS and CHO. |
| Identifier | sNSSAIList that are analyzed. |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR and UE location information. |
| QoE Data | The QoE data as defined in TS 26.247 [29] and TS 26.114 [30]. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs related with mobility. |

Note: 	The above parameters may not be the complete list.

##### 6.5.4.3.3	Analytics report for mobility performance analysis

The following table shows the potential information carried in the analytics report of mobility performance analysis.

| Analytics Report of mobility performance | Attribute Name | Description |
| --- | --- | --- |
|  | Mobility performance issue identifier | The identifier of the mobility performance issue analysis; |
|  | Root cause of mobility performance issue | The root cause of mobility performance issues, e.g., too long mobility interruption time for latency sensitive services, low handover successful rate due to poor coverage of the cell-edge, too-early/too-late/ping-pong handovers due to inappropriate handover parameters |
|  | Recommended handover mechanisms | Recommended handover mechanisms according to network conditions, e.g., DAPS, CHO, RACH-less handover; See note. |
|  | Recommended handover related parameters | Corresponding configurations of handover related parameters, e.g., the range of handover offset. |
|  | Time duration | The time duration the identified handover mechanism or handover related parameters are recommended to apply. |
|  | Location | The geographical area or the cells where the identified handover mechanism and handover related parameters are applied. |
| Note: The DAPS and CHO mechanism are mutually exclusive. |  |  |

#### 6.5.4.4	Evaluation

The solution described in clause 6.5.4.3 requires the analytics inputs as described in in clause 6.5.4.3.2, wherein

-	the performance data (measurements and KPIs) are either already available in TS 28.552 [8] and TS 28.554 [7] or can be defined in TS 28.552 [8].

-	the MDT data are available in TS 32.422 [25].

-	QoE data as defined in TS 26.247 [29] and TS 26.114 [30] can be acquired through the procedures defined in TS 28.405 [31].

-	Configuration Data are defined in TS 28.541 [20].

With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.5.4.3.3 can be derived.

Therefore, this solution is a feasible candidate for mobility performance analysis.

### 6.5.5	Handover optimization based on UE trajectory

#### 6.5.5.1	Use case

Handover optimization can benefit from knowledge about the trajectory on which the user may be moving. A trajectory here is a sequence of location coordinates, i.e. a vector that captures the sequences of coordinates within a certain time interval, derived from historical UE location data, that identifies the different directions which users may take starting at a given point. At a city junction, for example, the probability of a handover success for a user moving straight through the junction may be different from that for a user who is turning left or turning right. So, at that junction, the straight, left and right direction indicate three possible trajectories. The MDAS producer should be able to analyse historical handover performance data in combination with historical UE location and the radio characteristics like RSRP and SINR considering the possible user trajectories to identify the optimal handover configurations and target cell prioritization.

#### 6.5.5.2	Potential requirements

REQ-HO_OPT_TR_CON-1 		The MDAS producer should have a capability to provide an analytics report indicating the possible candidate user trajectories across the cell boundaries and provide radio configuration and target gNB selection specific to each trajectory.

#### 6.5.5.3	Possible solutions

##### 6.5.5.3.1		Solution description

The solution considers UE trajectories and radio conditions for selecting the target gNB. The current resource consumption is analysed with the future/predicative resource consumption to decide if the target gNB is optimal for handover or not.

##### 6.5.5.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Consumed Radio Resource | Radio resource utilization: The physical radio resource utilization of the target gNB, see clause 5.1.1.2 of TS 28.552 [8]; |
| MDT data | UE measurements related to RSRP, RSRQ, SINR time-stamped and annotated with UE location information. |

##### 6.5.5.3.3		Analytics report on user trajectory-based handover optimization

The gNB trajectory-based handover optimization analytics report contains the following information.

| Analytics Report of user trajectory-based handover | Attribute Name | Description |
| --- | --- | --- |
|  | User trajectory | The predicted UE location that identifies a trajectory across a cell boundary. |
|  | Radio characteristics on user trajectory | Radio parameters e.g. RSRP, SINR, on the predicted user trajectory across a cell boundary. |
|  | Recommended actions | Recommendation for optimal gNB configuration and/or target gNB selection/prioritization based on the user trajectory information. |

#### 6.5.5.4 	Evaluation

The solution described in clause 6.5.5.3.1 requires the analytics inputs as described in clause 6.5.5.3.2, wherein

-	Input PM on RAN utilization is specified in TS 28.552 [8].

-	Input MDT data is specified in TS 37.320 [12].

-	To specify MDAS report.

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.5.5.3.3 can be derived.

Therefore, this solution is a feasible candidate for Handover optimization based on UE trajectory.

### 6.5.6	Handover optimization based on UE Load

#### 6.5.6.1	Use case

The target node, eNB, may not have adequate resources to accept certain handover requests. In the context of network virtualization, these resources may include not only legacy radio resources, but also virtual resources such as processor and memory. Handover optimization can benefit from knowledge about the projected UE load on the target cell including additional radio and virtual resources.

#### 6.5.6.2	Potential requirements

REQ-HO_OPT_TR_CON-1	The MDAS producer should have a capability to provide an analytics report indicating the projected UE load with respect to virtual resource and radio resource on the target cell.

#### 6.5.6.3	Possible solutions

##### 6.5.6.3.1		Solution description

The MDAS producer will periodically collect UE Load statistics from gNBs. Data collected over a period of time is analysed to predict aggregated UE resource requirements on the target gNB. MDAS can utilize historical data and AI/ML (for example, time series based) algorithm to predict UE requirements of the gNB. The target cell, on receiving, a handover request will request UE load analytics report (ULAR) for the migrating UE. The ULAR will provide predictions for the additional radio and virtual resource requirements for the UE on the target cell in near future. The target cell will use the ULAR to decide whether to accept or reject the UE handover.

##### 6.5.6.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| UE Load Statistics | UE Identifier: Anonymous id e.g. C-RNTI |
|  | Cell ID: Cell ID represents the cell id of the cells to which the UE (identified by anonymous id e.g. C-RNTI) has been previously connected. |
|  | DL load: DL loads are the loads that the UE imposed on previously connected cells. These loads are represented by the number of assigned DL PRBs, DL traffic volume and the amount of virtual resource consumed within the collection period. |
|  | UL load: UL loads are the loads that the UE imposed on previously connected cells. These loads are represented by the number of assigned UL PRBs, UL traffic volume and the amount of virtual resource consumed within the collection period. |
|  | Load on virtual processors: Virtual processor usage for the UE |
|  | Load on memory: Virtual memory usage for the UE |
|  | Load on storage: Virtual storage usage for the UE |
|  | Record time: Recording time is the time instance when the statistics were gathered |
|  | Collection Period: elapsed time for which the statistics are collected, e.g. last 10000 seconds. |

Editor’s Note: The availability mechanisms of the Input data defined above is FFS.

##### 6.5.6.3.3		Analytics report on UE Load

The UE load analytics report contains the following information.

| UE load analytics report | Attribute Name | Description |
| --- | --- | --- |
|  | Projected UE virtual resource requirements | Projected requirement for virtual processor usage for the UE. Projected requirement for virtual memory usage for the UE. Projected requirement for virtual storage usage for the UE. |
|  | Projected UE radio resource requirements | Projected number of required DL and UL PRBs, DL and UL traffic volumes within the collection period. |
|  | Projection Time | The future timestamp to which the projections applies. |

#### 6.5.6.4	Evaluation

The solution described in clause 6.5.6.3 requires the analytics inputs as described in in clause 6.5.6.3.2, wherein

-	The UE Identifier and the Cell ID can be accessed from MDT reports.

-	The required UE radio and virtual resource load can be/should be defined as part of MDT data.

With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.5.6.3.3 can be derived.

Therefore, this solution is a feasible candidate for HO optimization based on UE Load.

## 6.6	Energy efficiency related issues

### 6.6.1	MDA assisted energy saving

#### 6.6.1.1	Use case

Energy saving is a critical issue for the 5G operators. Energy saving is achieved by activating the energy saving mode of the NR capacity booster cell or 5GC NF (e.g. UPF etc.), and the energy saving activation decision making may be based on the various information such as load information of the related cells/UPFs, the energy saving policies set by operators as specified in TS 28.310 [14] and the energy saving instructions provided by MDAS producer as described in TR 28.813 [23].

As the conclusion from clause 7.2 of the TR 21.866 [15], "The EE Control and Coordination Function: a self-managed automated process to control and coordinate system wide power saving operations including the access networks, core network, backhaul/fronthaul transmission networks, backbone networks and other subsystems", the management system has the overall view of network load information and it could also take the inputs from the control plane analysis (e.g. the analytics provided by NWDAF). The management system may provide the network wide analytics and cooperate with Core and RAN domains and decide on which cell/UPF should move into energy saving mode in a coordinated manner.

There are various performance measurements could be used as inputs by MDA for energy saving analysis, for example, the EE related performance measurements, (e.g. PDCP data volume of cells, PNF temperature, and PNF power consumption etc.) for the gNBs, and the data volume, number of PDU sessions with SSC mode 1 (see TS 23.501 [13]), delay related measurements, and VR usage for UPFs, and the traffic load variation related performance measurements, (e.g. the PRB utilization rate, RRC connection number).

The composition of the traffic load could be also considered as inputs for energy saving analysis. (e.g., the percentage of high-value traffic in the traffic load). The variation of traffic load may be related to the network data (e.g., historical handover information of the UEs or network congestion status, packet delay). Collecting and analysing the network data with machine learning tools may provide predictions related to the trends of traffic load. The composition and the trend of the traffic load may be used as references for making decision on energy saving.

There are many prediction data models which may use machine learning tools for predicting the energy saving related information, such as traffic load. MDAS may also take these prediction data models as input, make analysis and select the optimal prediction data models to provide more accurate prediction results as references for making energy saving decision. The more accurate the prediction results are, the better the energy-saving decision based on the prediction results will be. The prediction data models are related to services (e.g., traffic load, resource utilization, service experience), which can be provided by consumer.

MDAS may also obtain NF location or other inventory information such as energy efficiency and the energy cost of the data centers, while analysing historical network information. Based on the collected information, MDAS producer makes analysis and gives suggestions to network management in optimization suggestion for 5G Core NF deployment options in high-value traffic region (e.g. location of VNF in context of energy saving). The information from control plane data analysis from NWDAF, such as UE Communication analytics (see TS 23.288 [18]), may also be used as input for energy saving analysis and instruction.

The decision of core NF and RAN node energy saving should be coordinated by management system to guarantee the overall network and service performance are not affected as much as possible.

To achieve an optimized balance between the energy consumed and the performance provided by the network, MDAS can be used to provide an analytics report by analysing the above information comprehensively to assist the energy saving.

#### 6.6.1.2	Potential requirements

REQ-MDA_ES-CON-1	The MDAS producer should have a capability to provide the analytics report describing the energy saving instruction.

REQ-MDA_ES-CON-2	The analytics report describing the energy saving instruction should contain the following information:

-	The identifier of the energy saving instruction described in the analytics report;

-	Location of geographical area where the unreasonable energy consumption exists or optimization recommended;

-	Root cause of the energy consumption issue;

-	Recommended optimal prediction data models to assist the energy saving related decision making;

-	Recommended NR Cells (ES-Cell) to enter energySaving state;

-  	Recommended candidate cells with precedence for taking over the traffic of each ES-Cell.

-	Recommended UPFs (ES-UPF) to enter energySaving state;

- 	Recommended candidate UPFs with precedence for taking over the traffic of each ES-UPF.

-	Predictions related to the trends of traffic load which could be used as references for making decision on energy saving.

-	Recommended 5GC NF deployment options in high-value traffic region (e.g. location of VNF in context of energy saving).

REQ-MDA_ES-CON-3	The MDAS producer should have a capability to update the recommended decisions and fine-tune the accuracy of the analytics result after being informed of the service experience changes and traffic load changes since the recommended decisions have been are taken to make energy saving decisions.

#### 6.6.1.3	Possible solutions

##### 6.6.1.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to assist the management energy saving function to make energy efficiency decisions. As the table in clause 6.6.1.3.3 shows, the analytics report is able to be provided by the MDAS producer to describe the analytics result and recommendations of energy saving. This procedure may be triggered by the request or periodically.

Energy saving activation decision making may be based on the various information such as load information. The prediction result of these information can be used by operators to make energy-saving policies. There are many prediction data models which may use ML algorithms for predicting these information, such as energy-saving scenarios prediction data models and traffic load prediction data models.

The prediction data models are trained to be able to produce the expected training output from the training input. The data for models training may include the traffic load variation (e.g., RRC connection number, PRB utilization etc.), the network data related to the traffic load variations (e.g., UE throughput, rate of successful handover executions, air-interface delay, CQI related measurements etc.), energy consumption, service experience, etc. After the training process, the prediction data models for predicting information used to make energy-saving policies can be obtained.

The more accurate the information prediction results are, the better the energy-saving policies based on the information prediction results will be. According to the result of information prediction and the energy-saving benefit, the MDA service can assist the energy saving policy decision making by recommending the optimal  prediction data models which can provide more accurate information prediction results for consumers.

The MDAS producer is informed when the recommended decisions are taken by the MDAS consumer to make energy saving decisions, so that the MDAS producer can get the service experience changes and traffic load changes to evaluate energy-saving benefits.  By maximizing the expected sum of energy-saving benefits, the MDAS producer may update recommended decisions and fine-tune the accuracy of the analytics result.

For example, the MDAS producer may take base station energy saving scenarios prediction data model and the traffic load prediction data model together as input. Based on the results of energy saving scenarios prediction and traffic load prediction, the MDAS producer may use the ML algorithms to calculate the energy-saving benefit based on base station related information (e.g., service experience changes), the traffic load changes as well as the prediction of the energy saving scenarios. And by maximizing the expected sum of energy-saving benefits, the MDAS producer may provide the optimal prediction data models for making recommendation of base station energy saving policies accordingly.

##### 6.6.1.3.2	Data required for MDA assisted energy saving

The following table shows the potential data required to analyse the energy saving issue.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | PNF Power Consumption: Power consumed over the measurement period, see clause 5.1.1.19.2 of TS 28.552 [8]; PNF Energy consumption: The energy consumed, see clause 5.1.1.19.3 of TS 28.552 [8]; PNF Energy Temperature: The temperature over the measurement period, see clause 5.1.1.19.4 of TS 28.552 [8]; PNF Voltage: The voltage, see clause 5.1.1.19.5 of TS 28.552 [8]; PNF Current: The current, see clause 5.1.1.19.6 of TS 28.552 [8]; PNF humidity consumption: The percentage of humidity during the measurement period, see clause 5.1.1.19.7 of TS 28.552 [8]; PDCP Data Volume of NR cells: The transmitted PDCP data volume, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [8]; Virtual resource usage of NF: The resource usage of virtual network functions, see clause 5.7.1 of TS 28.552 [8]. Data volume of UPF: see clause 5.4 of TS 28.552 [8]. Delay related measurements of UPF: see clause 5.4 of TS 28.552 [8]. Number of PDU sessions with SSC mode 1 on UPF. Traffic load variation: PRB utilization rate, see clause 5.1.1.2 of TS 28.552[2] RRC connection number, see clause 5.1.1.4 of TS 28.552[8] etc Network data: UE throughput, see clause 5.1.1.3 of TS 28.552[2] the mobility management data (e.g. rate of successful handover executions), see clause 5.1.1.6 of TS 28.552[2] packet delay data (e.g., air-interface delay), see clause 5.1.1.1 of TS 28.552[2] etc. |
| QoE Data | The measurements that are collected are DASH (see TS 26.247 [16]) and MTSI (see TS 26.114 [17]) measurements (see TS 28.406 [24]). |
| NRM | MOIs of the cells, UPFs and SMFs, see TS 28.541 [20]. |
| Alarm information | The alarm information of the cells, UPFs and SMFs. |
| Network analytics data | The control plane analysis result from the NWDAF defined in TS 23.288 [18], e.g., observed service experience related network data analytics. |
| Prediction Data Models | The prediction data models, which may be based on ML algorithms and trained to be able to produce the expected training output for consumers, e.g., base station energy-saving scenarios prediction data models, traffic load prediction data models. The prediction data models, including the model related information (e.g., including Model Id), are related to service (e.g., traffic load, resource utilization, service experience), which can be provided by consumer. NOTE: The model related information may include but not only Model Id. Maybe more work on the model related information is needed. |

##### 6.6.1.3.3	Analytics report for MDA assist energy saving

The following table shows the potential information carried in the analytics report of MDA assist energy saving.

| \Analytics Report of MDA assisted energy saving | Attribute Name | Description |
| --- | --- | --- |
|  | Energy saving analytics identifier | The identifier of the MDA assisted energy saving |
|  | Location | The geographical area, UPFs or the cells where the unreasonable energy consumption exists or recommended 5GC NF deployment options or NR cells in high-value traffic geographical area. |
|  | Root cause | The root cause of the part of the energy consumption that may be conserved, e.g., ultra-low traffic load area with energy consumption, excessive energy consumption |
|  | Trends of the traffic load | Predictions related to the trends of traffic load(e.g.,PRB utilization rate, RRC connection number, etc.)which could be used as references for making decision on energy saving |
|  | Recommended prediction data models | The optimal prediction data models which can be selected from the prediction data models (e.g. including Model Id) to provide more accurate information prediction results to assist the energy saving related decision making. |
|  | Current Energy Saving State of the cells | The information of the current energy saving state of the cells, e.g. the cells which are currently in the energy saving state. |
|  | Energy saving recommendations | For ES on NR cells. It may contain a set of - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - a time period, during which energy saving is or not allowed. - predictions related to the trends of traffic load which could be used as references for making decision on energy saving of NR cells. For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. - a time period, during which energy saving is or not allowed. - predictions related to the trends of traffic load which could be used as references for making decision on energy saving of UPFs. |

#### 6.6.1.4	Evaluation

The solution described in clause 6.6.1.3 requires the analytics inputs as described in in clause 6.6.1.3.2, wherein

-	the performance measurements on number of PDU sessions with SSC mode 1 on UPF can be defined in TS 28.552 [8], while other measurements are already available in TS 28.552 [8].

-    Traffic load variation and network data are all available in TS 28.552 [8].

-	QoE data are available in TS 28.406 [24].

-	NRMs are available in TS 28.541 [20].

-	the inventory information can be defined in NRMs.

-	the alarm information is supported by fault supervision MnS (see TS 28.545 [22] and TS 28.541 [20]).

-	the network analytics data can be accessed from NWDAF according to TS 23.288 [18].

-	the prediction data models are a kind of data provided by consumer, which include the model related information to be used for identifying the existed ML model. The identifier of existed ML models can be defined in ML model training for MDA.

With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.6.1.3.3 can be derived.

## 6.7	Software management related issues

### 6.7.1		RAN Node Software Upgrade

#### 6.7.1.1		Use case

As per the current mechanism of software upgrade at RAN node results in service disruption or huge operational cost. Consider a scenario, when a RAN Node is required to shut down manually to undergo critical maintenance for a very short duration of time. Software upgrade can be one such critical maintenance scenario. In such cases, all the resources (bearer, security functions, mobility management) that are managed by this RAN Node need to be purged and reconfigured at another RAN Node (standby RAN Node) or if another RAN Node is not available then resources will be reconfigured again when former RAN Node comes up after software upgrade. Both the situations lead to additional operational expenses and data loss. Operational expense in terms of all the resources to be released/attached again and data loss for all GBR sessions/bearer.

It is expected to use MDAS to optimize the procedure of software upgrade at RAN Node. The software upgrade should be automatically initiated by the OAM system, once configured, at the time when the expected impacts are minimum i.e. at the Optimal Time when there would be minimum expected operational cost and data loss. The Optimal Time (current or futuristic) can be derived by collecting and analysing the data related to DRBs including GBR/non-GBR, state, modification count, ongoing handover etc. MDAS can utilize historical data and AI/ML (e.g., time series based) algorithm to derive the future optimal time for software upgrade.

Note: 	RAN Node above refers to CU-CP in case of gNB split case.

#### 6.7.1.2		Potential Requirements

REQ-SW_UPG_CON-1: The MDAS producer should have a capability allowing the authorized consumer to get the DRB info analytics report describing the DRBs info at a particular RAN Node(s).

REQ-SW_UPG_CON-2: The MDAS producer should have a capability to provide the DRB info analytics report describing the DRB info based on DRB characteristics including GBR/non-GBR, state, modification count, handover etc.

REQ-SW_UPG_CON-3: The DRB info analytics report describing the DRB info should contain the following information:

-	Timestamp: Time at which the report is generated

-	CurrentUpgradeOptimal: Whether RAN Node is optimal for upgrade at present

-	DRB status: Total number of GBR and non-GBR DRBs at present

-	FutureUpgradeOptimal: Whether RAN Node will be optimal for upgrade at a future point of time. This will also provide a future timestamp.

-	DRB status: Total number of GBR and non-GBR DRBs at future point of time. This will also provide a future timestamp.

#### 6.7.1.3		Possible Solutions

##### 6.7.1.3.1	Solution description

The solution requires MDAS producer to collect current DRB info as defined in clause 6.7.1.3.2 and produce the report, as defined in clause 6.7.1.3.3, providing the optimal time for software upgrade i.e. the time at which the impact is minimal.

##### 6.7.1.3.2	Data required

The following table shows the data required to generate the DRB info analytics report. The data is collected per DRB from gNB.

| Data Category | Required Data |
| --- | --- |
| Bearer Statistics | QCI: Indicates resource bearer type (GBR, non-GBR). |
|  | Radio bearer State: Radio bearer State (Idle, Active) The state indicates the status of bearer connection. The state will be used to deduce total number of idle/active connection. Based on the number of idle/active connections probable time for SW Upgrade can be decided. |
|  | Bearer Modification Count: This count indicates number of times, this bearer has gone for modification since its creation. This count will be used to detect how frequent this session has undergone bearer modification. With history database, if this session is vulnerable to many bearer modifications, the upgrade can be differed. |
|  | Handover In Progress: This flag indicates whether the bearer is undergoing handover or not. This flag will help to deduce number of sessions which are undergoing handover. MDAS may choose to defer SW upgrade based on number of sessions which are undergoing handover. The best practise is to go for SW upgrade, when active handovers are minimum. |
|  | GTP Error Indication: This flag indicates GTP Path has gone to error state and system is waiting for recovery. This flag will help to deduce number of sessions which are recovering due to error in GTP Path. MDAS may choose to defer SW upgrade based on number of sessions which are in error state. The best practise is to go for SW upgrade, when number of sessions suffering from this error are minimal. |
|  | Timestamp: This parameter indicates timestamp during which this information has been collected |

##### 6.7.1.3.3	Analytics report

The DRB info analytics report contains the following

| DRB info Analytics Report | Attribute Name | Description |
| --- | --- | --- |
|  | Timestamp | Time at which the report is generated |
|  | CurrentUpgradeOptimal | Boolean attribute indicating whether RAN Node can be upgrade at present. |
|  | No. of GBR DRB | Total number of GBR bearer |
|  | No. of Non-GBR DRB | Total number of non-GBR bearer |
|  | FutureUpgradeOptimal | Boolean attribute indicating whether RAN Node can be upgrade in future. This will provide the time duration in future. |
|  | No. of GBR DRB | Total number of GBR bearer |
|  | No. of Non-GBR DRB | Total number of non-GBR bearer |

#### 6.7.1.4		Evaluation

The solution described in clause 6.7.1.3 requires the analytics inputs as described in in clause 6.7.1.3.2. All the required bearer related inputs can be defined as part of DRB related measurements in TS 28.552 [8].

With these analytics inputs which can be defined in the normative work, the analytics output as described in 6.7.1.3.3 can be derived.

Therefore, this solution is a feasible candidate for RAN node software upgrade.

## 6.8	MDA assisted SON coordination

### 6.8.1	SON conflict prevention and resolution

#### 6.8.1.1	Use case

Some SON functions, such as the MRO function and the MLB function, may modify the same parameters of an NR cell and potentially cause conflict. For instance, the MRO function may need to modify the HO parameters causing a handover to occur later (i.e., when the signal strength of the neighbour cell become stronger). However, the MLB function may need to modify the same HO parameters causing the HO to occur sooner, in order to offload some traffic towards the same neighbour cell. Similarly, the conflict may arise between a SON function and a non-SON function (e.g., eMIMO).

The potential SON conflict (which could be between SON functions or between a SON function and a non-SON function) should be prevented from happening as much as possible, and if the conflict happens it should be resolved as soon as possible. The SON conflict prevention and resolution can be assisted by MDA.

The MDA could analyse the following data for identifying the potential SON conflict or detecting whether a SON conflict occurred:

- 	historical and the most recent changes made by the SON functions and non-SON functions;

-	the current network configurations;

-	historical and current network performance data related to the SON function(s) and the relevant non-SON functions. For instance, load information of the NR cells, handover performance measurements (too early HOs, too late HOs, etc.);

-	Policies and targets for the SON functions.

- 	historical and current MDT/RLF data.

- 	for a SON function or a non-SON function, considering potential affected parameters.

If a potential SON conflict is identified, the MDAS producer provides an analytics report to describe the potential conflict and the recommended actions to prevent such conflicts from happening.

If a SON conflict is detected, the MDAS producer provides an analytics report to describe the conflict including the recommended actions to resolve it.

The recommended actions for SON conflict prevention and resolution may be one or more of the following:

-	modify the policies and targets for the SON function(s);

-	change the priority of the SON function(s);

-	set or change the range of the attributes value that the SON function(s) are allowed to modify;

-	update the attributes value to correct the conflict (if already occurred);

-	temporarily switch off one or more SON function(s);

-	undo the most recent configuration undertaken by the SON function;

-	provide a report that describes how SON function(s) can potentially affect the common parameters.

#### 6.8.1.2	Potential requirements

REQ-MDA_SONCO-CON-1	The MDAS producer should have a capability to provide the analytics report describing the identified potential conflict between SON functions or between a SON function and a non-SON function with recommended actions to prevent the conflict from happening.

REQ-MDA_SONCO-CON-2	The MDAS producer should have a capability to provide the analytics report describing the detected conflict between SON functions or between a SON function and a non-SON function with recommended actions to resolve the conflict.

REQ-MDA_SONCO-CON-3	The MDAS producer should have a capability to provide the analytics report that describes the relations (i.e. whether certain conditions trigger conflicts) between observed conflicts and observed conditions.

#### 6.8.1.3	Possible solutions

##### 6.8.1.3.1	Solution description

The MDAS producer correlates and analyses the data described in the following subclause within a time period on a regular basis or trigged by events (e.g., the RLF report) to identify a potential SON conflict or a SON conflict already occurred.

Once a potential or already occurred SON conflict is identified, the MDAS producer provides the analytics report to describe the SON conflict as shown in subclause 6.8.1.3.3.

##### 6.8.1.3.2	Data required for SON conflict analysis

The following table describes the data required for SON conflict analysis:

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Radio Measurements for gNB: RRC connection, see clause 5.1, TS 28.552 [8], e.g., RRC connection number, RRC connection establishment, RRC connection re-establishment, RRC connection resuming; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]. Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Performance measurements related to the SON functions, see TS 28.313 [19]. |
| CM notification | Notifications of the NRM updates made by SON functions and non-SON functions. The notification needs to include the Id (e.g., DN) of the SON function or non-SON function who made the NRM updates. |
| MDT/RLF Data | UE measurements related to RLF containing RSRP, RSRQ, of the serving cell and neighbour cells, SINR with anonymous id (e.g., C-RNTI) and UE location information. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration Data (NRM) | The execution data including the changes or the configuration of the MOIs. This should include historical UE configurations The attributes of the MOIs to be analysed, see TS 28.541 [20]. The policy and targets of the SON functions, see TS 28.313 [19]. |
| Potential affected parameters | SON functions parameters affected including the degree of impact (low, medium, high). |

Note:	The above parameters may not be the complete list.

##### 6.8.1.3.3	Analytics report SON conflict prevention and resolution

The following table provides the potential contents of the analytics report for SON conflict prevention and resolution:

| Analytics Report of SON conflict prevention and resolution | Attribute Name | Description |
| --- | --- | --- |
|  | Conflicting Functions Id | Indicates the Id of the conflicting SON functions or non-SON functions. |
|  | Conflict type | Indicates the conflict is a potential conflict or an already occurred conflict. |
|  | Conflicting attributes | The MOI and attributes where the conflict occurred or is potentially to occur. |
|  | Conflict reason | The description of the reason for the conflict. |
|  | Conflicting metrics | The performance measurements and KPI(s) over which the SON or non-SON Functions conflict. |
|  | Attribute conflict status | The relative level (low, medium, high) of impact of each conflicting attribute on the SON or non-SON functions. |
|  | Recommended actions | The MOI and attributes recommended to be configured/changed: - The policy and targets of the SON functions, see TS 28.313 [19]; - The range of attributes value each conflicting SON function can change; - The priority of each conflicting SON function; - Value of the attributes of the affected MOIs (e.g., NRCellCU) where the conflict already occurred; - Switch off the conflicting SON functions, see TS 28.313 [19]. |

#### 6.8.1.4 	Evaluation

The solution described in clause 6.8.1.3.1 requires the analytics inputs as described in clause 6.8.1.3.2 wherein:

-	PM related to Radio Measurements for gNB are specified in TS 28.552 [8].

-	KPI related to RAN UE throughput, Throughput for network slice instance are specified in TS 28.554 [7].

-	PM related to the SON functions, see TS 28.313 [19].

-	CM notifications made by SON functions as specified in TS 28.532.

-	MDT data is available in TS 37.320 [12].

-	QoE can be provided by the service experience analytics by NWDAF TS 23.288 [18].

-	Configuration Data (NRM) TS 28.541 [20].

-	Potential affected SON parameters are available. NRM extension are also needed to reflect the degree of impact (low, medium, high).

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.8.1.3.3 can be derived.

Therefore, this solution is a feasible candidate for SON conflict prevention and resolution.

## 6.9	Security related issues

### 6.9.1	Security risk assessment

#### 6.9.1.1	Use case

Security risk assessment can detect anomalies in managed objects, e.g. NF, identify potential security risks and propose countermeasures to mitigate them. Security risks may originate from a variety of different sources and target distinct network objects resulting in different abnormal behaviours, e.g. sudden increase in computing and storage in a virtualization environment, sudden increase of load in network links, abnormal communication patterns between NFs, excessive latency in accessing a NF, relocating a NF in an unexpected location or interrupting the relocation of a NF.

To assess security risks, the management plane can leverage the benefits of MDAS. An MDAS producer can identify a security risk issue by correlating different performance measurements and alarms, i.e. performing a root cause analysis to locate the malicious source and the network objects affected. Such activity can be triggered when certain performance measurements and KPIs indicate an abnormal network behaviour, which is not related to another known faults.

The MDAS producer should correlate the usage, e.g. considering the NF procedures for supporting a number of UEs or other events, with the corresponding resource usage, e.g. in terms of CPU, storage and disk. If no other fault alarms indicate another reason, an unexpected resource usage should trigger a security risk analysis to identify the issue based on a private security risk database that resides on the MDAS producer. The MDAS producer can notify the MDAS consumer, e.g. MnF or NE, and provide recommendations to mitigate the identified security risk. The MDAS consumer based on the recommendations can isolate the malicious network objects, e.g. NF, and can also provide recommendations to harden the network in order to avoid similar security risks in the future.

#### 6.9.1.2	Potential requirements

REQ-SEC_CON-1 The MDAS producer should have a capability to provide the analytics report describing the security risk to authorized consumers based on the correlation of current and predicted performance measurements and alarms.

REQ-SEC_CON-2 The analytics report describing the security issue should contain the following information describing the current and future security risk issue:

-	Identify the type of security risk.

-	Location and network objects affected by the security risk.

-	Root cause analysis of the security risk issue.

-	Recommended action to isolate and/or restrict the security risk issue and harden the network security.

#### 6.9.1.3	Possible solutions

##### 6.9.1.3.1	Solution description

The solution considers security risk assessment related to network. The MDAS producer correlates and analyses performance measurements and alarm data considering the network topology and configuration data related to statistics or predictions to identify the type of security risk.

##### 6.9.1.3.2	Data required

The following data is required to perform the corresponding security risk assessment analysis.

| Data category | Required data |
| --- | --- |
| Alarm Data | Alarm information - types of alarms |
| Service Data | S-NSSAI as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information |
| Performance Measurements | Failures and disruptions: - Number of abnormal releases: DRB, QoS flows, PDU sessions, UE context in serving cells as per TS 28.552 [8] - Disruption measurements: CQI/MCS as per TS 28.552 [8] - Excessive delay in accessing NFs (e.g. AMF or SMF) - NF abnormal and excessive communication, e.g. AMF uses a different SMF without performing the expected selection process or it overloads an SMF unexpectedly. - Intra/inter-gNB handover: failures - long handover time as per TS 28.552 [8] Virtualized resources/behaviour: - Virtual resource usage of NF: The resource usage of virtual network functions, see clause 5.7.1 of TS 28.552 [8] - Virtual NF Re-location: Timing/duration and success rate - Frequency of virtual NF Re-location: Rate of relocation - Virtual NF location: NF location with respect to a data network NF context information as per TS 28.552 [8]: - Number of UEs/periodic registration updates (AMF) - Number of PDU sessions/modifications, QoS flows (SMF) - N4/N6/N9 measurements or packet delay, traffic volume, link usage, packet loss (UPF) - Number of application trigger requests rejected (NEF) - Number of failed NF service registration/update, discoveries due to unauthorized NF/error (NRF) |
| Configuration Data | NRM attributes affecting the location and virtual NF resource allocation and configuration NRM update reports (notification and log) containing the creation or changes of the MOIs affecting the virtual NFs |
| Network Topology | Topology of the network |

##### 6.9.1.3.3	Analytics report

The MDAS producer offers a new Security Analytics service to the MDAS consumer, which supports security risk assessment related providing the following analytics results:

| Analytics Report of Security risk assessment | Attribute Name | Description |
| --- | --- | --- |
|  | Security Incident Identifier | Identifier that indicates the security risk (e.g. DDoS, malicious NF, etc.) |
|  | Type of Analytics | Statistics or Prediction of security risks |
|  | Location | Geographical location that the security risk affects |
|  | Affected Objects | NF, PDU session, QoS Flow, Slice |
|  | Start/Stop Time | Starts/stop time of the security risk issue |
|  | Root Cause | The originator of security issue to isolate fast the problem. |
|  | Severity Level | The severity level (e.g. critical, medium, not important) of the security risk issue |
|  | Recommended Actions | Recommendation actions to resolve the security risk issue: - Isolate/terminate NF, terminate PDU session, throttle signalling from NF or UE, block UE, etc. - Harden security on specific NF, firewall update, scaling resources, load balancing, admission control, etc. |

#### 6.9.1.4	Evaluation

The solution described in clause 6.9.1.3.1 requires the analytics inputs as described in clause 6.9.1.3.2, wherein

-	Input related to alarm data, configuration data and network topology are available.

-	Input PMs: Number of abnormal releases, Disruption measurements, Intra/inter-gNB handover and Excessive delay in accessing NFs (e.g. AMF or SMF) are specified in TS 28.552[8].

-	Virtualized resources/behaviour are specified in TS 28.552[8].

-	IP address of virtual NF is available.

-	NF context information for AMF, SMF, UPF, NEF, NRF are specified in TS 28.552 [8].

-	The following PMs related to 5GC NFs shall be specified:

-	NF abnormal and excessive communication.

-	The following PMs related to virtual NFs shall be specified:

-	Virtual NF Re-location: Timing/duration and success rate.

-	Frequency of virtual NF Re-location: Rate of relocation.

-	the network topology can be reflected by the NRMs defined in TS 28.541 [20].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.9.1.3.3 can be derived.

Therefore, this solution is a feasible candidate for security risk assessment.

## 6.10	MDA management aspects

### 6.10.1	ML model training for MDA

#### 6.10.1.1	Use case

The MDA process may rely on ML technologies. To optimize the accuracy of MDA result, the ML model of the MDA process may need to be trained.

For training the ML model of the MDA process, the consumer provides the training data including training input and the desired output to the MDAS producer. The MDAS producer uses the training input and the desired output to train the ML model, i.e., to train the algorithm of the ML model to generate the desired output as accurately as possible by analysis of the training input. The MDAS producer provides an ML model training report to the consumer.

With a trained ML model for MDA, the MDAS producer can analyse the analytics input and generate the analytics report as output data of the analysis to the consumer.

The consumer may validate the output data provided by the MDAS producer. The output data to be validated may be the analytics report and/or the ML model training report as described above. The consumer may provide the validation data as feedback to the MDAS producer, and the MDAS producer will use the validation data for further ML model training for MDA with the input data that were used to generate the validated report. As a result of validation, the consumer may also provide the training data and request the MDAS producer to train the ML model.

#### 6.10.1.2	Potential requirements

REQ-MDA_MGMT-CON-1	The MDAS producer should have a capability allowing the consumer to provide the data for training the ML model for MDA.

REQ-MDA_MGMT-CON-2	The MDAS producer should have a capability to provide ML model training report to the consumer.

REQ-MDA_MGMT-CON-3	The MDAS producer should have a capability to receive the validation data from the consumer and train the ML model for MDA based on the received validation data.

#### 6.10.1.3	Possible solutions

##### 6.10.1.3.1	Solution description

The MDAS producer trains the ML model with the training data or the validation data received from consumer. The ML model training should have no or minimal impact to the normal data analytics.

For the ML model training with the training data, the training data (see subclause 6.10.1.3.2) should include the training Id, the training input and expected training output:

-	The training Id is used to identify the ML model training request, and to associate with the training report.

-	The training input is a set of training input data with the indicated data type (e.g., performance measurements, MDT report, NRM, etc.).

-	The expected training output specifies the analysis result that the ML model should aim to achieve based on the training input.

For the ML model training with the validation data, the validation data (see subclause 6.10.1.3.2) should include the validation report Id, analytics report Id that was validated, and the validated information including the data that are rectified. The MDAS producer looks up the historical data that are associated with the validated analytics report and trains the ML model with the historical data and the validation data.

The MDAS producer provides a training report (as shown in subclause 6.10.1.3.3) to the consumer, with indication of whether the training (identified by the training Id or validation Id) is successful and possibly the failure cause if the training is not fully successful.

##### 6.10.1.3.2	Data required for ML model training for MDA

The following table describes the data required for ML model training for MDA:

| Data category | Required data |
| --- | --- |
| Training data | The training data include a training identifier, the training input and the expected training output. Training Id: Identifier of the training. The training input contains the same types of information that should be included in a normal analytics input. The expected training output contains the same types of information that should be included in a normal analytics report. |
| Validation data | The training data include a validation report Id, analytics report Id that was validated, and the data that are rectified. Example of the rectified data is the originally reported root cause is not accurate by validation and should be updated to another. |

##### 6.10.1.3.3	ML model training report

Following table provides the potential contents of the ML model training report.

| ML model training report | Attribute Name | Description |
| --- | --- | --- |
|  | Training Id | The training Id that the training report is associated to. |
|  | Validation report Id | The validation report Id that the training report is associated to. |
|  | Training result | Indication of whether the training is successful, partially successful or failed. |
|  | Failure cause | The cause of the failure if the training is not fully successful. |

#### 6.10.1.4	Evaluation

The solution described in clause 6.10.1.3 requires the input s described in in clause 6.10.1.3.2, wherein

-	the training data are fully aligned with the analytics input and output of the concrete analytics use cases.

-	and validation data can be provided by the consumer.

Therefore, this solution is a feasible candidate for ML model training for the use cases that are concluded with a feasible solution.

### 6.10.2	Requesting and reporting of Management Data Analytics Reports

#### 6.10.2.1	Use case

A MDAS Producer may provide several management data analysis reports. Multiple consumers may wish to receive a selection of these reports.

The consumer submits a request to MDAS producer to request MDA reports. This request may include a filter to specify the scope of MDA reports (e.g., type of analytics report such as coverage issue analysis, resource utilization analysis, the managed objects to be analysed, etc.). The MDAS producer activates the data collection if it is not already active. In the request, the consumer may indicate the method that the MDA reports are to be reported.

For all reports, the MDAS producer collects data, analyses the data, and generates the analytics report.

The MDAS producer provides the MDA reports based on the reporting method designated by the consumer.

The consumer may send a request to MDAS producer to modify, i.e. adjust the filter scope of the MDA reporting, or  terminate the MDA reporting request.

#### 6.10.2.2	Potential requirements

REQ-MDA_SUB-1		The MDAS producer should have a capability to allow an MDAS consumer to request  an analytics report. The  reporting request should optionally allow the MDAS consumer to filter the scope of data in the analytics report.

REQ-MDA_SUB-2		The MDAS producer should have a capability to provide the analytics report to the requesting consumers.

REQ-MDA_SUB-3		The MDAS producer should have a capability to allow an MDAS consumer to terminate an analytics reporting request.

#### 6.10.2.3	Possible solutions

The MDAS consumer sends the MDA reporting request to MDAS producer, with the following information included:

-	identifier of the reporting request;

-	reporting method, i.e., file reporting or streaming based reporting or notification based reporting;

-	streaming target if the reporting method is designated to streaming based reporting;

-	file information (e.g., file location, ready time, expiration time, size, compression and format) if the reporting method is designated to file reporting;

- 	notification target if the reporting method is designated to provide notification based reporting;

-	filter for the scope of the MDA analytics report (e.g., type of analytics report, start/stop time, managed objects to be analysed, etc.).

The MDA reporting request may be modelled as an IOC and managed via provisioning related operations (such as CreateMOI, ModifyMOI, DeleteMOI).

The MDAS producers provides a response indicating the status of the request.

For the MDA reporting request designating the reporting method of file based reporting and streaming based reporting are already defined in TS 28.532 [28], while notification based reporting needs to be further investigated during the normative phase. Figure 6.10.2.3-1 shows MDA reporting with the multiple reporting options.

Figure 6.10.2.3-1: MDA reporting with multiple options

NOTE:  The file server where MDA report is fetched from in steps a5 and a6 can be deployed separately from the MDAS producer.

#### 6.10.2.4	Evaluation

The solution described in clause 6.10.2.3 include multiple reporting options:

- the generic streaming based reporting service and file based reporting service as defined in TS 28.532 [28] and can be reused for MDA data reporting,

- notification based reporting, for which the detailed solution can be further investigated and determined during the normative phase.

Therefore, this solution is a feasible candidate for MDA reporting request and reporting.

### 6.10.3	Retrieval of historical Management Data Analytics Reports

#### 6.10.3.1	Use case

Besides the subscription and reporting of the MDA reports, the consumer may need to retrieve some historical MDA reports which were produced by MDAS producer in the past. The consumer may retrieve the historical MDA reports for some reason, for example the reports have been received but lost by the consumer, or the reports have been received but could not be successfully parsed by the consumer, or even the reports were not successfully received by the consumer.

The consumer submits a request to MDAS producer to retrieve the historical MDA reports. This request may include a time frame and a filter to specify the scope of MDA reports to be retrieved (e.g., type of analytics report such as coverage issue analysis, resource utilization analysis, the managed objects analysed, etc.).

The MDAS producer retrieves the historical MDA reports, and sends the results to the consumer.

#### 6.10.3.2	Potential requirements

The handling of historical management data in being addressed in some other work items, such as MADCOL (UID: 880028), it is assumed that the generic requirements on historical management data handling are applicable to MDA reports. No specific potential requirements are identified for retrieval of historical MDA reports.

#### 6.10.3.3	Possible solutions

Handling of historical management data in being addressed in some other work items, such as MADCOL (UID: 880028), and the solutions derived from those work items can be applied to retrieval of historical MDA reports.

NOTE:	How to apply the solutions on historical management data handling made by these work items to MDA reports is not addressed in this study.

#### 6.10.3.4	Evaluation

The retrieval and reporting of historical MDA reports can reuse the solutions for handling historical management data being addressed in other work items, such as MADCOL (UID: 880028). This study does not address how to apply these solutions to MDA reports.

### 6.10.4	Confidence indicator in analysis results

A consumer of MDAS should treat the results of Management Data Analytics with caution. A decision based on analytics should take into account the degree of confidence of the analysis result, especially in the case when multiple analysis results are based on different models or different source data.

Therefore it is proposed that the result of Management Data Analytics should contain an attribute which indicates the degree of confidence of the analysis.

NOTE: 	How to evaluate a degree of confidence and how it should be expressed as an attribute are not addressed in this study. More work is needed in this area.

# 7	Conclusions and recommendations

The technical report described the MDA functionality and service framework (see clause 4), MDA process, MDA role in the management loop, and interactions between MDAS producer and NWDAF and gNB (see clause 5).

The technical report identified and documented a wide range of use cases for management data analytics and the MDA management aspects, derived the corresponding potential requirements, and developed and evaluated the possible solutions (see clause 6).

For enhancement of MDAS in normative work in 3GPP Rel-17, it is recommended:

To describe:

>	MDA service framework illustrating the service relations between MDAS producer, potential MDAS consumers and other network/management functions/services, including:

>> 	relation of MDAS producer with NWDAF;

>> 	relation of MDAS producer with other management functions/services;

>> 	relation between cross domain MDAS producer and domain specific MDAS producer, including 3GPP RAN/CN domain and non-3GPP management domains (e.g., transport, virtualized resources);

>> 	other relations related to MDA, where necessary.

>	MDA role in the management loop (including open control loop and closed control loop supporting Closed Loop SLS Assurance (COSLA) and eCOSLA).

>	How the MDA output is aligned to Analytics Insights defined in COSLA and eCOSLA work.

And, to specify:

>	MDA input and report (output):

>> 	Common information elements of MDA report (output);

>> 	3GPP domain specific MDA input data (including new performance measurements, NRMs, MDT data, etc.) and MDA output attributes (as extension to the common attributes of MDA report) pertaining to the specific MDA use cases, based on the use cases studied, evaluated, concluded and recommended in TR 28.809 with the following issues prioritized for Rel-17:

-	Alarm incident analysis

-	SLS analysis related issues

-	MDA assisted Energy Saving

-	Coverage related issues

-	Mobility management related issues

>> 	Mechanisms to address multiple relevant use cases by a consolidated MDA report.

>	Requirements and mechanisms on MDA report request and reporting;

>	Requirements on retrieval of historical data related to MDA (including MDA input and output), and mechanisms by reusing the solutions for historical data handling defined by work item MADCOL (UID: 880028) and possibly other work items if relevant;

>	Support for ML model training for MDA, where ML can be applicable without introducing tight coupling and dependencies on MDAS.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | SA5#127 | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2019-10 | SA5#127 | S5-196752 | - | - | - | pCR Add skeleton of draft TR 28.809 | 0.1.0 |
| 2019-10 | SA5#127 | S5-196812 | - | - | - | pCR Add overview for MDA | 0.1.0 |
| 2019-10 | SA5#127 | S5-196877 | - | - | - | pCR Add UC on coverage issue analysis | 0.1.0 |
| 2019-10 | SA5#127 | S5-196876 | - | - | - | pCR Add use case of MDAS assisted user data congestion analysis | 0.1.0 |
| 2019-12 | SA5#128 | S5-197807 | - | - | - | pCR on MDA role in management loop | 0.2.0 |
| 2019-12 | SA5#128 | S5-197710 | - | - | - | pCR on possible solution for coverage issue analysis | 0.2.0 |
| 2019-12 | SA5#128 | S5-197711 | - | - | - | pCR Add UC on resource utilization optimization | 0.2.0 |
| 2019-12 | SA5#128 | S5-197707 | - | - | - | pCR Update the overview descriptions of MDAS | 0.2.0 |
| 2019-12 | SA5#128 | S5-197709 | - | - | - | pCR on RAN user plane congestion | 0.2.0 |
| 2019-12 | SA5#128 | S5-197714 | - | - | - | pCR on E2E latency analysis as one of the SLS assurance related issues | 0.2.0 |
| 2019-12 | SA5#128 | S5-197808 | - | - | - | pCR on alarm incident analysis as one of the fault management related issues | 0.2.0 |
| 2019-12 | SA5#128 | S5-197809 | - | - | - | pCR Add UC on handover optimization | 0.2.0 |
| 2019-12 | SA5#128 | S5-197810 | - | - | - | pCR Add UC on MDA assisted energy saving | 0.2.0 |
| 2019-12 | SA5#128 | S5-197832 | - | - | - | pCR Add management interaction with NWDAF | 0.2.0 |
| 2020-03 | SA5#129e | S5-201315 | - | - | - | pCR Use case and potential solutions of MDA assisted EE | 0.3.0 |
| 2020-03 | SA5#129e | S5-201491 | - | - | - | pCR Add gNB as the consumer of MDAS | 0.3.0 |
| 2020-03 | SA5#129e | S5-201492 | - | - | - | pCR Add the use cases related to SLA assurance | 0.3.0 |
| 2020-03 | SA5#129e | S5-201327 | - | - | - | pCR Addition of resource utilization analysis solution | 0.3.0 |
| 2020-06 | SA5#131e | S5-203441 | - | - | - | pCR on MDA process | 0.4.0 |
| 2020-06 | SA5#131e | S5-203442 | - | - | - | pCR Add UC on MDA process training | 0.4.0 |
| 2020-06 | SA5#131e | S5-203443 | - | - | - | Clarify fault management use case | 0.4.0 |
| 2020-06 | SA5#131e | S5-203453 | - | - | - | Enhancing the Handover optimization use case in subclause 6.5 | 0.4.0 |
| 2020-06 | SA5#131e | S5-203206 | - | - | - | pCR solution for handover optimization | 0.4.0 |
| 2020-06 | SA5#131e | S5-203444 | - | - | - | Add use case and potential solutions of network slice throughput analysis | 0.4.0 |
| 2020-06 | SA5#131e | S5-203237 | - | - | - | Add use cases of UE uplink/downlink throughput analysis | 0.4.0 |
| 2020-06 | SA5#131e | S5-203148 | - | - | - | Update resource utilization use case | 0.4.0 |
| 2020-06 | SA5#131e | S5-203208 | - | - | - | pCR use case for paging optimization | 0.4.0 |
| 2020-06 | SA5#131e | S5-203209 | - | - | - | pCR use case RAN Node software upgrade | 0.4.0 |
| 2020-06 | SA5#131e | S5-203446 | - | - | - | pCR addition of network slice load analysis requirements and solution | 0.4.0 |
| 2020-06 | SA5#131e | S5-203447 | - | - | - | Add use case and potential solutions of cross-slice resource optimization analysis | 0.4.0 |
| 2020-06 | SA5#131e | S5-203448 | - | - | - | pCR Add UC on MDA assisted SON coordination | 0.4.0 |
| 2020-06 | SA5#131e | S5-203449 | - | - | - | New use case on Inter-gNB Beam Selection Optimization | 0.4.0 |
| 2020-06 | SA5#131e | S5-203450 | - | - | - | Add the use case for NAS level congestion control optimization | 0.4.0 |
| 2020-06 | SA5#131e | S5-203451 | - | - | - | Add use case and potential solutions of KPI anomaly analysis | 0.4.0 |
| 2020-06 | SA5#131e | S5-203445 | - | - | - | Modification of the resource utilization analysis use case | 0.4.0 |
| 2020-06 | SA5#131e | S5-203452 | - | - | - | Add use case of jitter analysis | 0.4.0 |
| 2020-08 | SA5#132e | S5-204559 | - | - | - | pCR Add solution on MDA assisted SON coordination | 0.5.0 |
| 2020-08 | SA5#132e | S5-204560 | - | - | - | pCR 28.809 Add load balancing optimization use case | 0.5.0 |
| 2020-08 | SA5#132e | S5-204561 | - | - | - | pCR 28.809 Add use case and potential requirements of mobility performance analysis | 0.5.0 |
| 2020-08 | SA5#132e | S5-204562 | - | - | - | Update of inter-gNB beam handover use case solution | 0.5.0 |
| 2020-08 | SA5#132e | S5-204563 | - | - | - | Update of Handover use case solution | 0.5.0 |
| 2020-08 | SA5#132e | S5-204564 | - | - | - | Update of Handover use case and solution considering user-trajectory info | 0.5.0 |
| 2020-08 | SA5#132e | S5-204565 | - | - | - | pCR 28.809 Update use case and potential solutions of resource utilization analysis | 0.5.0 |
| 2020-08 | SA5#132e | S5-204566 | - | - | - | pCR TR 28.809 Further clarifications relating to network resource allocation | 0.5.0 |
| 2020-08 | SA5#132e | S5-204567 | - | - | - | Update of KPI anomaly analysis use case and solution | 0.5.0 |
| 2020-08 | SA5#132e | S5-204568 | - | - | - | pCR Add description for analytics report | 0.5.0 |
| 2020-08 | SA5#132e | S5-204569 | - | - | - | pCR 28.809 Add Subscription to Management Data Analytics Reports | 0.5.0 |
| 2020-08 | SA5#132e | S5-204249 | - | - | - | pCR 28.809 Add Request for Management Data Analytics Reports | 0.5.0 |
| 2020-08 | SA5#132e | S5-204247 | - | - | - | pCR 28.809 Editorial improvements | 0.5.0 |
| 2020-08 | SA5#132e | S5-204570 | - | - | - | pCR 28.809 Add missing abbreviations | 0.5.0 |
| 2020-08 | SA5#132e | S5-204571 | - | - | - | pCR 28.809 Use case and potential solutions of fault prediction analysis | 0.5.0 |
| 2020-08 | SA5#132e | S5-204254 | - | - | - | pCR 28.809 updating fault analysis use case | 0.5.0 |
| 2020-08 | SA5#132e | S5-204572 | - | - | - | pCR 28.809 updating E2E latency analysis use case | 0.5.0 |
| 2020-08 | SA5#132e | S5-204573 | - | - | - | pCR 28.809 updating network slice throughput analysis use case | 0.5.0 |
| 2020-08 | SA5#132e | S5-204574 | - | - | - | pCR 28.809 Update use case and potential requirements of network slice load analysis | 0.5.0 |
| 2020-08 | SA5#132e | S5-204575 | - | - | - | pCR 28.809 Solution for Paging Optimization | 0.5.0 |
| 2020-08 | SA5#132e | S5-204576 | - | - | - | pCR 28.809 solution for RAN Node software upgrade | 0.5.0 |
| 2020-08 | SA5#132e | S5-204577 | - | - | - | pCR 28.809 MDAS Analysis confidence levels | 0.5.0 |
| 2020-08 | SA5#132e | S5-204578 | - | - | - | pCR 28.809 Add overview of MDA functionality | 0.5.0 |
| 2020-08 | SA5#132e | S5-204251 | - | - | - | pCR 28.809 Add possible solution for NAS congestion control | 0.5.0 |
| 2020-08 | SA5#132e | S5-204579 | - | - | - | pCR 28.809 Add the potential requirements and solutions for service experience related analysis | 0.5.0 |
| 2020-08 | SA5#132e | S5-204580 | - | - | - | Security risk assessment analytics use case | 0.5.0 |
| 2020-08 | SA5#132e | S5-204581 | - | - | - | MDAS assisted network slice traffic projections | 0.5.0 |
| 2020-08 | SA5#132e | S5-204587 | - | - | - | Add potential solutions for the MDA assisted energy saving analysis | 0.5.0 |
| 2020-08 | SA5#132e | S5-204505 | - | - | - | pCR 28.809 Add CN related KPI anomaly analysis | 0.5.0 |
| 2020-09 | SA#89e | SP-200755 |  |  |  | Presented for information | 1.0.0 |
| 2020-09 |  |  |  |  |  | EditHelp review | 1.0.1 |
| 2020-10 |  |  |  |  |  | New version due to corrupted file | 1.0.2 |
| 2020-11 | SA5#134e | S5-206375 | - | - | - | Update potential solutions for the MDA assisted energy saving analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206376 | - | - | - | pCR Update on MDA assisted energy saving | 1.1.0 |
| 2020-11 | SA5#134e | S5-206377 | - | - | - | pCR Add conclusion for resource utilization analytics | 1.1.0 |
| 2020-11 | SA5#134e | S5-206378 | - | - | - | pCR Add conclusion for coverage issue analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206379 | - | - | - | Gap Analysis for KPI Anomaly Analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206380 | - | - | - | HO Optimization based on UE Load | 1.1.0 |
| 2020-11 | SA5#134e | S5-206381 | - | - | - | Gap Analysis for Handover Optimization based on UE trajectory | 1.1.0 |
| 2020-11 | SA5#134e | S5-206382 | - | - | - | Load Balancing Optimization Solutions based on Resource Consumption and Service Specifics | 1.1.0 |
| 2020-11 | SA5#134e | S5-206383 | - | - | - | pCR Update solutions on subscription and reporting of Management Data Analytics Reports | 1.1.0 |
| 2020-11 | SA5#134e | S5-206384 | - | - | - | pCR Update solutions on request of Management Data Analytics Report | 1.1.0 |
| 2020-11 | SA5#134e | S5-206385 | - | - | - | Suggested conclusion for Alarm incident analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206386 | - | - | - | Conclusion for NAS congestion control | 1.1.0 |
| 2020-11 | SA5#134e | S5-206092 | - | - | - | Resubmission pCR Add solution on MDA ML model training | 1.1.0 |
| 2020-11 | SA5#134e | S5-206093 | - | - | - | pCR Update based on comments provided by Edithelp | 1.1.0 |
| 2020-11 | SA5#134e | S5-206387 | - | - | - | Update the solution of Jitter analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206388 | - | - | - | Add the conclusion on network slice load analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206389 | - | - | - | Add the conclusion on E2E latency analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206390 | - | - | - | Add the conclusion on service experience analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206391 | - | - | - | Add the conclusion on throughput analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206392 | - | - | - | New use case on slice coverage and throughput optimization | 1.1.0 |
| 2020-11 | SA5#134e | S5-206393 | - | - | - | Gap analysis for inter-gNB beam selection Optimization | 1.1.0 |
| 2020-11 | SA5#134e | S5-206327 | - | - | - | Alarm Malfunction Analytics | 1.1.0 |
| 2020-11 | SA5#134e | S5-206394 | - | - | - | Gap Analysis for Security Risk Assessment | 1.1.0 |
| 2020-11 | SA5#134e | S5-206395 | - | - | - | Add conclusion for fault predication analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206396 | - | - | - | Gap Analysis for MDA assisted SON coordination | 1.1.0 |
| 2020-11 | SA5#134e | S5-206397 | - | - | - | Gap Analysis for Mobility performance Analysis | 1.1.0 |
| 2020-11 | SA5#134e | S5-206398 | - | - | - | MDAS detection of outlying MEs | 1.1.0 |
| 2021-02 | SA5#135e | S5-211447 | - | - | - | Update solution on MDA assisted energy saving | 1.2.0 |
| 2021-02 | SA5#135e | S5-211500 | - | - | - | Update on MDA assisted energy saving | 1.2.0 |
| 2021-02 | SA5#135e | S5-211153 | - | - | - | Update solution on MDA assisted energy saving to provide necessary information in output | 1.2.0 |
| 2021-02 | SA5#135e | S5-211449 | - | - | - | Update the use case of traffic projection | 1.2.0 |
| 2021-02 | SA5#135e | S5-211483 | - | - | - | pCR Update diagram of MDA process | 1.2.0 |
| 2021-02 | SA5#135e | S5-211473 | - | - | - | Slice coverage optimization: Use case and solution enhancements | 1.2.0 |
| 2021-02 | SA5#135e | S5-211484 | - | - | - | pCR Rapporteur clean-up | 1.2.0 |
| 2021-02 | SA5#135e | S5-211073 | - | - | - | pCR 28.809 Alignment of internal references | 1.2.0 |
| 2021-02 | SA5#135e | S5-211072 | - | - | - | pCR 28.809 Update alarm malfunction analytics | 1.2.0 |
| 2021-02 | SA5#135e | S5-211485 | - | - | - | pCR Add scope for TR 28.809 | 1.2.0 |
| 2021-02 | SA5#135e | S5-211135 | - | - | - | Update the analytics report for alarm incident analysis | 1.2.0 |
| 2021-02 | SA5#135e | S5-211237 | - | - | - | RAN node software upgrade Evaluation | 1.2.0 |
| 2021-02 | SA5#135e | S5-211502 | - | - | - | Paging Optimization Evaluation | 1.2.0 |
| 2021-02 | SA5#135e | S5-211503 | - | - | - | HO Optimization based on UE load Evaluation | 1.2.0 |
| 2021-03 | SA5#136e | S5-212229 | - | - | - | pCR re-arranging paging UC | 1.3.0 |
| 2021-03 | SA5#136e | S5-212230 | - | - | - | pCR re-arranging projection UC | 1.3.0 |
| 2021-03 | SA5#136e | S5-212445 | - | - | - | Report Validation | 1.3.0 |
| 2021-03 | SA5#136e | S5-212418 | - | - | - | Enhancing analytics request and reporting use case | 1.3.0 |
| 2021-03 | SA5#136e | S5-212049 | - | - | - | pCR Add evaluation for cross-slice resource optimization | 1.3.0 |
| 2021-03 | SA5#136e | S5-212419 | - | - | - | pCR Add evaluation for RAN user plane congestion analysis | 1.3.0 |
| 2021-03 | SA5#136e | S5-212420 | - | - | - | pCR Add evaluation for UE throughput in network slice analysis | 1.3.0 |
| 2021-03 | SA5#136e | S5-212421 | - | - | - | pCR Update and evaluate the solution for jitter analysis | 1.3.0 |
| 2021-03 | SA5#136e | S5-212422 | - | - | - | pCR Update and evaluate the mobility performance analysis | 1.3.0 |
| 2021-03 | SA5#136e | S5-212446 | - | - | - | pCR Add general conclusions and recommendations | 1.3.0 |
| 2021-03 | SA5#136e | S5-212142 | - | - | - | pCR Update on MDA assisted energy saving | 1.3.0 |