| 3GPP TR 32.847 V0.2.0 (2021-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects Charging management Study on Charging Aspects for Network Slicing Phase 2 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	General background	7

4.1	Background from GSMA	7

4.2	Background from 3GPP network architecture	8

4.3	Network Slice Management Charging	9

5	Potential charging requirements	9

6	Key Issues	10

6.1	Key Issue #1: Converged Charging for max. number of UEs per network slice	10

6.1.1	General Description	10

6.1.2	Solution#1	10

6.1.3	Solution#2	10

6.1.x	Evaluation	10

6.2	Key Issue #2: Converged Charging for max. number of PDU sessions per network slice	10

6.2.1	General Description	10

6.3	Key Issue #34: Volume based Converged Charging per network slice	11

6.3.1	General Description	11

6.4	Key Issue #4: Converged Charging for max duration per network slice	11

6.4.1	General Description	11

6.5	Key Issue #5: Converged charging for number of simultaneous UEs per network slice	11

6.5.1	General Description	11

6.5.2	Solution#1	12

6.5.3	Solution#2	12

6.5.x	Evaluation	12

6.6	Key Issue #6: Converged Charging for number of unique UEs per network slice	12

6.6.1	General Description	12

6.6.2	Solution#1	13

6.6.3	Solution#2	13

6.6.x	Evaluation	13

6.7	Key Issue #7: Converged charging for number of concurrent PDU sessions per network slice	13

6.7.1	General Description	13

6.7.2	Solution#1	13

6.7.3	Solution#2	13

6.7.x	Evaluation	13

Annex A (informative): Change history	14

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document continues the study on charging aspects of network slicing covered by 3GPP TR 32.845 [2] but no longer limited to charging data collection functionalities without quota management as specified for Network Slice management charging in 3GPP TS 28.202 [3] and for Network slice performance and analytics charging in 3GPP TS 28.201 [4].

The following is studied:

-	Network slice converged charging for Network Slices enabled over 3GPP 5GS;

-	Correlation aspects between Network Slice charging and the end user charging accessing the Network Slice;

-	Inter-Provider (B2B) charging aspects by considering the different Organizations (e.g. MNO, Verticals).

The Converged Charging service aspects associated to the 3GPP TR 23.700-40 [5] conclusion on network slice information is also considered.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 32.845: "Charging management; Study on charging aspects of network slicing”.

[3]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[4]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[5]	3GPP TR 23.700-40: "Study on enhancement of network slicing; Phase 2”.

[6]	GSMA 5GJA NG.116: "Generic Network Slice Template".

[7]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[8]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GJA		5G Joint Activity (Initiative of NG)

CSC	Communication Service Customer

CSP	Communication Service Provider

EPS	Evolved Packet System

GSM	Global System for Mobile Communications

GSMA	GSM Association

GST	Generic Network Slice Template

NEST	Network Slice Types

NG	Networks Group (GSMA WG)

NOP	Network Operator

NSC	Network Slice Customer

NSI	Network Slice Instance

NSP	Network Slice Provider

# 4	General background

## 4.1	Background from GSMA

GSMA 5GJA has specified in NG.116 [6] the concept of Generic Network Slice Template (GST) from which several Network Slice Types (NESTs) can be derived by assigning values to applicable attributes defined in the GST.

As per 3GPP TS 23.501 [7] is the network slice a logical network that provides specific network capabilities and network characteristics, and GSMA tailored the network slice based on the specific requirements adhered to a Service Level Agreement (SLA) agreed between Network Slice Customer (NSC) and Network Slice Provider (NSP).

Multiple roles related to network slicing are defined by GSMA and displayed in figure 4.1-1, based on some roles specified in 3GPP TS 28.530 [8] and described in 3GPP TR 32.845 [2].

The Charging model will fullfill the following used roles:

-	Communication Service Customer (CSC): Uses communication services, e.g. end user, tenant, vertical.

-	Communication Service Provider (CSP): Provides communication services. Designs, builds and operates its communication services. The CSP provided communication service can be built with or without network slice.

-	Network Operator (NOP): Designs, builds and operates networks and provides related services, including network services and network slices.

-	Network Slice Customer (NSC): The CSP or CSC who uses Network Slice as a Service.

-	Network Slice Provider (NSP): The CSP or NOP who provides Network Slice as a Service.

Depending on the scenarios an organisation can play one or several roles simultaneously, and apply charging models based on corresponding business relationship, e.g.:

-	Charging models for CSP to charge CSC for using communication services

-	Charging models for NOP to charge CSP for using their Networks and related Network services

-	Charging models for NOP playing NSP Role to charge CSP playing NSC Role for using their Networks and related network slices

Figure 4.1-1: Model roles in network slicing

The GST is a set of generic attributes that can characterise a type of network slice/service and is not tied to any specific network deployment.

The NEST is a GST filled with values. The attributes and their values are assigned to fulfil a given set of requirements derived from a network slice customer use case. The NEST is an input to the network slice preparation performed by NSP. NEST-related attributes can be differentiated by a Service Category.  For instance, when an attribute of a NEST includes a Service Category parameter and the UE subscription information includes a parameter value associated with that Service Category, then the UE will be subject to the differentiated service associated with the Service Category in the network slice.

Figure 4.1-2 places the GST and NEST in the context of the network slice lifecycle. As first step, the NSC provides the service requirements for the use case that it would like to fulfil, to the NSP. The NSP maps the service requirements into the attributes of the GST with the appropriate values generating a NEST.

Figure 4.1-2: GST and NEST in context of the network slice lifecycle

Editor’s Note: The addition of the concept of Network Slice Instance (NSI) is ffs.

## 4.2	Background from 3GPP network architecture

The architectural requirements per 3GPP TR 23.700-40 [5] are copied below to derive potential charging requirements:

"-	Solutions shall build on the 5G System architectural principles as in 3GPP TS 23.501 [7], including flexibility and modularity for newly introduced functionalities.

-	Subject to operator policy and national/regional regulations, solutions shall be able to exempt Emergency services, Multimedia Priority Service, and Mission Critical Service from count limits.

NOTE:	Handling of UE/PDU Session count exemption and any pre-emption to other non-mission critical services is expected to be controlled by operator policy.

-	While the support of network slicing applies only to 5GS, solutions supports subscriptions that allows EPS/5GS interworking. Therefore EPS/5GS interworking needs to be considered for the enforcement of counts for maximum number of UEs per network slice and for maximum number of PDU Sessions per network slice."

## 4.3	Network Slice Management Charging

Network Slice Management Charging specified by 3GPP TS 28.202 [3], enables the Converged Charging System (CCS) to capture for each Network Slice instance per 3GPP TS 28.530 [8] definition, the supported list of service profiles representing the Service Level Specification (SLS) between Network Slice Customer (NSC) and Network Slice Provider (NSP).

For each S-NSSAI, associated attributes and their values representing the Service Level Specification (SLS) are available to the Converged Charging system, via service profiles captured with the NetworkSlice instance.

These information are captured in NSM charging CHF record generated by CHF upon NetworkSlice instance creation, modification and termination.

# 5	Potential charging requirements

REQ-NSCH-01	The 5G charging framework should support converged charging per S-NSSAI.

REQ-NSCH-02	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs".

REQ-NSCH-03	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions".

REQ-NSCH-04	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs", exempting Emergency Registered UEs

REQ-NSCH-05	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions", exempting Emergency services, Multimedia Priority Service, and Mission Critical Service.

REQ-NSCH-06	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs", including UEs connected in the EPS.

REQ-NSCH-07	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions", including PDN connections in the EPS.

REQ-NSCH-08	The 5G charging framework should support converged charging per tenant based on assigned group of S-NSSAI(s).

REQ-NSCH-09	The 5G charging framework should support converged charging per S-NSSAI, based on "aggregated PDU sessions volume".

REQ-NSCH-10	The 5G charging framework should support converged charging per S-NSSAI, based on "PDU session maximum duration".

# 6	Key Issues

## 6.1	Key Issue #1: Converged Charging for max. number of UEs per network slice

### 6.1.1	General Description

This key issue is for investigating how to support network slice access based converged charging service on max. number of UEs.

Maximum number of UEs is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice and defined as the maximum number of UEs that can use the network slice simultaneously. The max. number of UEs that can use the network slice simultaneously is one criteria for usage control of network slice, it is therefore an aspect to be considered for converged charging.

This investigation covers the following:

-	determination of max. number of UEs per slice that can use the network slice simultaneously to be used as a criteria for converged charging service;

-	identification of the main charging information to be collected in addition to max. number of UEs;

-	determination of the main interactions with the NF or service usage to obtain the network slice authorization and usage based on max. number of UEs;

- 	determination of all needed charging information;

### 6.1.2	Solution#1

### 6.1.3	Solution#2

### 6.1.x	Evaluation

## 6.2	Key Issue #2: Converged Charging for max. number of PDU sessions per network slice

### 6.2.1	General Description

This key issue is for investigating how to support network slice access based converged charging service. on max. number of PDU sessions.

Maximum number of PDU sessions is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice and defined as maximum number of concurrent PDU supported by the network slice. The number of max. number of PDU sessions is one criteria for usage control of network slice, it is therefore an aspect to be considered for converged charging.

This investigation covers the following:

-	determination of max. number of PDU sessions per slice that can use the network slice simultaneously to be used as a criteria for converged charging service;

-	identification of the main charging information to be collected in addition to max. number of PDU sessions;

-	determination of the main interactions with the NF or service usage to obtain the network slice authorization and usage based on max number of PDU sessions;

- 	determination of all needed charging information.

## 6.3	Key Issue #34: Volume based Converged Charging per network slice

### 6.3.1	General Description

This key issue is for investigating how to support network slice usage converged charging based on total PDU sessions volume per S-NSSAI.

The volume considered is the aggregated PDU sessions volume.

This investigation covers the following:

-	determination of which entity/entities in the 5G system is suitable to provide the network slice usage information based on volume;

-	determination of the main interactions required to obtain network slice usage information based on volume.

-	identification of the main charging information to be collected;

## 6.4	Key Issue #4: Converged Charging for max duration per network slice

### 6.4.1	General Description

This key issue is for investigating how to support network slice usage converged charging based on PDU sessions maximum duration per S-NSSAI.

This investigation covers the following:

-	determination of which entity/entities in the 5G system is suitable to provide the network slice usage information based on PDU sessions duration;

-	determination of the main interactions required to obtain network slice usage information based on PDU sessions duration.

-	identification of the main charging information to be collected;

## 6.5	Key Issue #5: Converged charging for number of simultaneous UEs per network slice

### 6.5.1	General Description

This key issue is for investigating how to support network slice access based converged charging service on number of simultaneous UEs.

The number of UEs is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice (one S-NSSAI) and defined as the maximum number of UEs that can use the network slice simultaneously. Since the maximum number of simultaneous UEs can part of the offering and in the extension part of the price for the offering, then also the number of UEs that use the network slice simultaneously can be a criterion for the charging (i.e. rate and cost). For example, if during a period it never goes above a specific number of simultaneous UEs one price and if it does go above another price.

This investigation covers the following:

-	determination of number of simultaneous UEs passing a threshold for a network slice;

-	identification of the main charging information to be collected in addition to number of UEs;

-	identification of the main interactions with the NFs to obtain the charging information;

- 	determination of need for online and offline charging methods.

### 6.5.2	Solution#1

### 6.5.3	Solution#2

### 6.5.x	Evaluation

## 6.6	Key Issue #6: Converged Charging for number of unique UEs per network slice

### 6.6.1	General Description

This key issue is for investigating how to support network slice access based converged charging service on number of UEs.

The number of Ues is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice (one S-NSSAI) and defined as the maximum number of Ues that can use the network slice simultaneously. This means that the maximum number of simultaneous Ues can part of the offering and in the extension part of the price for the offering. Simultaneous Ues will currently mean unique Ues (i.e. with a unique identifier) and at the same time. This can be extended to also allow to charge based on the number of unique Ues that connect to a network slice over a period of time. For example, if during a period the number of unique Ues is below a specific value one price and if it does go above another price for the network slice.

This investigation covers the following:

-	determination of number of unique Ues accessing the network slice passing a threshold;

-	identification of the main charging information to be collected in addition to number of Ues;

-	identification of the main interactions with the NFs to obtain the charging information;

- 	determination of need for online and offline charging methods.

Editor’s Note: how to determine that a UE is unique is ffs.

### 6.6.2	Solution#1

### 6.6.3	Solution#2

### 6.6.x	Evaluation

## 6.7	Key Issue #7: Converged charging for number of concurrent PDU sessions per network slice

### 6.7.1	General Description

This key issue is for investigating how to support network slice access based converged charging service on number of concurrent PDU sessions.

The number of PDU sessions is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice (one S-NSSAI) and defined as the maximum number of concurrent PDU sessions supported by the network slice. Since the maximum number of concurrent PDU sessions can be part of the offering and in the extension part of the price for the offering, then also the number of PDU sessions that use the network slice concurrently can be a criterion for the charging (i.e. rate and cost). For example, if during a period it never goes above a specific number of concurrent PDU sessions one price and if it does go above another price.

This investigation covers the following:

-	determination of number of concurrent PDU sessions passing a threshold for a network slice;

-	identification of the main charging information to be collected in addition to number of PDU sessions;

-	identification of the main interactions with the NFs to obtain the charging information;

- 	determination of need for online and offline charging methods.

### 6.7.2	Solution#1

### 6.7.3	Solution#2

### 6.7.x	Evaluation

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 01-2021 | SA5#135e | S5-211042 |  |  |  | Initial skeleton | 0.0.0 |
| 02-2021 | SA5#135e | S5-211416 S5-211418 S5-211044 S5-211045 S5-211421 S5-211385 S5-211387 |  |  |  | Update on skeleton Rel-17 pCR TR 32.847 – add scope Rel-17 pCR TR 32.847 – add new subclause on background Rel-17 pCR TR 32.847 – add new subclause on key Issues Rel-17 pCR TR 32.847 – Introduction of key Issue #1 & #2 Rel-17 pCR 32.847 Introduction of background on NS charging information Rel-17 pCR 32.847 Introduction of potential requirements | 0.1.0 |
| 02-2021 | SA5#135e |  |  |  |  | EditHelp review | 0.1.1 |
| 03-2021 | SA5#136e | S5-212475 S5-212476 S5-212477 S5-212478 |  |  |  | Rel-17 pCR TR 32.847 - update subclause on background from GSMA Rel-17 pCR 28.847 Introduction of new requirements and Key issues - volume and duration Rel-17 pCR 32.847 Adding key issue for number of UEs per network slice Rel-17 pCR 32.847 Adding key issue for number of PDU sessions per network slice | 0.2.0 |