3GPP TSG-SA5 Meeting #136-e	S5-212108

# Online, 1st – 9th March 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 00xx | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.535 Clean up the concepts and background |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCOSLA |  |  |  |  |  | Date: |  |  | 2021-02-19 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing concepts and background material includes uneccessary information related to the lifecycle of a communication service, basic infroamtion regarding open and closed loop that is not needed and repetitive text on the notion of the closed loop. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete section 4.1 and 4.2.2. Delete figure 4.2.1.2 and corresponding text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Document would contain unesseccary infroamtion that it may create confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.1, 4.2.2, 4.3.X (new), 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.536 CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[4]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[5]	ETSI GS ZSM 002 (V1.1.1) (2019-08): "Zero-touch network and Service Management (ZSM); Reference Architecture".

[6]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[7]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[10]	3GPP TS 28.530: “Management and orchestration; Concepts, use cases and requirements”.

2nd Modified Section

# 4	Concepts and background

Preparation

Commissioning

Operation

Decommissioning

## 4.1	Management control loops

Communication Service Assurance (CSA) applies to different phases in the life of communication services these lifecycle phases are; preparation, commissioning, operation and decommissioning [10].

For communication service assurance one can identify two interactions of management control loops:

1)	Between the CSC and the CSP: In this case, the CSC provides the requirements for an assured communication service to the CSP, the CSP provides the corresponding communication service, the CSP also provides feedback to the CSC. The CSP adjusts the resources used by a communication service or the CSC adjusts the SLS continuously to achieve the assured requirements.

2)	Between the CSP and the NSP: the communication service provided by CSP requires the network capabilities. For example, the CSP requires a certain network latency. The NSP management system adjusts the network or CSP adjusts the latency requirement continuously to satisfy the latency requirement.

Figure 4.1.1: Communication service closed control loop assurance

Figure 4.1.1 gives a high level description of interaction process involved in the management closed control loop.

A control loop is a building block for management of networks and services. The basic principle of any control loop is to adjust the value of a measured or observed variable (expressed as for example an attribute) to equal the value of a desired goal (expressed as for example an attribute). The producer of the measurements or observations, the control service, and the controlled entity are all required to create a control loop.

For the control loop to act on input in the context of the set goal, the control loop provided through following four steps that continuously consume and produce information from each other in a loop in the following sequence monitor, analyse, decide and execute.

## 4.2	Communication service assurance service

Communication service assurance relies on a set of management services that together provide the CSP with the capability to assure the communication service as per agreement (for example an SLS) with a CSC (e.g. enterprise). The overall solution and information flows between management services and the closed control loop steps [5] are shown in Figure 4.2.1.

Figure 4.2.1: Overview of closed control loop information flows

In Figure 4.2.1 the controlled entity represents the resources used by a communication service and the assurance of this communication service is provided by the closed control loop between the different management services provided by the management system.

The input to the closed control loop is the data concerning the resources used by the communication service and corrresponding service KPIs which is monitored by the closed control loop and step "Monitor", analyzed by the closed control loop step “Analyze”, a decision on potential solution by the closed control loop step "Decide" which may be a possible action for the closed control loop step "Execute", The role of the decision support services is to provide variable degrees of automated decision making and human oversight support. The following two examples demonstrate how a closed control loop can be used:

- when a service experience degradation is detected (for example due to resource shortage or faults in the network), the resources used by a communication service may be adjusted automatically to improve the service experience

- the data associated with the communication service is monitored by the management services for data collection, this management service provides information to an assurance root cause analysis management service (example of an analytics service) and based on that information the assurance root cause analysis takes place, followed by proposing activities, mitigation or suggestions to solve the problem. The proposed activities, for example mitigation or problem-solving suggestion(s) are executed through provisioning services to bring the behaviour of the communication service within the requested boundaries of the metrics (SLS goals) that are controlled by the closed control loop.

The management services available for the closed control loop steps for "Monitor", "Analyze" and "Decide" are based on file transfer described in TS 28.550 [3], or data streaming described in TS 28.550 [3] and notifications described in TS 28.545 [6].

The information provided from the "Monitor" step to the "Analyze" step includes performance measurements (see TS 28.552 [7]), KPI’s (see TS 28.554 [8]), performance threshold monitoring events and fault supervision events (see TS 28.532 [9]).

The insights provided from the "Analyze" step to the "Decide" step includes analytics outcomes that are not specified in the present document.

The decision support services provided from the "Decide" step to the "Execute" step are not specified in the present document.

End of modifications