3GPP TSG SA WG5 Meeting 136-e		TDoc S5-212224

# electronic meeting, online, 1 - 9 March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0169 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add operations for discovery of management services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S5 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GDMS |  |  |  |  |  | Date: |  |  | 2021-03-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Operations are missing for discovery of management services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add definitions of missing operations. Add RESTful HTTP-based solution set. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.x (new), 12.x (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | forge.3gpp.org/rep/sa5/MnS/commit/e6e2682c8637782796532f5c18e5b67e75191156 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

11.x	Management services discovery service

11.x.1	Operations and notifications

11.x.1.1	publishMnS operation (M)

11.x.1.1.1	Definition

This operation enables the producer of a Management Service (MnS) to publish data describing the MnS. This data includes a descriptive name for the MnS, and the information needed by a potential MnS consumer to uniquely address the operations, notifications, and information model related to the MnS.

11.x.1.1.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| mnsName | M | String | Descriptive name for the Management Service. |
| mnsType | M | String | Identifies the 3GPP solution that is supported by the Management Service. For OpenAPI-based services, this is defined by the parameter “title”. For NETCONF-based services, this is defined by the module-set name. |
| mnsVersion | CM | String | Identifies the version of 3GPP solution that is supported by the Management Service. For OpenAPI-based services, this is defined by the parameter “MnSVersion”. Not relevant for NETCONF-based services. The condition is that the solution set supports version information. |
| uriRoot | CM | String | Root URI for Management Service operations and notifications. For OpenAPI-based services, this is defined by the parameter “MnSRoot”. For NETCONF-based services, this is defined by the parameter “namespace”. The condition is that the Management Service contains Component Type A (operations and notifications). |
| dnRoot | CM | String | Root DN for Management Service resource model. For OpenAPI-based services, this is defined by the parameter “URI-LDN-first-part”. For NETCONF-based services, this is defined by the parameter “datastore”. The condition is that the Management Service contains Component Type B (information model). |

11.x.1.1.3	Output parameters

| Parameter Name | Support Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| mnsDataId | M | String | A unique identifier to be used in the removeMnS operation. |
| status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

11.x.1.2	removeMnS operation (M)

11.x.1.2.1	Definition

This operation enables the producer of a Management Service (MnS) to remove published data describing the MnS.

11.x.1.2.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| mnsDataId | M | String | A unique identifier from the publishMnS operation. |

11.x.1.2.3	Output parameters

| Parameter Name | Support Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| Status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

11.x.1.3	queryMnS operation (M)

11.x.1.3.1	Definition

This operation enables the potential consumer of a Management Service (MnS) to retrieve data describing the available MnSs in the 3GPP Management System.

By default, the reponse contains a list of all available MnSs in the 3GPP Management System. The query may optionally include parameters to reduce the scope of the query, and thereby reduce the size of the response to include only available MnSs which have matching parameter values.

11.x.1.3.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| mnsType | O | String | Identifies the 3GPP solution that is supported by the Management Service. For OpenAPI-based services, this is defined by the parameter “title”. For NETCONF-based services, this is defined by the module-set name. |
| mnsVersion | O | String | Identifies the version of 3GPP solution that is supported by the Management Service. For OpenAPI-based services, this is defined by the parameter “MnSVersion”. Not relevant for NETCONF-based services. The condition is that the solution set supports version information. |

11.x.1.3.3	Output parameters

| Parameter Name | Support Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| mnsList | M | List of MnS data | A list of the MnS data for the available MnSs in the 3GPP Management System. An empty list indicates that there are no available MnSs in the 3GPP Management System. If input parameter mnsType and/or mnsVersion was specified, the list contains the available MnSs which have matching parameter values, or an empty list if there are no matching MnSs in the 3GPP Management System. |
| Status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

The following table defines an item of mnsList.

| Parameter name | Support Qualifier | Matching information | Comment |
| --- | --- | --- | --- |
| mnsName | M | String | Descriptive name for the Management Service. |
| mnsType | M | String | Identifies the 3GPP solution that is supported by the Management Service. For OpenAPI-based services, this is defined by the parameter “title”. For NETCONF-based services, this is defined by the module-set name. |
| mnsVersion | CM | String | Identifies the version of 3GPP solution that is supported by the Management Service. For OpenAPI-based services, this is defined by the parameter “MnSVersion”. Not relevant for NETCONF-based services. The condition is that the solution set supports version information. |
| uriRoot | CM | String | Root URI for Management Service operations and notifications. For OpenAPI-based services, this is defined by the parameter “MnSRoot”. For NETCONF-based services, this is defined by the parameter “namespace”. The condition is that the Management Service contains Component Type A (operations and notifications). |
| dnRoot | CM | String | Root DN for Management Service resource model. For OpenAPI-based services, this is defined by the parameter “URI-LDN-first-part”. For NETCONF-based services, this is defined by the parameter “datastore”. The condition is that the Management Service contains Component Type B (information model). |

2nd change

12.x	Management services discovery service

12.x.1	RESTful HTTP-based solution set

12.x.1.1	Mapping of operations

12.x.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.x.1.1.1-1.

Table 12.1.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| publishMnS | POST | {MnSRoot}/DiscoveryMnS/{MnSVersion} | M |
| removeMnS | DELETE | {MnSRoot}/DiscoveryMnS/{MnSVersion}/mnsDataId | M |
| queryMnS | GET | {MnSRoot}/ DiscoveryMnS/{MnSVersion} | M |

12.x.1.1.2	Operation "publishMnS"

This operation publishes MnS data.

Table 12.x.1.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| mnsName | request body | mnsName | string | M |
| mnsType | request body | mnsType | string | M |
| mnsVersion | request body | mnsVersion | string | O |
| uriRoot | request body | uriRoot | string | O |
| dnRoot | request body | dnRoot | string | O |

Table 12.x.1.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP PUT)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| mnsDataId | response body | mnsDataId | string | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for creating a resource is as follows:

1.	The MnS consumer sends a HTTP POST request to the MnS producer.

- The message body shall carry the complete representation of the MnS data.

2.	The MnS producer sends a HTTP POST response to the MnS consumer.

- On success, "201 Created" shall be returned. The response message body contains a unique identifier.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

12.x.1.1.3	Operation "removeMnS"

This operation removes published MnS data.

Table 12.x.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| mnsDataId | path | /{mnsDataId} | string | M |

Table 12.x.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for creating a resource is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

- The message path shall include a unique identifier for the MnS data.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

- On success, "204 No content" shall be returned.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

12.x.1.1.4	Operation "queryMnS"

This operation retrieves MnS data for available Management Services.

Table 12.x.1.1.4-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| mnsType | query | mnsType | string | O |
| mnsVersion | query | mnsVersion | string | O |

Table 12.x.1.1.4-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| mnsList | response body | mnsList | Array of object | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for retrieval of one or multiple resources is as follows:

1.	The MnS consumer sends a HTTP GET request to the MnS producer.

- If present, the query parameters indicate that the scope of the query should be limited to the specified MnS type and/or version.

- The query parameters are applied to the set of available MnSs. Only MnSs passing the criteria are targeted.

2.	The MnS producer sends a HTTP GET response to the MnS consumer.

- On success, "200 OK" shall be returned.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

3rd change

A.x	Management services discovery service

A.x.1	OpenAPI document "discoveryMnS.yaml"

openapi: 3.0.1

info:

title: Management Services Discovery MnS

version: 16.6.0

description: >-

OAS 3.0.1 definition of the Management Services Discovery MnS

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532 V16.6.0; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/DiscoveryMnS/{MnSversion}'

variables:

MnSRoot:

description: See subclause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSversion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/:

get:

summary: Query MnS data

description: Read data describing available Management Services

parameters:

- name: mnsType

in: query

required: false

schema:

type: string

- name: mnsVersion

in: query

required: false

schema:

type: string

responses:

'200':

description: >-

Success case ("200 OK").

The MnS data is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ManagementServicesDataList'

default:

description: Response in case of error.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

post:

summary: Publish MnS data

description: Publish data describing an available Management Service

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ManagementServicesSubmit'

responses:

'201':

description: >-

Success case ("201 Created "). The operation is successful.

The MnS data identifier is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ManagementServicesId'

default:

description: Response in case of error.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

/{mnsDataId}:

delete:

summary: Remove MnS data

description: Remove data describing an available Management Service

parameters:

- name: mnsDataId

in: path

description: Identifies the MnS data to be deleted.

required: true

schema:

type: string

responses:

'204':

description: >-

Success case ("204 No Content"). The operation is successful.

The response message body is absent.

default:

description: Response in case of error.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

ManagementServicesSubmit:

type: object

properties:

mnsName:

type: string

mnsType:

type: string

mnsVersion:

type: string

default: ""

uriRoot:

type: string

default: ""

dnRoot:

type: string

default: ""

ManagementServicesId:

type: object

properties:

mnsDataId:

type: string

ManagementServicesData:

type: object

properties:

mnsDataId:

type: string

mnsName:

type: string

mnsType:

type: string

mnsVersion:

type: string

uriRoot:

type: string

dnRoot:

type: string

ManagementServicesDataList:

type: array

items:

$ref: '#/components/schemas/ManagementServicesData'

End of changes