3GPP TSG SA WG5 Meeting 137-e		TDoc S5-213435

# electronic meeting, online, 10 - 19 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0046 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-introduce use cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S5 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Work item code: | eCOSLA |  |  |  |  |  | Date: |  |  | 2021-04-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following two use cases where removed from Rel-16 last meeting and Rel-17 this meeting (mirror CR XXX) - obtaining resource requirements for a communication service - interaction with core network for service assurance This CR proposes to re-introduce the modified use case in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses are introduced |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.3	Obtaining resource requirements for a communication service

Once a request for a communication service is received, the 3GPP management system needs to identify the network resources to be used by this service in order to do service assurance. For example, during the feasibility study, in order to assure the performance, the 3GPP management system should be able to determine the network availability for that service. This could be done by informing the MDAS provider about the network requirements and check if the requirements can be met with the available network resources.

MDAS provider may already have network requirement for a given service requirement, which is obtained by the historical analysis using offline or online monitoring of network resource usage of similar services. The 3GPP management system may check the feasibility of provisioning the communication services by using the existing network, and if feasible, provision the communication services using that network to go to the operational phase If the network requirement cannot be determined (e.g. not sufficient prior data), the 3GPP management system may assign certain amount of the initial network resources and limit the number of users admitted by configuring the appropriate CN functions. The resource usage information and the services using those resources in a given time period with their performance (e.g. delay) is monitored by the 3GPP management system for different number of UEs to learn the network resource requirement for different service requirements. This data could be used to determine network requirements for future service requests during the provisioning phase or to adjust network resources to reflect the changing service demands for the already admitted communication services.

It may be a continuous learning process in the run-time phase since service degradation could happen due to various reasons and network resources may need to be adjusted to address such situations.

REQ-CSA_RR-CON-01 The 3GPP management system shall be able to determine the network resource requirement for a given communication service requirement.

REQ-CSA_RR-CON-02 The 3GPP management system shall be able to allocate certain amount of network resources for a communication service and configure the 5GC functions to limit the number of users of a given communication service.

### 5.1.4	Interaction with core network for service assurance

The goal is to enable the 3GPP management system to take early action to prevent service degradation.

The 3GPP management system configures the control plane functions (e.g. NWDAF) so as to report potential service degradation according to the SLS. Service load can be determined by considering both NF(s) load in 5GC and network utilization in access network. If the service degradation occurs or is predicted when the network capacity is decreased, network capacity could be increased to solve the issue. Therefore, it is necessary for the 3GPP management system to configure the 5GC functions such that in the event of a potential service degradation or prediction of overloading, a notification is sent to the 3GPP management system. This can be done by configuring the overloading conditions (e.g. triggering parameters) in the 5GC functions for a selected service. The 3GPP management system could configure the 5GC functions to trigger when the service load is increased or predicted to be increased beyond a certain threshold level. The 3GPP management system could then increase the network capacity or use an MDAS to find a proper solution.

Similarly, when the network resources are underutilized the 3GPP management system could decrease the network capacity.

REQ-CSA_RR-CON-01 The 3GPP management system shall be able to configure the 5GC functions to enable reporting of a potential service load increase beyond a certain threshold so that the 3GPP management system can increase the network resource capacity in time without impacting the SLS.

REQ-CSA_RR-CON-02 The 3GPP management system shall be able to determine the service load thresholds that need to be used by the 5GC functions to report, so that a potential network resource overprovisioning situation can be ascertained.

REQ-CSA_RR-CON-03 The 3GPP management system shall be able to reduce network resource capacity when a network resource overprovisioning situation is detected, and the overprovisioned capacity is not needed.

End of change