| 3GPP TS 28.100 V0.5.0 (2021-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Levels of autonomous network; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Concepts and background	8

4.1	Autonomous network concept	8

4.2	Autonomous network level concept	8

4.3	Dimensions for evaluating autonomous network levels.	8

4.3.1 	Introduction	8

4.3.2	Scenarios	8

4.3.3	Management scope	9

4.3.4	Workflow	9

5	Framework approach for evaluating autonomous network levels	10

6	Use cases and requirements	11

6.1	Network and service planning scenarios	11

6.2	Network and service deployment scenarios	12

6.2.1 Autonomous network level for RAN NE deployment	12

6.3	Network and service maintenance scenarios	12

6.3.1 Autonomous network level for fault management	12

6.3.1.1	Introduction	12

6.3.1.2	Workflow	12

6.3.1.3	Classification of autonomous network level	13

6.3.1.4	Requirements	14

6.3.1.4.1 Requirements to support autonomous network level 1	14

6.3.1.4.2 Additional requirements to support autonomous network level 2	15

6.3.1.4.3 Additional requirements to support autonomous network level 3	15

6.3.1.4.4 Additional requirements to support autonomous network level 4	15

6.3.1.4.5 Additional requirements to support autonomous network level 5	16

6.4	Network and service optimization scenarios	16

6.4.1 Autonomous network level for radio network coverage optimization	16

6.4.2 Autonomous network level for RAN UE throughput optimization	16

7	Generic autonomous network level	16

7.1	Generic autonomous network level for network optimization	17

7.1.1	Generic workflow	17

7.1.2 Generic classification of autonomous network level	17

7.1.3 Generic requirements	19

7.1.3.1 Requirements to support autonomous network level 1	19

7.1.3.2 Additional requirements to support autonomous network level 2	19

7.1.3.3 Additional requirements to support autonomous network level 3	19

7.1.3.4 Additional requirements to support autonomous network level 4	20

7.1.3.5 Additional requirements to support autonomous network level 5	20

7.2	Generic autonomous network level for RAN NE deployment	20

7.2.1	Generic workflow	20

7.2.2 Generic autonomous network level	21

7.2.3 Generic requirements	22

7.2.3.1 Requirements to support autonomous network level 1	22

7.2.3.2 Requirements to support autonomous network level 2	22

7.2.3.3 Requirements to support autonomous network level 3	22

7.2.3.4 Requirements to support autonomous network level 4	23

7.2.3.5 Requirements to support autonomous network level 5	23

7.3	Generic autonomous network level for fault management	23

7.3.1	Generic workflow	23

7.3.2 Generic autonomous network level	24

7.3.3 Generic requirements	25

7.3.3.1 Requirements to support autonomous network level 1	25

7.3.3.2 Additional requirements to support autonomous network level 2	26

7.3.3.3 Additional requirements to support autonomous network level 3	26

7.3.3.4 Additional requirements to support autonomous network level 4	26

7.3.3.5 Additional requirements to support autonomous network level 5	27

Annex A (informative): Examples of Autonomous network level for network and service optimization	27

A.1 Autonomous network level for radio network coverage optimization	27

A.1.1 Workflow	27

A.1.2 Classification of autonomous network level	28

A.2 Autonomous network level for RAN UE throughput optimization	29

A.2.1 Workflow	29

A.2.2 Classification of autonomous network level	30

Annex <X> (informative): Change history	32

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

Should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

Can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

Will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

Autonomous network is one of the important topics in 5G network. Complexity of 5G network increases with large number of devices and diversity of services. Different autonomous mechanisms are introduced by the industry to reduce the complexity of mobile network and service management. Moving from a manual operating network to a fully autonomous network requires a stepwise progression. For each step there are different capability and performance level of autonomy. Thus the concept of autonomous network levels and corresponding requirements are introduced to describe and evaluate each level in details.

1	Scope

The present document specifies the concepts, use cases and requirements for the levels of autonomous network.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[3]	3GPP TS 28.535: "Management Services for Communication Service Assurance; Requirements".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Autonomous Network: telecommunication system (including management system and network) with autonomy capabilities which is able to be governed by itself with minimal to no human intervention.

Autonomous Network Level: describes the level of autonomy capabilities in the autonomous network.

Network and Service Planning: Editor's Note: network and service planning is FFS.

Network and Service Deployment: processes of allocation, installation, configuration, activation and verification of specific network and service.

Network and Service Maintenance: processes of monitoring, analysing and healing of the network and service issues.

Network and Service Optimization: processes of monitoring, analysing and optimizing the network and service performance.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<ABBREVIATION>	<Expansion>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AON	Autonomous Network

ANL	Autonomous Network Level

4	Concepts and background

Editor's note: this clause will contain the concept of autonomous network levels, dimensions for evaluating autonomous network levels.

4.1	Autonomous network concept

The network become complex due to large number of devices and diversity of services. Different autonomy mechanisms are introduced in the industry to reduce the complexity of network management and control. The ultimate goal for autonomous network is to enable telecommunication system (including management system and network) to be governed by itself with minimal to no human intervention by utilizing the autonomy mechanisms (including intelligence mechanism, e.g. AI/ML, and automation mechanism, e.g. rule-based automatic control, and other mechanisms to enable the autonomous network). Autonomous network can reduce the operating expenditure (OPEX) associated with the autonomous management and control of the complexity network and improve the service experience to enable various vertical industries (e.g. autonomous vehicle, smart city). Following are concepts related to autonomous network.

- Autonomous network is telecommunication system (including management system and network) with autonomy capabilities which is able to be governed by itself with minimal to no human intervention.

- Autonomous network level is used to describe the level of autonomy capabilities in the autonomous network.

- Self-Organization Network, Management data analytics, Intent driven management, closed loop SLS assurance are examples of enablers for autonomous network.

4.2	Autonomous network level concept

Different autonomy mechanisms in the telecommunication system may lead to different capabilities of autonomy and different operation efficiency on network management and control workflow, and indicates the level of autonomy of the network. The term Autonomous network level is used to describe the levels of autonomy capabilities in the autonomous network to improve the efficiency for network management and control. Participation of the human and telecommunication system in the network management and control workflow are different for each level and are important factors to evaluate the autonomous network levels. For each autonomous network level, some tasks can be performed by telecommunication system, some performed by human, and some performed by cooperation of human and telecommunication system. For example, in the highest autonomous network level, all tasks are performed by telecommunication system.

Editor's Note: the concept of autonomous network and autonomous network level need to be revised and coordinated with other SDOs.

4.3	Dimensions for evaluating autonomous network levels.

4.3.1 	Introduction

This clause describes the dimensions i.e. scenarios, management scope and workflow, which can be used for evaluating autonomous network level.

4.3.2	Scenarios

The autonomous network can be implemented for different scenarios, the complexity of autonomous network depends on the detailed scenarios where it is applied. Also it will be more challenging for the telecommunication system to achieve the autonomous network for full scenarios than for certain scenarios.  The autonomy capabilities of the scenarios will impact the autonomous network level for the whole autonomous network.

Following are scenario types categorized by network and service management process for autonomous network:

- Network and service planning

- Network and service deployment

- Network and service maintenance

- Network and service optimization

Editor's note: network and service planning is FFS.

4.3.3	Management scope

The autonomy can be implemented in different scopes, the complexity of autonomous network depends on its applicable scope. For example, it will be more challenging for the telecommunication system to achieve the autonomous network on cross domain network layer than domain network layer, because more autonomy mechanism needs to be introduced for the coordination between different domains. The autonomy capabilities of the management scope will impact the autonomous network level for the whole autonomous network.

Following are applicable scopes for autonomous network:

-	Autonomy in NE/NF layer, which means the autonomy mechanisms are executed in the NE/NF.

-	Autonomy in domain network layer, which means the autonomy mechanisms are executed in the MnF(s) in domain.

-	Autonomy in cross domain network layer, which means the autonomy mechanisms are executed in the MnF(s) in cross domain.

-	Autonomy in communication service layer, how to execute the autonomy mechanisms are executed in MnF(s) for communication service.

Editor's Note: autonomy in communication service layer is FFS.

Figure 4.3.3-1: Autonomy for different management scope

4.3.4	Workflow

Workflow is used to describe the necessary steps to achieve certain management and control purposes. A workflow is composed of one or more management and control tasks. Each workflow task may be accomplished by human, or accomplished by telecommunication system with human assistance, or accomplished by telecommunication system without human intervention. The autonomy capabilities of the tasks in the workflow may impact the autonomous network level.

Following are the categorization of the tasks in a workflow:

-	Intent handling: The group of tasks which translate network or service intent from operator or customer into detailed operations and/or policies which may affect one or more of the following groups of tasks (i.e. awareness, analysis, decision, execution), also evaluate and feedback intent fulfilment information (e.g. the intent is satisfied or not) based on the detailed network and service information. More information of intent handling see the Intent driven management defined in 28.312[1].

-	Awareness: The group of tasks which include network and service data (e.g. configuration data, performance data, alarm data, etc.) collection and necessary data pre-processing (e.g. data cleaning, filtering, statistics, etc.) with the purpose of monitoring network and service information (including network and service performance, network and service anomaly, network and service event, etc.). More information of Awareness see corresponding Monitor of Management control loops defined in 28.535[2].

-	Analysis: The group of tasks which analyse the obtained network and service information (e.g. network and service status, network and service issues and so on) or based on the historical network and service information to further predict the future change trend of the above network and service status, and make recommendation for decision. More information of Analysis see corresponding Analytic of Management control loops defined in 28.535[2].

-	Decision: The group of tasks which evaluate and decide the necessary operation for execution, e.g. network configuration or adjustment. More information of Decision see corresponding Decision of Management control loops defined in 28.535[2].

-	Execution: The group of tasks which execute the operations. More information of Execution see corresponding Execution of Management control loops defined in 28.535[2].

Figure 4.3.4-1: Categorization of the tasks in the workflow for evaluating autonomous network levels

5	Framework approach for evaluating autonomous network levels

Editor's Note: the framework approach for evaluating autonomous network levels need to be revised and coordinated with other SDOs.

A framework approach for evaluating autonomous network levels is as following, which is used for evaluating the autonomy capability of telecom system. In the following framework table,

-	“Human” represents corresponding tasks are accomplished by human or human utilizing the tools for network and service management and orchestration.

-	 “Human & System” represents corresponding tasks are accomplished by collaboration of human and telecom system, the detailed collaboration pattern depends on the scenario, which is not addressed in the framework approach for evaluating autonomous network levels.

-	 “System” represents corresponding tasks are fully accomplished by telecom system.

Table 5-1: Framework approach for evaluating autonomous network levels

| Network autonomy level |  | Task categories |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Execution | Awareness | Analysis | Decision | Intent handling |
| L0 | Manual operating network | Human | Human | Human | Human | Human |
| L1 | Assisted operating network | Human & Telecom system | Human & Telecom system | Human | Human | Human |
| L2 | Preliminary autonomous network | Telecom system | Human & Telecom system | Human & Telecom system | Human | Human |
| L3 | Intermediate autonomous network | Telecom system | Telecom system | Human & Telecom system | Human & Telecom system | Human |
| L4 | Advanced autonomous network | Telecom system | Telecom system | Telecom system | Telecom system | Human & Telecom system |
| L5 | Full autonomous network | Telecom system | Telecom system | Telecom system | Telecom system | Telecom system |
| Note 1: Human reviewed decision have the highest authority in each level if there is any confliction between human reviewed decision and telecom system generated decision. Note 2: The order of above five task categories does not reflect the workflow sequence. |  |  |  |  |  |  |

Level 0 manual operating network: No categorization of the tasks is accomplished by telecom system itself.

Level 1 assisted operating network: A part of the execution and awareness tasks are accomplished automatically by telecom system itself based on human defined rules. At this level, telecom system can assist human to improve the execution and awareness efficiency.

Level 2 preliminary autonomous network: All the execution tasks are accomplished automatically by telecom system itself. A part of the awareness and analysis tasks are accomplished automatically by telecom system itself based on human defined policies. At this level, telecom system can assist human to achieve the closed loop based on human defined policies.

Level 3 intermediate autonomous network: All the execution and awareness tasks are accomplished automatically by telecom system itself. A part of the analysis and decision tasks are accomplished automatically by telecom system itself based on human defined policies. At this level, the telecom system can achieve the closed loop automation based on the human defined closed loop automation policies.

Level 4 advanced autonomous network: All the execution, awareness, analysis and decision tasks are accomplished automatically by telecom system itself. And intent handling tasks can be partly accomplished automatically by telecom system itself based on human defined intent handling policies. At this level, telecom system can achieve the intent driven closed loop automation based on human defined intent handling policies, which means the telecom system can translate the intent to the detailed closed loop automation policies and translate the detailed network and service information to intent fulfilment information (e.g. the intent is satisfied or not) based on human defined intent handling policies.

Level 5 fully autonomous network: The entire network autonomy workflow is accomplished automatically by telecom system without human intervention. At this level, telecom system can achieve the whole network autonomy.

Note: 	Above framework approach for evaluating autonomous network levels is applicable for evaluating the autonomous network level from both management scope and scenario perspective. The overall autonomous network level of the whole telecom system is a comprehensive reflection of autonomous network level of the individual management scope and scenarios, which means in fully autonomous network level, the telecom system can achieve the whole network autonomy for all management scopes and scenarios.

6	Use cases and requirements

6.1	Network and service planning scenarios

Editor’s Note: this clause will contain the typical management use cases corresponding to telecom network and communication service planning scenarios and related management requirements for each autonomous levels. The management requirements include architecture requirements, enhancements of management and orchestration, important data and data collection policy and key management requirements on autonomous network related features.

6.2	Network and service deployment scenarios

Editor's Note: this clause will contain the typical management use cases corresponding to telecom network and communication service deployment scenarios and related management requirements for each autonomous levels. The management requirements include architecture requirements, enhancements of management and orchestration, important data and data collection policy and key management requirements on autonomous network related features.

6.2.1 Autonomous network level for RAN NE deployment

RAN NE deployment use case refers to the entire workflow of deploying an RAN NE, full autonomy of RAN NE deployment can help the network operator to reduce OPEX by reducing manual involvement in such tasks. However, full autonomy of RAN NE deployment is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing network autonomy mechanisms in corresponding steps. The requirements for each autonomous level for RAN NE deployment are different.

6.3	Network and service maintenance scenarios

Editor's Note: this clause will contain the typical management use cases corresponding to telecom network and communication service maintenance scenarios and related management requirements for each autonomous levels. The management requirements include architecture requirements, enhancements of management and orchestration, important data and data collection policy and key management requirements on autonomous network related features.

6.3.1 Autonomous network level for fault management

Editor's Note: whether the following use case need to be generalized for common maintenance scenarios is FFS.

6.3.1.1	Introduction

Fault management use case refers to the entire workflow of network fault management, autonomy of fault management can help the network operator to reduce OPEX by reducing manual involvement in such tasks and to enhance user experience and reduce network and service failure time by reducing the time for network fault supervision and recovery. However, full autonomy of fault management is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing network autonomy mechanism in corresponding steps. The requirements for each autonomous level for fault management autonomy are different.

6.3.1.2	Workflow

Following are the entire workflow for fault management:

Intent handling:

-	Task A: Fault management policies generation. The group of tasks of generating the fault management related rules and/or policies (e.g., alarm filtering rules, fault recognition and root cause analysis policies, fault recovery policies) based on fault management intent (e.g. reduce fault recovery response time to a certain value, reduce network and service failure times to a certain value within a specific duration).

-	Task B: Fault management intent fulfilment evaluation. The group of tasks of evaluating fault management intent fulfilment information (e.g. corresponding fault recovery response time is satisfied or not).

Awareness:

-	Task C: Fault related information collection. The group of tasks which collect the alarm information and other fault related information (e.g. performance information and configuration information etc.).

-	Task D: Alarm filtering. The group of tasks which filter the alarms collected in Task C based on the filtering rules specified. A single network fault may generate a large number of correlative alarms over space and time, therefore it is considered advantageous to have methods filtering the redundant alarms. Reporting only effective alarms without redundant alarms would improve the efficiency of alarm management.

Analysis:

-	Task E: Fault recognition. The group of tasks which recognize the fault and its category (including fault domain, fault type) based on the alarm information and other fault related information.

-	Task F: Fault prediction. The group of tasks which predict the potential fault and its category based on the performance information and other fault related information.

-	Task G: Fault root cause analysis (RCA). The group of tasks which analyse the detailed root cause of the network and service failure.

-	Task H: Fault recovery mechanism analysis. The group of tasks which analyse the possible fault recovery mechanisms based on the fault root cause, thereby generate the feasible options (e.g. recommended recovery solutions).

Decision:

-	Task I: Fault management action generation. The group of tasks which evaluate the feasible options and determine the fault recovery solutions and other corresponding actions (e.g. clear of alarms, storage and retrieval of alarms).

Execution:

-	Task J: Fault management action execution. The group of tasks which execute the fault recovery actions and other corresponding actions.

Note: If the faulty resource has no redundancy (e.g. backup equipment/board/battery/transport link) and all the fall back recovery actions are not available nor effective, then the subsequent fault recovery actions (e.g. replace physical equipment/board/battery, repair the physical connector/fibre/cable, repair the power supply, etc.) are considered as beyond the capabilities of the telecom system. And those actions execution is excluded from the consideration of autonomous network level classification.

6.3.1.3	Classification of autonomous network level

Level 0:

-	All the tasks in the fault management workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G, Task H,  Task I,  Task J) are accomplished by human.

Level 1:

-	Telecom system executes the tasks of fault related information collection (Task C) automatically based on predefined rules, and the tasks of fault management action execution (Task J) based on specified action execution rules. Telecom system can also execute the tasks of alarm filtering (Task D) based on specified alarm filtering rules.

-	All the other tasks in the fault management workflow (Task A, Task B, Task E, Task F, Task G, Task H,  Task I) are accomplished by human.

Level 2:

-	Compared to Level 1, telecom system can additionally executes the tasks of fault recognition (Task E) based on specified rules.  The tasks of fault management action execution (Task J), fault related information collection (Task C) and alarm filtering (Task D) are fully accomplished by telecom system based on specified rules.

-	All the other tasks (Task A, Task B, Task F, Task G, Task H,  Task I) are accomplished by human.

Level 3:

-	Compared to Level 2, telecom system can additionally execute the tasks of fault root cause analysis based on specified analysis policies (Task G). For certain network faults, telecom system can also additionally execute the tasks of fault recovery mechanism analysis (Task H) and action generation (Task I) based on specified fault management policies.

-	All the other tasks (Task A, Task B, Task F) are accomplished by human.

Level 4:

-	Compared to Level 3, the telecom system can additionally execute the task of fault prediction (Task F). For certain scenario, telecom system additionally execute the tasks of fault management policies generation (Task A) and fault management intent fulfilment information evaluation (Task B) based on specified intent handling policies. The tasks of fault recovery mechanism analysis (Task H) and action generation (Task I) are accomplished automatically by telecom system without human intervention.

-	Intent handling policies can be pre-defined and specified by human.

Level 5:

-	The entire fault management workflows is accomplished by telecom system without human intervention and human predefined rules or policies.

-	Human can optionally supervise the fault management action generated by telecom system.

Figure 6.3.1.3-1: Classification of autonomous network level for fault management

6.3.1.4	Requirements

Following are the functional requirements and interface requirements for 3GPP management system to support each autonomous network level (except for level 0) for fault management.

Editor’s note: all requirements that do not have any stage 2 and 3 solution shall be deleted before this TS is made public.

6.3.1.4.1 Requirements to support autonomous network level 1

REQ-ANL-Level_1-FM-FUN-1 The 3GPP management system shall have autonomy capability to execute the network recovery actions (including reset the hardware/software, switch to the backup hardware, rollback to the backup software/firmware) based on the specified fault recovery rules.

REQ-ANL-Level_1-FM-FUN-2 The 3GPP management system shall have autonomy capability to collect fault related data (including alarms, events, signalling data, performance data, configuration data and environment data) based on specified collection rules.

REQ-ANL-Level_1-FM-FUN-3 The 3GPP management system shall have autonomy capability to filter the alarms based on the specified filtering rules.

REQ-ANL-Level_1-FM-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault recovery rules.

REQ-ANL-Level_1-FM-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault related data collection rules.

REQ-ANL-Level_1-FM-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the alarm filtering rules

REQ-ANL-Level_1-FM-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain fault related data (including alarms, events, signalling data, performance data, configuration data and environment data).

REQ-ANL-Level_1-FM-MnS-5 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the filtered alarm data.

6.3.1.4.2 Additional requirements to support autonomous network level 2

REQ-ANL-Level_2-FM-FUN-1 The 3GPP management system shall have the autonomy capability to recognize the fault category (including fault domain and type) based on specified fault recognition rules.

REQ-ANL-Level_2-FM-MnS-1. The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault recognition rules.

REQ-ANL-Level_2-FM-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the fault recognition information (including fault domain and type information).

6.3.1.4.3 Additional requirements to support autonomous network level 3

REQ-ANL-Level_3-FM-FUN-1 The 3GPP management system shall have autonomy capability to analyse the root cause of the network fault based on specified fault root cause analysis policies.

REQ-ANL-Level_3-FM-FUN-2 The 3GPP management system shall have autonomy capability to analyse and generate the recommended fault recovery mechanism and determine the fault recover actions to be executed based on specified fault recovery mechanism analysis and decision policies.

REQ-ANL-Level_3-FM-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault root cause analysis policies.

REQ-ANL-Level_3-FM-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault recovery mechanism analysis policies.

REQ-ANL-Level_3-FM-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault recovery mechanism decision policies.

REQ-ANL-Level_3-FM-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the root cause of the network fault.

REQ-ANL-Level_3-FM-MnS-5 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the recommended fault recovery mechanism.

REQ-ANL-Level_3-FM-MnS-6 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the fault recovery actions to be executed.

6.3.1.4.4 Additional requirements to support autonomous network level 4

REQ-ANL-Level_4-FM-FUN-1 The 3GPP management system shall have autonomy capability to generate or update fault management policies according to fault management intent based on specified intent translation policies.

REQ-ANL-Level_4-FM-FUN-2 the 3GPP management system shall have autonomy capability to evaluate fault management intent fulfilment based on specified intent evaluation policies.

REQ-ANL-Level_4-FM-FUN-3 The 3GPP management system shall have autonomy capability to predict the potential fault.

REQ-ANL-Level_4-FM-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault management intent.

REQ-ANL-Level_4-FM-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault management intent translation policies.

REQ-ANL-Level_4-FM-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault management evaluation policies.

REQ-ANL-Level_4-FM-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the fault management fulfilment information.

REQ-ANL-Level_4-FM-MnS-5 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the potential fault prediction information.

6.3.1.4.5 Additional requirements to support autonomous network level 5

REQ-ANL-Level_5-FM-FUN-1 The 3GPP management system shall have autonomy capability to generate the fault management intent translation and evaluation policies.

6.4	Network and service optimization scenarios

Editor's Note: this clause will contain the typical management use cases corresponding to telecom network and communication service optimization scenarios and related management requirements for each autonomous levels. The management requirements include architecture requirements, enhancements of management and orchestration, important data and data collection policy and key management requirements on autonomous network related features.

6.4.1 Autonomous network level for radio network coverage optimization

Radio networks are geographically distributed, and mobile user activity varies significantly in different places and at different times of day. To achieve the optimal coverage, a set of initial coverage configuration parameters (e.g. coverageShape, digitalTilt and digitalAzimuth) may not always meet the requirements. Therefore, the coverage configuration parameters need to be adjusted in a differentiated manner with the change of the radio network environment. It’s complex for the adjustment due to multiple factors needs to be considered, e.g. interference control, huge data and frequent traffic changes. So introducing the autonomous network level for radio network coverage optimization will benefit for operator to achieve the full autonomy goal step by step and have clear view on which typical issues can be addressed by utilizing autonomy mechanism in corresponding steps. The requirements for each autonomous level for radio network coverage optimization are different.

6.4.2 Autonomous network level for RAN UE throughput optimization

RAN UE throughput is a key performance for radio network, and, numerous radio feature parameters with broad value ranges affect the RAN UE throughput. There are many radio feature parameters of RAN NE level or cell level, for examples, MLB related parameters, resource schedule related parameters, cell reselection related parameters, handover related parameters, different radio feature parameters affect each other and the combination number of various parameter values is large. Therefore, it’s complex to adjust radio feature configuration parameters to achieve the optimized RAN UE throughput. So introducing the autonomous network level for RAN UE throughput optimization will be benefit for operator to achieve the full autonomy goal step by step and have clear view on which typical issues can be addressed by utilizing autonomy mechanism in corresponding steps. The requirements for each autonomous level for RAN UE throughput optimization are different.

7	Generic autonomous network level

Editor’s Note: this clause will contain the generic requirements for autonomous network level.

7.1	Generic autonomous network level for network optimization

7.1.1	Generic workflow

Following is the generic entire workflow for the network optimization:

Intent handling:

-	Task A: Network optimization policies generation and determination. The tasks of generating and determining the network optimization related policies (e.g., network issue identification policies, network issue analysis policies and network parameters adjustment policies) based on received network optimization intent (e.g. network targets for the specified areas).

-	Task B: Network optimization intent fulfilment evaluation. The tasks of evaluating network optimization intent fulfilment information (e.g. corresponding network targets are satisfied or not).

Awareness:

-	Task C: Network related information collection. The tasks of collecting network related data, including network performance data (i.e. performance measurement, MDT data), network configuration data and environment data (e.g. electronic map)).

Analysis:

-	Task D: Network issues identification. The tasks of analysing the network performance (e.g. geographical grid based network performance) and identifying the network issues (e.g. coverage related issues, RAN UE throughput related issues) and corresponding location.

-	Task E: Network deterioration prediction. The tasks of analysing current network performance and historical network performance, predicting the network performance trend in the future and identifying potential network performance deterioration in advance.

-	Task F: Network issue analysis. The tasks of analysing the root cause (e.g. affected Cells) of the identified or predicted network issues.

-	Task G: Network adjustment solutions generation. The tasks of generating the recommended network adjustment solution which can address the identified or predicted network issues.

Decision:

-	Task H: Network adjustment solutions evaluation and determination. The tasks of evaluating the recommended network adjustment solutions, and deciding the network adjustment solutions to be executed.

Execution:

- 	Task I: Network adjustment solutions execution. The tasks of adjusting and configuring the network configuration parameters.

7.1.2 Generic classification of autonomous network level

Level 0:

-	All the tasks in the network optimization workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G, Task H,  Task I) are accomplished by human.

Level 1:

-	Telecom system executes the tasks of network adjustment solutions execution based on the specified network related parameters (Task I). Telecom system also can execute the tasks of network related information collection based on the specified collection rule (Task C). At this level, telecom system can assist human to improve the execution and awareness efficiency for network optimization.

-	All the other tasks in the network optimization workflow (Task A, Task B, Task D, Task E, Task F, Task G, Task H) are accomplished by human.

Level 2:

-	Compared to Level 1, telecom system additionally executes the tasks of network issues identification, network issue analysis based on the specified network issue identification and analysis rule (Task D, Task F). The tasks of network adjustment solutions execution (Task I) and network related information collection (Task C) are fully accomplished by telecom system. At this level, telecom system can assist human to achieve the closed loop for network optimization based on human defined rules.

-	All the other tasks in the network optimization workflow (Task A, Task B, Task E, Task G, Task H) are accomplished by human.

Level 3:

-	Compared to Level 2, telecom system additionally executes the tasks of network adjustment solutions generation (Task G) and network adjustment solutions evaluation and determination (Task H) based on the specified network adjustment policies and evaluation policies. In this level, telecom system also can execute the tasks of deterioration prediction (Task E). The tasks of network issues identification (Task D) and network issue analysis (Task F) are fully accomplished by telecom system. At this level, the telecom system can achieve the closed loop automation for network optimization based on the human defined optimization policies.

-	All the other tasks in the network optimization workflow (Task A, Task B, Task E) are accomplished by human.

Level 4:

-	Compared to Level 3, the telecom system additionally executes the tasks of network optimization policies determination (Task A), and network optimization intent evaluation (Task B) based on received network optimization intent and intent translation/evaluation policies. The tasks of network adjustment solutions generation (Task G), network adjustment solutions evaluation and determination (Task H), and network deterioration prediction (Task E) are fully accomplished by telecom system. At this level, telecom system can achieve the intent driven closed loop automation for network optimization based on human defined intent translation and evaluation policies.

-	The intent translation and evaluation policies maybe pre-defined and specified by human to assist the telecom system.

Level 5:

-	Telecom system can autonomously execute the entire workflow of radio network optimization for all scenarios, which means the telecom system can achieve the full autonomy for network optimization for full scenarios.

Figure 7.1.2-1 Classification of generic autonomous network level for network optimization

7.1.3 Generic requirements

Following are the functional requirements and interface requirements for 3GPP management system to support each autonomous network level (except for level 0) for network optimization.

Editor’s note: all requirements that do not have any stage 2 and 3 solution shall be deleted before this TS is made public.

7.1.3.1 Requirements to support autonomous network level 1

REQ-ANL-NetOpt-Level_1- FUN-1 The 3GPP management system shall have autonomy capability to adjust network based on the specified network adjustment solution.

REQ-ANL-NetOpt-Level_1-FUN-2 The 3GPP management system shall have autonomy capability to collect network related information (including network performance data, network configuration data and environment data) based on specified collection rules.

REQ-ANL-NetOpt-Level_1-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network adjustment solution.

REQ-ANL-NetOpt-Level_1-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network related information collection rules.

REQ-ANL-NetOpt-Level_1-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network related information (including network performance data, network configuration data and environment data).

7.1.3.2 Additional requirements to support autonomous network level 2

REQ-ANL-NetOpt-Level_2-FUN-1 The 3GPP management system shall have autonomy capability to identify the network issue based on the specified network issue identification rule.

REQ-ANL-NetOpt-Level_2-FUN-2 The 3GPP management system shall have autonomy capability to analyse the network issue based on the specified network issue analysis rule.

REQ-ANL-NetOpt-Level_2-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network issue identification rule.

REQ-ANL-NetOpt-Level_2-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network issue.

REQ-ANL-NetOpt-Level_2-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network issue analysis rule.

REQ-ANL-NetOpt-Level_2-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network issue analysis result.

7.1.3.3 Additional requirements to support autonomous network level 3

REQ-ANL-NetOpt-Level_3-FUN-1 The 3GPP management system shall have the autonomy capability to analyse and generate the recommended network adjustment solutions based on specified network adjustment policies.

REQ-ANL-NetOpt-Level_3-FUN-2 The 3GPP management system shall have autonomy capability to evaluate the recommended network adjustment solutions and determine the network adjustment solutions to be executed based on specified network adjustment decision policies.

REQ-ANL-NetOpt-Level_3-FUN-3 The 3GPP management system shall have autonomy capability to predict network performance deterioration.

REQ-ANL-NetOpt-Level_3-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network adjustment policies.

REQ-ANL-NetOpt-Level_3-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the recommended network adjustment solution.

REQ-ANL-NetOpt-Level_3-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network adjustment decision policies.

REQ-ANL-NetOpt-Level_3-MnS-5 The 3GPP management system shall have the capability to obtain the network performance deterioration prediction information.

7.1.3.4 Additional requirements to support autonomous network level 4

REQ-ANL-NetOpt-Level_4-FUN-1 The 3GPP management system shall have autonomy capability to determine or update network optimization policies according to network optimization intent based on specified intent translation policies.

REQ-ANL-NetOpt-Level_4-FUN-2 The 3GPP management system shall have autonomy capability to evaluate network optimization intent fulfilment result based on specified intent evaluation policies.

REQ-ANL-NetOpt-Level_4-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network optimization intent.

REQ-ANL-NetOpt-Level_4-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network optimization intent translation policies.

REQ-ANL-NetOpt-Level_4-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network optimization evaluation policies.

REQ-ANL-NetOpt-Level_4-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network optimization fulfilment information.

7.1.3.5 Additional requirements to support autonomous network level 5

REQ-ANL-NetOpt-Level_5-FUN-1 The 3GPP management system shall have autonomy capability to generate the network optimization intent translation and evaluation policies.

7.2	Generic autonomous network level for RAN NE deployment

7.2.1	Generic workflow

Following are the entire workflow for RAN NE deployment:

Intent handling:

-	Task A:  RAN NE deployment policy generation based on received RAN NE deployment intent. The tasks of generating and determining the RAN NE deployment related policies (e.g. RAN NE configuration data generation policies, RAN NE commissioning polices, RAN NE dialling test policies).

-	Task B: RAN NE deployment intent evaluation, the tasks of evaluating RAN NE deployment fulfilment information (e.g. satisfied or not).

Awareness:

-	Task C: RAN NE information collection, the tasks of collecting the RAN NE information (e.g., IP address, hardware information).

Analysis:

-	Task D: RAN NE configuration data analysis, the tasks of analysing and generating the recommended network configuration data for the RAN NE based on the network planning data for the RAN NE and hardware information collected.

-	Task E: RAN NE commissioning test, the tasks of performing the RAN NE commissioning test to put RAN NE into service (e.g. ensure the software and configuration (including radio configuration and transport configuration) is correct, and alarms are cleared).

-	Task F: RAN NE dialling test, the tasks of performing the RAN NE dialling test to check whether RAN NE services are normal by testing whether the communication can be established between two subscribers.

Decision:

- Task G: RAN NE configuration data determination, the tasks of evaluating and determining the RAN NE configuration data to be download and activated.

Execution:

-	Task H: RAN NE configuration data and software download and activation, the tasks of downloading and activating network configuration data and software in RAN NE.

7.2.2 Generic autonomous network level

Level 0:

-	All the tasks in the RAN NE deployment workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G, Task H) are accomplished by human.

Level 1:

-	Telecom system executes tasks of downloading and activating the available RAN NE network configuration data and software prepared by human (Task H) and tasks of collecting part of RAN NE information (including IP address) based on specified rules(Task C).

-	All the other tasks in the RAN NE deployment workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G) are accomplished by human.

Level 2:

-	Compared to Level 1, telecom system additionally executes the tasks of collecting RAN NE information (including hardware information) (Task C). Telecom system also can execute the tasks of analysing the network configuration data for the RAN NE based on the network configuration data generation rules (e.g. configuration template) specified by human(Task D). The tasks of RAN NE configuration data and software download and activation (Task H) are fully accomplished by telecom system.

-	All the other tasks in the RAN NE deployment workflow (Task A, Task B, Task E, Task F, Task G) are accomplished by human.

Level 3:

-	Compared to Level 2, the telecom system additionally executes tasks of analysing and determining the network configuration data for the RAN NE based on the network configuration data generation policies specified by human (Task D, Task G). Telecom system also can executes tasks of performing the RAN NE commissioning test based on commissioning policies specified by human (Task E), and tasks of performing the RAN NE dialling test based on dialling test policies specified by human(Task F). The tasks of RAN NE information collection (Task C) are fully accomplished by telecom system.

-	All the other tasks in the RAN NE deployment workflow (Task A, Task B) are accomplished by human.

Level 4:

-	Compared to Level 3, the telecom system can additionally execute the tasks of RAN NE deployment policy generation (Task A) and RAN NE deployment intent evaluation (Task B) based on intent handling policies specified by human. The tasks of RAN NE configuration data analysis (Task D), RAN NE configuration data determination (G), RAN NE commissioning test (Task E) and RAN NE dialling test (Task F) are fully accomplished by telecom system.

-	The intent handling policies maybe pre-defined and specified by human.

Level 5:

-	Telecom system can autonomously execute the entire workflow of RAN NE deployment for all scenarios,

Figure 7.2.2-1 Classification of autonomous network level for RAN NE deployment

7.2.3 Generic requirements

Following are the functional requirements and interface requirements for 3GPP management system to support each autonomous network level (except for level 0) for RAN NE deployment.

Editor’s note: all requirements that do not have any stage 2 and 3 solution shall be deleted before this TS is made public.

7.2.3.1 Requirements to support autonomous network level 1

REQ-ANL-RanNeDeploy-Level_1-FUN-1 The 3GPP management system shall have autonomy capability to download and activate the specified RAN NE network configuration data and software.

REQ-ANL-RanNeDeploy-Level_1- FUN-2 The 3GPP management system shall have autonomy capability to collect RAN NE information based on the specified RAN NE information collection rule.

REQ-ANL-RanNeDeploy-Level_1-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the RAN NE network configuration data and software.

REQ-ANL-RanNeDeploy-Level_1-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the RAN NE information collection rule.

7.2.3.2 Requirements to support autonomous network level 2

REQ-ANL-RanNeDeploy-Level_2- FUN-1 The 3GPP management system shall have autonomy capability to analyse the network configuration data for the RAN NE based on the specified network configuration data generation rules.

REQ-ANL-RanNeDeploy-Level_2-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network configuration data generation rules.

7.2.3.3 Requirements to support autonomous network level 3

REQ-ANL-RanNeDeploy-Level_3- FUN-1 The 3GPP management system shall have autonomy capability to analyse and determine the network configuration data for the RAN NE based on the specified network configuration data generation policies.

REQ-ANL-RanNeDeploy-Level_3- FUN-2 The 3GPP management system shall have autonomy capability to perform the RAN NE commissioning test based on specified commissioning test policies.

REQ-ANL-RanNeDeploy-Level_3- FUN-3 The 3GPP management system shall have autonomy capability to perform the RAN NE dialling test based on specified dialling test policies.

REQ-ANL-RanNeDeploy-Level_3-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network configuration data generation policies.

REQ-ANL-RanNeDeploy-Level_3-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the commissioning test policies.

REQ-ANL-RanNeDeploy-Level_3-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to specify the dialling test policies.

7.2.3.4 Requirements to support autonomous network level 4

REQ-ANL-RanNeDeploy-Level_4-FUN-1 The 3GPP management system shall have autonomy capability to determine or update RAN NE deployment policies according to RAN NE deployment intent based on specified intent translation policies.

REQ-ANL-RanNeDeploy-Level_4- FUN-2 The 3GPP management system shall have autonomy capability to evaluate RAN NE deployment intent fulfilment result based on specified intent evaluation policies.

REQ-ANL-RanNeDeploy-Level_4-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the RAN NE deployment intent.

REQ-ANL-RanNeDeploy-Level_4-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the RAN NE deployment intent translation policies.

REQ-ANL-RanNeDeploy-Level_4-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the RAN NE deployment intent evaluation policies.

7.2.3.5 Requirements to support autonomous network level 5

REQ-ANL-RanNeDeploy-Level_5-FUN-1 The 3GPP management system shall have autonomy capability to generate the RAN NE deployment intent translation and evaluation policies.

7.3	Generic autonomous network level for fault management

7.3.1	Generic workflow

Following is the generic workflow for fault management:

Intent handling:

-	Task A: Fault management policies generation. The group of tasks of generating the fault management related rules and/or policies (e.g., alarm filtering rules, fault recognition and root cause analysis policies, fault recovery policies) based on fault management intent (e.g. reduce fault recovery response time to a certain value, reduce network and service failure times to a certain value within a specific duration).

-	Task B: Fault management intent fulfilment evaluation. The group of tasks of evaluating fault management intent fulfilment information (e.g. corresponding fault recovery response time is satisfied or not).

Awareness:

-	Task C: Fault related information collection. The group of tasks which collect the alarm information and other fault related information (e.g. performance information and configuration information etc.).

-	Task D: Alarm filtering. The group of tasks which filter the alarms collected in Task C based on the filtering rules specified. A single network fault may generate a large number of correlative alarms over space and time, therefore it is considered advantageous to have methods filtering the redundant alarms. Reporting only effective alarms without redundant alarms would improve the efficiency of alarm management.

Analysis:

-	Task E: Fault recognition. The group of tasks which recognize the fault and its category (including fault domain or NE, fault type) based on the alarm information and other fault related information.

-	Task F: Fault prediction. The group of tasks which predict the potential fault and its category based on the performance information and other fault related information.

-	Task G: Fault root cause analysis (RCA). The group of tasks which analyse the detailed root cause of the network and service failure.

-	Task H: Fault recovery mechanism analysis. The group of tasks which analyse the possible fault recovery mechanisms based on the fault root cause, thereby generate the feasible options (e.g. recommended recovery solutions).

Decision:

-	Task I: Fault management action generation. The group of tasks which evaluate the feasible options and determine the fault recovery solutions and other corresponding actions (e.g. clear of alarms, storage and retrieval of alarms).

Execution:

-	Task J: Fault management action execution. The group of tasks which execute the fault recovery actions and other corresponding actions.

Note: If the faulty resource has no redundancy (e.g. backup equipment/board/battery/transport link) and all the fall back recovery actions are not available nor effective, then the subsequent fault recovery actions (e.g. replace physical equipment/board/battery, repair the physical connector/fibre/cable, repair the power supply, etc.) are considered as beyond the capabilities of the telecom system. And those actions execution is excluded from the consideration of autonomous network level classification.

7.3.2 Generic autonomous network level

Level 0:

-	All the tasks in the fault management workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G, Task H,  Task I,  Task J) are accomplished by human.

Level 1:

-	Telecom system executes the tasks of fault related information collection (Task C) automatically based on predefined rules, and the tasks of fault management action execution (Task J) based on specified action execution rules. Telecom system can also execute the tasks of alarm filtering (Task D) based on specified alarm filtering rules.

-	All the other tasks in the fault management workflow (Task A, Task B, Task E, Task F, Task G, Task H,  Task I) are accomplished by human.

Level 2:

-	Compared to Level 1, telecom system can additionally executes the tasks of fault recognition (Task E) based on specified rules.  The tasks of fault management action execution (Task J) and fault related information collection (Task C) are fully accomplished by telecom system.

-	All the other tasks (Task A, Task B, Task F, Task G, Task H,  Task I) are accomplished by human.

Level 3:

-	Compared to Level 2, telecom system can additionally execute the tasks of fault root cause analysis based on specified analysis policies (Task G). For certain network faults, telecom system can also additionally execute the tasks of fault prediction (Task F), fault recovery mechanism analysis (Task H) and action generation (Task I) based on specified fault management policies. The tasks of alarm filtering (Task D) and fault recognition (Task E) are fully accomplished by telecom system.

-	All the other tasks (Task A, Task B) are accomplished by human.

Level 4:

-	Compared to Level 3, for certain scenario, telecom system additionally execute the tasks of fault management policies generation (Task A) and fault management intent fulfilment information evaluation (Task B) based on specified intent handling policies. The tasks of fault prediction (Task F), fault root cause analysis (Task G), fault recovery mechanism analysis (Task H) and action generation (Task I) are accomplished automatically by telecom system without human intervention.

-	Intent handling policies can be pre-defined and specified by human.

Level 5:

-	The entire fault management workflows is accomplished by telecom system without human intervention and human predefined rules or policies.

-	Human can optionally supervise the fault management action generated by telecom system.

Figure 7.3.2-1: Generic classification of autonomous network level for fault management

7.3.3 Generic requirements

Following are the functional requirements and interface requirements for 3GPP management system to support each autonomous network level (except for level 0) for fault management.

Editor’s note: all requirements that do not have any stage 2 and 3 solution shall be deleted before this TS is made public.

7.3.3.1 Requirements to support autonomous network level 1

REQ-ANL-FM-Level_1-FUN-1 The 3GPP management system shall have autonomy capability to execute the network recovery actions (including reset the hardware/software, switch to the backup hardware, rollback to the backup software/firmware) with corresponding network elements based on the specified fault recovery rules.

REQ-ANL-FM-Level_1-FUN-2 The 3GPP management system shall have autonomy capability to obtain fault related data (including alarms, events, signalling data, performance data, configuration data and environment data) based on specified collection rules.

REQ-ANL-FM-Level_1-FUN-3 The 3GPP management system shall have autonomy capability to filter the alarms based on the specified filtering rules.

REQ-ANL-FM-Level_1-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault recovery rules.

REQ-ANL-FM-Level_1-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault related data collection rules.

REQ-ANL-FM-Level_1-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the alarm filtering rules

REQ-ANL-FM-Level_1-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain fault related data (including alarms, events, signalling data, performance data, configuration data and environment data).

REQ-ANL-FM-Level_1-MnS-5 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the filtered alarm data.

7.3.3.2 Additional requirements to support autonomous network level 2

REQ-ANL-FM-Level_2-FUN-1 The 3GPP management system shall have the autonomy capability to recognize the fault category (including fault domain and type) based on specified fault recognition rules.

REQ-ANL-FM-Level_2-MnS-1. The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault recognition rules.

REQ-ANL-FM-Level_2-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the fault recognition information (including fault domain and type information).

7.3.3.3 Additional requirements to support autonomous network level 3

REQ-ANL-FM-Level_3-FUN-1 The 3GPP management system shall have autonomy capability to analyse the root cause of the network fault based on specified fault root cause analysis policies.

REQ-ANL-FM-Level_3-FUN-2 The 3GPP management system shall have autonomy capability to analyse and generate the recommended fault recovery mechanism and determine the fault recover actions to be executed based on specified fault recovery mechanism analysis and decision policies.

REQ-ANL-FM-Level_3-FUN-4 The 3GPP management system shall have autonomy capability to predict the potential fault.

REQ-ANL-FM-Level_3-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault root cause analysis policies.

REQ-ANL-FM-Level_3-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault recovery mechanism analysis policies.

REQ-ANL-FM-Level_3-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault recovery mechanism decision policies.

REQ-ANL-FM-Level_3-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the root cause of the network fault.

REQ-ANL-FM-Level_3-MnS-5 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the recommended fault recovery mechanism.

REQ-ANL-FM-Level_3-MnS-6 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the fault recovery actions to be executed.

REQ-ANL-FM-Level_3-MnS-7 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the potential fault prediction information.

7.3.3.4 Additional requirements to support autonomous network level 4

REQ-ANL-FM-Level_4-FUN-1 The 3GPP management system shall have autonomy capability to generate or update fault management policies according to fault management intent based on specified intent translation policies.

REQ-ANL-FM-Level_4-FUN-2 the 3GPP management system shall have autonomy capability to evaluate fault management intent fulfilment based on specified intent evaluation policies.

REQ-ANL-FM-Level_4-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault management intent.

REQ-ANL-FM-Level_4-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault management intent translation policies.

REQ-ANL-FM-Level_4-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the fault management evaluation policies.

REQ-ANL-FM-Level_4-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the fault management fulfilment information.

7.3.3.5 Additional requirements to support autonomous network level 5

REQ-ANL-FM-Level_5-FUN-1 The 3GPP management system shall have autonomy capability to generate the fault management intent translation and evaluation policies.

Annex A (informative):
Examples of Autonomous network level for network and service optimization

A.1 Autonomous network level for radio network coverage optimization

A.1.1 Workflow

Following are the entire workflow for the radio network coverage optimization:

Intent handling:

-	Task A: Coverage optimization policies generation and determination. The tasks of generating and determining the coverage optimization related policies (e.g., weak coverage issue identification policies, weak coverage analysis policies and coverage parameters adjustment policies) based on received coverage optimization intent (e.g. coverage targets(e.g. weak coverage ratio) for the specified areas).

-	Task B: Coverage optimization intent evaluation. The tasks of evaluating coverage optimization intent fulfilment information (e.g. corresponding coverage targets are satisfied or not).

Awareness:

-	Task C: Coverage related information collection. The tasks of collecting coverage related data, including coverage performance data (i.e. performance measurement, MDT data), coverage configuration data (e.g. coverageShape, digitalTilt and digitalAzimuth) and environment data (e.g. electronic map)).

Analysis:

-	Task D: Coverage issues identification. The tasks of analysing the coverage performance (e.g. geographical grid based coverage performance) and identifying the coverage issues (e.g. weak coverage, coverage hole, overshoot coverage) and corresponding location.

-	Task E: Coverage deterioration prediction. The tasks of analysing current coverage performance and historical coverage performance, predicting the coverage performance trend in the future and identifying potential coverage performance deterioration in advance.

-	Task F: Coverage issue analysis. The tasks of analysing the root cause (e.g. affected Cells) of the identified or predicted coverage issues.

-	Task G: Coverage adjustment solutions generation. The tasks of generating the recommended coverage adjustment solution which can address the identified or predicted coverage issues.

Decision:

-	Task H: Coverage adjustment solutions evaluation and determination. The tasks of evaluating the recommended coverage adjustment solutions, and deciding the coverage adjustment solutions to be executed.

Execution:

- 	Task I: Coverage adjustment solutions execution. The tasks of adjusting and configuring the coverage related parameters (e.g. selection of coverageShape, digitalTilt and digitalAzimuth).

A.1.2 Classification of autonomous network level

Level 0:

-	All the tasks in the radio network coverage optimization workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G, Task H,  Task I) are accomplished by human.

Level 1:

-	Telecom system executes the tasks of coverage adjustment solutions execution based on the specified coverage related parameters (Task I). Telecom system also can execute the tasks of coverage related information collection based on the specified collection rule (Task C). At this level, telecom system can assist human to improve the execution and awareness efficiency for radio network coverage optimization.

-	All the other tasks in the radio network coverage optimization workflow (Task A, Task B, Task D, Task E, Task F, Task G, Task H) are accomplished by human.

Level 2:

-	Compared to Level 1, telecom system additionally executes the tasks of coverage issues identification, coverage issue analysis based on the specified coverage issue identification and analysis rule (Task D, Task F). The tasks of coverage adjustment solutions execution (Task I) and coverage related information collection (Task C) are fully accomplished by telecom system. At this level, telecom system can assist human to achieve the closed loop for radio network coverage optimization based on human defined rules.

-	All the other tasks in the radio network coverage optimization workflow (Task A, Task B, Task E, Task G, Task H) are accomplished by human.

Level 3:

-	Compared to Level 2, telecom system additionally executes the tasks of coverage adjustment solutions generation (Task G) and coverage adjustment solutions evaluation and determination (Task H) based on the specified coverage adjustment policies and evaluation policies. In this level, telecom system also can execute the tasks of coverage deterioration prediction (Task E) based on the specified deterioration prediction policies. The tasks of coverage issues identification (Task D) and coverage issue analysis (Task F) are fully accomplished by telecom system. At this level, the telecom system can achieve the closed loop automation for coverage optimization based on the human defined optimization policies.

-	All the other tasks in the radio network coverage optimization workflow (Task A, Task B, Task E) are accomplished by human.

Level 4:

-	Compared to Level 3, the telecom system additionally executes the tasks of coverage optimization policies determination (Task A), and coverage optimization intent evaluation (Task B) based on received coverage optimization intent and intent translation/evaluation policies. The tasks of coverage adjustment solutions generation (Task G), coverage adjustment solutions evaluation and determination (Task H) and coverage deterioration prediction (Task E) are fully accomplished by telecom system. At this level, telecom system can achieve the intent driven closed loop automation for radio network coverage optimization based on human defined intent translation and evaluation policies

-	The intent translation and evaluation policies maybe pre-defined and specified by human to assist the telecom system.

Level 5:

-	Telecom system can autonomously execute the entire workflow of radio network coverage optimization for all scenarios, which means the telecom system can achieve the full autonomy for radio network coverage optimization for full scenarios.

Figure A.1.2-1 Classification of autonomous network level for coverage optimization

A.2 Autonomous network level for RAN UE throughput optimization

A.2.1 Workflow

Following are the entire workflow for the RAN UE throughput optimization:

Intent handling:

-	Task A: RAN UE throughput optimization policies generation and determination. The tasks of generating and determining the RAN UE throughput optimization related policies (e.g., RAN UE throughput issue identification policies, RAN UE throughput analysis policies and RAN UE throughput parameters adjustment policies) based on received RAN UE throughput optimization intent (e.g. RAN UE throughput targets(e.g. low RAN UE throughput ratio, target average RAN UE throughput) for the specified areas).

-	Task B: RAN UE throughput optimization intent fulfilment evaluation. The tasks of evaluating RAN UE throughput optimization intent fulfilment information (e.g. corresponding RAN UE throughput targets are satisfied or not).

Awareness:

-	Task C: RAN UE throughput related information collection. The tasks of collecting RAN UE throughput related data, including RAN UE throughput performance data (e.g., AverageUL/ DL UE throughput in gNB, Distribution of UL/DL UE throughput in gNB ), RAN UE throughput configuration data (e.g., cellIndividualOffset, isHOAllowed and isMLBAllowed of corresponding NRCellRelation(s)) and environment data (e.g. electronic map)).

Analysis:

-	Task D: RAN UE throughput issues identification. The tasks of analysing the RAN UE throughput performance (e.g. geographical grid based RAN UE throughput performance) and identifying the RAN UE throughput issues (e.g. low average RAN UE throughput, large number of UEs with low RAN UE throughput) and corresponding location.

-	Task E: RAN UE throughput deterioration prediction. The tasks of analysing current RAN UE performance and historical RAN UE throughput performance, predicting the RAN UE throughput performance trend in the future and identifying potential RAN UE throughput performance deterioration in advance.

-	Task F: RAN UE throughput issue analysis. The tasks of analysing the root cause (e.g. unbalanced load between different cells, frequently handover among several cells) of the identified or predicted RAN UE throughput issues.

-	Task G: RAN UE throughput adjustment solutions generation. The tasks of generating the recommended RAN UE throughput adjustment solution (e.g. reconfigure the NRCellRelation IOC and DMROFunction IOC of corresponding RAN NEs) which can address the identified or predicted RAN UE throughput issues.

Decision:

-	Task H: RAN UE throughput adjustment solutions evaluation and determination. The tasks of evaluating the recommended RAN UE throughput adjustment solutions, and deciding the RAN UE throughput adjustment solutions to be executed.

Execution:

- 	Task I: RAN UE throughput adjustment solutions execution. The tasks of adjusting and configuring the RAN UE throughput related parameters (e.g. cellIndividualOffset, isHOAllowed and isMLBAllowed of corresponding NRCellRelation(s), maximumDeviationHoTrigger of corresponding DMROFunction).

A.2.2 Classification of autonomous network level

Level 0:

-	All the tasks in the RAN UE throughput optimization workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G, Task H,  Task I) are accomplished by human.

Level 1:

-	Telecom system executes the tasks of RAN UE throughput adjustment solutions execution based on the specified RAN UE throughput related parameters (Task I). Telecom system also can execute the tasks of RAN UE throughput related information collection based on the specified collection rule (Task C). At this level, telecom system can assist human to improve the execution and awareness efficiency for RAN UE throughput optimization.

-	All the other tasks in the RAN UE throughput optimization workflow (Task A, Task B, Task D, Task E, Task F, Task G, Task H) are accomplished by human.

Level 2:

-	Compared to Level 1, telecom system additionally executes the tasks of RAN UE throughput issues identification, RAN UE throughput issue analysis based on the specified RAN UE throughput issue identification and analysis rule (Task D, Task F). The tasks of RAN UE throughput adjustment solutions execution (Task I) and RAN UE throughput related information collection (Task C) are fully accomplished by telecom system. At this level, telecom system can assist human to achieve the closed loop for RAN UE throughput optimization based on human defined rules.

-	All the other tasks in the RAN UE throughput optimization workflow (Task A, Task B, Task E, Task G, Task H) are accomplished by human.

Level 3:

-	Compared to Level 2, telecom system additionally executes the tasks of RAN UE throughput adjustment solutions generation (Task G) and RAN UE throughput adjustment solutions evaluation and determination (Task H) based on the specified RAN UE throughput adjustment policies and evaluation policies. In this level, telecom system also can execute the tasks of RAN UE throughput deterioration prediction (Task E) based on the specified deterioration prediction policies. The tasks of RAN UE throughput issues identification (Task D) and RAN UE throughput issue analysis (Task F) are fully accomplished by telecom system. At this level, the telecom system can achieve the closed loop automation for RAN UE throughput optimization based on the human defined optimization policies.

-	All the other tasks in the RAN UE throughput optimization workflow (Task A, Task B, Task E) are accomplished by human.

Level 4:

-	Compared to Level 3, the telecom system additionally executes the tasks of RAN UE throughput optimization policies determination (Task A), and RAN UE throughput optimization intent evaluation (Task B) based on received RAN UE throughput optimization intent and intent translation/evaluation policies. The tasks of RAN UE throughput adjustment solutions generation (Task G), RAN UE throughput adjustment solutions evaluation and determination (Task H) and RAN UE throughput deterioration prediction (Task E) are fully accomplished by telecom system. At this level, telecom system can achieve the intent driven closed loop automation for RAN UE throughput optimization based on human defined intent translation and evaluation policies

-	The intent translation and evaluation policies maybe pre-defined and specified by human to assist the telecom system.

Level 5:

-	Telecom system can autonomously execute the entire workflow of RAN UE throughput optimization for all scenarios, which means the telecom system can achieve the full autonomy for RAN UE throughput optimization for full scenarios.

Figure A.2.2-1 Classification of autonomous network level for RAN UE throughput optimization

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | SA5#132e | S5-204084 |  |  |  | Initial draft TS | 0.0.0 |
| 2020-08 | SA5#132e | S5-204555 S5-204086 S5-204087 |  |  |  | 1. pCR 28.100 Add skeleton 2. pCR 28.100 Add Scope 3. pCR 28.100 Add definitions and abbreviations | 0.1.0 |
| 2020-11 | SA5#134e | S5-206180 S5-206352 S5-206353 S5-206354 S5-206355 S5-206356 S5-206357 |  |  |  | 1. pCR 28.100 Update skeleton for clause 5 2. Add concept of autonomous network and autonomous network level 3. pCR TS 28.100 Add use case and requirements for coverage optimization 4. pCR 28.100 Add Introduction 5. pCR 28.100 Add definition for network and service management scenario types 6. pCR 28.100 Add dimensions for evaluating autonomous network levels 7. pCR 28.100 Add framework approach for evaluating autonomous network levels | 0.2.0 |
| 2021-02 | SA5#135e | S5-211229 S5-211443 S5-211459 |  |  |  | 1. pCR 28.100 Update clause 4.3.4 workflow 2. pCR TS 28.100 Update use case for coverage optimization 3. pCR 28.100 Add use case and requirements for fault management | 0.3.0 |
| 2021-03 | SA5#136e | S5-212386 S5-212387 S5-212388 S5-212389 |  |  |  | 1. pCR TS 28.100 Add autonomous network level for RAN NE deployment 2. pCR TS 28.100 Add autonomous network level for RAN UE throughput optimization 3. pCR 28.100 Update Clause 4.1 Autonomous network concept 4. pCR 28.100 Add generic classification of autonomous network level for network optimization | 0.4.0 |
| 2021-05 | SA5#137e | S5-213514 S5-213515 S5-213516 S5-213517 S5-213518 S5-213519 S5-213520 |  |  |  | 1. Add requirements for generic autonomous network level for network optimization 2. Update several existing classification of autonomous network level 3. Add generic autonomous network level for RAN NE deployment 4. pCR TS 28.100 Add description for Table 5-1 and update corresponding figures 5. pCR TS 28.100 Add figure in clause 4.3.4 Workflow 6. pCR TS 28.100 Add generic autonomous network level for fault management 7. Add a note about the relationship between the categorization of the tasks and standardized features | 0.5.0 |