# 3GPP TSG-SA5 Meeting #138-e 	S5-214242rev02

# e-meeting, 23 - 31 August 2021

# Source:	China Telecom

# Title:	Discussion paper on the generic principle for EE KPI definition for 5GC

# Document for:	Endorsement

# Agenda Item:	6.5.1

1	Decision/action requested

The group is asked to discuss and endorse on the proposal.

2	References

[1]	3GPP TR 28.813: "Study on new aspects of Energy Efficiency (EE) for 5G "

[2]	3GPP TS 23.501: "5G; System Architecture for the 5G System"

[3] 	ETSI ES 203 539: "Environmental Engineering (EE); Measurement method for energy efficiency of Network Functions Virtualisation (NFV) in laboratory environment"

[4]	3GPP TS 28.552: "5G Performance Measurements"

3	Rationale

3.1			Background

The 5GC is consisted of the User Plane Functions (UPF) and Control Plane functions. In TS 28.813[1], the generic EE KPI for 5GC is defined as:

The Data Volume (DV) is meaningful for the User Plane Functions. However, for the Control Plane functions, the DV is less meaningful.  Consequently, applying the EE KPI defined based on the Data Volume of the Control Plane functions within 5GC can be controversial. In this contribution, a generic principle for defining the EE KPI which is applicable for both User Plane Functions (UPF) and control plane functions in 5GC is discussed.

3.2			Discussion

In TS 28.813[1], the generic definition of the EE KPI for 5GC is based on the User Plane traffic volume, where two potential alternatives are proposed as follows:

In the proposed alternative1, both incoming and outgoing traffic are measured at the GTP-U layer on the N3 interface. This potential alternative applies only to UPFs with N3 interfaces connected, where DV5GC, expressed in bits, is defined as:

In the proposed alternative2, both incoming and outgoing traffic are measured at the IP layer on the N6 interface. This potential alternative applies only to UPFs with N6 interfaces connected, where DV5GC, expressed in bits, is defined as:

Observation 1: the generic EE KPI for 5GC defined in TR 28.813 is designed based on the data volume measured on the 5GC User Plane.

This data volume may not be suitable for measuring the 5GC control plane.

In ES 203 539[1], the Useful Output was defined as the maximum capacity of the system under test which is depending on the different functions in both data plane/user plane or control plane. The Useful Output can be expressed as the number of Erlang (Erl), Packets/s (PPS), Subscribers (Sub) or Simultaneously Attached Users (AU).  More specifically, the Useful Output may be the throughput (e.g., bps) of data plane VNF or capacity (e.g., subscribers, sessions) for control plan VNF.

Observation 2: The Useful Output may be the throughput (e.g., bps) of data plane VNF or capacity (e.g., subscribers, sessions) of control plan VNF.

Based on the observation 1 and observation 2, the Data Volume measured on the 5GC user plane is as same as the Useful Output of the 5GC user plane. And in cases where the Data Volume is applicable, i.e. the performance of the user plane of the 5GC is considered, the Data Volume in the current generic EE KPI can be expressed as the Useful Output of the 5GC user plane.

Observation 3: When the performance of the user plane of the 5GC is considered, the Data Volume in the current generic EE KPI can be expressed as the Useful Output of the 5GC user plane.

The Useful Output may be the capacity (e.g., subscribers, sessions) for control plan VNF. At the same time, in TS 28.552[4], the performance measurement of the AMF and SMF have been provided. The performance measurement of the AMF can be the mean number of registered subscribers on AMF, and the performance measurement of the SMF can be the mean number of the PDU sessions on SMF. Therefore, the Useful Output of the AMF and SMF may be measured based the defined performance measurement in [4], correspondently.

As the AMF and SMF play important roles in the 5GC control plane, it was proposed in the endorsed DP S5-213551 to take the Useful Output of the AMF or the Useful Output of SMF as the estimation of the Useful Output of the 5GC control plane.

Observation 4: the Useful Output of the 5GC control plane can be estimated as the mean number of registered subscribers on AMFs or the mean number of PDU sessions on SMFs within the 5GC control plane.

Based on the Observation 2 to 4, we think that the Useful Output of the 5GC user plane and 5GC control plane can be estimated.

Moreover, like what we have found in Observation 3, we propose to use the Useful Output of 5GC control plane to derive the 5GC EE KPI when the performance of the 5GC control plane is considered.

Proposal 1: when the performance of the 5GC control plane is considered, the 5GC EE KPI can be defined based on the Useful Output of 5GC control plane.

The capacity of the entire 5GC can be estimated by the capacity of 5GC user plane or by the capacity of the 5GC control plane. One step further with the generic EE KPI, based on the observations and proposal above, we propose to use the capacity of the entire 5GC, i.e. Useful Output of 5GC, as the generic principle for the 5GC EE KPI definition.

Proposal 2: A generic principle for the 5GC EE KPI definition can be provided as the ratio of the Useful Output of 5GC to the Energy Consumption of the entire 5GC(EC5GC):

EE5GC=UsefulOutput5GCEC5GC

And in case when the performance of the 5GC user plane is considered, UsefulOutput5GC can be estimated as the Useful Output of the 5GC user plane, and:

EE5GC=EE5GC,UPuser plane

EE5GC,User Plane Functions=UsefulOutput5GC,User Plane FunctionsEC5GC=DV5GCEC5GCEE5GC,UPuser plane=UsefulOutput5GC,uUPser planeEC5GC=DV5GCEC5GC

And in case when the performance of the 5GC control plane is considered, UsefulOutput5GC can be estimated as the Useful Output of the 5GC control plane, and:

EE5GC=EE5GC,CPcontrol plane

EE5GC,CPcontrol plane=UsefulOutput5GC,control planCPeEC5GC

3.3	Conclusion

It is proposed to take the proposals above as the way forwards on the definition of EE KPI for 5GC Use Plane and Control Plane.

EE5GC=UsefulOutput5GCEC5GC

EE5GC,User Plane Functions=UsefulOutput5GC,User Plane FunctionsEC5GC=DV5GCEC5GC

EE5GC,control plane=UsefulOutput5GC,control planeEC5GC

4	Detailed proposals

SA5 is asked to endorse the followings as the way forwards for the discussions on the definition of EE KPI for 5GC User Plane and Control Plane:

- When the performance of the 5GC control plane is considered, the 5GC EE KPI can be defined based on the Useful Output of 5GC control plane.

- A generic principle for the 5GC EE KPI definition can be provided as the ratio of the Useful Output of 5GC to the Energy Consumption of the entire 5GC:
EE5GC=UsefulOutput5GCEC5GC

And in case when the performance of the 5GC user plane is considered, UsefulOutput5GC can be estimated as the Useful Output of the 5GC user plane, and:

EE5GC=EE5GC,user plaUPne

EE5GC,User Plane Functions=UsefulOutput5GC,User Plane FunctionsEC5GC=DV5GCEC5GCEE5GC,UPuser plane=UsefulOutput5GC,UPuser planeEC5GC=DV5GCEC5GC

And in case when the performance of the 5GC control plane is considered, UsefulOutput5GC can be estimated as the Useful Output of the 5GC control plane, and:

EE5GC=EE5GC,cCPontrol plane

EE5GC,conCPtrol plane=UsefulOutput5GC,CPcontrol planeEC5GC