3GPP TSG-SA5 Meeting #139-e 	S5-215225

# e-meeting, 11 - 20 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | DraftCR | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhance generic management service to support authentication of MnS consumer, stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSAC |  |  |  |  |  | Date: |  |  | 2021-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to conclusion of TR 28.817, the generic management service should be updated to support authentication and authorization, the exact change will be decided in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new operation to support authentication capability, stage 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Access control for management service cannot be realized, that results in big security risk on 3GPP management system, hence imperil 3GPP networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 12.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Input for Rel-17 DraftCR 28.532 for MSAC |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

# 2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.526: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[3]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	3GPP TS 28.531: "Management and orchestration ; Provisioning; ".

[6]	3GPP TS 28.554: "Management and orchestration ; 5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 22.261: "Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1".

[8]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	ITU-T Recommendation X.734 (1992): "Information technology - Open Systems Interconnection - Systems management: Event report management function".

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[16]	3GPP TS 32.302:	 "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[18]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[19]	3GPP TS 32.401: "Telecommunication management; Perfomance Measurement (PM); Concept and requirements".

[20]	ISO 8601:2004: "Data elements and interchange formats – Information interchange – Representation of dates and times".

[21]	Void.

[22]	Void.

[23]	Void.

[24]	Void.

[25]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects ".

[26]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[27]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[28]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[29]	W3C REC-xml-names-19990114: "Namespaces in XML".

[30]	Void.

[31]	3GPP TS 32.111-2: " Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[32]	IETF RFC 6241 "Network Configuration Protocol (NETCONF)".

[33]	3GPP TS 32.160 " Management and orchestration; Management service template ".

[34]	IETF RFC 7950 "The YANG 1.1 Data Modeling Language".

[35]	OpenAPI: "OpenAPI 3.0.1 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.1.md.

[36]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[37]	IETF RFC 7396: "JSON Merge Patch".

[38]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[39]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[40]	IETF RFC 6455: "The WebSocket Protocol".

[41]	IETF RFC 793: "Transmission Control Protocol".

[42]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[43]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[44]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[45]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://github.com/onap/vnfrqts-requirements/blob/05f26fac2b941513a7d0e856b99fd8c61d688299/docs/Chapter8/ves7_1spec.rst#resource-structure.

[46]	3GPP SA5 FORGE OpenAPI definitions: https://forge.3gpp.org/rep/sa5/MnS/tree/Rel-16/OpenAPI.

[47]	3GPP TS 32.404: "Performance Management (PM); Performance measurements; Definitions and template".

[48]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[x]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[y]	OpenID: OpenID connect protocol:

https://openid.net/specs/openid-connect-core-1_0.html

[z]	IETF RFC 7519: "JSON Web Token (JWT)".

End of modification

Start of 2nd modification

## 12.x	Access control service

### 12.x.1	RESTful HTTP-based solution set

#### 12.x.1.1	Mapping of operations

##### 12.x.1.1.1	Introduction

The OpenID connect protocol (see [y]) and OAuth 2.0 authorization code grant (see [x]) are used to authenticate and authorize human management service consumer, as shown in Figure 12.x.1.1.1-1.

Figure 12.x.1.1.1-1 Authentication and authorization for human MnS consumer

- The authenticaiton service producer implements OpenID Provider (OP), and authorization endpoint of Oauth 2.0 which authenticates the end user in OAuth 2.0 authorization code grant scenario.

- The MnS consumer is end user of OpenID connect protocol and resource owner of OAuth 2.0.

- The client acting on behalf of human MnS consumer implements Relaying Party (RP) of OpenID connect protocol, and confidential client of OAuth 2.0.

- The authorization service producer implements token endpoint of OAuth 2.0, which issue access token to the client.

- The management service producer implements resource server of OAuth 2.0.

Note: Authentication of human MnS consumer includes two steps, the client on behalf of human user sends authenticaiton request is the first step, and the human user logins with credentials is the second step.  The steps to redirect user agent (e.g. browser) to autthentication service producer (by the client on behalf of MnS consumer) to send authentication request, and redirect user agent back to the client (by autthentication service producer) to send authentication response are ignored in the figure for sake of simplicity and conciseness.

OAuth 2.0 client credential grant (see [x]) is used to authenticate and authorize machine management service consumer, as shown in Figure 12.x.1.1.1-2.

Figure 12.x.1.1.1-2 Authentication and authorization for machine MnS consumer

- The authenticaiton service producer authenticates management service consumer  by validate the client credential.

- The management service consumer implements confidential client of OAuth 2.0.

- The authorization service producer implements token endpoint of OAuth 2.0, which issue access token to the client.

- The management service producer implements resource server of OAuth 2.0.

Note: Authentication service producer is preconfigured in authorization service producer, and vice versa. Trust relationship between the two entities is established.

The IS operations are mapped to SS equivalents according to table 12.x.1.1.1-1.

Table 12.x.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| authentication | GET | /oauth2/authorize | M |
| authorization | POST | /oauth2/token | M |

##### 12.x.1.1.2	authentication

The IS operation parameters are mapped to SS equivalents according to table 12.x.1.1.2-1 and table 12.x.1.1.2-2.

Table 12.x.1.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| IS parameter name | SS parameter location | SS parameter name |  | S |  | Remark |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| identifier | query | consumer_id |  | M |  | A unique identifer of a MnS consumer. For machine MnS consumer, it could be DN, FQDN, etc. It is included in authentication request. For human MnS consumer, it could be user name, email address, phone number, etc. It is included in both of authentication request from the client (actining on behalf of human MnS consumer) to authentication service producer and login request from user agent to authentication service producer. |  |
| credential | query | credential_type | CM | Different credential types will be used according to authentication policy of the MnS consumer, e.g. it could be secret (e.g. password) or certificate based assertion (e.g. jwt-bear, see [z]) For human MnS consumer, it is included in login request from user agent to authentication service producer. For machine MnS consumer, it is in authentication request. |
| credential | query | credential | CM | It is secret or certificate based assertion. For human MnS consumer, it is included in login request from user agent to authentication service producer. For machine MnS consumer, it is in authentication request. |
| assClient | query | client_id |  | CM |  | It is used only for human MnS consumer scenario. It is part of assoicated client acting on behalf of the human consumer. It is unique id, e.g. DN, FQDN, assigned to the client. |  |
| assClient | query | redirect_uri |  | It is used only for human MnS consumer scenario. It is part of assoicated client acting on behalf of the human consumer. It is redirection URI to which the authentication response from authentication service producer will be sent. |
|  | query | response_type | CM | It is oauth2 and OpenID connect specific parameter. It presents and its value is "code" in the authentication request from the client (actining on behalf of human MnS consumer) to authentication service producer for human MnS consumer authentication. It is empty in the authentication request from authorization service producer (actining on behalf of machine MnS consumer) to authentication service producer for machine MnS consumer authentication |

Table 12.x.1.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS parameter name | SS parameter location | SS parameter name | S | Remark |
| --- | --- | --- | --- | --- |
| status | response status codes/body | status | M | It is response status code, and optional error description in response body for error response. |
| identifier | response body | consumer_id | M | same to identifier in the request |
| sessionId | response body | session_id | O | It could be used to uniquely identy a successful authentication for a MnS consumer in different entities included in authentication workflow. |
| context | response body | context | O | It could be used when there's no assertion in the response. |
| assertion | response body | code | CM | It is oauth2 and OpenID connect specific parameter. It presents and its value is set to authorization code generated by the authentication service producer. |

Informative example: human MnS consumer authentication :

Authentication request from clien on behalf of human MnS consumer to authentication service producer:

GET /oauth2/authorize?

consumer_id=consumer1@example.com

&client_id=client.example.com

&redirect_uri=https%3A%2F%2Fclient.example.com%2Fac

&response_type=code

Host: authenticationserver.example.com

Login request from user agent to authentication service producer:

GET /oauth2/authorize?

consumer_id=consumer1@example.com

&credential_type=secret

&credential=SHJKUJUYKKLH

Host: authenticationserver.example.com

Authentication response:

HTTP/2 302 Found

Location: https://client.example.org/ac?

consumer_id=consumer1@example.com

&code=SplxlOBeZQQYbYS6WxSbIA

Informative example: machine MnS consumer authentication :

Authentication request:

GET /oauth2/authorize?

consumer_id=consumer1.example.com

&credential_type=jwt

&credential=eyJhbGciOiJSUzI1NiIsIng1dCI6Imd4OHRHeXN5amNScUtq

Host: authenticationserver.example.com

Authentication response:

HTTP/2 200

consumer_id=consumer1.example.com

End of modification