3GPP TSG-SA5 Meeting #139-e 	S5-215235

# e-meeting, 11 - 20 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0344 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS charging information data types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adding the IMS charging information data types |  |  |  |  |  |  |  |  |
| Summary of change: |  | The initial IMS charging information data types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging for IMS cannot be supported by converged charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.8.a (new), 6.1.6.2.8.b (new), 6.1.6.2.8.c (new), 6.1.6.2.8.d (new), 6.1.6.2.8.e (new), 6.1.6.2.8.f (new), 6.1.6.2.8.g (new), 6.1.6.2.8.h (new), 6.1.6.2.8.i (new), 6.1.6.2.8.j (new), 6.1.6.2.8.k (new), 6.1.6.2.8.l (new), 6.1.6.2.8.m (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

###### 6.1.6.2.8.a	Type SIPEventType

Table 6.1.6.2.8.x-1: Definition of type SIPEventType

| Attribute name | Data type | P |  | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| sIPMethod | string | OC |  | 0..1 | This field holds holds the name of the SIP Method (INVITE, UPDATE etc.). |  |
| eventHeader | string | OC |  | 0..1 | This field holds the content of the "Event" header |  |
| expiresHeader | Uint32 | OC |  | 0..1 | This field holds the content of the "Expires" header |  |

Second change

###### 6.1.6.2.8.b	Type ISUPCause

Table 6.1.6.2.8.x-1: Definition of type ISUPCause

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| iSUPCauseLocation | OctetString | OC | 0..1 | This field identifies the network in which the event causing the call release. Values described in TS 29.078 [259]. |  |
| iSUPCauseValue | Uint32 | OC | 0..1 | This field identifies the reason a voice call service is released. Values described in TS 29.078 [259]. |  |
| iSUPCauseDiagnostics | Uint32 | OC | 0..1 | This field holds the diagnostics field associated with the release of the voice call service. Values described in TS 29.078 [259]. |  |

Third change

###### 6.1.6.2.8.c	Type CalledIdentityChange

Table 6.1.6.2.8.x-1: Definition of type CalledIdentityChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| calledIdentity | string | OC | 0..1 | This field holds the address (Public User ID: SIP URI, E.164, etc.) of the called party after a change. |  |
| changeTime | DateTime | OC | 0..1 | This field holds the time in UTC format when the change was registered. |  |

Fourth change

###### 6.1.6.2.8.d	Type InterOperatorIdentifier

Table 6.1.6.2.8.x-1: Definition of type InterOperatorIdentifier

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| originatingIOI | string | OC | 0..1 | This field holds the Inter Operator Identifier (IOI) for the originating network as generated by the IMS node as described in RFC 7315 [405] and TS 24.229 [258]. |  |
| terminatingIOI | string | OC | 0..1 | This field holds the Inter Operator Identifier (IOI) for the terminating network as generated by the IMS node as described in RFC 7315 [405] and TS 24.229 [258]. |  |

Fifth change

###### 6.1.6.2.8.e	Type EarlyMediaDescription

Table 6.1.6.2.8.x-1: Definition of type EarlyMediaDescription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sDPTimeStamps | SDPTimeStamps | OC | 0..1 | This field holds the time of the SDP offer and the SDP answer. |  |
| sDPMediaComponent | array(SDPMediaComponent) | OC | 0..N | This field contains information about media used for a IMS session. |  |
| sDPSessionDescription | array(string) | OC | 0..N | This field holds the content of the SDP line (i=, c=, b=, k=, a=, etc.) in the session description, as described in RFC 8866 [407]. |  |

Sixth change

###### 6.1.6.2.8.f	Type SDPMediaComponent

Table 6.1.6.2.8.x-1: Definition of type SDPMediaComponent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sDPMediaName | string | OC | 0..1 | This field holds the content of the SDP "m=" line in a media description, as described in RFC 8866 [407]. |  |
| SDPMediaDescription | array(string) | OC | 0..N | This field holds the content of SDP lines (i=, c=, b=, k=, a=, etc.) related to a media description, as described in RFC 8866 [407]. |  |
| localGWInsertedIndication | boolean | OC | 0..1 | This field indicates if the local GW (TrGW, IMS-AGW) is inserted or not for the SDP media component. Set to true if inserted. |  |
| ipRealmDefaultIndication | boolean | OC | 0..1 | This field indicates whether the IP realm used for the SDP media component is the default IP realm or not. Set to true if it is the default IP realm is used. |  |
| transcoderInsertedIndication | boolean | OC | 0..1 | This field indicates if a transcoder is inserted or not for the SDP media component. Set to true if it is inserted. |  |
| mediaInitiatorFlag | MediaInitiatorFlag | OC | 0..1 | This field indicates which party has requested the session modification. |  |
| mediaInitiatorParty | string | OC | 0..1 | This field it holds the address (SIP URI or Tel URI) of the party (Public User ID or Public Service ID) who initiates the media action. |  |
| threeGPPChargingId | OctetString | OC | 0..1 | This field contains a charging identifier. |  |
| accessNetworkChargingIdentifierValue | OctetString | OC | 0..1 | This field contains a charging identifier (e.g. GCID). |  |
| sDPType | SDPType | OC | 0..1 | This field holds information if the SDP media component was of type SDP offer or SDP answer |  |

Seventh change

###### 6.1.6.2.8.g	Type ServerCapabilities

Table 6.1.6.2.8.x-1: Definition of type ServerCapabilities

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mandatoryCapability | array(Uint32) | OC | 0..N | This field can represent a single determined mandatory capability or a set of capabilities of an S-CSCF, as described in TS 29.228 [260] clause 6.7. |  |
| optionalCapability | array(Uint32) | OC | 0..N | This field can represent a single determined optional capability or a set of capabilities of an S-CSCF, as described in TS 29.228 [260] clause 6.7. |  |
| serverName | array(string) | OC | 0..N | This field contains a SIP-URL (as defined in IETF RFC 3261 [406] and IETF RFC 3986 [404]), used to identify a SIP server (e.g. S-CSCF name). |  |

Eighth change

###### 6.1.6.2.8.h	Type TrunkGroupID

Table 6.1.6.2.8.x-1: Definition of type ServerCapabilities

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| incomingTrunkGroupID | string | OC | 0..1 | This field identifies the incoming PSTN leg. |  |
| outgoingTrunkGroupID | string | OC | 0..1 | This field identifies the outgoing PSTN leg. |  |

Nineth change

###### 6.1.6.2.8.i	Type MessageBody

Table 6.1.6.2.8.x-1: Definition of type ServerCapabilities

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| contentType | string | M | 0..1 | This field holds the media type (e.g. application/sdp, text/html) of the message-body, as described in RFC 3261 [406]. |  |
| contentLength | Uint32 | M | 0..1 | This field holds the size of the message-body, as described in RFC 3261 [406]. |  |
| contentDisposition | string | OC | 0..1 | This field indicates how the message body, or a message body part is to be interpreted (e.g. session, render), as described in RFC 3261 [406]. |  |
| originator | OriginatorPartyType | OC | 0..1 | This field indicates the originating party of the message body. |  |

Tenth change

###### 6.1.6.2.8.j	Type AccessTransferInformation

Table 6.1.6.2.8.x-1: Definition of type AccessTransferInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| accessTransferType | AccessTransferType | OC | 0..1 | This field indicates which type of transfer occurred for IMS service continuity. |  |
| accessNetworkInformation | array(OctetString) | OC | 0..N | This field indicates one instance of the SIP P-header "P-Access-Network-Info". |  |
| cellularNetworkInformation | OctetString | OC | 0..1 | This field indicates one instance of the SIP header "Cellular-Network-Info". |  |
| interUETransfer | UETransferType | OC | 0..1 | This field contains information about type of the transfer. If this AVP is not present, this means that the type of transfer is Intra-UE transfer. |  |
| userEquipmentInfo | Pei | OC | 0..1 | This field contains the identity and capability of the terminal the subscriber is using. |  |
| instanceId | string | OC | 0..1 | This field contains a URN generated by the device that uniquely identifies a specific device amongst all other devices. |  |
| relatedIMSChargingIdentifier | string | OC | 0..1 | This field holds the Related IMS Charging Identifier (ICID) as generated by the Enhanced MSC Server or the P-CSCF for the target access leg of an SRVCC access transfer. |  |
| relatedIMSChargingIdentifierNode | Address | OC | 0..1 | This field holds the identifier of the Enhanced MSC Server or the P-CSCF that generated the Related IMS Charging Identifier (ICID). |  |
| changeTime | DateTime | OC | 0..1 | This field holds the time in UTC format when the change was registered. |  |

Eleventh change

###### 6.1.6.2.8.k	Type AccessNetworkInfoChange

Table 6.1.6.2.8.x-1: Definition of type AccessNetworkInfoChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| accessNetworkInformation | array(OctetString) | OC | 0..N | This field indicates one instance of the SIP P-header "P-Access-Network-Info". |  |
| cellularNetworkInformation | OctetString | OC | 0..1 | This field indicates one instance of the SIP header "Cellular-Network-Info". |  |
| changeTime | DateTime | OC | 0..1 | This field holds the time in UTC format when the change was registered. |  |

Twelfth change

###### 6.1.6.2.8.l	Type NNIInformation

Table 6.1.6.2.8.x-1: Definition of type NNIInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionDirection | NNISessionDirection | OC | 0..1 | This field indicates whether the NNI is used for an inbound or outbound service request on the control plane in case of interconnection and roaming. |  |
| nNIType | NNIType | OC | 0..1 | This field indicates whether the type of used NNI is non-roaming, roaming without loopback routing or roaming with loopback routing |  |
| relationshipMode | NNIRelationshipMode | OC | 0..1 | This field indicates whether the other functional entity (e.g. contact point of the neighbouring network) is regarded as part of the same trust domain. |  |
| Neighbour-Node-Address | Address | OC | 0..1 | This field holds the control plane IP address of the neighbouring network contact point that handles the service request in case of interconnection and roaming |  |

Thirteenth change

###### 6.1.6.2.8.m	Type TADIdentifier

Table 6.1.6.2.8.x-1: Definition of type NNIInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionDirection | NNISessionDirection | OC | 0..1 | This field indicates whether the NNI is used for an inbound or outbound service request on the control plane in case of interconnection and roaming. |  |
| nNIType | NNIType | OC | 0..1 | This field indicates whether the type of used NNI is non-roaming, roaming without loopback routing or roaming with loopback routing |  |
| relationshipMode | NNIRelationshipMode | OC | 0..1 | This field indicates whether the other functional entity (e.g. contact point of the neighbouring network) is regarded as part of the same trust domain. |  |
| Neighbour-Node-Address | Address | OC | 0..1 | This field holds the control plane IP address of the neighbouring network contact point that handles the service request in case of interconnection and roaming |  |

End of changes