3GPP TSG-SA5 Meeting #139-e 	S5-215081

# e-meeting, 11 - 20 October 2021

# Source:	Huawei

# Title:	Remove obsolete editor notes

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TS 28.557 Management of non-public networks; Stage 1 and stage 2 v1.0.0

3	Rationale

There are two Editor's NOTEs saying that "Whether modifications in NR NRM fragment [7] are needed or not is FFS" in clause 4.6.2 NG-RAN related management aspects. These two ENs are obsolete now since the group has agreed S5-214579 (adding NPN Identity Information to NRCellDU in TS 28.541) at last meeting.

It is proposed to remove the two obsolete editor notes in draft TS 28.557 [1].

4	Detailed proposal

This document proposes the following changes in TS 28.557 [1].

1st Change

4.6.2	NG-RAN related management aspects

A NG-RAN node can serve multiple NPNs, including SNPNs and PNI-NPNs. To that end, the NPN-OP shall configure the NG-RAN node accordingly, using the 3GPP management system.

For NG-RAN non-split deployments, the gNB needs to be configured (via 3GPP management system) with lists of NID(s) and CAG(s) it supports, for SNPN and PNI-NPN, respectively. In the NG Application Protocol (NGAP), this information is used as follows.

-	The gNB communicates supported NID(s) to AMF in the following NGAP messages: NG SETUP REQUEST (see clause 9.2.6.1 of TS 38.413 [9]) and RAN CONFIGURATION UPDATE (see clause 9.2.6.4 of TS 38.413 [9]).

-	The gNB does not communicate supported CAG(s) to the AMF; instead, it keeps this cell-level information internally. The gNB uses information on supported CAG(s) to accept/reject handover requests from AMF in the following NGAP message: HANDOVER REQUEST (see clause 9.2.3.4 of TS 38.413 [9])

For NG-RAN split deployments, individual gNB-DU needs to be configured (via 3GPP management system) with lists of NID(s) and CAG(s) it supports, for SNPN and PNI-NPN, respectively. In the F1 Application Protocol (F1AP), this information is used as follows:

-	Each gNB-DU communicates supported NID(s) to the gNB-CU in the following F1AP messages: F1 SETUP REQUEST (see clause 9.2.1.4 of TS 38.473 [10]) and gNB-DU CONFIGURATION UPDATE (see clause 9.2.1.7 of TS 38.473 [10]). With this information, the gNB-CU knows NPN support information about the cells configured in this gNB-DU.

-	Upon receiving the above information from individual gNB-DUs, the gNB-CU knows which NID(s) are available for use. The reason is that not all distributed gNB-DUs under the same gNB-CU may necessarily the same NIDs. The gNB-CU can then decide

-	Based on this information, the gNB-CU can decide on which specific cells need to be activated on individual gNB-DUs. The gNB-CU communicate this information in the following F1AP messages: F1 SETUP RESPONSE (see clause 9.2.1.5 of TS 38.473 [10]) and gNB-DU CONFIGURATION ACKNOWLEDGE (see clause 9.2.1.8 of TS 38.473 [10]).

-	gNB-DUs do not communicate supported CAG(s) to the gNB-DU; instead, they keep this cell-level information internally.

There could be scenarios where the NG-RAN node supporting NPNs is shared using 5G MOCN. In all these NPN sharing scenarios, each Cell Identity as specified in TS 38.331 [11] is associated with one of the following configuration options:

-	one or multiple SNPNs;

-	one or multiple PNI-NPNs (with CAG);

-	one or multiple PLMNs only.

For more details on these configuration options, see clause 5.18 of TS 23.501 [3].

End of change