# 3GPP TSG SA WG5 Meeting #139e	S5-215149

# Online, , 11 Oct 2021- 20 Oct 2021

# Source:	Samsung

# Title:	DP 28.538 DN NRM

# Document for:	Endorsement

Agenda Item:	6.4.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

- The following is the EDGEAPP architecture defined in 23.558.

Observation 1: EDN contains EAS(s) and EES(s)

- The ANNEX A.2 of TS 23.558 provides the following deployment option for a EDN

Observation 2: An EDN is deployed as a DN/LADN.

- The TS 23.548 provides the following architecture for supporting edge computing

Observation 3: The EAS is deployed as part of LADN (Local part of DN)

- TS 28.541 defines the following NRM support

Observation 4: Although the name is DNFunction the DNFunction IOC does not inherit from ManagedFunction. Hence, cannot be equated with other 5GC network functions

Depending on the above observations the following are the two options to support the same in EDGE NRM.

- Option-A

Edge NRM containment naming relationship

Edge NRM Inheritance

The DNFunction can be used to represent EDN. The DNFunction when used for Edge Computing will contain the following attributes. This can be achieved by defining the attributes as “Conditional Manadatory” for DNFunction IOC.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| EASFunctionRef | CM | T | T | F | T |
| EESFunctonRef | CM | T | T | F | T |

| Name | Definition |
| --- | --- |
| EASFunctionRef Support Qualifier | Condition: DNFunction is used for Edge Computing i.e representing an EDN |
| EESFunctonRef Support Qualifier | Condition: DNFunction is used for Edge Computing i.e representing an EDN |

- Option-B

Edge NRM containment naming relationship

Edge NRM Inheritance

The EdgeDataNetwork IOC can be defined representing an EDN. The EdgeDataNetwork IOC will contain the reference to the constituent EAS and EES. There will be no requirement to manage the EDN itself i.e there won’t be any requirement for a consumer to request for EDN LCM/provisioning, PM and FS.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| EASFunctionRef | CM | T | T | F | T |
| EESFunctonRef | CM | T | T | F | T |

# 4	Detailed proposal

The WG is requested to endorse either of the follwing options.

- Option-A: Use of DNFunction IOC to represent EDN i.e S5-215147

- Option-B: Defining EdgeDataNetwork IOC to represent EDN i.e S5-215148