3GPP TSG-SA5 Meeting #139-e 	S5-215236

# e-meeting, 11 - 20 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0345 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS charging information enumerations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adding the IMS charging information enumerations |  |  |  |  |  |  |  |  |
| Summary of change: |  | The initial IMS charging information enumerations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging for IMS cannot be supported by converged charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.3.a (new), 6.1.6.3.b (new), 6.1.6.3.c (new), 6.1.6.3.d (new), 6.1.6.3.e (new), 6.1.6.3.f (new), 6.1.6.3.g (new), 6.1.6.3.h (new), 6.1.6.3.i (new), 6.1.6.3.j (new), 6.1.6.3.k (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 6.1.6.3.a	Enumeration: RoleOfIMSNode

Table 6.1.6.3.x-1: Enumeration RoleOfIMSNode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ORIGINATING | The node is applying an originating role, serving the calling party. |  |
| TERMINATING | The node is applying a terminating role, serving the called party. |  |
| FORWARDING | The node is applying a originating role, serving the forwarding party. |  |

Second change

##### 6.1.6.3.b	Enumeration: IMSSessionPriority

Table 6.1.6.3.x-1: Enumeration IMSSessionPriority

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PRIORITY_0 | Mapped from the value received by the CSCF. (NOTE 1) |  |
| PRIORITY_1 | Mapped from the value received by the CSCF. (NOTE 1) |  |
| PRIORITY_2 | Mapped from the value received by the CSCF. (NOTE 1) |  |
| PRIORITY_3 | Mapped from the value received by the CSCF. (NOTE 1) |  |
| PRIORITY_4 | Mapped from the value received by the CSCF. (NOTE 1) |  |
| NOTE 1: The mapping from TS 24.229 [258] table A.162 is operator specific. NOTE 2: PRIORITY_0 is the highest priority. |  |  |

Third change

##### 6.1.6.3.c	Enumeration: MediaInitiatorFlag

Table 6.1.6.3.x-1: Enumeration MediaInitiatorFlag

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CALLED_PARTY | The called party initiated the modification. (NOTE 1) |  |
| CALLING_PARTY | The calling party initiated the modification. |  |
| UNKNOWN | It’s unkown who initiated the modification. |  |
| NOTE 1: The default is called party. |  |  |

Fourth change

##### 6.1.6.3.d	Enumeration: SDPType

Table 6.1.6.3.x-1: Enumeration LocalGWInsertedIndication

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| OFFER | The SDP media component was of type SDP offer. |  |
| ANSWER | The SDP media component was of type SDP answer. |  |

Fifth change

##### 6.1.6.3.e	Enumeration: OriginatorPartyType

Table 6.1.6.3.x-1: Enumeration OriginatorPartyType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CALLING | The calling party is the originator. |  |
| CALLED | The called party is the originator |  |

Sixth change

##### 6.1.6.3.f	Enumeration: AccessTransferType

Table 6.1.6.3.x-1: Enumeration AccessTransferType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PS_TO_CS | Transferred from packet switched to circuit switched. |  |
| CS_TO_PS | Transferred from circuit switched to packet switched |  |
| PS_TO_PS | Transferred from packet switched to packet switched |  |
| CS_TO_CS | Transferred from circuit switched to circuit switched |  |

Seventh change

##### 6.1.6.3.g	Enumeration: UETransferType

Table 6.1.6.3.x-1: Enumeration UETransferType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| INTRA_UE | The type of transfer is intra-UE. |  |
| INTER_UE | The type of transfer is inter-UE. |  |

Eighth change

##### 6.1.6.3.h	Enumeration: NNISessionDirection

Table 6.1.6.3.x-1: Enumeration NNISessionDirection

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| INBOUND | NNI is used for an inbound service request. |  |
| OUTBOUND | NNI is used for an outbound service request. |  |

Eighth change

##### 6.1.6.3.i	Enumeration: NNIType

Table 6.1.6.3.x-1: Enumeration NNIType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NON_ROAMING | Type of used NNI is non-roaming. |  |
| ROAMING_NO_LOOPBACK | Type of used NNI is roaming without loopback routing. |  |
| ROAMING_LOOPBACK | Type of used NNI is roaming with loopback routing. |  |

Nineth change

##### 6.1.6.3.j	Enumeration: NNIRelationshipMode

Table 6.1.6.3.x-1: Enumeration NNIRelationshipMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TRUSTED | Is regarded as part of the same trust domain. |  |
| NON_TRUSTED | Is not regarded as part of the same trust domain. |  |

Tenth change

##### 6.1.6.3.k	Enumeration: TADIdentifier

Table 6.1.6.3.x-1: Enumeration TADIdentifier

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CS | The session shall be terminated in a circuit switched access network. |  |
| PS | The session shall be terminated in a packet switched access network. |  |

End of changes