3GPP TSG-SA5 Meeting #140-e 	S5-216275

# e-meeting, 15 - 24 November 2021

# Source:	China Mobile

# Title:	pCR draft TS28.104 add slice coverage analysis

# Document for:	Approval

Agenda Item:	6.4.18

1	Decision/action requested

For approval

2	References

[1] 	3GPP TR28.809, v17.0.0, Management and orchestration; Study on enhancement of Management Data Analytics (MDA).

[2] 	3GPP TS28.104, v0.2.0,  Management and orchestration; Management Data Analytics (MDA).

3	Rationale

The MDA use case of slice coverage analysis has bee studied in TR 28.809 [1], and it is included in the outline of TS28.104 [2]. The contribution is to add use case and requirement of slice coverage analysis.

4	Detailed proposal

1st modification

## 5.1	General framework

#### 7.2.1.2	Slice coverage analysis (NS-COV_MDA)

##### 7.2.1.2.1	Description

The 3GPP management system shall have the capability to provide the slice coverage analysis

##### 7.2.1.2.2	Use case

The slice coverage is one of the indicators when a 3rd party (i.e. slice tenant) issues a slice request and is mapped into the desired geographical coverage area with the available radio coverage which depends on the base station planning and deployment. In order to map the desired slice coverage perfectly, MDAS can be used to optimize the slice coverage and load distribution on the slice instantiation and runtime considering (i) slice-aware statistics, e.g., slice-UE distributions and mobility patterns, (ii) slice SLA and (iii) access node capabilities.

##### 7.2.1.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- NS-COV _MDA-CON-1 | 3GPP management system shall be able to provide the analytics report describing the slice coverage, slice availability and slice prediction information. | (NS-COV_MDA) Slice coverage analysis |
| REQ- NS-COV _MDA-CON-2 | 3GPP management system shall be able to provide the mapping between slice coverage and actual radio deployment, | (NS-COV_MDA) Slice coverage analysis |

End of modifications