3GPP TSG-SA5 Meeting #140-e 	S5-216235

# e-meeting, 15 - 24 November 2021

# Source:	Huawei

# Title:	Add potential solution and conclusion for Issue#5

# Document for:	Approval

Agenda Item:	6.5.6

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	3GPP TR 28.925 enhancement of service based management architecture v0.3.0

3	Rationale

It is proposed to add solutions to add potential solution and conclusion for Issue#5 in TR 28.925 [1].

4	Detailed proposal

This document proposes the following changes in TR 28.925 [1].

1st Change

4.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs

4.5.1	Description

There are some relevant works on the architectures and frameworks for automation of management and orchestration in other Standards Developing Organizations (SDOs) in industry. This clause provides a brief overview on the works in the related SDOs.

4.5.2	Potential solutions

4.5.2.1	ETSI ISG ZSM

4.5.2.1.1	Introduction

The goal of ETSI ZSM is to enable zero-touch automated network and service management in a multivendor environment.

As documented in ETSI GS ZSM 002 [6], it provides the ZSM framework reference architecture as shown in figure 4.5.2.1.1-1 and distributed management and data services, organized into management domains and integrated via an integration fabric. The integration fabric is used to enable management service consumption, communication, and integration with 3rd party management systems. The cross-domain data service allows data sharing across domains. All management services provide a set of capabilities for their consumption.

Figure 4.5.2.1.1-1: ZSM framework reference architecture

In figure 4.5.2.1.1-1, every management domain, as well as the E2E service management domain, provides a set of ZSM service capabilities by management functions that expose and/or consume set of service end-points. The cross-domain integration fabric facilitates providing capabilities and accessing endpoints cross-domain. Some services are only provided and consumed locally inside the management domain.

4.5.2.1.2	Potential solutions #5-1

3GPP management service deployment framework based on ZSM framework reference architecture is captured in clause 5.3 of TS 28.533 [2]. 3GPP SBMA is aligned with ZSM framework reference architecture.

2nd Change

5	Conclusion and Recommendation

5.x Issue #5: SBMA supporting management architecture and frameworks in other SDOs

3GPP SBMA is aligned with ZSM framework reference architecture, and related description is captured in TS 28.533[2]. No further modification is needed to existing specifications.

End of change