3GPP TSG-SA5 Meeting #141-e 	S5-221163

# e-meeting, 17 -26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.663 | CR | 0021 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.663 Provide YAML solution set for Generic RAN NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSA_SBMA |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A YAML solution set is needed for Generic Radio Access Network NRM,but haven’t been defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A YAML solution set is needed for Generic Radio Access Network NRM is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,4,Annex X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge Link: https://forge.3gpp.org/rep/sa5/MnS/tree/28.663_Rel17_CR_TS_Provide_YAML_solution_set_for_Generic_RAN_NRM |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-216165 is the revision of S5-215133 S5-221163 is the revision of S5-216165 |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 28.662: "Generic Radio Access Network (RAN) Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)”.

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	3GPP TS 32.606: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[7]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[8]	W3C REC-xml11-20060816: "Extensible Markup Language (XML) 1.1 (Second Edition)".

[9]	Void.

[10]	W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures.

[11]	W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes.

[12]	W3C REC-xml-names-20060816: "Namespaces in XML 1.1 (Second Edition)".

[13]	3GPP TS 28.623: "Generic network resources Integration Reference Point (IRP); Solution Set (SS) definition".

[14]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[Y]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

2nd Change

4	Solution Set (SS) definition

This specification defines the following 3GPP Generic RAN NRM IRP Solution Set Definitions:

-	3GPP Generic RAN NRM IRP CORBA SS (Annex A)

-	3GPP Generic RAN NRM IRP XML Definitions (Annex B)

-	3GPP Generic RAN NRM YAML Definitions (Annex X)

3rd Change

Annex X (normative):
OpenAPI definition of the Generic RAN NRM

X.1	General

This annex contains the OpenAPI definition of the Generic RAN NRM in YAML format.

The Information Service (IS) of the Generic RAN NRM is defined in 3GPP TS 28.662[4].

Mapping rules to produce the OpenAPI definition based on the IS are defined in TS 32.160 [Y].

X.2	OpenAPI document "GenericRanNrm.yaml"

openapi: 3.0.1

info:

title: Generic RAN NRM

version: 17.0.0

description: >-

OAS 3.0.1 specification of the Generic RAN NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.663; Generio RAN NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.663/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

EutranFqband:

type: string

EutranFqbandList:

type: array

items:

$ref: '#/components/schemas/EutranFqband'

NrFqband:

type: string

NrFqbandList:

type: array

items:

$ref: '#/components/schemas/NrFqband'

UtranFDDFqband:

type: string

UtranFDDFqbandList:

type: array

items:

$ref: '#/components/schemas/UtranFDDFqband'

UtranTDDFqband:

type: string

UtranTDDFqbandList:

type: array

items:

$ref: '#/components/schemas/UtranTDDFqband'

AngleValue:

type: integer

minimum: 0

maximum: 3600

SharedTechnologies:

type: string

enum:

- GSM

- UMTS

- LTE

- GSM_UMTS

- GSM_LTE

- UMTS_LTE

- GSM_UMTS_LTE

Latitude:

type: integer

minimum: -90

maximum: 90

Longitude:

type: integer

minimum: -180

maximum: 180

CtrlConnMode:

type: string

enum:

- GSM_SMS

- WCDMA_SMS

- CIRCLE_SWITCH_DATA_CSD

- PACKAGE_SWITCH_DATA_IP

- SERIAL_PORT

RepeaterType:

type: string

enum:

- WideBandReptFunction

- FreqSelReptFunction

- FiberReptFunction

- IndoorReptFunction

- FreqShiftReptFunction

#-------- Definition of concrete IOCs --------------------------------------------

ManagedElement-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

SectorEquipmentFunction:

$ref: '#/components/schemas/SectorEquipmentFunction-Multiple'

AntennaFunction:

$ref: '#/components/schemas/AntennaFunction-Multiple'

TMAFunction:

$ref: '#/components/schemas/TMAFunction-Multiple'

CommonBsFunction:

$ref: '#/components/schemas/CommonBsFunction-Multiple'

RepeaterFunction:

$ref: '#/components/schemas/RepeaterFunction-Multiple'

SectorEquipmentFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: >-

genericNrm.yaml#/components/schemas/ManagedFunction-Attr

- type: object

properties:

fqBand:

type: integer

eUTRANFqBands:

$ref: '#/components/schemas/EutranFqbandList'

nRFqBands:

$ref: '#/components/schemas/NrFqbandList'

uTRANFDDFqBands:

$ref: '#/components/schemas/UtranFDDFqbandList'

uTRANTDDFqBands:

$ref: '#/components/schemas/UtranTDDFqbandList'

confOutputPower:

type: integer

theTMAList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

theAntennaList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

theCellList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

theNRSectorCarrierList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

AntennaFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: >-

genericNrm.yaml#/components/schemas/ManagedFunction-Attr

- type: object

properties:

retTiltValue:

type: integer

bearing:

type: string

retGroupName:

type: integer

height:

type: integer

maxAzimuthValue:

$ref: '#/components/schemas/AngleValue'

minAzimuthValue:

$ref: '#/components/schemas/AngleValue'

horizBeamwidth:

$ref: '#/components/schemas/AngleValue'

vertBeamwidth:

$ref: '#/components/schemas/AngleValue'

theCellList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

TMAFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: >-

genericNrm.yaml#/components/schemas/ManagedFunction-Attr

- type: object

properties:

tmaSubunitNumber:

type: integer

tmaStateFlag:

type: integer

tmaFunctionFlag:

type: integer

tmaMinGain:

type: integer

tmaMaxGain:

type: integer

tmaResolution:

type: integer

tmaGainFigure:

type: integer

tmaNumberOfSubunits:

type: integer

tmaBaseStationId:

type: string

tmaSectorId:

type: string

tmaAntennaBearing:

type: integer

tmaInstalledMechanicalTilt:

type: integer

tmaSubunitType:

type: integer

tmaSubunitRxFrequencyBand:

type: integer

tmaSubunitTxFrequencyBand:

type: integer

tmaGainResolution:

type: integer

theCellList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

CommonBsFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: >-

genericNrm.yaml#/components/schemas/ManagedFunction-Attr

- type: object

properties:

sharedTechnologies:

$ref: '#/components/schemas/SharedTechnologies'

theProxyBsList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

RepeaterFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: >-

genericNrm.yaml#/components/schemas/ManagedFunction-Attr

- type: object

properties:

priority:

type: integer

latitude :

$ref: '#/components/schemas/Latitude'

longitude:

$ref: '#/components/schemas/Longitude'

ctrlConnMode:

$ref: '#/components/schemas/CtrlConnMode'

environmentInfo:

type: string

powerSwitch:

type: boolean

ulAttenuation:

type: integer

dlAttenuation:

type: integer

firmwareVer:

type: string

repeaterType:

$ref: '#/components/schemas/RepeaterType'

externalUTRANCell:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

#-------- Definition of arrays for name-contained IOCs ----------------------

SectorEquipmentFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/SectorEquipmentFunction-Single'

AntennaFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AntennaFunction-Single'

TMAFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/TMAFunction-Single'

CommonBsFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/CommonBsFunction-Single'

RepeaterFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/RepeaterFunction-Single'

#-------- Definitions in TS 28.663 for TS 28.532 ---------------------------------

resources-grNrm:

oneOf:

- $ref: '#/components/schemas/SectorEquipmentFunction-Single'

- $ref: '#/components/schemas/AntennaFunction-Single'

- $ref: '#/components/schemas/TMAFunction-Single'

- $ref: '#/components/schemas/CommonBsFunction-Single'

- $ref: '#/components/schemas/RepeaterFunction-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

End of Changes