3GPP TSG-SA5 Meeting #141-e 	S5-221308

# e-meeting, 17 -26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0367 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting charging rule base name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The charging rule base name is not part of PDU session information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding separate sections for PDU and QFI container information instead of having them under each information element as well as moving the charging rule base name to PDU container information where it belongs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The charging rule base name could be interpreted as belonging to the PDU session information which could lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMF for 5G data connectivity converged charging or offline only charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.2.1 defines the basic structure of the supported fields in the Charging Data Request message for 5G data connectivity converged charging or offline only charging.

Table 6.2.2.1: Supported fields in Charging Data Request message

| Information Element | Functionality of SMF | FBC | QBC | FBC |  | QBC |
| --- | --- | --- | --- | --- | --- | --- |
|  | Charging Service | Converged Charging | Converged Charging | Offline Only Charging |  | Offline Only Charging |
|  | Supported Operation Types | I/U/T/E | I/U/T/E | I/U/T/E |  | I/U/T/E |
| Session Identifier |  | -UT- | -UT- | -UT- |  | -UT- |
| Subscriber Identifier |  | IUT- | IUT- | IUT- |  | IUT- |
| NF Consumer Identification |  | IUT- | IUT- | IUT- |  | IUT- |
| Invocation Timestamp |  | IUT- | IUT- | IUT- |  | IUT- |
| Invocation Sequence Number |  | IUT- | IUT- | IUT- |  | IUT- |
| Retransmission Indicator |  | IUT- | IUT- | IUT- |  | IUT- |
| Notify URI |  | IU- | IU- | IU- |  | IU- |
| Supported Features |  | IU- | IU- | - |  | - |
| Service Specification Information |  | IUT- | IUT- | IUT- |  | IUT- |
| Triggers |  | -UT- | -UT- | -UT- |  | -UT- |
| Multiple Unit Usage |  | IUT- | - | IUT- |  | - |
| Rating Group |  | IUT- | - | IUT- |  | - |
| Requested Unit |  | IU-- | - | - |  | - |
| Used Unit Container |  | -UT- | - | -UT- |  | - |
| Triggers |  | -UT- | - | -UT- |  | - |
| UPF ID |  | IUT- | IUT- | IUT- |  | IUT- |
| PDU Session Charging Information |  | IUT- | IUT- | IUT- |  | IUT- |
| Charging Id |  | IUT- | IUT- | IUT- |  | IUT- |
| Home Provided Charging Id |  | -UT- | -UT- | -UT- |  | -UT- |
| User Information |  | IUT- | IUT- | IUT- |  | IUT- |
| User Location Info |  | IUT- | IUT- | IUT- |  | IUT- |
| MA PDU Non 3GPP User Location Info |  | IUT- | IUT- | IUT- |  | IUT- |
| User Location Time |  | IUT- | IUT- | IUT- |  | IUT- |
| MA PDU Non 3GPP User Location Time |  | IUT- | IUT- | IUT- |  | IUT- |
| UE Time Zone |  | IUT- | IUT- | IUT- |  | IUT- |
| Presence Reporting Area Information |  | -UT- | -UT- | -UT- |  | -UT- |
| PDU Session Information |  | IUT- | IUT- | IUT- |  | IUT- |
| PDU Session ID |  | IUT- | IUT- | IUT- |  | IUT- |
| Network Slice Instance Identifier |  | IUT- | IUT- | IUT- |  | IUT- |
| PDU Type |  | IUT- | IUT- | IUT- |  | IUT- |
| PDU Address |  | IUT- | IUT- | IUT- |  | IUT- |
| SSC Mode |  | IUT- | IUT- | IUT- |  | IUT- |
| MA PDU session information |  | IUT- | IUT- | IUT- |  | IUT- |
| SUPI PLMN ID |  | IUT- | IUT- | IUT- |  | IUT- |
| Serving Network Function ID |  | IUT- | IUT- | IUT- |  | IUT- |
| Serving CN PLMN ID |  | IUT- | IUT- | IUT- |  | IUT- |
| RAT Type |  | IUT- | IUT- | IUT- |  | IUT- |
| MA PDU Non 3GPP RAT Type |  | IUT- | IUT- | IUT- |  | IUT- |
| Data Network Name Identifier |  | IUT- | IUT- | IUT- |  | IUT- |
| DNN Selection Mode |  | IUT- | IUT- | IUT- |  | IUT- |
| Authorized QoS Information |  | IUT- | IUT- | IUT- |  | IUT- |
| Subscribed QoS Information |  | IUT- | IUT- | IUT- |  | IUT- |
| Authorized Session-AMBR |  | IUT- | IUT- | IUT- |  | IUT- |
| Subscribed Session-AMBR |  | IUT- | IUT- | IUT- |  | IUT- |
| PDU session start Time |  | I--- | I--- | I--- |  | I--- |
| PDU session stop Time |  | --T- | --T- | --T- |  | --T- |
| Diagnostics |  | --T- | --T- | --T- |  | --T- |
| Enhanced Diagnostics |  | --T- | --T- | --T- |  | --T- |
| Charging Characteristics |  | IUT- | IUT- | IUT- |  | IUT- |
| Charging Characteristics Selection Mode |  | IUT- | IUT- | IUT- |  | IUT- |
| 3GPP PS Data Off Status |  | IUT- | IUT- | IUT- |  | IUT- |
| Session Stop Indicator |  | --T- | --T- | --T- |  | --T- |
| Redundant Transmission Type |  | IUT- | IUT- | IUT- |  | IUT- |
| PDU Session Pair ID |  | IUT- | IUT- | IUT- |  | IUT- |
| Unit Count Inactivity Timer |  | IU-- | - | IU-- |  | - |
| RAN Secondary RAT Usage Report |  | -UT- | -UT- | -UT- |  | -UT- |
| PDU Container Information |  | -UT- | - | -UT- |  | - |
| Time of First Usage |  | -UT- | - | -UT- |  | - |
| Time of Last Usage |  | -UT- | - | -UT- |  | - |
| QoS Information |  | -UT- | - | -UT- |  | - |
| QoS Characteristics |  | -UT- | - | -UT- |  | - |
| AF Charging Identifier |  | -UT- | - | -UT- |  | - |
| AF Charging Id String |  | -UT- | - | -UT- |  | - |
| User Location Information |  | -UT- | - | -UT- |  | - |
| UE Time Zone |  | -UT- | - | -UT- |  | - |
| Presence Reporting Area Information |  | -UT- | - | -UT- |  | - |
| Serving Network Function ID |  | -UT- | - | -UT- |  | - |
| RAT Type |  | -UT- | - | -UT- |  | - |
| Sponsor Identity |  | -UT- | - | -UT- |  | - |
| Application Service Provider Identity |  | -UT- | - | -UT- |  | - |
| Charging Rule Base Name |  | -UT- | - | -UT- |  | - |
| 3GPP PS Data Off Status |  | -UT- | - | -UT- |  | - |
| MA PDU Steering functionality |  | -UT- | - | -UT- |  | - |
| MA PDU Steering mode |  | -UT- | - | -UT- |  | - |
| Roaming QBC information |  | - | IUT- | - |  | IUT- |
| Multiple QFI container |  | - | IUT- | - |  | IUT- |
| UPF ID |  | - | IUT- | - |  | IUT- |
| Roaming Charging Profile |  | - | IU-- | - |  | IU-- |
| QFI Container Information |  | -UT- | - | -UT- |  | - |
| QFI Container Information |  | -UT- | - | -UT- |  | - |
| QoS Flow Id |  | -UT- | - | -UT- |  | - |
| Time of First Usage |  | -UT- | - | -UT- |  | - |
| Time of Last Usage |  | -UT- | - | -UT- |  | - |
| QoS Information |  | -UT- | - | -UT- |  | - |
| QoS Characteristics |  | -UT- | - | -UT- |  | - |
| User Location Information |  | -UT- | - | -UT- |  | - |
| UE Time Zone |  | -UT- | - | -UT- |  | - |
| Presence Reporting Area Information |  | -UT- | - | -UT- |  | - |
| RAT Type |  | -UT- | - | -UT- |  | - |
| Report Time |  | -UT- | - | -UT- |  | - |
| Serving Network Function ID |  | -UT- | - | -UT- |  | - |
| 3GPP PS Data Off Status |  | -UT- | - | -UT- |  | - |
| QFI Container Information |  | -UT- | - | -UT- |  | - |

Table 6.2.2.2 defines the basic structure of the supported fields in the Charging Data Response message for 5G data connectivity converged charging or offline only charging.

Table 6.2.2.2: Supported fields in Charging Data Response message

| Information Element | Functionality of SMF | FBC | QBC | FBC |  | QBC |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Charging Service | Converged Charging | Converged Charging | Offline Only Charging |  | Offline Only Charging |  |
|  |  |  | Supported Operation Types |  |  |  |  | I/U/T/E |  |  |  |  |  | I/U/T/E |  |  |  |  |  | I/U/T/E |  |  |  |  |  | I/U/T/E |  |  |  |  |  |
| Session Identifier |  |  |  |  |  |  |  | I--- |  |  |  |  |  | I--- |  |  |  |  |  | I--- |  |  |  |  |  | I--- |  |  |  |  |  |
| Invocation Timestamp |  |  |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  |
| Invocation Result |  |  |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  |
| Invocation Sequence Number |  |  |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  | IUT- |  |  |  |  |  |
| Session Failover |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | IUT- |  |  |  |  |  | IU-- |  |  |  |  |  | IUT- |  |  |  |  |  |
| Supported Features |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Triggers |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  |
| Multiple Unit information |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  |
| Result Code |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  |
| Rating Group |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  |
| UPF ID |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  |
| Granted Unit |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Validity Time |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Final Unit Indication |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Time Quota Threshold |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Volume Quota Threshold |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Unit Quota Threshold |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Quota Holding Time |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Triggers |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  |
| PDU Session Charging Information |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  |
| Charging Id |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Home Provided Charging Id |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| User Information |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| User Location Info |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| MA PDU Non 3GPP User Location info |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| User Location Time |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| MA PDU Non 3GPP User Location Time |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| UE Time Zone |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Presence Reporting Area Information |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  | IU-- |  |  |  |  |  |
| PDU Session Information |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| PDU Session ID |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Network Slice Instance Identifier |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| PDU Type |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| PDU Address |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| SSC Mode |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| MA PDU session information |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| SUPI PLMN ID |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Serving Network Function ID |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Serving CN PLMN ID |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| RAT Type |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| MA PDU Non 3GPP RAT Type |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Data Network Name Identifier |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| DNN Selection Mode |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Authorized QoS Information |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Subscribed QoS Information |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Authorized Session-AMBR |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Subscribed Session-AMBR |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| PDU session start Time |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| PDU session stop Time |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Diagnostics |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Enhanced Diagnostics |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Charging Characteristics |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Charging Characteristics Selection Mode |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| 3GPP PS Data Off Status |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Session Stop Indicator |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Redundant Transmission Type |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| PDU Session Pair ID |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Unit Count Inactivity Timer |  |  |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| RAN Secondary RAT Usage Report |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| PDU Container Information |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Time of First Usage |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Time of Last Usage |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| QoS Information |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| QoS Characteristics |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| AF Charging Identifier |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| AF Charging Id String |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| User Location Information |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| UE Time Zone |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Presence Reporting Area Information |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Serving Network Function ID |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| RAT Type |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Sponsor Identity |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Application Service Provider Identity |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Charging Rule Base Name |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| 3GPP PS Data Off Status |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| MA PDU Steering functionality |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| MA PDU Steering mode |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Roaming QBC information |  |  |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  |
| Multiple QFI container |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| UPF ID |  |  |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Roaming Charging Profile |  |  |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  | - |  |  |  |  |  | IU-- |  |  |  |  |  |
| QFI Container Information |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| QoS Flow Id |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Time of First Usage |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Time of Last Usage |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| QoS Information |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| QoS Characteristics |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| User Location Information |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| UE Time Zone |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Presence Reporting Area Information |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| RAT Type |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Report Time |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| Serving Network Function ID |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |
| 3GPP PS Data Off Status |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |

End of changes