3GPP TSG-SA5 Meeting #141-e	S5-221759

# Online, 17th Jan 2022 - 26th Jan 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | Draft CR | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Add file download NRM fragment, YANG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FIMA |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Requirements for file download were agreed at SA5#138. This contribution proposes the corresponding file download control NRM fragment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add FileDownloadJob IOC to NRM, including support for common async operation monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI FIMA solution will lack download support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR 28.622 CR 0144 |  |  |
| Other comments: |  | Baseline DraftCR for FIMA: S5-214758. Definition uses a common datatype (ProcessMonitor) as defined in S5-221549. Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/tree/S5-221330_28.622_Rel-17_Input_to_DraftCR_Add_file_download_control_NRM_fragment |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-221371 |  |  |  |  |  |  |  |  |

First modification

## D.2.11	module _3gpp-common-filemanagement.yang

<CODE BEGINS>

module _3gpp-common-filemanagement {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-filemanagement";

prefix "filemgmt3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix yang3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the FileDownloadJob Information Object

Class (IOC) that is part of the Generic Network Resource Model (NRM).";

reference "3GPP TS 28.623 Generic Network Resource Model (NRM)";

revision 2022-02-10 { reference "Initial revision, S5-221757"; }

grouping FileDownloadProcessMonitor {

description "Provides specialisations of the ProcessMonitor datatype.";

uses yang3gpp:ProcessMonitor {

refine resultStateInfo {

description "If status is FAILED resultStateInfo will be one of

the following or empty.";

must '../status != "FAILED"

or . = "UNKNOWN"

or . = "NO_STORAGE"

or . = "LOW_MEMORY"

or . = "NO_CONNECTION_TO_REMOTE_SERVER"

or . = "FILE_NOT_AVAILABLE"

or . = "DNS_CANNOT_BE_RESOLVED"

or . = "TIMER_EXPIRED"

or . = "OTHER"

or . = "NULL" ';

}

refine status {

description "Status must not be 'PARTIALLY_FAILED'.";

must '. != "PARTIALLY_FAILED" ';

}

}

}

grouping FileDownloadJobGrp {

description "Represents the FileDownloadJob IOC.";

reference "3GPP TS 28.622";

uses top3gpp:Top_Grp;

leaf fileLocation {

description "Provides the location of a file.

allowedValues:  File URI (See RFC 8089)";

mandatory true;

type string;

}

leaf notificationRecipientAddress {

description "Address of the notification recipient.";

type string;

}

leaf cancelJob {

description "Setting this attribute to TRUE cancels the file download job.

As specified in the definition of ProcessMonitor, cancellation is possible in

the NOT_STARTED and RUNNING state. Setting the attribute to FALSE has no

observable result.";

type boolean;

mandatory true;

}

list jobMonitor {

key "id";

description "Provides monitoring for the file download job.";

uses FileDownloadProcessMonitor;

}

}

grouping FileDownloadJobSubtree {

description "Helps augmenting FileDownloadJob into multiple places.";

list FileDownloadJob {

description "Specifies the FileDownloadJob IOC, see 3GPP TS 28.622.";

key "id";

uses top3gpp:Top_Grp;

container attributes {

uses FileDownloadJobGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses FileDownloadJobSubtree;

}

augment "/me3gpp:ManagedElement" {

uses FileDownloadJobSubtree;

}

}

<CODE ENDS>

End of modifications