3GPP TSG-SA5 Meeting #141-e 	S5-221321

- e-meeting, 17 -26 January 2022

- Source:	Ericsson

- Title:	Corrections to clause 7.1

- Document for:	Approval

Agenda Item:	7.5.4

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: " Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Correcting the figures in clause 7.1 as well as adding general clause for the 7.1.4.3.

# 4	Detailed proposal

First change

#### 7.1.1.2	Use case #1b: Visited MNO charging home MNO for 5G connection and mobility provided to the home MNO’s CSCs

This use case focuses on visited MNO and home MNO business roles.

A CSC (UE) has a subscription with the home MNO which allows usage of 5G data connectivity connection and mobility.

The visited MNO to home MNO charging is based number of home MNO’s UEs (i.e. unique SUPI) connected to the visited MNO’s network over a specific period of time.

Second change

##### 7.1.4.1.1	General

A possible solution for key issue 1a, wholesale charging for 5G data connectivity provided to the home MNO by the visited MNO, covering both home routed and local breakout case.

Third change

##### 7.1.4.1.2	Reference architecture

Figure 7.1.4.1.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.1.4.1.2-2: Roaming 5G data connectivity scenario in reference point representation

##### The SMF with CTF interact with CHF using Nchf interface. The CDRs produced by CHF may send this for interconnect charging (shown as dashed in the figures), which then can be used to aggregate and calculate the amount of data per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.1.4.1.3	Message flows

For both home routed roaming and LBO, the flows for 5G data connectivity charging would be the same as in clause 5.2.2.12 of TS 32.255 [4] with the difference that for the LBO case would be that there would be no interaction with the home PLMN’s UPF, SMF and CHF.

Fourth change

##### 7.1.4.2.2	Reference architecture

Figure 7.1.4.2.2-1: Roaming 5G connection and mobility scenario in service-based interface representation

Figure 7.1.4.2.2-2: Roaming 5G connection and mobility scenario in reference point representation

The AMF with CTF interact with CHF using Nchf interface. The CDRs produced by CHF may be send for interconnect charging (shown as dashed in the figures), which then can be used to aggregate and calculate the number of connected UEs per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

Fifth change

##### 7.1.4.3.1	General

A possible solution for key issue 1a, wholesale charging for 5G data connectivity provided to the home MNO by the visited MNO, covering local breakout case. In this case there is also charging information generated in the home MNO.

Sixth change

##### 7.1.4.3.2	Reference architecture

Figure 7.1.4.3.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.1.4.3.2-2: Roaming 5G data connectivity scenario in reference point representation

Seventh change

##### 7.1.4.3.3	Message flows

For local breakout roaming, the flows for 5G data connectivity charging are present as the following figure 7.1.4.3.3-1.

Figure 7.1.4.3.3-1: Local breakout roaming for 5G data connectivity charging

1-3a.	UE initiates a new PDU session. V-SMF selection by the AMF.

4ch-a. The UE is identified as a roamer (PLMN ID of the received SUPI is different from VPLMN PLMN ID), the V-CHF is selected accordingly. A Charging Data Request [Initial, Roaming Charging profile] is sent to V-CHF, indicating "in-bound roamer".

4ch-b. The V-CHF opens a CDR (indicating "in-bound roamer")

4ch-c. The V-CHF acknowledges by sending Charging Data Response [Initial, Roaming Charging profile] to the V-SMF.

7ch-a. A Charging Data Request [Update] is sent to V-CHF with charging information received from V-SMF.

7ch-b. The V-CHF updates the CDR.

7ch-c. The V-CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

10ch-a. A Charging Data Request [Termination] is sent to V-CHF.

10ch-b. The V-CHF closes the CDR

10ch-c. The V-CHF acknowledges by sending Charging Data Response [Termination] to the V-SMF.

Step 5, Step 8 and Step 11 are detailed in clause 7.2.4.2.3.

End of changes