| 3GPP TR 28.824 V0.5.0 (2022-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on network slice management capability exposure (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	7

4.1	General	7

4.1.1	Concepts related to network management capability exposure	7

4.1.2	Roles related to network management capability exposure	10

4.1.3	Types of interface for the exposure of network slice	11

4.1.4	Procedures related to consumption of exposed network management capabilities	14

4.2	Issues	18

4.2.1	Issue #1: Types of NSCs	18

4.2.2	Issue #2: Types of capabilities available for exposure	18

4.2.3	Issue #3: EGMF/MCEG	19

4.2.4	Issue #4: NSC-NSP service interaction	19

4.2.5	Issue #5: Relation to other SA5 work/study items	19

4.2.6	Issue #5: Network slice management capability exposure interface via OSS	19

5	Use cases for network management capability exposure	20

5.1	Network slice management capability exposure	20

5.1.1	Description	20

5.1.2	Issue and gaps	20

5.2	Exposure of MnS for monitoring QoS of video application	20

5.2.1	Description	20

5.2.2	Issue and gaps	21

5.3	eMnS discovery service	21

5.3.1	Description	21

5.3.2	Issue and gaps	21

5.4	eMnS support to discovery systems	21

5.4.1	Description	21

5.4.2	Issue and gaps	21

5.5	Exposure of network slice as a product	22

5.5.1	Description	22

5.6	Exposure of network slice as a service	25

5.6.1	Description	25

5.7	Network slice management capability consumption	29

5.7.1	Description	29

5.7.2	Issue and gaps	29

6	Potential requirements for network management capability exposure	30

6.1	Potential requirements related to eMnS discovery service	30

6.2	Potential requirements related to exposure interface via OSS	30

7	Possible solutions for network management capability exposure	30

7.1	Possible solution for “exposed MnS support to discovery systems” – Scenario 5.4	30

7.2	Possible solutions for eMnS discovery service	31

7.3	  Potential solution for MnS discovery service for exposure	31

7.4	  Potential solution for exposed MnS consumption via OSS interface	32

7.5	Solution for internal service order after receiving product order	34

7.6	Solution for external product order after receiving product order	34

7.7	Solution for external service order after receiving product order	35

7.8	Potential solution for product onboarding	35

8	Conclusion and Recommendation	36

8.X	Issue #X	36

8.Y Issue #Y	36

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

This report is to study on network slice management capability exposure.

1	Scope

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	TM Forum TMF622 Product Order API REST Specification

[3]		TM Forum TMF641 Service Ordering API

[4]	TM Forum TMF652 Resource Order Management API

[5]		3GPP TS 28.531: "Management and orchestration; Concepts, use cases and requirements"

[6]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2"

[7]	3GPP TR23.700-99 “Study on Network Slice Capability Exposure for Application Layer Enablement (NSCALE)”

[8]	3GPP TS23.434 “Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows.”

[9]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[10]	3GPP TS 28.537: "Management and orchestration; Management capabilities"

[11]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[12]	TM Forum TMF633 Service Catalogue Management API

[13]	TM Forum TMF620 Product Catalogue Management API

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSS	Business Support System

CSP	Communication Service Provider

NOP	Network OPerator

4	Overview

4.1	General

4.1.1	Concepts related to network management capability exposure

4.1.1.1	Exposed Management Services

Exposed MnS (eMnS) represents the MnS that can be exposed by MnS producer to the external MnS consumer. eMnS may rely on a dedicated MnF (e.g. EGMF defined in 3GPP or function defined in other standard like TMF) that manages the exposure aspects.

Editor’s notes: Whether eMnS is exposed transparently to external MnS consumer via BSS or being processed through a dedicated exposure platform is FFS.

4.1.1.2	Exposure of Management Services

Exposure of management services supports the case that an external MnS consumer which is outside 3GPP management system can indrectly consume management capability offered by MnS producer within 3GPP management system. Even though the eMnS complies with the same Technical Specification as a MnS, the actual operational behavior and managed data may be constrained by the network slice provider.

4.1.1.3	Exposure via BSS

4.1.1.3.1	General

Exposure of service data to companies that are external to the operator are regulated by contracts. Different customers may have access to different management capabilities. It may differ on what attributes/policies/intents are allowed to be changed, which value ranges changes are allowed, and which performance metrics are allowed to be exposed.

The network slice data with the ServiceProfile is located in the Network Management Layer.

Even if the external interface always goes via BSS, there are different scenarios for how a customer, e.g. vertical, can influence the ServiceProfile.

The capabilities required by a Vertical have to be the same on the interface between NOP Network Management Layer and CSP Service manager. The external interface is external to the administrative domain of the operator, while the internal interface is inside the administrative domain of the operator.

4.1.1.3.2	Exposure scenarios

Scenario 1: Consumption of exposed MnS by applications

Editor’s note: This scenario doesn’t fit in the level of details of this section.  Where to incorporate this section is FFS.

The operator has other non-management entities such as the middleware or application servers (AS) defined by 3GPP SA6 that could consume management services as shown in Figure 4.1.1.3.2-1. In such a case the BSS may or may not be directly involved.  An example of an external application could be a V2X application server may use the management system to provision V2X slices in a certain geography (AS2 or AS3 in Figure).  An example for an internal application could be the operators eMBB application server discovering a newly supported coverage area and provisioning the operator eMBB network slice instance in that area (AS1 in Figure).   AS1 and AS2 access the 3GPP management system from an operator internal enabler server (see TR23.700-99), another enabler server could be located in the vertical premises and therefore external to the operator. In TR23.700-99 both such options are considered. In this scenario the operator MnSs are directly consumed by internal or external entities (subject to prior agreements) without going through the BSS. In addition to application servers and application enabler server, any internal of external authorized application function may also access exposed MnS.

In Figure 4.1.1.3.2-1 AS1 and AS2 may or may not be aware that they use exposed MnSs from the operator. The respective enabler servers could hide this internal implementation. Bother enabler servers may access exposed MnS subject to respective authorization. However, it is likely that the application enabler server A and AF1 have direct access to management services without a BSS, whereas the application enabler server B and AF2 would need some sort of involvement of the BSS.

Figure 4.1.1.3.2-1 Exposure to application server within and outside operator network

4.1.1.4	Exposure without going through BSS

4.1.1.4.1	General

Exposure of service data to companies that are external to the operator are regulated by contracts. The exposure usually goes through BSS, as explained in clause 4.1.1.2. However, based on specific contract, the customer may interact with the Operator without going through BSS. There are different scenarios for how a customer having such contract interacts with Operator for the network slice management capability exposure.

4.1.1.4.2	Exposure scenarios

Scenario 1: The NOP and NSP belong to the same Operator. The OSS/SML and OSS/NML are connected via internal interface. The external customer has an external interface with the NSP based on the contract between the external customer and NSP.

NOTE 1: External customer may have connection with NSP BSS for the product-level interaction. If not, the OSS/SML may have an embedded BSS functionalities for the product-level interaction.

NOTE 2: The Company-B BSS connects to the BSS part of external customer and the Company-B OSS/SML connect to the OSS/SML part of external customer.

Figure 4.1.1.4.2-1 NSP OSS/SML to customer being an external interface

Scenario 2: The NOP may interface to an external NSP that has a contract with NOP for the exposure directly via OSS. The NOP’s OSS/SML have a direct machine to machine interface with the NSP’s OSS/SML. The external customer has an external interface with the NSP OSS/SML based on the contract between external customer and NSP.

NOTE 3: External customer may have connection with NSP BSS for the product-level interaction. If not, the NSP OSS/SML may have an embedded BSS functionalities for the product-level interaction.

NOTE 4: If the external customer can get access to the OSS directly, it must maintain a copy of a part of the operator’s MIB. if the customer wants to e.g. receive alarms or performance measurements or KPIs related to the network slice the customer has ordered to the NSP, these alarms / perf. meas / KPIs need to relate to some MOIs known at customer side. All these MOIs shall be part of a containment tree in the copy of the Operators’ MIB maintained by the customer.

Editor’s notes: Whether and how does the external customer obtain the copy of a part of the operator’s MIB is FFS.

NOTE 5: The Company-B BSS connects to the BSS part of external customer and the Company-B OSS/SML connect to the OSS/SML part of external customer.

Figure 4.1.1.4.2-2 NOP OSS/SML to NSP OSS/SML interface being an external interface

Scenario 3: The NOP OSS/SML may interface to an external NSP OSS/SML based on the contract between the two parties. The "External NSP OSS/SML" has internal interface with NSP BSS. The NSP might have a machine to machine interface towards their customer (e.g. a vertical) via their BSS.

NOTE 6: External customer may have connection with BSS for the product-based interaction. If not, the NSP OSS/SML may have an embedded BSS functionalities for the product-based interaction.

NOTE 7: If the external customer can get access to the OSS directly, it must maintain a copy of a part of the operator’s MIB. if the customer wants to e.g. receive alarms or performance measurements or KPIs related to the network slice the customer has ordered to the NSP, these alarms / perf. meas / KPIs need to relate to some MOIs known at customer side. All these MOIs shall be part of a containment tree in the copy of the Operators’ MIB maintained by the customer.

Editor’s notes: Whether and how does the external customer obtain the copy of a part of the operator’s MIB is FFS.

Figure 4.1.1.4.2-3 NOP OSS/SML to NSP OSS/SML being an external interface, NSP BSS to customer being an external interface

4.1.2	Roles related to network management capability exposure

4.1.2.1 Exposed MnS

The roles related to network management capability exposure are the Exposed MnS consumer and Exposed MnS producer with the interface for the Exposed MnS (eMnS). The roles and interface are shown in Figure 4.1.2.1.1

Figure 4.1.2.1.1 Roles related to network management capability exposure

4.1.2.2	Exposed MnS consumer

The logical entity consuming management capability offered by an Exposed MnS producer is called an Exposed MnS consumer. An eMnS consumer is equivalent to an MnS consumer with the difference that it is outside the trust domain of the CSP or NOP. An eMnS consumer is owned by an external customer (e.g. vertical such as Industry, Internet Company, etc) which may take the role of a CSC or NSC. The external customer usually has specific service requirements on a 5G network.

4.1.2.3	Exposed MnS producer

The logical entity offering management capability that can be consumed by an Exposed MnS consumer is called Exposed MnS producer. An eMnS producer is owned by a service provider which may take the role of a CSP or NSP.

4.1.3	Types of interface for the exposure of network slice

4.1.3.1	Introduction

Different types of interfaces may be used during the whole lifecycle of network slice, including ordering, provisioning, operation phases. These interfaces can be categorized as:

- Inter-organization interfaces (represented East-West):

- Between the organizational entity playing the Network Slice Customer (NSC) role, named Company V here, and the organizational entity playing the Network Slice Provider (NSP) role, named Company A here:

- Interface Type-A to:

- create a product (network slice) order,

- retrieve information concerning a product (network slice) order,

- update a product (network slice) order,

- delete a product (network slice) order;

Candidate APIs for Interface Type-A include, but are not limited to, TMF API 622 (Product ordering).

Interface Type-A is not in the scope of 3GPP.

- Between the organizational entity playing the Network Slice Provider (NSP) role, named Company A here, and the two organizational entities playing the Communication Service Provider (CSP) role, acting as ‘partners’ and named Company X and Company Y respectively here

- Interface Type-X to:

- create a product order,

- retrieve information concerning a product order,

- update a product order,

- delete a product order;

Candidate APIs for Interface Type-X include, but are not limited to, TMF API 622 (Product ordering).

Interface Type-X is not in the scope of 3GPP.

- Intra-organization interfaces (represented North-South), internal to the organizational entity playing the Network Slice Provider (NSP) role:

- Between its BSS and its OSS/SML

- Interface Type-1 to:

- create a service order,

- retrieve information concerning a service order,

- update a service order,

- delete a service order;

Candidate APIs for Interface Type-1 include, but are not limited to, TMF API 641 (Service ordering).

Interface Type-1 is not in the scope of 3GPP.

- Between its OSS/SML and its OSS/NML

- Interface Type-2 to:

- create a network slice / network slice subnet,

- retrieve information concerning a network slice / network slice subnet,

- update a network slice / network slice subnet,

- delete a network slice / network slice subnet,

- collect PM and FM data concerning a network slice / network slice subnet;

Candidate APIs for Interface Type-2 include, 3GPP TS 28.531, TS 28.532, TS 28.545, TS 28.550.

Interface Type-2 is in the scope of 3GPP/SA5.

The following clauses describe a non-exhaustive set of use cases.

In case of additional use cases, new types of interfaces may be needed.

4.1.3.2	Use case No.1: simple case

In this use case, Company-V (as the NSC) chooses a product from Company-A product offerings and sends a request to Company-A to order the product which is a network slice via Interface Type-A.

As Company-A can satisfy Company-V’s requirements by itself, it doesn’t have to count on partners.

In this use case, Interface Type-A, Type-1 and Type-2 are involved.

Figure 4.1.3.2.1 Use case No.1: simple case

4.1.3.3	Use case No.2: partners involved

In this use case, Company-V (as the NSC) chooses a product from Company-A product offerings and sends a request to Company-A to order the product which is a network slice via Interface Type-A.

As Company-A can’t satisfy Company-V’s requirements by itself, it has to count on partners (Company-X and Company-Y), e.g. to provide RAN coverage in their respective countries.

In this use case, Interface Type-A, Type-1, Type-2 and Type-X are involved.

Figure 4.1.3.3.1 Use case No.2: partners involved

4.1.4	Procedures related to consumption of exposed network management capabilities

4.1.4.1	Introduction

When an NSP receives an order from an NSC for a network slice enabled product, the order is decomposed by the NSP’s BSS. Depending if the NSP employs services from 3rd party CSP’s different procedures may apply for the same order. The different procedures applicable to the same order may be invoked asynchronously and treated as independent procedures, however it may not result in loss of traceability between the original order and the orders that are created as result of decomposition. The following procedures have been identified:

- Procedure invoking internal service order after receiving product order from NSC

- Procedure invoking external product order after receiving product order from NSC

- Procedure invoking external service order after receiving product order from NSC

4.1.4.2	Procedure invoking internal service order after receiving product order from NSC

The procedure for invoking a service order internal to the NSP after receiving a product order from an NSC is shown in 4.1.4.2.1. The interface through which the NSC can order a product from the NSP is on BSS level. The steps as shown in Figure 4.1.4.2.1 are described in the subsequent paragraphs.

Figure 4.1.4.2.1 Procedure invoking internal service order after receiving product order from NSC

1) The NSP receives a product order from the NSC through the interface to BSS. The interface between the NSC and BSS of the NSP is used.

2) The BSS processes the product order and when applicable converts it to appropriate service order(s) for the OSS Service Management Layer. This is internal to BSS and there are no interface requirements.

3) The OSS Service Management Layer receives a service order from the BSS. The interface between the BSS of the NSP and the OSS_SML of the NSP is used.

4) The OSS Service Management Layer processes the service order and when applicable converts it to appropriate request(s) for the OSS Network Management Layer as requests for management and orchestration of resources. This is internal to the OSS Service Management Layer and there are no interface requirements.

5) The OSS Network Management Layer receives a request from the OSS Service Management Layer. An interface between the OSS Service Management Layer and OSS Network Management Layer may be used.

6) The OSS Network Management Layer processes the request and when applicable converts it to appropriate request(s) for the network. An interface between the OSS Network Management Layer and Network Layer (not shown) may be used.

7) The OSS Network Management Layer notifies the OSS Service Management Layer that the resource order(s) have been completed.  An interface between the OSS Service Management Layer and the Network Layer  may be used.

8) The OSS Service Management Layer notifies the BSS that the service order has been completed. The interface between the OSS Service Management Layer and the BSS is used.

9) The BSS notifies the NSC that the product order has been completed. The NSC may start using the services included in the product order. The interface between the NSC and the BSS is used.

4.1.4.3	Procedure invoking external product order after receiving product order from NSC

The procedure for invoking a product order external to the NSP after receiving a product order from an NSC is shown in 4.1.4.3.1. The interface through which the NSC can order a product from the NSP is on BSS level. The steps as shown in Figure 4.1.4.3.1 are described in the subsequent paragraphs.

Figure 4.1.4.3.1 Procedure invoking external product order after receiving product order from NSC

1) The NSP receives a product order from the NSC through the interface to BSS. The interface between the NSC and BSS of the NSP is used.

2) The NSP BSS processes the product order and when applicable converts it to appropriate product order(s) towards a 3rd party CSP BSS. This is internal to BSS and there are no interface requirements.

NOTE: When the BSS_NSP receives a product order the BSS_NSP splits the product order into service orders. A service order that can be fulfilled by the NSP will be processed by the NSP_OSS_SML (see also Figure 4.1.4.2.1) while a service order that cannot be fulfilled by NSP will be ordered from the CSP through a product order.

3) The CSP BSS receives a product order from the NSP BSS. The interface between the BSS of the CSP and the BSS of the NSP is used.

4) The CSP BSS processes the product order and when applicable converts it to appropriate service order(s) for the CSP OSS. This is internal to the BSS and there are no interface requirements.

5) The CSP OSS receives a service order from the CSP BSS. The interface between the BSS and the OSS both belonging to the CSP is used.

6) The CSP OSS processes the service order until the service order is completed.

7) The CSP OSS notifies the CSP BSS that the service order has been completed. The interface between the OSS and the BSS both belonging to the CSP is used.

8) The CSP BSS notifies the NSP BSS that the product order has been completed. The interface between the BSS of the CSP and the OSS of an NSP is used.

9) The BSS notifies the NSC that the product order has been completed. The NSC may start using the services included in the product order.

4.1.4.4	Procedure invoking external service order after receiving product order from NSC

The procedure for invoking a service order external to the NSP after receiving a product order from an NSC is shown in 4.1.4.4.1. The interface through which the NSC can order a product from the NSP is on BSS level. The steps as shown in Figure 4.1.4.4.1 are described in the subsequent paragraphs.

Figure 4.1.4.4.1 Procedure invoking external service order after receiving product order from NSC

1) The NSP receives a product order from the NSC through the interface to BSS. The interface between the NSC and BSS of the NSP is used.

2) The NSP BSS processes the product order and when applicable converts it to appropriate service order(s) for the OSS producer. This is internal to BSS producer and there are no interface requirements.

3) The NSP OSS receives a service order from the NSP BSS. The interface between the OSS and the BSS, both belonging to the same NSP, is used.

4) The OSS processes the service order and when applicable converts it to appropriate service order(s) for a 3rd party CSP OSS. This is internal to the OSS producer and there are no interface requirements.

5) The CSP OSS receives a service order from the NSP OSS producer. The interface between the OSS of the CSP and the OSS of the NSP is used.

6) The CSP OSS processes the service order until the service order is completed. This is internal to the OSS producer and there are no interface requirements.

7) The CSP OSS notifies the CSP BSS that the service order has been completed. The interface between the OSS and the BSS, both belonging to the same CSP, is used.

8) The CSP OSS notifies the NSP OSS producer (may occur at the same time as or before step 7) that the service order has been completed. The interface between the OSS of the CSP and the OSS of the NSP is used.

9) The NSP BSS notifies the NSC that the product order has been completed. The NSC may start using the services included in the product order.

#### 4.1.4.5	Procedure for product onboarding

Figure 4.1.4.5.1 Procedure  related to product onboarding

- OSS_SML obtains 3GPP management services in the network through management service discovery;

- OSS_SML governs the rules and policies of MnS and configures the available MnS (e.g. eMnS) to BSS_NSP. For example, if the RAN NE is dedicated to external customers, the performance monitoring service of a RAN NE should be exposed. Otherwise, it should not be exposed;

- Optional, the BSS_NSP may send information to request the list of available services from OSS_SML;

- OSS_SML provides the list of available services to BSS_NSP;

- BSS_NSP configures (groups or package the services into a product) the services from OSS_SML such that they can be exposed to NSC;

- NSC should request the product catalog from BSS_NSP.

- BSS_NSP provides product catalog to NSC.

Editor’s Note: “EGMF can have the functionality of eMnS data Whether registration to an external discovery system is FFS.”

4.2	Issues

4.2.1	Issue #1: Types of NSCs

The problem of network slice capability exposure is mostly relevant for B2B/B2B2C market, where Network Slice as a Service (NSaaS) [1] model applies. In this regard, different types of NSCs can be found.

- Baseline vertical customer: it corresponds to a NSC which is only interested in monitoring the network slice, to verify it behaves as expected, according to the SLA. This NSC does typically have no telco experience, and is associated to a network slice that is entirely deployed on a PLMN. The capabilities offered to a baseline vertical customer includes the ability for this NSC to receive information on subscribed items, including network slice status (e.g. active, inactive) and subscribed management data (e.g., KPIs, events/logs, trace data, etc.). The profile of this NSC type is a ‘passive NSC’.

- Advanced vertical customer: it corresponds to an NSC which requests (to the NSP) a dedicated network slice for the provision of PNI-NPN services. In this scenario, a portion of the network slice is deployed within the NSC premises (e.g., RAN, UPF) and the other portion (e.g., 5GC control plane functions) is hosted by one or more PLMN nodes. Unlike the baseline vertical customer, this new NSC does typically have (yet limited) telco knowledge, and wants to retain certain control over the allocated network slice. The capabilities offered to an advanced vertical customer might include (i) monitoring capabilities, i.e. the same capabilities offered to a baseline vertical customer and (ii) device configuration capabilities, i.e. provision of parameters for battery, mobility and communication patterns associated to the device, (iii) edge discovery/selection, e.g. in case the vertical wants to deploy workloads on the telco edge cloud.

- Hyperscaler: it corresponds to a NSC which requests (to the NSP) a dedicated network slice to provide a service-tailored connectivity pipe to a NSC’s customer. With some enterprises (i.e., NSC’s customer) starting to migrate workloads towards hyperscaler nodes, it is necessary for the hyperscaler (i.e., NSC) to provide SLA guarantees to these enterprises, especially for critical processes/services. However, the hyperscaler does not have network resources between its cloud nodes and customer premises, and therefore has to ask the mobile network operator (i.e., NSP) to set up a network slice between these endpoints. The capabilities offered to a hyperscaler might include (i) monitoring capabilities, i.e., the same capabilities offered to a baseline vertical customer; (ii) quality on demand, i.e. dynamic QoS and bandwidth management; (iii) policy control.

- Mobile (Virtual) Network Operator.

NOTE 1: In all the above cases, the NSP role is assumed to be played by an MNO.

NOTE 2: The capabilities mentioned above are neither exhaustive nor accurate, but examples to motivate the need for considering different NSC types.

The NSC types within the scope of FS_NSCE should be use case driven.

4.2.2	Issue #2: Types of capabilities available for exposure

When referring to the capabilities a NSC might be interested to consume, we have three big groups of capabilities that a NSP can make available for consumption:

- Application layer capabilities, within the scope of SA6.

- Management layer capabilities, within the scope of SA5.

- Network layer capabilities, within the scope of SA2.

There is the need to have one single exposure layer to make all the capabilities available for NSCs. This exposure layer should integrate network layer capabilities (@NEF, SA2), management layer capabilities (@MCEG, SA5), application layer capabilities (@SEAL, SA6), together with non-3GPP capabilities (e.g. cloud related capabilities). Otherwise, if every SA WG starts defining their own exposure fabric, (i) the likelihood of encountering incompatibilities/duplicities across these WG specific solutions is high; (ii) the operators may come up with increased integration efforts, which ultimately may make their systems very hard to build and maintain. Which 3GPP working group is responsible to provide the single exposure layer is FFS.

4.2.3	Issue #3: EGMF/MCEG

The Exposure Governance Management Function (EGMF) was originally defined in TS 28.533 [2] as an MnF providing management capability exposure governance (MCEG). However, the current definition needs more elaboration on the following questions:

- The functional scope of management capability exposure governance, and its relationship with the access control and with existing API GW solutions in carrier networks.

- The impact of management capability exposure governance on the Network Slice NRM fragment. Is within the scope of SA5 or not? If in-scope, then:

  - what NSC related information (e.g., NSC id, NSC granted capabilities) does the NSP send to the NOP?

  - how does the NOP manage this information in relation to the existing NetworkSlice and NetworkSliceSubnet IOCs?

- The need to standardize EGMF. Does SA5 really need to define this MnF? Doesn’t this approach mean moving away from producer centric model of SA5, i.e., focus on service producers rather than MnFs?

- If EGMF standardization is within the scope of SA5, then does SA5 need to provide details on EGMF internals? Does SA5 need to decide whether the EGMF is positioned on the Network Management Layer (NML), or the Service Management Layer (SML), or BSS layer?

4.2.4	Issue #4: NSC-NSP service interaction

The NSC-NSP service interactions work (i.e., APIs made available by the NSP, for consumption by the NSCs) is out of the 3GPP scope. Based on the proposal #2, which argues in favour of having one single exposure layer integrating 3GPP SA2/SA5/SA6 capabilities and non-3GPP capabilities, together with the fact that a high number of NSCs are not familiar with 3GPP models, it makes sense to let these interactions be covered in other industry fora. The potential group to provide the single exposure layer is FFS.

4.2.5	Issue #5: Relation to other SA5 work/study items

The work conducted in the FS_NSCE is related to other Rel-17 SI/WIs, including:

- MSAC (Management Service Access Control), on the access control aspects inherent to exposure to 3rd parties. NSP shall expose capabilities to NSCs in a controlled, secure and auditable way.

- OAM_NPN (Management of Non-Public Networks), when the network slice is used for the provisioning of a PNI-NPN. In this case, the modes 1b defined in [3] applies.

- eMEMTANE (Management of enhanced tenant concept), on the need to associate tenants to different NSCs, and manage the corresponding information in the NRM.

- 5GDMS (Discovery of management services in 5G), on the need for NSCs to discover capabilities available for consumption.

The work in FS_NSCE is to leverage outcomes from the Rel-17 study/work items which are listed above. It seems these study/work items provides most (if not all) the ingredients for the network slice capability exposure topic, so the mission of FS_NSCE should be to find out the recipe to combine them and provide overall exposure picture.

4.2.6	Issue #5: Network slice management capability exposure interface via OSS

Use cases regarding exposure interface via OSS has been introduced in Section 6. NSC can make contract with the NSP or CSP regarding exposure via BSS. Upon the completion of the contact, the NSC can directly get access to the OSS within the NSP or CSP for access the exposed MnS. Several key issues exist for the use cases. For example, how does the NSC identifies the address of MnS producer for the exposed MnS within the OSS and how to consume the exposed MnS directly from OSS needs to be addressed.

5	Use cases for network management capability exposure

5.1	Network slice management capability exposure

5.1.1	Description

A use case of network slice management capability exposure can be described as follows:

1. NSP selects the MnS that can be exposed externally.

2. NSP decides on any constraints that shall be applied to the MnS when it is exposed externally. For example, NSP may decide to disallow certain operations, limit the Managed Object Instances that may be managed, or aggregate/anonymize sensitive data.

3. NSP implements and deploys a Management Function which consumes the MnS, applies any constraints, and exposes the resulting functionality as an eMnS.

4. NSP may publish the eMnS in a service catalog or service directory.

5.1.2	Issue and gaps

Gap:

Whether and how to publish eMnS which can be exposed to BSS to a suitable eMnS producer for network management capability exposure is not specified in existing 3GPP management system.

5.2	Exposure of MnS for monitoring QoS of video application

5.2.1	Description

A use case of exposure of MnS for monitoring QoS of video application can be described as follows:

1. A live concert with high-resolution video application service is provided with service provider A (i.e. associate to eMnS consumer). A local hosting network (e.g. SNPN) is available and provides localized services for high-resolution video service of the concert. The local hosting network is managed by the service provider A. The high-resolution video streaming service of the live concert is available from both the local hosting network and PLMN. Service provider A and the provider of PLMN (i.e. associate to eMnS provider) are two different parties.

2. The service provider A of the local hosting network can have an offer from the Operator of PLMN before identifying the correct MnS for exposure through BSS (e.g. by using Product Catalog). The BSS may obtain the information of MnS that is allowed to be exposed using a MnS service for exposure provided by OSS. The contract may contain the agreement on the exposure of MnSs for consuming certain management MnS related to QoS. According to the contract, the service provider A can have the permission to use the MnS for consuming certain performance MnS related to QoS for the PLMN. The performance MnS can be related to NR and 5GC, e.g. Average DL UE throughput in gNB.

3. Once the offer has been accepted, the corresponding exposure governance management service within the 3GPP management system (of the Operator of PLMN) is configured with permission rule through the interface between BSS and OSS. The permission rule defines that the provider of the local hosting network can have the right to access certain management MnS regarding QoS.

4. Through obtaining the measurement MnS using exposure capability, the provider of the local hosting network can determine the situation when PLMN cannot support the high-resolution video service with satisfied QoS. In this case, the service provider A can notify its customers about the local hosting network that may improve the QoS of the high-resolution video application.

5.2.2	Issue and gaps

Gap:

The definition and the format of the permission rule for the network management capability exposure regarding NR and 5GC related performance MnS is not specified in existing 3GPP management system.

Whether and how to publish MnS which can be exposed to externals to a suitable MnS service producer for the exposure of performance MnS regarding NR and 5GC is not specified in existing 3GPP management system.

5.3	eMnS discovery service

5.3.1	Description

A use case of eMnS discovery service is described as follows:

1. MNO A provides eMnSs from 3GPP management system for external customers and these eMnSs go through BSS for exposure.

2. In order to provide such discovery service to external customer, MNO A wishes to use an eMnS discovery service producer that takes responsibility for making eMnS data available for it to be discovered by the BSS.

5.3.2	Issue and gaps

Gap:

Study is needed whether the eMnS discovery service is to be provided by the MnS discovery service producer or by a dedicated eMnS discovery service producer (e.g. EGMF).

Study is needed on whether the MnS data as defined in 28.533 can also be re-used for eMnS data, or if any extensions are necessary.

5.4	eMnS support to discovery systems

5.4.1	Description

An eMnS should be allowed to register to an authorized supported discovery system such that interested authorized consumers (within or external to the operator) are able to discover it. This implies that the eMnS is only exposed to a discovery system where a pre-existing contract allows for such an exposure.

1. An operator would like to register its eMnS to a trusted discovery system. The operator configures the eMnS with the discovery system’s address and the appropriate level of exposure for the registration. The eMnS is registered at the discovery system with the appropriate level of exposure.

2. The operator performs changes in its management system that  impacts the information exposed by the eMnS. The changed information is automatically updated in the discovery system.

3. In case the relationship between the operator and a discovery system ends, which implies the system is no longer trusted, then the eMnS automatically requests deletion of its registration in the discovery system.

Editor’s Note: How the “is no longer trusted” would be handled by the authentication and authorization and system is FFS, since it is not clear how a non-trust party can be trusted to delete a registration entry.

5.4.2	Issue and gaps

There are several issues that need to be resolved.

Issues:

There is a difference if the discovery system is external or internal to the operator. A discovery system for internal use may still exist outside the scope of management.

There is an issue with managing which consumers have access to the discovery system and could theoretically consume the management service.

There is an issue with the trust (i.e., authentication and authorization) between the three parties (MnS producer/operator, MnS consumer/customer and discovery system owner) in this use case.

Gap:

To limit issues the exposure from a discovery system of the operator may only provide “read” permissions (w.r.t the eMnS) without authentication and authorization. To execute the discovered eMnS the consumer still needs to be authenticated and authorized by the management system. Therefore, there is a gap in the difference in exposure for consumption, and exposure for discovery which needs to be solved.

5.5	Exposure of network slice as a product

5.5.1	Description

This use case involves the following roles:

- NSP: Network Slice Provider

- NSC: Network Slice Customer

- NOP: Network Operator

- CSC: Communication Service Customer

- CSP: Communication Service Provider

, and the following systems:

- BSS: Business Support System

- OSS: Operations Support System, made up of the two following sub-systems:

- SML: Service Management Layer

- NML: Network Management Layer (for sake of simplicity, network management and network element / function management are both in the NML).

5.5.1.1	Sub-use case 1: NSP and NOP played by the same organization

In this scenario, the following organizations play aforementioned roles as follows:

- Company-V plays the role of NSC

- Company-A plays the role of NSP and NOP

- As NSP, it has:

- a BSS, e.g. to manage its customers, products, contracts, and

- a SML, to manage the services that support its products

- As NOP, it has:

- its own 5G network (RAN + core). In this sub-use case, Company-A owns the whole set of network resources used by the service required to support the product ordered by Vertical V

- a NML, to manage the network resources used by services

Figure 5.5.1.1-1: Sub-use case 1 - NSP and NOP played by the same organization

Company-A product catalogue proposes the following product offerings:

- Network Slice eMBB with different flavours: Silver, Gold, Platinum

- Network Slice URLLC with different flavours: Silver, Gold, Platinum

Network Slice MIoT with different flavours: Silver, Gold, Platinum.

In this sub-use case 1:

1. Company-V (as the NSC) chooses a product from Company-A product offerings

2. Company-V sends a request to Company-A (as the NSP) to order the product ‘Network Slice eMBB Platinum’. To achieve this, a candidate API is TMF API 622 (Product Ordering)

2.1 Company-A BSS determines which service supports the product being ordered by Company-V and issues a request to its OSS/SML to order this service. This service can be e.g. a network slice. To achieve this, a candidate API is TMF API 641 (Service Ordering)

2.2 OSS / SML determines which network resources support the service being ordered and issues a request to the OSS / NML to allocate required network resources, e.g. network slice subnet(s), network functions, etc. To achieve this, candidate APIs are from 3GPP TS 28.531 and TS 28.532

2.3 OSS / NML allocates network resources required to support the service and informs OSS / SML back about the characteristics of the network resources being allocated

2.4 OSS / SML associates the allocated network resources to the service and informs its BSS back about the characteristics of the service supporting the product

3. Company-A (as the NSP) sends a reply to Company-V to inform that the product ordered is now available to Company-V.

NOTE: in this use case, aspects related to Transport Network(s) are not addressed as they are out of 3GPP scope.

5.5.1.2	Sub-use case 2: NOP role played simultaneously by different organizations

In this scenario, the following organizations play aforementioned roles as follows:

- Company-V plays the role of NSC

- Company-A plays the role of NSP and NOP

- As NSP, it has:

- a BSS, e.g. to manage its customers, products, contracts, and

- a SML, to manage the services that support its products

- As NOP, it has:

- its own 5G core network. In this sub-use case, Company-A owns the whole set of 5G core network resources used by the service required to support the product ordered by Vertical-V

- a NML, to manage the 5G core network resources used by services

As Company-A has no RAN in all requested areas, it relies on external organizations, namely Company-X and Company-Y, to provide RAN coverage in the US and in Spain respectively. Therefore:

- Company-A plays the role of Communication Service Customer (CSC) wrt. Company-X and Company-Y who both play the role of Communication Service Provider (CSP)

- Both Company-X and Company-Y have their own catalogue of products to offer RAN coverage in their respective countries

- Both Company-X and Company-Y play the role of CSP (for their respective product offerings) and NOP (for their respective RAN).

Figure 5.5.1.2-1: Sub-use case 2 - NOP role played simultaneously by different organizations

Company-A product catalogue proposes the following product offerings:

- Network Slice eMBB with different flavours: Silver, Gold, Platinum

- Network Slice URLLC with different flavours: Silver, Gold, Platinum

Network Slice MIoT with different flavours: Silver, Gold, Platinum.

In this sub-use case 2:

1. Company-V (as the NSC) chooses a product from Company-A product offerings

2. Company-V sends a request to Company-A (as the NSP) to order the product ‘Network Slice eMBB Platinum’. To achieve this, a candidate API is TMF API 622 (Product Ordering)

2.1 Company-A BSS determines which service supports the product being ordered by Company-V and issues a request to its OSS/SML to order this service. This service can be e.g. a network slice. To achieve this, a candidate API is TMF API 641 (Service Ordering)

2.2 Company-A OSS / SML determines which network resources support the service being ordered and:

2.2.1 based on its knowledge that required 5G core network resources are available internally, it issues a request to its own OSS / NML to allocate required 5G core network resources, e.g. network slice subnet(s) for its 5G core network, etc. To achieve this, candidate APIs are from 3GPP TS 28.531 and TS 28.532

2.2.2 Company-A OSS / NML allocates 5G core network resources required to support the service and informs OSS / SML about the characteristics of the network resources being allocated

2.2.3 based on its knowledge that required RAN resources are not available internally, it informs Company-A BSS about missing RAN resources

2.3 Company-A BSS:

2.3.1 acting as a CSC, issues a request to Company-X to order product X-1 (e.g. from the Wholesale offerings) to get RAN coverage in the US. To achieve this, a candidate API is TMF AP 622 (Product ordering). Company-X, as the CSP, receives the product order. Company-X BSS determines which service supports the product being ordered by Company-A and issues a request to its OSS/SML to order this service. This service can be e.g. a network slice. To achieve this, a candidate API is TMF API 641 (Service Ordering). Company-X OSS / SML determines which network resources support the service being ordered, etc. Once completed, Company-X BSS informs Company-A BSS that the product which has been ordered is now available to Company-A

2.3.2 acting as a CSC, issues a request to Company-Y to order product Y-1 (e.g. from the inter-operator network slice offerings) to get RAN coverage in Spain. To achieve this, a candidate API is TMF AP 622 (Product ordering). Company-Y, as the CSP, receives the product order. Company-Y BSS determines which service supports the product being ordered by Company-A and issues a request to its OSS/SML to order this service. This service can be e.g. a network slice. To achieve this, a candidate API is TMF API 641 (Service Ordering). Company-Y OSS / SML determines which network resources support the service being ordered, etc. Once completed, Company-Y BSS informs Company-A BSS that the product which has been ordered is now available to Company-A

2.3.3 informs its own OSS/SML that required RAN resources are available

2.4 Company-A OSS / SML associates the network resources allocated either internally or externally (by Company-X or Company-Y) to the service and informs its BSS about the characteristics of the service supporting the product

3. Company-A (as the NSP) sends a reply to Company-V to inform that the product ordered is now available to Vertical-V.

NOTE: in this use case, aspects related to Transport Network(s) are not addressed as they are out of 3GPP scope.

5.5.1.3	Exposure of the network slice as a product

In both sub-use case 1 and sub-use case 2, the characteristics of the network slice ordered by the NSC to the NSP are exposed by the NSP to the NSC at product-level, i.e. as specified by the product specification in the NSP catalogue. The product specification provides the characteristics of the product being offered by the NSP at business level and is not subject to standardization. These characteristics are generally more abstract than attributes defined in the 3GPP 5G NRM (cf. TS 28.541) and performance measurements defined in TS 28.552.

In these two sub-use cases, there is no exposure of service or network resources directly to the NSC.

5.6	Exposure of network slice as a service

5.6.1	Description

This use case involves the following roles:

- NSP: Network Slice Provider

- NSC: Network Slice Customer

- NOP: Network Operator

- CSC: Communication Service Customer

- CSP: Communication Service Provider

, and the following systems:

- BSS: Business Support System

- OSS: Operations Support System, made up of the two following sub-systems:

- SML: Service Management Layer

- NML: Network Management Layer (for sake of simplicity, network management and network element / function management are both in the NML).

5.6.1.1	Sub-use case 1: NSP and NOP play by the same organization

In this scenario, the following organizations play aforementioned roles as follows:

- Company-V, which has a contract with Company-A for the exposure directly via OSS, plays the role of NSC

- Company-A plays the role of NSP and NOP

- As NSP, it has:

- a BSS, e.g. to manage its customers, products, contracts, and

- a SML, to manage the services that support its products,

- As NOP, it has:

- its own 5G network (RAN + core). In this sub-use case, Company-A owns the whole set of network resources used by the service that can potentially support the service required by Company-V

- a NML, to manage the network resources used by services

NOTE: NSC may have connection with Company-A BSS for the product-level interaction. If not, the OSS/SML may have an embedded BSS functionalities for the product-level interaction.

Figure 5.6.1.1-1 Sub-use case – NSP and NOP played by the same organization

Company-A proposes the following product offering together with the exposure capability:

- Network Slice eMBB with the exposure capability of related KPI monitoring and alarm notification, etc.

In this sub-use case 1:

1. Company-V (as the NSC) gets the information regarding eMnSs that are available via the eMnS discovery service from the Company-A.

2. Company-V sends a request to Company-A (as the NSP) for the access to exposed MnS set ‘Network Slice eMBB’, which contains the exposure capabilities such as  related KPI monitoring and alarm notification, etc. To achieve this, a candidate API is the interface with the MnF that controls the exposure governance (e.g. EGMF).

2.1 Company-A SML determines which service supports the exposed MnS being requested by Company-V.

2.2 Company-A OSS / SML sents a response, including the authentication materials (e.g. key, token) for access to the chosen exposed MnS.

3. The company-V can direct consume the exposed MnS (e.g. KPI monitoring and alarm notification) from SML of the Company-A’s 3GPP management system.

5.6.1.2	Sub-use case 2: NOP role played simultaneously by different organizations

In this scenario, the following organizations play aforementioned roles as follows:

- Company-V, which has a contract with Company-A for the exposure directly via OSS, plays the role of NSC

- Company-A plays the role of NSP and NOP

- As NSP, it has:

- a BSS, e.g. to manage its customers, products, contracts, and

- a SML, to manage the services that support its products,

- As NOP, it has:

- its own 5G core network. In this sub-use case, Company-A owns the whole set of 5G core network resources used by the service that can potentially support the service required by Company-V

- a NML, to manage the 5G core network resources used by services

As Company-A has no RAN in all requested areas, it relies on a different organization with a specific contract (e.g. exposure directly via OSS), namely Company-B Spain and Company-C USA, to provide RAN coverage in the Spain and in the USA respectively. Therefore:

- Company-A plays the role of Communication Service Customer (CSC) wrt. Company-B Spain and Company-C USA who both play the role of Communication Service Provider (CSP)

- Both Company-B Spain and Company-C USA have their own services to offer RAN coverage in their respective countries

- Both Company-B Spain and Company-C USA play the role of CSP (for their respective product offerings) and NOP (for their respective RAN).

NOTE 1: NSC may have connection with Company-A BSS for the product-level interaction. If not, the OSS/SML may have an embedded BSS functionalities for the product-level interaction.

NOTE 2: If the external customer can get access to the OSS directly, it must maintain a copy of a part of the operator’s MIB. If the customer wants to e.g. receive alarms or performance measurements or KPIs related to the network slice the customer has ordered to the NSP, these alarms / perf. meas / KPIs need to relate to some MOIs known at customer side. All these MOIs shall be part of a containment tree in the copy of the Operators’ MIB maintained by the customer.

Figure 5.6.1.2-1 Sub-use case – NOP role played simultaneously by different organizations

Company-A proposes the following product offering together with the exposure capability:

- Network Slice eMBB with the exposure capability of related KPI monitoring and alarm notification, etc.

In this sub-use case 2:

1. Company-V (as the NSC) gets the information regarding eMnSs that are available via the eMnS discovery service from the Company-A.

2. Company-V sends a request to Company-A (as the NSP) for the access to exposed MnS set ‘Network Slice eMBB Platinum’, which contains the exposure capabilities such as related KPI monitoring and alarm notification, etc. To achieve this, a candidate API is the interface with the MnF that controls the exposure governance (e.g. EGMF).

2.1 Company-A SML determines which service supports the exposed MnS being requested by Company-V.

2.2 Company-A OSS / SML determines which network resources support the service being requested and:

2.2.1 based on its knowledge that required RAN resources are not available internally, the SML, acting as a CSC, issues a request to Company-C USA to request service to get RAN coverage in the US. To achieve this, a candidate API is the interface with the MnF that controls the exposure governance (e.g. EGMF) of Company-C USA. Company-C USA, as the CSP, receives the service request. Company-C USA SML determines which service supports the exposed MnS being requested by Company-A. This service can be e.g. a network slice. Company-C USA OSS / SML determines which network resources support the service being requested, etc. Once completed, Company-C USA SML informs Company-A SML that the exposed MnS which has been requested is now available to Company-A. The Company-A SML can consume the eMnS from Company-C USA directly via its SML.

2.2.3 acting as a CSC, issues a request to Company-B Spain to request service to get RAN coverage in Spain. To achieve this, a candidate API is the interface with the MnF that controls the exposure governance (e.g. EGMF) of Company-B Spain. Company-B Spain, as the CSP, receives the service request. Company-B Spain SML determines which service supports the exposed MnS being requested by Company-A. This service can be e.g. a network slice. Company-B Spain OSS / SML determines which network resources support the service being requested, etc. Once completed, Company-B Spain SML informs Company-A SML that the exposed MnS which has been requested is now available to Company-A. The Company-A SML can consume the eMnS from Company-B Spain directly via its SML.

3. Company-A (as the NSP) sends a reply to Company-V to inform that the exposed MnS requested is now available to Company-V. The reply may also include the authentication materials (e.g. key, token) for access to the chosen exposed MnS.

NOTE 3: NSP may not have network at all. In this case, if and how NSP can offer exposure services to NSC is FFS.

NOTE 4: in this use case, aspects related to Transport Network(s) are not addressed as they are out of 3GPP scope.

5.7	Network slice management capability consumption

5.7.1	Description

A use case of network slice management capability consumption can be described as follows:

1. In order to enable the consumption of network slice related eMnS, a NSC firstly makes a contract with the NSP, which contains the agreement and conditions for consuming an eMnS. The condition can be a certain constraint of eMnS consumption based on the contract, e.g. the usage quota of certain eMnS, the usage frequency of certain eMnS, etc. The NSC negotiates its specific requirements for the network slice management capability consumption with the NSP. The negotiation can be done via the following ways:

a) For NSC which is small enterprise, it can directly have a view on the network slice related management capability through the BSS (e.g. by using Product Catalog). Based on that, the NSC can select the network slice related eMnSs which will be covered by the contract.

b) For NSC which is large enterprise (i.e. Internet giants that have their own service customer), it can select the network slice related eMnSs that are available to be exposed offline (e.g. through a F2F meeting). The NSP can proceed with the service ordering through BSS based on the contract.

2. The BSS may interact with the OSS in order to complete certain configuration (i.e. permission regarding what eMnS, optionally under what condition, can be consumed) regarding the consumption of eMnS based on the customized requirement from the NSC.

3. NSP authorizes NSC to consume the eMnS as defined in the contract, and provides the relevant authentication keys to NSC.

4. The NSC can get access to the network slice related management capability offered by eMnS producer within 3GPP management system. The access may need the interaction with BSS (e.g. through Service Catalog).

5.7.2	Issue and gaps

Gap:

NSC needs to apply for the access of network slice management capability through BSS. However, there is no discussion and agreement on whether an eMnS is exposed transparently through the BSS or being processed through a dedicated exposure platform before exposing to the NSC.

The definition and the format of permission for the consumption of network slice related eMnS and its potential impact on internal interface with BSS is not discussed in current SA5 work.

6	Potential requirements for network management capability exposure

6.1	Potential requirements related to eMnS discovery service

- REQ-NSCE-01 The 3GPP management system may provide capabilities allowing to discover exposed MnS and related eMnS producers that are managing a specific managed entity.

- REQ-NSCE-02 The 3GPP management system may have functionalities to register exposed MnS to an appropriate discovery service/system (e.g. eMnS discovery service producer).

Editor’s notes: Whether the discovery system is internal or external of 3GPP management system is FFS.

6.2	Potential requirements related to exposure interface via OSS

- REQ-NSCE-03 The 3GPP management system may provide capabilities to authenticate and authorize NSC to consume exposed MnS directly from 3GPP management system.

7	Possible solutions for network management capability exposure

Editors note: reallocate section according to agreement in S5-221089.

7.1	Possible solution for “exposed MnS support to discovery systems” – Scenario 5.4

The steps of the solution are as follows:

- A MnS consumer configures using the appropriate MnS (for example the generic provisioning service) the details of the external discovery service location and other supporting details (for e.g., authentication and authorization). Further, the detail on which parts of which MnS (component A) IOCs and which instances of the corresponding MOI (component B) and corresponding data (component C) can be registered with the discovery service/system.  As an example, the operator may want to register her the ability to provision (Management object A) a particular slice type (NSSAI-ID) at a certain coverage area (coverageArea item 6.3.3 TS28.541) with some additional details (example: supported latency or maxNumberofUEs or delayTolerance) to an external discovery service or system.

- In addition to the information of what is externally registered, information relating to the address of the exposed MnS needs to be provided. This could be default information based on the operator in the external discovery system or in case of a trusted discovery the address of the actual exposed management service.

- The 3GPP Management System registers the exposed management component A, B and C as configured in step 1 to the appropriate external discovery service/system.

- Eventually, if any of the exposure details change – for example the same slice type can now be supported in a new coverageArea – the registration to the external discovery system may now need to be updated.

Editor’s note: provide a picture explain the operator and JV relationship below.

The solution for the trust issue between the three MnS discovery system owner, MnS consumer and the MnS producer is scenario dependent. Let’s take for example a multi-operator network scenario. In this scenario let’s take operator A as an MnS producer, a joint venture of multiple operators as a MnS discovery system owner and operator B as an MnS consumer. Both Operator A and Operator B have business relations with the joint venture thereby creating a trust relationship between Operator A and the joint venture (JV) AND operator B and the joint venture. The JV provides the authorization and authentication details for each of the operator to use the discovery service. Operator A may then expose aspects of its MnS that it considers ok to expose with the JV. Operator B can then discover the MnS offered by operator A and contact operator A to access them. If operator B requires a higher level of access than operator A provides in the JV by default, then they need to form a new business relationship.

7.2	Possible solutions for eMnS discovery service

To enable communication between eMnS consumers and eMnS producers, eMnS consumers need a mechanism to discover eMnS producers that are available in the 3GPP management system, this is called eMnS discovery service.

When the operator decides to expose a management service (eMnS), the operator must decide which MnS(s) should be exposed, which internal MnS operations should be abstracted/filtered, and which internal MnS data should be abstracted/filtered. As part of this decision, the operator may use the MnS Discovery Service to collect information. The operator exposes the MnS and registers it with the eMnS discovery service, this may be done using an EGMF. The eMnS discovery service consumer sends a request to appropriate discovery service (e.g. eMnS discovery service producer) to obtain the eMnS data.

The eMnS discovery service consumer sends a request to appropriate discovery service (e.g. eMnS discovery service producer) to obtain the eMnS data.

Editor’s Note: Whether the use of an MnS discovery service to collect information about services for exposure is subject to standardization by SA5 is FFS.

7.3			Potential solution for MnS discovery service for exposure

The MnS data can be accessed by different kinds of discovery service consumers based on different use cases.

In some use cases, the exposure usually goes through BSS. The BSS can obtain MnS data from the MnS discovery service on behalf of the NSC.

There are different scenarios where the NSC could directly interact with the MnS producer exposure interface via OSS.

Figure 7.3.1 Procedure for MnS discovery service for exposure via BSS

- The BSS requests for MnS data from the MnS discovery service for NSC on behalf of the NSC. The MnS data may contain the address of the exposed producer.

- MnS discovery service producer for NSC provides discovery service to NSC. The MnS discovery service producer returns the MnS data to BSS. The BSS may provide discovery service to NSC based on MnS data.

- The NSC completes the product order via the interaction with BSS_NSP.

Figure 7.3.2 Procedure for MnS discovery service for exposure via OSS

- The NSC completes a product order via the interaction with BSS_NSP. As a result of the product ordering, the NSC obtains the address of the MnS discovery service producer for exposure.

NOTE: MnS discovery service producer providing discovery service for the exposed MnS to external.

- The NSC executes authentication and authorization for accessing MnS discovery service.

- The NSC requests for MnS data for exposed MnS from the MnS discovery service. The exposed MnS data for exposed MnS may contain the information of the exposed MnS instance and the address of the exposed MnS producer. The MnS data for exposed MnS is the data for authorized NSC to identify proper MnS producer which produces the exposed MnS.

- The MnS discovery service returns the MnS data for exposed MnS to the NSC.

- The NSC requests the services from the target exposed MnS producer.

- The target exposed MnS producer returns the responses.

7.4			Potential solution for exposed MnS consumption via OSS interface

This clause introduces the solution for the use case described in clause 5.6, where the NSC can directly interact with OSS for the consumption of exposed MnS.

Figure 7.4.1 Exposed MnS consumption via OSS interface

1. The NSP receives a product order from the NSC through BSS. The interface used towards the BSS is specified by TM Forum specifications [2].

2. The BSS processes the product order and when applicable converts it to appropriate service order(s) for the OSS Service Management Layer. This is internal to BSS and there are no interface requirements.

3. The OSS Service Management Layer receives a service order from the BSS. The interface used is specified by TM Forum specifications [3].

4. The MnS producer for NSC (e.g. NSMF) on the OSS Service Management Layer processes the service order and when applicable converts it to appropriate request(s) for the OSS Network Management Layer as requests for management and orchestration of resources. In addition, MnS producer on the OSS Service Management Layer can decides to expose MnS directly from OSS_SML and prepares the address of MnS discovery service producer for NSC that can be accessed by the NSC and related authorization information (e.g. token) for accessing the MnS discovery service for NSC. The service order may trigger a procedure of resource order with OSS_NML.

5. The MnS producer for NSC on OSS Service Management Layer notifies the BSS that the service order has been completed. In addition, the notification may contain the address of producer that manages the MnS discovery service for NSC to access and also identities of the related MOIs which is related to the exposed MnSs that the NSC requests. The interface used is specified by TM Forum specifications [3].

NOTE 1: The MnS discovery service producer for NSC can be within the OSS or outside the OSS.

6. The BSS notifies the NSC that the product order has been completed. In addition, the address of MnS discovery producer for NSC and the related authorization information (e.g. token) for accessing the MnS discovery service for NSC are sent to the NSC by the product order completed message. The interface used the interface towards the BSS is specified by TM Forum specifications [2].

7. If the notification in step 6 contains the address of MnS discovery service producer for NSC, the NSC conducts authentication and authorization for accessing exposed MnS discovery service.

8. After the authentication and authorization, the NSC obtains the MnS data for exposed MnS, which contains the information of the exposed MnS instance and the address of target MnS producer for NSC. The MnS data for exposed MnS is the data for authorized NSC to identify proper MnS producer which produces the exposed MnS.

9. After obtaining the information of the MnS data for exposed MnS, the NSC identifies the target MnS producer for NSC and consumes the exposed MnS. To consider the security, the MnS producer may leverage a dedicated MnF which controls the exposure governance as a proxy for exposing MnS.

7.5	Solution for internal service order after receiving product order

This clause describes a solution for the procedure described in clause 4.1.4.2. For each step in the procedure Table 7.5.1 identifies the following:

- if an interface is Internal to an operator, i.e. internal to a CSP or internal to an NSP, or External between a CSP and an NSP, or None in case the step is an internal process and there is no interface requirement,

- which operation or notification is used by that step, and

- which specification describes the interface (stage 2 and stage 3).

| Step |  | Description in step |  | Interface | Reference | Description in reference |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | product order |  | External |  | TM Forum TMF622 [2] | Product Order |
| 2 |  | None | - | - |
| 3 | service order |  | External |  | TM Forum TMF641 [3] | Service Order |
| 4 |  | None | - | - |
| 5 | resource order |  | Internal |  | TM Forum TMF652 [4] | Resource Order |
| 6 |  | Internal | 3GPP TS 28.531 [5], clause 6.5.1- | AllocateNsi |
| 7 | resource order completed |  | Internal |  | TM Forum TMF652 [4] | Service Order State Change Event |
| 8 | service order completed | External | TM Forum TMF641 [3] | Product Order State Change Event |
| 9 | product order completed |  | External |  | TM Forum TMF622 [2] | Product Order State Change Event |

Table 7.5.1 Solution for internal service order after receiving product order

7.6	Solution for external product order after receiving product order

This clause describes a solution for the procedure described in clause 4.1.4.3. For each step in the procedure Table 7.6.1 identifies the following:

- if an interface is Internal to an operator, i.e. internal to a CSP or internal to an NSP, or External between a CSP and an NSP, or None in case the step is an internal process and there is no interface requirement,

- which operation or notification is used by that step, and

- which specification describes the interface (stage 2 and stage 3).

| Step | Description in step | Interface | Reference |  | Description in reference |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | product order | External | TM Forum TMF622 [2] | Product Order |  |
| 2 |  | None | - | - |  |
| 3 | product order | External | TM Forum TMF622 [2] | Product Order |  |
| 4 |  | None | - | - |  |
| 5 | service order | Internal | TM Forum TMF641 [3] | Service Order |  |
| 6 |  | None | - | - |  |
| 7 | service order completed | Internal | TM Forum TMF641 [3] | Service Order State Change Event |  |
| 8 | product order completed | External | TM Forum TMF622 [2] | Product Order State Change Event |  |
| 9 | product order completed | External | TM Forum TMF622 [2] | Product Order State Change Event |  |

Table 7.6.1 Solution for external product order after receiving product order

7.7	Solution for external service order after receiving product order

This clause describes a solution for the procedure described in clause 4.1.4.4. For each step in the procedure Table 7.7.1 identifies the following:

- if an interface is Internal to an operator, i.e. internal to a CSP or internal to an NSP, or External between a CSP and an NSP, or None in case the step is an internal process and there is no interface requirement,

- which operation or notification is used by that step, and

- which specification describes the interface (stage 2 and stage 3).

| Step | Description in step | Interface | Reference | Description in reference |
| --- | --- | --- | --- | --- |
| 1 | product order | External | TM Forum TMF622 [2] | Product Order |
| 2 |  |  | None |  | - |  | - |  |
| 3 | service order | Internal | TM Forum TMF641 [3] | Service Order |
| 4 |  |  | None |  | - |  | - |  |
| 5 | service order | External | TM Forum TMF641 [3] | Service Order |
| 6 |  |  | None |  | - |  | - |  |
| 7 | notify service order completed | Internal | 3GPP TS 28.202 [6] | Service Order Completed Notification |
| 8 |  | service order completed | External |  | TM Forum TMF641 [3] |  | Service Order State Change Event |  |
| 9 | service order completed | External | TM Forum TMF641 [3] | Service Order State Change Event |

Table 7.7.1 Solution for external product order after receiving product order

7.8	Potential solution for product onboarding

This clause describes a solution for the procedure described in clause 4.1.4.5. For each step in the procedure Table 7.8.1 identifies the following:

- if an interface is Internal to an operator, i.e. internal to the NSP  or External between a NSC and NSP, or None in case the step is an internal process and there is no interface requirement,

- which operation or notification is used by that step, and

- which specification describes the interface (stage 2 and stage 3).

| Step | Description in step | Interface | Reference | Description in reference |
| --- | --- | --- | --- | --- |
| 1 | service discovery | Internal | 3GPP TS 28.537[10], clause 5- | Editor’s Note: further details are FFS |
| 2 |  |  | None | - | - |  |
| 3 | request list of available services | Internal | TM Forum TMF633[12] | Service Catalogue |
| 4 |  | provide list of available services | Internal | TM Forum TMF633[12] | Service Catalogue |  |
| 5 |  | None | - | - |
| 6 |  | product catalogue request | External | TM Forum TMF620[13] | Product Catalogue |  |
| 7 | product catalogue | External | TM Forum TMF620[13] | Product Catalogue |

Table 7.8.1 Solution for product onboarding

8	Conclusion and Recommendation

8.X	Issue #X

8.Y Issue #Y

Annex A (informative):
Appendix with UML code of the sequence diagrams

A.1			UML code for Figure 4.1.4.2.1

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

autonumber

actor NSC

participant BSS_NSP

participant OSS_SML

participant OSS_NML

NSC --> BSS_NSP : product order

BSS_NSP --> BSS_NSP : process product order

BSS_NSP --> OSS_SML : service order(s)

OSS_SML --> OSS_SML : process service order(s)

OSS_SML --> OSS_NML : resource order(s)

OSS_NML --> OSS_NML : process resource order(s)

OSS_SML <-- OSS_NML : resource order(s) completed

BSS_NSP <-- OSS_SML : service order(s) completed

NSC <-- BSS_NSP : product order completed

@enduml

A.2			UML code for Figure 4.1.4.3.1

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

autonumber

actor NSC

participant BSS_NSP

participant BSS_CSP #lightgrey

participant OSS_CSP #lightgrey

NSC --> BSS_NSP : product order

BSS_NSP --> BSS_NSP : process product order

BSS_NSP --> BSS_CSP : product order(s)

BSS_CSP --> BSS_CSP : process product order(s)

BSS_CSP --> OSS_CSP : service order(s)

OSS_CSP --> OSS_CSP : process service order(s)

BSS_CSP <-- OSS_CSP : service order(s) completed

BSS_NSP <-- BSS_CSP : product order(s) completed

NSC <-- BSS_NSP : product order completed

note right of OSS_CSP

BSS_NSP belongs to the company

fulfilling the NSP role.

BSS_CSP (grey) and OSS_CSP (grey) belong

to the company fulfilling the CSP role.

end note

@enduml

A.3			UML code for Figure 4.1.4.4.1

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

autonumber

actor NSC

participant BSS_NSP

participant OSS_NSP

participant BSS_CSP #lightgrey

participant OSS_CSP #lightgrey

NSC --> BSS_NSP : product order

BSS_NSP --> BSS_NSP : process product order

BSS_NSP --> OSS_NSP : service order(s)

OSS_NSP --> OSS_NSP : process service order(s)

OSS_NSP --> OSS_CSP : service order(s)

OSS_CSP --> OSS_CSP : process service order(s)

OSS_CSP --> BSS_CSP : notify service order(s) completed

OSS_NSP <-- OSS_CSP : service order(s) completed

BSS_NSP <-- OSS_NSP : service order(s) completed

NSC <-- BSS_NSP : product order completed

note right of OSS_CSP

BSS_NSP and OSS_NSP belong

to the company fulfilling the NSP role.

BSS_CSP (grey) and OSS_CSP (grey) belong

To the company fulfilling the CSP role.

end note

@enduml

A.4			UML code for Figure 4.1.2.1.1

@startuml

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam rectangleBackgroundColor white

skinparam rectangleBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor black

skinparam arrowColor black

hide circle

hide members

left to right direction

rectangle Exposed_MnS_Consumer

rectangle Exposed_MnS_Producer

Exposed_MnS_Consumer -(0- Exposed_MnS_Producer: eMnS

@enduml

A.5			UML code

@startuml

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

actor NSC

participant BSS_NSP

participant OSS_SML

participant OSS_NML

OSS_SML--> OSS_NML: 1. service discovery

OSS_SML---> OSS_SML: 2. configure the available MnS service

opt

BSS_NSP -->OSS_SML: 3. request list of available services

end

OSS_SML---> BSS_NSP: 4. provide list of available services

BSS_NSP --> BSS_NSP: 5. configure the eMnS service for exposure

NSC --> BSS_NSP: 6. product catalogue request

BSS_NSP --> NSC: 7. product catalogue

@enduml

A.6		UML code for 7.3.1

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

actor NSC

participant BSS_NSP

participant MnS_Discovery_service_producer

BSS_NSP --> MnS_Discovery_service_producer: 1. request to obtain MnS data

MnS_Discovery_service_producer --> BSS_NSP: 2. response for MnS data

Ref over NSC, BSS_NSP : 3. procedure of product order

@enduml

A.7		UML code for 7.3.2

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

actor NSC

participant BSS_NSP

participant MnS_Discovery_service_producer

participant OSS_SML

Ref over NSC, OSS_SML: 1. procedure of product order

Ref over NSC, OSS_SML: 2. authentication and authorization for accessing exposed MnS discovery service

NSC --> MnS_Discovery_service_producer: 3. request to obtain MnS data for exposed MnS

MnS_Discovery_service_producer --> NSC: 4. response for exposed MnS data for exposed MnS

NSC ---> OSS_SML: 5. request to use an exposed MnS

OSS_SML---> NSC: 6. response for using an exposed MnS

@enduml

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-05 | SA5#137e | S5-213046 | - | - | - | 28.824 skeleton-0.0.0 | 0.0.0 |
| 2021-05 | SA5#137e | S5-213565 | - | - | - | pCR 28.824 External exposure via BSS | 0.1.0 |
| 2021-05 | SA5#137e | S5-213566 | - | - | - | pCR 28.824 Add Description of Concept and Roles to TR 28.824 | 0.1.0 |
| 2021-05 | SA5#137e | S5-213658 | - | - | - | pCR 28.824 Use case – network slice management capability exposure | 0.1.0 |
| 2021-05 | SA5#137e | S5-213564 | - | - | - | pCR 28.824 Use case – Exposure of MnS for monitoring QoS of video application | 0.1.0 |
| 2021-08 | SA5#138e | S5-214645 | - | - | - | pCR 28.824 Use case – eMnS discovery service | 0.2.0 |
| 2021-08 | SA5#138e | S5-214646 | - | - | - | pCR 28.824 eMNs discovery by other systems | 0.2.0 |
| 2021-08 | SA5#138e | S5-214128 | - | - | - | pCR 28.824 Improve description of roles | 0.2.0 |
| 2021-10 | SA5#139e | S5-215523 | - | - | - | pCR 28.824 Add use case for the exposure of network slice as a product | 0.3.0 |
| 2021-10 | SA5#139e | S5-215524 | - | - | - | pCR 28.824 Types of interfaces for the exposure of network slice | 0.3.0 |
| 2021-10 | SA5#139e | S5-215525 | - | - | - | pCR 28.824 Service management scenarios between CSC, CSP and NSP | 0.3.0 |
| 2021-10 | SA5#139e | S5-215526 | - | - | - | pCR 28.824 Key issues relative to network slice management capabilities exposure | 0.3.0 |
| 2021-10 | SA5#139e | S5-215578 | - | - | - | Possible solution to support eMnS discovery service | 0.3.0 |
| 2021-10 | SA5#139e | S5-215579 | - | - | - | Solution for external discovery | 0.3.0 |
| 2021-11 | SA5#140e | S5-216583 | - | - | - | Clarification on clause 4 overview | 0.4.0 |
| 2021-11 | SA5#140e | S5-216616 | - | - | - | Add Use Case for exposure without going through BSS | 0.4.0 |
| 2021-11 | SA5#140e | S5-216617 | - | - | - | Exposure of network slice as a service | 0.4.0 |
| 2021-11 | SA5#140e | S5-216624 | - | - | - | Consolidate potential requirements of use cases for eMnS discovery service | 0.4.0 |
| 2021-11 | SA5#140e | S5-216585 | - | - | - | Add procedures related to product onboarding | 0.4.0 |
| 2021-11 | SA5#140e | S5-216586 | - | - | - | Clarification on clause 5 use cases | 0.4.0 |
| 2021-11 | SA5#140e | S5-216385 | - | - | - | Clarification on clause 7 solution | 0.4.0 |
| 2021-11 | SA5#140e | S5-216582 | - | - | - | Concept definition for Exposed Management Service | 0.4.0 |
| 2021-11 | SA5#140e | S5-216623 | - | - | - | Exposure to SA6 applications or middleware | 0.4.0 |
| 2021-11 | SA5#140e | S5-216625 | - | - | - | Add text to procedures related to management capability exposure | 0.4.0 |
| 2022-02 | SA5#141e | S5-221570 | - | - | - | Add new solution for MnS discovery service for exposure | 0.5.0 |
| 2022-02 | SA5#141e | S5-221713 | - | - | - | Skeleton restructuring proposal | 0.5.0 |
| 2022-02 | SA5#141e | S5-221714 | - | - | - | Key issue and solution on exposure interface via OSS | 0.5.0 |
| 2022-02 | SA5#141e | S5-221719 | - | - | - | pCR 28.824 Clarification on access to exposed MnS | 0.5.0 |
| 2022-02 | SA5#141e | S5-221742 | - | - | - | Add solution for product and service order procedures to clause 7 | 0.5.0 |
| 2022-02 | SA5#141e | S5-221743 | - | - | - | Update procedure for product on-boarding | 0.5.0 |
| 2022-02 | SA5#141e | S5-221744 | - | - | - | Resolve EN in 5.4 | 0.5.0 |