| 3GPP TS 28.405 V17.1.0 (2022-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Quality of Experience (QoE) measurement collection	8

4.1	Management based activation in UTRAN	8

4.1.1	Activation of measurement collection job and reporting of collected information in UTRAN	8

4.1.2	Handling of measurement collection at handover in UTRAN	10

4.1.2.1	Handover between cells within an RNC	10

4.1.2.2	Handover between RNCs	10

4.1.3	Deactivation of measurement collection job in UTRAN	11

4.1.3.1	Forced deactivation in UTRAN	11

4.1.3.2	Deactivation of recording session in UTRAN	11

4.2	Management based activation in LTE	11

4.2.1	Activation of measurement collection job and reporting of collected information in LTE	11

4.2.2	Handling of measurement collection at handover in LTE	12

4.2.2.1	Handover between cells within an eNB	12

4.2.2.2	Handover between eNBs	13

4.2.3	Deactivation of measurement collection job in LTE	14

4.2.3.1	Forced deactivation	14

4.2.3.2	Deactivation of recording session	14

4.2.4	Void	14

4.3	Signalling based activation in UTRAN	14

4.3.1	Activation of measurement collection job in UTRAN	14

4.3.2	Handling of measurement collection at handover in UTRAN	16

4.3.3	Deactivation of measurement collection job in UTRAN	16

4.3.3.1	Pre-set time has elapsed in UTRAN	16

4.33.2	Forced deactivation in UTRAN	16

4.3.3.3	Deactivation of recording session in UTRAN	16

4.4	Signalling based activation in LTE	16

4.4.2	Activation of measurement collection for a UE in LTE	16

4.4.2.0	General	16

4.4.2.1	Activation of QoE measurement task before UE attaches to the network	16

4.4.2.2	Activation of QoE measurement task after UE attachement to the network	17

4.4.3	Handling of measurement collection at handover in LTE	19

4.4.4	Deactivation of measurement collection for a UE in LTE	19

4.4.4.1	Pre-set time has elapsed in LTE	19

4.4.4.2	Forced deactivation in LTE	19

4.4.4.3	Deactivation of recording session in LTE	19

4.5	Management based activation in NR	19

4.5.1	Activation of measurement collection job and reporting of collected information in NR	19

4.5.3	Deactivation of measurement collection job in NR	21

4.5.3.1	Forced deactivation	21

4.5.3.2	Deactivation of recording session	21

4.5.4	Temporary stop and restart of QoE information reporting during RAN overload in NR	21

5	Quality of Experience (QoE) measurement management parameters	21

5.1	QoE collection entity address (M)	21

5.2	QoE reference (M)	21

5.3	PLMN target (CM)	22

5.4	Area scope (CM)	22

5.5	QMC configuration file (container) (M)	22

5.6	QMC target (M)	22

5.7	Recording session id (M)	22

5.8	Service type (M)	23

5.9	Slice scope (CM)	23

Annex A (informative): Change history	24

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management, as identified below:

TS 28.404:	"Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements";

TS 28.405:	"Quality of Experience (QoE) measurement collection; Control and configuration";

TS 28.406:	"Quality of Experience (QoE) measurement collection; Information definition and transport".

One main motivation of mobile network evolution is to improve the user experience, which is why the evaluation of the user experience at the UE side is vital to network operators. This is especially true when the operators provide high bit rate real-time services like streaming services (typically video services), where even intermittent quality degradation is very annoying. Many of these streaming services are a significant part of the commercial traffic growth rate, therefore the focus is on the end users' experience.

Quality of Experience (QoE) information collection provides detailed information at session level on a number of UEs.

The capability to log information within a UE, and in particular the QoE of an end user service, initiated by an operator, provides the operator with QoE information. The collected information (specified in 3GPP TS 26.247 [6]) cannot be deduced from performance measurements in the mobile network.

The QoE information is information collected by the end user application in the UE.

The QoE information is collected by the management system for analysis and/or KPI calculations.

# 1	Scope

The present document addresses the mechanisms used for the function Quality of Experience (QoE) measurement collection in 3GPP networks . The measurements that are collected are DASH [6] and MTSI [7] measurements and Virtual Reality (VR) (see TS 26.118 [13]).

The function includes collecting QoE information from UEs frequenting a specified area or an individual UE for a specified end user service/end user service type. The document describes the activation and deactivation of a network request session, UE request session and recording session and also the reporting of recorded information [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.404: "Telecommunication management;Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements".

[3]	3GPP TS 28.308: "Management of Quality of Experience (QoE) measurement collection Integration Reference Point (IRP); Information Service (IS)".

[4]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[5]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[6]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[7]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[8]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[10]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[11]	3GPP TS 38.331: " NR; Radio Resource Control (RRC); Protocol specification".

[12]	3GPP TS 38.413: " NG-RAN; NG Application Protocol (NGAP)".

[13]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DM	Domain Manager

MCE	Measurement Collector Entity

NB	Node B

QMC	QoE Measurement Collection

QoE	Quality of Experience

VR	Virtual Reality

# 4	Quality of Experience (QoE) measurement collection

## 4.1	Management based activation in UTRAN

### 4.1.1	Activation of measurement collection job and reporting of collected information in UTRAN

The parameters for the network request session are sent from the management system to the RNCs that host the cells that are included in the collection job request in the activateAreaQMCJob operation [3]. The RNC starts a network request session, with the Network request session id [3] given in activateAreaQMCJob operation [3]. For the duration of the network request session, the RNC(s) checks for connections where the UE has the UE Application Layer Measurement Capability [10]. The UE Application Layer Measurement Capability is sent from the UE to RNC (via the core network) in message INITIAL UE MESSAGE [10].

When a session is found that has a UE with the UE Application Layer Measurement Capability, the RNC starts a UE request session by sending a MEASUREMENT CONTROL message [4] to the UE via the NB.

The AT command +CAPPLEVMC [5] activates provisioning of measurement reporting from the lower layers to the application within the UE by an unsolicited result code +CAPPLEVMC. When information about a measurement report is received in the UE from the network, the unsolicited result code +CAPPLEVMC contains the relevant parameters (service_type, start_stop-indication and config-file & config-file length if applicable) that are provided from the lower layers in the UE to the application.

Figure 4.1.1-1: QMC activation and reporting in UTRAN

1.	The NM sends activateAreaQMCJob to DM/EM that controls the impacted RNC(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qMCTarget, qoEReference and QMC configuration file.

2.	The DM/EM forwards activateAreaQMCJob to impacted RNC(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qMCTarget, qoEReference and QMC configuration file.

3.	The RNC checks for connections where the UE has the UE Application Layer Measurement Capability [10] that match the criteria for serviceType in the activateAreaQMCJob.

4.	When a connection is found that has the UE Application Layer Measurement Capability [10], the RNC start a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC to application level and includes the following: serviceType, qoEReference and QMC configuration file.

6.	When the application in the serviceType starts, the QMC is initiated.

7.	The application layer sends the AT command +CAPPLEVMR including a recording session indication that a session has started to the access stratum.

8.	The UE sends the message MeasurementReport including the recording session indication to the RNC.

9.	The RNC sends a notification including the recording session indication to the NM.

10.	When the QMC is completed, the recorded information is collected in a QMC report [6], [7], including qoEReference and recordingSessionId. The qoEReference, Client Id [6], [7] in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

11.	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

12.	The UE sends the message MeasurementReport including qoEReference and the QMC report to the RNC.

13.	The RNC sends the QMC report to the MCE associated to the qoEReference.

### 4.1.2	Handling of measurement collection at handover in UTRAN

#### 4.1.2.1	Handover between cells within an RNC

When handover is made, the measurement area is checked.

-	If the mobile is inside the measurement area after the handover, the RNC sends in an indication that the UE is inside the measurement area in the RRCConnectionReconfiguration message [4] to the UE. The UE send the +CAPPLEVMC AT command [5] to the application to inform that recording shall start if the application becomes active (when the UE moves into the measurement area).

-	If the mobile has moved outside the measurement area, there is no indication that the UE is inside the measurement area in the RRCConnectionReconfiguration message [4] from the RNC to the UE. The UE sends the +CAPPLEVMC AT command [5] to the application to inform that recording shall not be started even if the application becomes active.

#### 4.1.2.2	Handover between RNCs

The figure 4.1.2.2-1 and the text below describes the handling at handover between RNCs.

Figure 4.1.2.2-1: Handling of QMC activation in case of handover in UTRAN

1.	Source RNC sends the message HandoverRequest to Target RNC and includes areaScope, qoECollectionEntityAddress and a recording session indication indicating whether a recording session has started.

2.	If a recording session has not started, the target RNC checks if the target cell is inside the areaScope or not.

3.	The target RNC sends the message Handover Request Ack to the source RNC.

4.	The source RNC sends the message RRCReconfiguration to the UE.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC to application level.

6.	The UE sends the message RRCReconfigurationComplete to the RNC.

7.	When the application becomes active, QMC is initiated if the UE is inside the areaScope.

8.	The application layer sends the AT command +CAPPLEVMR including a recording session indication to the access stratum.

9.	The UE sends the message MeasurementReport including the recording session indication to the RNC.

10.	The RNC sends a notification including the recording session indication to the NM.

11.	When the QMC is completed, the recorded information is collected in a QMC report [6] and [7]. including qoEReference and recordingSessionId. The qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session) and recordingSessionId are needed in the QoE collection entity for post processing purposes.

12.	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

13.	The UE sends the message MeasurementReport including qoEReference and the QMC report to the RNC.

14.	The RNC sends the QMC report to the MCE associated to the qoEReference.

### 4.1.3	Deactivation of measurement collection job in UTRAN

#### 4.1.3.1	Forced deactivation in UTRAN

If the operator technician or the management application wants to deactivate a measurement collection job before the pre-set time has expired, the management system sends the deactivateQMCJob operation [3] to the RNC. The RNC sets the network request session to ended, but does not delete the UE request session id and the Collection Entity Address parameters [3], as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the RNC sends the MeasurementControl message [4] with the DeactivateJob request to relevant UEs. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the DeactivateJob request. The application stops the recording session and stops recording of the requested information. The UE request session id and the Collection Entity Address parameters [3] in the RNC are deleted when the UE request session is ended.

#### 4.1.3.2	Deactivation of recording session in UTRAN

Regardless of whether the pre-set time has elapsed or not, the recording session continues to be active until the session for the application is ended.

## 4.2	Management based activation in LTE

### 4.2.1	Activation of measurement collection job and reporting of collected information in LTE

The parameters for the network request session are sent from the management system to the eNBs that host the cells that are included in the collection job request in the activateAreaQMCJob operation [3]. The eNB starts a network request session, with the Network request session id [3] given in activateAreaQMCJob operation [3]. For the duration of the network request session, the eNB(s) checks for connections where the UE has the QoE-MeasReport capability [8] for collection of streaming services or the QoE-MTSI-MeasReport capability [8] for collection of MTSI services. The UE capability is sent from the UE to eNB via the core network in message UE CAPABILITY INFO INDICATION [9].

When a session is found that has a UE with the wanted UE capability, the eNB starts a UE request session and sends a RRCConnectionReconfiguration [8] to the UE.

The AT command +CAPPLEVMC [5] activates provisioning of measurement reporting from the lower layers to the application within the UE by an unsolicited result code +CAPPLEVMC. When information about a measurement report is received in the UE from the network, the unsolicited result code +CAPPLEVMC contains the relevant parameters (service_type, start_stop-indication and config-file & config-file length if applicable) that are provided from the lower layers in the UE to the application.

Figure 4.2.1-1: QMC activation and reporting in LTE

1.	The NM sends activateAreaQMCJob to DM/EM that controls the impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

2.	The DM/EM forwards activateAreaQMCJob to impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

3.	The eNB checks for connections where the UE has the UE capability [9] that match the criteria for serviceType in the activateAreaQMCJob.

4.	When a connection is found that has the wanted UE capability [9], the eNB starts a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC to application level and includes the following: serviceType, qoEReference and QMC configuration file.

6.	When the application in the serviceType starts, the QMC is initiated.

7.	The application layer sends the AT command +CAPPLEVMR including a recording session indication that indicates that a session is started to the access stratum.

8.	The UE sends the message MeasReportAppLayer including the recording session indication to the eNB.

9.	The eNB sends a notification including the recording session indication to the NM.

10.	When the QMC is completed, the recorded information is collected in a QMC report [6], [7], including qoEReference and recordingSessionId. The qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

11.	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

12.	The UE sends the message MeasReportAppLayer including qoEReference and the QMC report to the eNB.

13.	The eNB sends the QMC report to the MCE associated to the qoEReference.

### 4.2.2	Handling of measurement collection at handover in LTE

#### 4.2.2.1	Handover between cells within an eNB

When handover is made and no Recording session id is provided, the measurement area is checked.

-	If the mobile is inside the measurement area after the handover, the eNB sends in an indication that the UE is inside the measurement area in the RRCConnectionReconfiguration message [8] to the UE. The UE send the +CAPPLEVMC AT command [5] to the application to inform that recording shall start if the application becomes active (when the UE moves into the measurement area).

-	If the mobile has moved outside the measurement area, there is no indication that the UE is inside the measurement area in the RRCConnectionReconfiguration message [8] from the eNB to the UE. The UE sends the +CAPPLEVMC AT command [5] to the application to inform that recording shall not be started even if the application becomes active.

#### 4.2.2.2	Handover between eNBs

The figure 4.2.2.2-1 and the text below describes the handling at handover between eNBs.

Figure 4.2.2.2-1: Handling of QMC activation in case of handover in LTE

1.	Source eNB sends the message HandoverRequest to Target eNB and includes areaScope and qoECollectionEntityAddress.

2.	The target eNB checks if the target cell is inside the areaScope or not.

3.	The target eNB sends the message HandoverRequestAcknowledge to the source eNB.

4.	The source eNB sends the message RRCConnectionReconfiguration to the UE.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC to application level.

6.	The UE sends the message RRCConnectionReconfigurationComplete to the eNB.

7.	When the application becomes active, QMC is initiated if the UE is inside the areaScope.

8.	The application layer sends the AT command +CAPPLEVMR including a recording session indication indicating that a session has started to the access stratum.

9.	The UE sends the message MeasReportAppLayer including the recording session indication to the eNB.

10.	The eNB sends a notification including the recording session indication to the NM.

11.	When the QMC is completed, the recorded information is collected in a QMC report [6], [7]. including qoEReference and recordingSessionId. The qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session) and recordingSessionId are needed in the QoE collection entity for post processing purposes.

12.	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

13.	The UE sends the message MeasReportAppLayer including qoEReference and the QMC report to the eNB.

14.	The eNB sends the QMC report to the MCE associated to the qoEReference.

### 4.2.3	Deactivation of measurement collection job in LTE

#### 4.2.3.1	Forced deactivation

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQMCJob operation [3] to the eNB. The eNB sets the network request session to ended, but does not delete the UE request session id and the Collection Entity Address parameters [3], as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the eNB sends the RRCConnectionReconfiguration message [8] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to discard application layer measurement report information in otherConfig [8]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the discard request. The application stops the recording session and stops recording of the requested information. The UE request session id and the Collection Entity Address parameters [3] in the eNB are deleted when the UE request session is ended.

#### 4.2.3.2	Deactivation of recording session

Regardless of whether the pre-set time has elapsed or not, the recording session continues to be active until the session for the application is ended.

### 4.2.4	Void

## 4.3	Signalling based activation in UTRAN

### 4.3.1	Activation of measurement collection job in UTRAN

In Signalling based activation, the QoE measurement collection is sent to the UE from the OAM via CN. The messages from EMS propagated to HSS, MSE Server/SGSN and RNC. QoE measurement collection activation request is propagated to UE.

When HSS activates the QoE measurement collection for a job, the MSC Server/SGSN can send the QoE measurement collection to the UE through RNC, the following configuration parameters shall be included in the message: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

1)	The NM sends activateAreaQMCJob to DM/EM that controls the impacted RNC(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qMCTarget, Individual UE Id, qoEReference and QMC configuration file.

2)	The DM/EM sends activateAreaQMCJob to HSS that controls the impacted RNC(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, Individual UE Id,qoEReference and QMC configuration file.

3)	The HSS sends an Insert Subscriber Data to the MSC Server/SGSN that is registered in the HSS in a previous Update Location Request message.

4)	The MSC Server/SGSN starts finding the UE and check if that the UE capability matches the criteria for ServiceType in activateAreaQMCJob

5)	The MSC Server/SGSN sends message CN INVOKE TRACE to the RNC and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget,qoEReference and QMC configuration file.

6)	The RNC checks for connections where the UE has the UE Application Layer Measurement Capability that match the criteria for serviceType in the activateAreaQMCJob.

7)	The RNC start a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

8)	The access stratum in the UE sends the AT command +CAPPLEVMC to application level and includes the following: serviceType, qoEReference and QMC configuration file.

9)	When the application in the serviceType starts, the QMC is initiated.

10)	The application layer sends the AT command +CAPPLEVMR including a recording session indication that a session has started to the access stratum.

11)	The UE sends the message MeasurementReport including the recording session indication to the RNC.

12)	The RNC sends a notification including the recording session indication to the NM.

13)	When the QMC is completed, the recorded information is collected in a QMC report, including qoEReference and recordingSessionId. The qoEReference, Client Id, in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

14)	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

15)	The UE sends the message MeasurementReport including qoEReference and the QMC report to the RNC.

16)		The RNC sends the QMC report to the MCE associated to the qoEReference.

### 4.3.2	Handling of measurement collection at handover in UTRAN

See clause 4.1.2.

### 4.3.3	Deactivation of measurement collection job in UTRAN

#### 4.3.3.1	Pre-set time has elapsed in UTRAN

When the pre-set time is elapsed, the HSS or SGSN and the RNC sets the network request session to ended, but do not delete the UE request session id and the Collection Entity Address parameters [3], due to that reports still can come from the UE that has to be transferred to the collection centre. The UE request session id and the Collection Entity Address parameters [3] are deleted when the session is ended.

#### 4.33.2	Forced deactivation in UTRAN

If the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQoEJob operation [3] to the HSS or SGSN, that propagate it to the RNC. The RNC sets the network request session to ended, but does not delete the UE request session id and the Collection Entity Address parameters [3], as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the RNC sends the MeasurementControl message [4] with the DeactivateJob request to relevant UEs. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the DeactivateJob request. The application stops the recording session and stops recording of the requested information. The UE request session id and the Collection Entity Address parameters [3] in the RNC are deleted when the UE request session is ended.

#### 4.3.3.3	Deactivation of recording session in UTRAN

See clause 4.1.3.3.

## 4.4	Signalling based activation in LTE

### 4.4.2	Activation of measurement collection for a UE in LTE

#### 4.4.2.0	General

Activation of measurement collection for a UE can be done before UE attachment or after UE is attached.

#### 4.4.2.1	Activation of QoE measurement task before UE attaches to the network

When HSS activates the QoE measurement collection for a job, to the MME the following configuration parameters shall be included in the message: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

At UE attachment the configuration parameters for the QoE measurement collection for a job is propagated to the UE.

The MME receives and stores QoE measurement collection job as part of Update Location Answer (or Insert Subscriber Information). Then the same procedure for activation of measurement collection job is applied as after UE is attached, described in see 4.4.2.2.

#### 4.4.2.2	Activation of QoE measurement task after UE attachement to the network

The QoE measurement collection is sent to the UE from the OAM via CN. The messages from EMS propagated to HSS, MME and eNodeB are the same as previous section. QoE measurement collection activation request is propagated to UE finally.

When MME can send the QoE measurement collection to the UE through eNodeB, the following configuration parameters shall be included in the message: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file. QoE measurement collection activation request is propagated to UE.

1)	The EMS sends activateAreaQMCJob to HSS that controls the impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

2)	The HSS inserts subscriber related data and forwards it to the MME.

3)	The MME forwards activateAreaQMCJob to impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

4)	The eNB checks if the UE capability that matches the criteria for serviceType in the activateAreaQMCJob.

5)	If the UE has the wanted UE capability, the eNB starts a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

6)	The access stratum in the UE sends the AT command +CAPPLEVMC to application level and includes the following: serviceType, qoEReference and QMC configuration file.

7)	When the application in the serviceType starts, the QMC is initiated.

8)	The application layer sends the AT command +CAPPLEVMR including a recording session indication that indicates that a session is started to the access stratum.

9)	The UE sends the message MeasReportAppLayer including the recording session indication to the eNB.

10)	The eNB sends a notification including the recording session indication to the NM.

11)	When the QMC is completed, the recorded information is collected in a QMC report [6], [7], including qoEReference and recordingSessionId. The qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

12)	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

13)		The UE sends the message MeasReportAppLayer including qoEReference and the QMC report to the eNB.

14)		The eNB sends the QMC report to the MCE associated to the qoEReference.

### 4.4.3	Handling of measurement collection at handover in LTE

See clause 4.2.2.

### 4.4.4	Deactivation of measurement collection for a UE in LTE

#### 4.4.4.1	Pre-set time has elapsed in LTE

When the pre-set time is elapsed, the MME and the eNB sets the network request session to ended, but do not delete the UE request session id and the Collection Entity Address parameters [3], due to that reports still can come from the UE that has to be transferred to the collection centre. The UE request session id and the Collection Entity Address parameters [3] are deleted when the session is ended.

#### 4.4.4.2	Forced deactivation in LTE

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQMCJob operation [3] to the MME that propgates to the eNB . The eNB sets the network request session to ended, but does not delete the UE request session id and the Collection Entity Address parameters [3], as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the eNB sends the RRCConnectionReconfiguration message [8] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to discard application layer measurement report information in otherConfig [8]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the discard request. The application stops the recording session and stops recording of the requested information. The UE request session id and the Collection Entity Address parameters [3] in the eNB are deleted when the UE request session is ended.

#### 4.4.4.3	Deactivation of recording session in LTE

See clause 4.2.3.2.

## 4.5	Management based activation in NR

### 4.5.1	Activation of measurement collection job and reporting of collected information in NR

The parameters for the network request session are sent from the management system to the gNBs that host the cells that are included in the collection job request in the activateAreaQMCJob operation [3]. The gNB starts a network request session, with the Network request session id [3] given in activateAreaQMCJob operation [3]. For the duration of the network request session, the gNB(s) checks for connections where the UE has the QoE-MeasReport capability [11] for collection of the Service type in the activateAreaQMCJob. The UE capability is sent from the UE to gNB via the core network in message UE CAPABILITY INFO INDICATION [12].

When a session is found that has a UE with the wanted UE capability, the gNB starts a UE request session and sends a RRCReconfiguration [11] to the UE.

The AT command +CAPPLEVMC [5] activates provisioning of measurement reporting from the lower layers to the application within the UE by an unsolicited result code +CAPPLEVMC. When information about a measurement report is received in the UE from the network, the unsolicited result code +CAPPLEVMC contains the relevant parameters (service_type, start_stop-indication and config-file & config-file length if applicable) that are provided from the lower layers in the UE to the application.

1. The NM sends activateAreaQMCJob to DM/EM that controls the impacted gNB(s), and includes the parameters: serviceType, areaScope, SliceScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

2.	The DM/EM forwards activateAreaQMCJob to impacted gNB(s), and includes the parameters: serviceType, areaScope, SliceScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

3.	The gNB checks for connections where the UE has the UE capability [12] that match the criteria for serviceType in the activateAreaQMCJob.

4.	When a connection is found that has the wanted UE capability [12], the gNB starts a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCReconfiguration to the UE, and includes the following: serviceType, MeasConfigAppLayerId and QMC configuration file.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC to application level and includes the following: serviceType, MeasConfigAppLayerId and QMC configuration file.

6.	When the application in the serviceType starts, the QMC is initiated.

7.	The application layer sends the AT command +CAPPLEVMR MeasConfigAppLayerId to the access stratum.

8.	The UE sends the message MeasConfigAppLayerId to the gNB.

9.	The gNB sends a notification including the recording session indication to the NM.

10.	When the QMC is completed, the recorded information is collected in a QMC report [6], [7], including qoEReference and recordingSessionId. The qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

11.	The application layer sends the AT command +CAPPLEVMR including MeasConfigAppLayerId and the QMC report to the access stratum.

12.	The UE sends the message including MeasConfigAppLayerId and the QMC report to the gNB.

13.	The gNB sends the QMC report to the MCE associated to the qoEReference and SliceScope.

### 4.5.3	Deactivation of measurement collection job in NR

#### 4.5.3.1	Forced deactivation

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQMCJob operation [3] to the gNB. The gNB sets the network request session to ended, but does not delete the UE request session id and the Collection Entity Address parameters [3], as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the gNB sends the RRCReconfiguration message [11] to relevant UEs. The RRCReconfiguration message is including measConfigAppLayerId set to discard application layer measurement report information in otherConfig [11]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the discard request. The application stops the recording session and stops recording of the requested information. The UE request session id and the Collection Entity Address parameters [3] in the gNB are deleted when the UE request session is ended.

#### 4.5.3.2	Deactivation of recording session

The recording session continues to be active until the session for the application is ended.

4.5.4	Temporary stop and restart of QoE information reporting during RAN overload in NR

In case of overload in RAN, the gNB may temporarily stop the reporting from the UE by sending the RRCReconfiguration message [11] to relevant UEs. The RRCReconfiguration message [11] is including measConfigAppLayerId set to temporarily stop application layer measurement reporting in otherConfig [11]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the temporary stop request. The application stops the reporting.The application continues and stops recording further information untilwhen the data storage capacity for the reporting is fully used or the UE request session ends. Then the recorded data is kept until it is reported or when the UE request session is ended.

When the overload situation in RAN is ended the gNB restart the reporting from the UE by send the RRCReconfiguration message [11] to relevant UEs. The RRCReconfiguration message is including measConfigAppLayerId set to restart application layer measurement reporting in otherConfig [11]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the restart request. The application restarts the reporting and recording if it was stopped.

# 5	Quality of Experience (QoE) measurement management parameters

## 5.1	QoE collection entity address (M)

This is a parameter which defines the IP address to which the QMC records shall be transferred. Ipv4 or Ipv6 address(es) may be used.

## 5.2	QoE reference (M)

The QoE reference parameter specify the network request session. The QoE reference shall be globally unique therefore it is composed as follows:

MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC  activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 byte Octet String.

The QMC ID is generated by the management system or the operator.

It is used to identify the QoE measurement collection job in the traffic nodes and in the measurement collection centre.

## 5.3	PLMN target (CM)

This parameter defines the PLMN for which sessions shall be selected in the network request session in case of area based QMC when several PLMNs are supported in the RAN (this means that shared cells and not shared cells are allowed for the specified PLMN. Furthermore, several PLMNs can be used for not shared RAN cases as well as for shared RAN cases.). Only the sessions may be selected where the PLMN that the UE reports as selected PLMN is the same as the PLMN Target.

Note that the PLMN Target might differ from the PLMN specified in the Network Request Session Id, as that specifies the PLMN that is containing the management system requesting the Network Request  Session from the NE.

The parameter is mandatory if network sharing is deployed.

## 5.4	Area scope (CM)

The area scope parameter defines the area in terms or cells or Tracking Area/Routing Area/Location Area where the QMC shall take place. If the parameter is not present the QMC shall be done throughout the PLMN specified in PLMN target.

The area scope parameter in UMTS is either:

-	List of cells, identified by CGI. Maximum 32 CGI can be defined.

-	List of Routing Area, identified by RAI. Maximum of 8 RAIs can be defined.

-	List of Location Area, identified by LAI. Maximum of 8 LAIs can be defined.

The area scope parameter in LTE is either:

-	list of cells, identified by E-UTRAN-CGI. Maximum 32 CGI can be defined.

-	List of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined.

The area scope parameter in NR is either:

-	list of cells, identified by N-CGI. Maximum 32 NCGI can be defined.

-	List of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined.

The parameter is mandatory if area based QMC is requested.

## 5.5	QMC configuration file (container) (M)

The QMC configuration file is a container that is specified in [6] or [7].

## 5.6	QMC target (M)

The QMC target parameter specifies it the QMC is area based or individual UE based.

-	Area based QMC (0)

-	Individual UE based QMC (1)

NOTE:	Void.

## 5.7	Recording session id (M)

This parameter shall be a 2 byte octet string. The recording session id shall be the same for the whole session in the application, while for each different session in the application the recording session id shall be changed. The recording session id is generated by the application in the UE.

It is used in the measurement collection centre to identify which session within a UE has collected information in the application.

## 5.8	Service type (M)

Which kind of service that shall be recorded.

-	DASH (0)

-	MTSI (1)

-	VR (2)

DASH measurements are specified in [6]. MTSI measurements are specific in [7]. VR measurements are specified in TS 26.118 [13].

## 5.9	Slice scope (CM)

This parameter contains a list of S-NSSIAs (Single Network Slice Selection Assistance Information). A Network Slice end to end is identified by S-NSSAI.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-07 | SA#88e |  |  |  |  | Upgrade to version under change control | 16.0.0 |
| 2021-09 | SA#93e | SP-210883 | 0001 | - | F | Aligning with RAN specifications 36.331 and 25.331 | 16.1.0 |
| 2021-12 | SA#94e | SP-211454 | 0004 | 1 | F | Corrent the description of QoE reference and PLMN target | 16.2.0 |
| 2021-12 | SA#94e | SP-211453 | 0003 | 1 | B | Adding Signalling Based Activation for UTRAN and LTE | 17.0.0 |
| 2021-03 | SA#95e | SP-220181 | 0005 | - | B | Adding Management Based Activation and Temporary stop and restart during RAN overload in NR | 17.1.0 |
| 2021-03 | SA#95e | SP-220181 | 0008 |  | B | Update to include NR and adding VR | 17.1.0 |