# 3GPP TSG SA WG5 Meeting #141e	S5-221634

# Online, , 17 Jan 2021- 26 Jan 2021

# Source:	Samsung

# Title:	pCR 28.538 ECS Performance assurance

# Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides ECS performance assurance procedures.

# 4	Detailed proposal

First modification

## 8.2	Performance Assurance

### 8.2.x	ECS performance assurance

#### 8.2.x.1	MnS component type A

Table 8.2.1.1-1: ECS performance assurance type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1.1 of TS 28.532 [5]: - createMOI operation | It is supported by Provisioning MnS to create PerfMetricJob IOC, as defined in TS 28.531 [9]. |
| Operations defined in clause 11.5 and 11.6 in TS 28.532 [3] and clause 6.1 of TS 28.550 [8]: - createMeasurementJob - stopMeasurementJob - listMeasurementJobs - establishStreamingConnection - notifyFileReady notification - reportStreamData operation | It is supported by Performance Assurance MnS for ECS, as defined in TS 28.550 [8]. |

#### 8.2.x.2	MnS Component Type C definition

Performance measurements related ECS are captured in Table 8.2.1.2.-1:

Table 8.2.1.2-1. ECS related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Mean virtual CPU usage | Includes the mean usage of the underlying virtualized CPUs for a virtualized 3GPP NF (see clause 5.7.1.1.1 in TS 28.552 [10]). |  |
| Mean virtual memory usage | Includes the mean usage of the underlying virtualized memories for a virtualized 3GPP NF (see clause 5.7.1.2.1 in TS 28.552 [10]). |  |
| Mean virtual disk usage | Includes the mean usage of the underlying virtualized disks for a virtualized 3GPP NF (see clause 5.7.1.3.1 in TS 28.552 [10]). |  |
| EES Registration | Includes the total, mean and successful number of EES Registration request processed by ECS (see clause 5.X.1 in TS 28.552 [10]). |  |
| Service Provisioning | Includes the total, mean and successful number of service provisioning request processed by ECS (see clause 5.X.1 in TS 28.552 [10]). |  |