3GPP TSG SA WG5 Meeting 142-e		S5-222362

# e-meeting, 4 -12 April 2022

# Source:	Nokia

# Title:	Modify the paging requirements based on geographical area

# Document for:	Approval

Agenda Item:	6.6.5

# 1	Decision/action requested

This contribution is for approval.

# 2	Rationale

This contribution proposes to modify the requirements of the MDA capability of “Paging optimization analysis” to provide the analytics output describing the paging patterns based on geographical area rather than based on users or group of users.

Users are mobile in the network and paging is performed to a particular UE based on it’s last known location. If there is a network problem (intentional/unintentional) in a particular geographical area because of which the paging fails to a particular UE, same problem would be witnessed for all the UEs in the same vicinity. So, it becomes redundant to share the same analytics as part of all the users facing the problem of paging.

Instead, it would make more sense to perform the paging analytics based on geographical area than based on users or group of users.

In requirement REQ-PAG_MDA-03, predictions have been proposed to be carried our everyday. This may not be essentially useful for all the networks and their traffic profiles. For instance, half-day analytics would be better and in some cases even lesser. Hence it is better to remove “evey day” and add the keyword “periodically” so that the intention is clear in the requirement and the actual periodicity is left to the implementation based on the target network in which the analytics is deployed.

# 3	Detailed proposal

1st Modified Section

#### 7.2.1.3	Paging optimization analysis

##### 7.2.1.3.1	Description

This MDA capability deals with enabling various functionalities related to paging optimization.

##### 7.2.1.3.2	Use Case

As per the current procedures, if the UE goes out-of-coverage (OOC) the paging which was initiated by the network Access and Mobility Management Function (AMF) fails. The re-attempts continue to fail until UE comes in the coverage and reacts to the paging attempts. This repetitive paging attempts result in the wastage of network resources. As an example, the use case includes a user or a group of users getting into an area, with no cellular coverage on a regular basis for a considerably long duration, for e.g., the user gets into a shielded room for some testing purpose every day for a defined period. The Network initiated paging for such users will fail until they are back in the area with cellular coverage. This would result in in-efficient network resource usage.

It is desirable to use MDAS (Management data analytic service) to optimize the current paging procedures in 5G networks. MDAS producer provides an analytics output containing the user(s) paging analytics indicating the time window at which the user is OOC on a regular basis at the particular location and hence will not be able to respond on a network-initiated paging. Based on the output MDAS consumer (e.g., AMF, gNB) decides on whether, when and where to initiate or not to initiate the paging procedures, thereby ensuring the efficient paging procedures and optimal network resource utilization, as paging can be initiated only when there are more chances for it to be successful.

##### 7.2.1.3.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-PAG_MDA-01 | MDA capability for paging optimization analysis shall be able to provide analytics output describing paging result patterns based on geographical area. | Paging optimization analysis |
| REQ-PAG_MDA-02 | MDA capability for paging optimization analysis shall be able to provide analytics output describing the paging result patterns based on successful and un-successful paging attempts at a particular time and duration based on geographical area. | Paging optimization analysis |
| REQ-PAG_MDA-03 | MDA capability for paging optimization analysis shall be able to provide analytics output describing the paging result patters to contain the following information: - Identify the geographical area of concern. - Prediction of the time window during which UE is out-of-coverage periodically. - Prediction of the last known location before UE going out-of-coverage periodically. - The recommended action which may suggest stopping paging the UE for Daily-OOC-Duration at Daily-OOC-Location. | Paging optimization analysis |

End of Modified Sections