3GPP TSG SA WG5 Meeting 142-e		222547

# Online, , 4th April 2022 - 12th April 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.160 | CR | draftCR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR YANG Mapping Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | adNRM |  |  |  |  |  | Date: |  |  | 2022-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Unclear or faulty mapping rules for YANG - Document that stage2 names should be used in the YANG SS - The “ordered-by user” statement is ignored in YANG if the leaf-list or list is config=false, but the current mapping rules still require it. This could give the false feeling that it means something. - The interplay between the defaultValue, isNullable properties and multiplicity, and their mapping to YANG was not clearly described. - The case of “isNullable=false for an attribute that is mapped to a list or leaf-lists” was not described, as the values false and true were interchanged due to an editing error. - Indicate that structured types are mapped to a grouping |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify mapping of isNullable, defaultValue and isOrdered and the mapping of structured types |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Faulty mapping of stage 2 specifications to YANG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.a, 6.2.10, 6.2.10.1, 6.2.10.2, 6.2.12, 6.2.12.0, 6.2.12.1, 6.2.13, 6.2.13.1, 6.2.13.2, 6.2.14, 6.2.14.0, 6.2.14.1, 6.2.15, 6.2.15.0, 6.2.15.1, 6.2.16, 6.2.16.0, 6.2.16.1, 6.2.21, 6.2.21.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This section is only used for new design work. As there is no new design work prior to Rel-17, there is no need for the same change in earlier releases. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire"

[4]	ITU-T Recommendation M.3020 (07/2017): "Management interface specification methodology".

[5]	3GPP TR 21.801: "Specification drafting rules".

[6]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[7]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[8]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[9]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[10]	ITU-T Recommendation M.3020 (07/2011): "Management interface specification methodology" – Annex E "Information type definitions – type repertoire".

[11]	IETF RFC 8407: "Guidelines for Authors and Reviewers of Documents Containing YANG Data Models, October 2018".

[12]	3GPP TS 28.532: " Management and orchestration; Generic management services"

[13]	IETF RFC 8528: "YANG Schema mount "

[14]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.1.md.

[15]	draft-wright-json-schema-01 (October 2017): "JSON Schema: A Media Type for Describing JSON Documents".

[16]	draft-wright-json-schema-validation-01 (October 2017: "JSON Schema Validation: A Vocabulary for Structural Validation of JSON".

[17]	draft-wright-json-schema-hyperschema-01 (October 2017): "JSON Hyper-Schema: A Vocabulary for Hypermedia Annotation of JSON.

[18]	IETF RFC 7950: "The YANG 1.1 Data Modeling Language, August 2016".

Next change

#### 6.2.1.a	YANG Naming

All YANG schema nodes and identifiers that are a direct mapping from the stage 2 specifications (including leafs, leaf-list, containers, lists, enumerations, enums, typedefs) shall have the exact same name as used in stage 2 definitions except if

- stage 2 name violates the allowed naming rules of the YANG language as defined in RFC7950 [18] section 6.2.

- specified otherwise in this document

Next change

### 6.2.10	Attribute – simple, single value

#### 6.2.10.1	Introduction

Reference TS 32.156 [3] clause 5.2.1

The multiplicity of the attribute is either 0..1 or 1..1. .

#### 6.2.10.2	YANG Mapping

Non-structured single value attributes are mapped to a "leaf".

// attribute single value, nonstructured

leaf myAttribute { type xxx; }

Next change

### 6.2.12	Attribute, structured

#### 6.2.12.0	Introduction

Reference TS 32.156 [3] clause 5.2.1

#### 6.2.12.1	YANG Mapping

Structured attributes are mapped to a grouping containing member parts; and a list using the grouping. (Structured attributes that are not used in multiple places may define the member parts directly in the list.)

// attribute, structured, isUnique=true OR isWritable=false

grouping pLMNIdGrp {

description "PLMN-Id= Mobile Country Codes (MCC) &

Mobile Network Codes(MNC)";

leaf MCC {

type t_mcc;

}

leaf MNC {

type t_mnc;

}

}

list pLMNIdList {

key "MCC MNC";

config true;

description "a list of PLMN-Ids";

ordered-by user;

uses pLMNIdGrp;

}

// attribute, structured, isUnique=false, isWritable=true

list pLMNIdList {

key "idx";

leaf idx { type uint32 ; };

leaf member1 { type xxx ; }

leaf member2 { type yyy ; }

}

YANG keys for the list shall be selected according to the following steps:

- If the attribute is isUnique=true and according to the descriptions of the sub-attributes, one or a combination of some subattributes are unique, and all these subattributes are mandatory, these subattribute(s) should be used as key(s) in YANG. (Note only mandatory subattributes should be considered for keys as declaring a subattribute a key makes it mandatory in YANG.)

- If suitable key(s) cannot be found in step 1 and the attribute is isWritable=true, an additional dummy index shall be defined in YANG. The name of the dummyIndex shall be “idx” and shall have a type uint32 or uint64. The dummy key “idx” usually does not appear on stage 2.

- If suitable cannot be found in step 1 and the attribute (and all its member parts) is isWritable=false, the YANG "key" statement should not be used. YANG allows defining ReadOnly lists without a key. Defining keys is allowed following the principles of the next paragraph.

### 6.2.13	defaultValue

#### 6.2.13.1	Introduction

Reference TS 32.156 [3] clause 5.2.1.1.

The 3GPP/UML defaultValue has a different meaning then the YANG "default" statement.

The 3GPP defaultValue could be considered an initialValue as it has effect only at object creation. If the attribute is later deleted the 3GPP defaultValue has no effect. In YANG the "default" is always used whenever a leaf/leaf-list does not have a value: both at creation of the parent object and if the leaf/leaf-list is deleted (set to null in 3GPP operation).

The 3GPP defaultValue, isNullable and multiplicity properties cannot be mapped one-to-one into YANG statements. A combination of these three stage 2 input properties shall result in a combination of the four YANG statements mandatory, min-elements, default,and yext3gpp:initial-value (defined in the YANG module _3gpp-common-yang-extensions.yang). The table below describes the combinations of input properties and the resulting YANG statements.

| Stage 2 properties |  |  | YANG mapping |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| multiplicity | isNullable | Stage-2 default | Yang mandatory | YANG Min-Elements > 0 | YANG default | YANG initial-value |
| 0..1 | True | none | N | N | N | N |
|  |  | defined | N | N | N | Y |
|  | False | none | N | N | N | N |
|  |  | defined | N | N | N | Y |
| 1 | True | none | N | N | N | N |
|  |  | defined | N | N | N | Y |
|  | False | none | Y/N | Y/N | N | N |
|  |  | defined | N | N | Y | N |
| 0..* | True | none | N | N | N | N |
|  |  | defined | N | N | N | Y |
|  | False | none | N | N | N | N |
|  |  | defined | N | N | N | Y |
| x..* x >= 1 | True | none | N | N | N | N |
|  |  | defined | N | N | N | Y |
|  | False | none | N | Y | N | N |
|  |  | defined | N | N | Y | N |

YANG mandatory indicates that the leaf shall have a “mandatory true;” substatement.

YANG min-elements > 0 indicates that the list or leaf-list shall have a “min-elements” substatement that has an argument that is greater than zero.

YANG default indicates that the leaf shall have a “default” substatement.

YANG initial-value indicates that the leaf should have a “yext3gpp:initial-value” substatement.

#### 6.2.13.2	YANG mapping

YANG “default” and “initial-value” statements are only used for simple attributes. For structured attributes describe the default in the YANG description. In some cases, the stage 2 default value is not defined as a specific value, but rather as a reference or defined in a human readable language. In these cases, the default value is described in the YANG description.

YANG default or yext3gpp:initial-value statements shall be used as specified in the table in clause 6.2.13.1.

Note 1: 	The YANG extension statement yext3gpp:initial-value is not understood or enforced by standard YANG tools, they need extra SW implementation.

### 6.2.14	multiplicity and cardinality

#### 6.2.14.0	Introduction

Reference TS 32.156 [3] clause 5.2.1.1

Reference TS 32.156 [3] clause 5.2.8

#### 6.2.14.1	YANG mapping

YANG mandatory, or min-elements statements shall be used as specified in the table in clause 6.2.13.1.

Multiplicity of attributes mapped to a list or leaf-list shall be mapped to the "min-elements" and "max-elements" YANG statements.

Cardinality for containment of classes shall be mapped to "min-elements" and "max-elements" on the list representing the child objects.

Cardinality for reference relationships shall be mapped to "mandatory”,  "min-elements" and "max-elements" on the reference attributes representing the reference.

### 6.2.15	isNullable

#### 6.2.15.0	Introduction

Reference TS 32.156 [3] clause 5.2.1.1

#### 6.2.15.1	YANG mapping

isNullable=false for attributes is not mapped to YANG. In this case the attribute’s multiplicity will dictate any YANG mandatory or min-elements statements. See table in clause 6.2.13.1.

isNullable=true shall not be mapped to YANG, because isNullable=true makes the attribute optional to use, which is the default case in YANG, thus it should not be explicitly stated.

A special case is an attribute that is

mapped to a list or leaf-lists, is isNullable=true and has a minimum multiplicity greater than zero. In this case a "must" statement shall be added to the list/leaf-list forbidding any multiplicity values between 1 and the minimum multiplicity (but allowing zero and the minimum). See example below:

list nullableListWithMinimumMultiplicityOf5 {

key idx;

must ‘count(.) = 0 or count(.) >= 5’;

leaf idx { type uint32 ; }

leaf nonUniqueSingleValueAttribute [ type int32; };

}

Next change

### 6.2.16	dataType

#### 6.2.16.0	Introduction

Reference TS 32.156 [3] clause 5.3.4

Reference TS 32.156 [3] clause 5.4.3

#### 6.2.16.1	YANG mapping

Mapping for predefined datatypes shall be the following:

-	integer -> One of the 8 YANG integer types

-	string - > string

-	Boolean -> Boolean

3GPP user-defined simple datatypes shall be mapped to the YANG "typedef" statement.

3GPP user-defined structured datatypes shall be mapped to the YANG "grouping" statement with the name <typeName>Grp.

Next change

### 6.2.21	isOrdered

Reference [TS 32.156 [3]  Model repertoire] clause 5.2.1.1

#### 6.2.21.1	YANG mapping

For isWritable=true attributes the property isOrdered=true shall be mapped to the "ordered-by user;"  YANG statement. For isWritable=false attributes the isOrdered property shall be described in the description statement of the YANG leaf-list, list representing the attribute.

Note: 	The “ordered-by user” statement is ignored in YANG if the leaf-list or list is config=false.

End of  changes