3GPP TSG-SA5 Meeting #143-e 	S5-223085

# e-meeting, 9 - 17 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0388 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting response codes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The handling of response codes in the flows doesn’t match what is described in the other clauses |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the unsuccessful and redirect response codes from the flows and adding a description for them instead in the description for the flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different descriptions and figures for the handling of unsuccessful or redirect response codes may lead to misunderstand and incompatibility issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.2.2	Service operations

#### 5.2.2.1	Introduction

The service operations defined for Nchf_ConvergedCharging are shown in table 5.2.2.1-1.

Table 5.2.2.1-1: Nchf_ConvergedCharging Operations

| Service operation name | Description | Initiated by | Corresponding converged charging messages in 3GPP TS 32.290 [58] |
| --- | --- | --- | --- |
| Nchf_ ConvergedCharging_Create | Initial request of unit reservation , and/or initial report of service usage. | CTF | Charging Data Request/Response [Initial] |
|  | One Time request for the service. | CTF | Charging Data Request/Response [Event] |
| Nchf_ ConvergedCharging_Update | Update request for subsequent unit reservation when: - the granted service unit for one rating group are spent - expiry of granted service units validity time - service events occur, which might affect the rating of the current service And/or update report of service usage. | CTF | Charging Data Request/Response [Update] |
| Nchf_ ConvergedCharging_Release | Termination request without any unit reservation, a nd/or last report of service usage. | CTF | Charging Data Request/Response [Termination] |
| Nchf_ ConvergedCharging_Notify | Request that the user be re-authorized, or the charging session context be terminated. | CHF | Charging Notify Request/Response |

#### 5.2.2.2	Nchf_ConvergedCharging_Create Operation

The Nchf_ConvergedCharging_Create service operation provides means for NF Service Consumer (CTF) to request quotas for service delivery or initial report of service usage.

The following procedures using the Nchf_ConvergedCharging_Create service operation are supported:

-	No existing charging data resource.

Figure 5.2.2.2-1: Nchf_ ConvergedCharging_Create Service Operation

1.	NF (CTF) sends a Nchf_ConvergedCharging_Create request to the CHF to create resource for charging. Requested quota and notification URI for Nchf_ConvergedCharging_Notify service operation are included in the request body.

2.	At successful operation, "201 Created" response is returned. In the "201 Created" response, the CHF includes a Location header field and the allocated quota in the body. The Location header field shall contain the URI of the created resource. The CTF shall use the URI received in the Location header in subsequent requests to the CHF for the same charging session.

If errors occur when processing the HTTP request, the CHF shall send an HTTP error response as specified in clause 6.1.7.

If CHF determines that the request needs to be redirection, an HTTP redirect response shall be returned as specified in clause 6.10.9 of 3GPP TS 29.500 [299].

#### 5.2.2.3	Nchf_ConvergedCharging_Update Operation

The Nchf_ConvergedCharging_Update service operation provides means for NF Service Consumer (CTF) to update the charging session.

The following procedures using the Nchf_ConvergedCharging_Update service operation are supported:

-	the granted service units for one rating group are spent

-	expiry of granted service units' validity time

-	charging events occur, which might affect the rating of the current service

-	receiving re-authorization notification from CHF

Figure 5.2.2.3-1: Nchf_ConvergedCharging_Update Service Operation

1.	NF (CTF) sends a Nchf_ConvergedCharging_Update request to the CHF. The {ChargingDataRef } in the URI identifies the "Charging Data" to be updated. The requested service unit and previous used service unit is included in the request body.

2.	At successful operation, "200 OK" response is returned. The CHF includes the granted service unit in the "200 OK" response.

If errors occur when processing the HTTP request, the CHF shall send an HTTP error response as specified in clause 6.1.7.

If CHF determines that the request needs to be redirection, an HTTP redirect response shall be returned as specified in clause 6.10.9 of 3GPP TS 29.500 [299].

#### 5.2.2.4	Nchf_ConvergedCharging_Release Operation

The Nchf_ConvergedCharging_Release service operation provides means for NF Service Consumer (CTF) to terminate the charging session.

The following procedures using the Nchf_ConvergedCharging_Release service operation are supported:

-	Expiry of unit count inactivity timer in CTF.

-	Abort notification is received from CHF.

-	Service termination in CTF.

Figure 5.2.2.4-1: Nchf_ConvergedCharging_Release Service Operation

1.	NF(CTF) sends a Nchf_ConvergedCharging_Release request to the CHF. The {ChargingDataRef } in the URI identifies the "Charging Data" to be updated and then released. The final used service unit is included in the request body.

2.	At successful operation, "204 No Content" response is returned.

If errors occur when processing the HTTP request, the CHF shall send an HTTP error response as specified in clause 6.1.7.

If CHF determines that the request needs to be redirection, an HTTP redirect response shall be returned as specified in clause 6.10.9 of 3GPP TS 29.500 [299].

#### 5.2.2.5	Nchf_ConvergedCharging_Notify Operation

The Nchf_ConvergedCharging_Notify service operation provides means for CHF to notify the NF Service Consumer (CTF), to update or terminate the charging session.

The following procedures using the Nchf_ConvergedCharging_Notify service operation are supported:

-	CHF determines re-authorization.

-	CHF determines abort of charging.

Figure 5.2.2.5-1: Nchf_ConvergedCharging_Notify Service Operation

1.	The CHF sends a Nchf_ConvergedCharging_Notify request to the CTF. The {notifyUri} identifies the notification URI which is sent in the Nchf_ConvergedCharging_Create request. The notification type is included in the request body.

2a.	At successful operation, "204 No Content" response is returned.

2b.	At successful operation with information, "200 OK" response is returned. The CTF may include the invocation result in the "200 OK" response.

After successful operation, when the CTF receives a Charging Notify Request while not waiting for any Charging Data Response from the CHF, CTF can send a new Charging Data Request.

If errors occur when processing the HTTP request, the CTF shall send an HTTP error response as specified in clause 6.1.7.

If CTF determines that the request needs to be redirection, an HTTP redirect response shall be returned as specified in clause 6.10.9 of 3GPP TS 29.500 [299].

End of changes