3GPP TSG-SA5 Meeting #143-e 	S5-223335

# e-meeting, 9 - 17 May 2022

# Source:	asiaInfo, Huawei

# Title:	add Service Support Expectation in clause 7.2.2 stage3

# Document for:	Approval

Agenda Item:	6.6.3

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v1.1.0”.

3	Rationale

This contribution proposes to add Service Support Expectation in clause 7.2.2 stage3, because other scenario specific expectation is TBD.

4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

7	Stage 3 definition for Intent Driven Management

7.2.2	OpenAPI document "TS28312_intentNrm.yaml"

openapi: 3.0.1

info:

title: Intent NRM

description: OAS 3.0.1 specification of the Intent NRM

version: 17.0.0

paths: {}

components:

schemas:

Intent-Single:

allOf:

- $ref: 'TS28623_genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

userLabel:

type: string

intentExpectations:

type: array

items:

$ref: "#/components/schemas/IntentExpectation"

intentContexts:

type: array

items:

$ref: "#/components/schemas/IntentContext"

intentFulfilmentInfo:

$ref: "#/components/schemas/IntentFulfilmentInfo"

IntentExpectation:

type: object

properties:

expectationId:

type: string

expectionVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/ExpectationObject"

expectationTargets:

type: array

items:

$ref: "#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

$ref: "#/components/schemas/ExpectationContext"

ExpectationObject:

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwrok  #value for Radio Network Expectation--#

- Service_Support  #value for Service Support Expectation--#

- TBD    #-This will be added based on defined scenario specfic intent expectation-#

objectInstance:

$ref: "TS28623_comDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

$ref: "#/components/schemas/ObjectContext"

ExpectationTarget:

oneOf:

#--ExpectationTargets defind for Radio Network Expectation (ObjectType is RAN_SubNetwork)- #

- $ref: "#/components/schemas/WeakRSRPRatioTarget"

- $ref: "#/components/schemas/LowSINRRatioTarget"

- $ref: "#/components/schemas/AveULRANUEThptTarget"

- $ref: "#/components/schemas/AveDLRANUEThptTarget"

- $ref: "#/components/schemas/LowULRANUEThptRatioTarget"

- $ref: "#/components/schemas/LowDLRANUEThptRatioTarget"

#--ExpectationTargets defind for Radio Network Expectation (ObjectType is RAN_SubNetwork)- #

#--ExpectationTargets defind for Service Support Expectation (ObjectType is ServiceSupport)- #

- $ref: "#/components/schemas/ DlThptPerUETarget"

- $ref: "#/components/schemas/ UlThptPerUETarget"

- $ref: "#/components/schemas/ DlLatencyTarget"

- $ref: "#/components/schemas/ UlLatencyTarget"

- $ref: "#/components/schemas/MaxNumberofUEsTarget"

- $ref: "#/components/schemas/ActivityFactorTarget"

- $ref: "#/components/schemas/UESpeedTarget"

#--ExpectationTargets defind for Service Support Expectation (ObjectType is ServiceSupport)- #

#--ExpectationTargets defind for other Scenario specific Expectation-----TBD---------------#

#--ExpectationTargets defind for other Scenario specific Expectation-----TBD---------------#

ObjectContext:

oneOf:

#--ObjectContext defind for Radio Network Expectation (ObjectType is RAN_SubNetwork)-------#

- $ref: "#/components/schemas/CoverageAreaPolygonContext"

- $ref: "#/components/schemas/CoverageTACContext"

- $ref: "#/components/schemas/PLMNContext"

- $ref: "#/components/schemas/NRFqBandContext"

- $ref: "#/components/schemas/RATContext"

#--ObjectContext defind for Radio Network Expectation (ObjectType is RAN_SubNetwork)-------#

#--ObjectContext defind for Service Support Expectation (ObjectType is ServiceSupport)-------#

- $ref: "#/components/schemas/EdgeIdenfiticationIdContext"

- $ref: "#/components/schemas/EdgeIdenfiticationLocContext"

- $ref: "#/components/schemas/CoverageAreaTAContext"

#--ObjectContext defind for Service Support Expectation (ObjectType is ServiceSupport)-------#

#--ObjectContext defind for other Scenario specific Expectation -----------TBD------------#

#--ObjectContext defind for other Scenario specific Expectation -----------TBD------------#

ExpectationContext:

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

oneOf:

#--ExpectationContext defind for Service Support Expectation (ObjectType is ServiceSupport)- #

- $ref: "#/components/schemas/ServiceStartTimeContext"

- $ref: "#/components/schemas/ServiceEndTimeContext"

- $ref: "#/components/schemas/UEMobilityLevelContext"

- $ref: "#/components/schemas/ResourceSharingLevelContext"

#--ExpectationContext defind for Service Support Expectation (ObjectType is ServiceSupport)- #

IntentContext:

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

Condition:

type: string

enum:

- Is_equal_to

- Is_less_than

- Is_greater_than

- Is_within_the_range

IntentFulfilStatus:

type: string

enum:

- FULFILLED

- NOT_FULFILLED

#-------Definition of the concrete ExpectationTarget dataType----------#

WeakRSRPRatioTarget:

type: object

properties:

targetName:

type: string

enum:

- WeakRSRPRatio

targetCondition:

type: string

enum:

- Is_less_than

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/WeakRSRPContext"

WeakRSRPContext:

type: object

properties:

contextAttribute:

type: string

enum:

- WeakRSRPThreshold

contextCondition:

type: string

enum:

- Is_less_than

contextValueRange:

type: number

LowSINRRatioTarget:

type: object

properties:

targetName:

type: string

enum:

- LowSINRRatio

targetCondition:

type: string

enum:

- Is_less_than

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowSINRContext"

LowSINRContext:

type: object

properties:

contextAttribute:

type: string

enum:

- LowSINRThreshold

contextCondition:

type: string

enum:

- Is_less_than

contextValueRange:

type: integer

AveULRANUEThptTarget:

type: object

properties:

targetName:

type: string

enum:

- AveULRANUEThpt

targetCondition:

type: string

enum:

- Is_greater_than

targetValueRange:

type: integer

AveDLRANUEThptTarget:

type: object

properties:

targetName:

type: string

enum:

- AveDLRANUEThpt

targetCondition:

type: string

enum:

- Is_greater_than

targetValueRange:

type: integer

LowULRANUEThptRatioTarget:

type: object

properties:

targetName:

type: string

enum:

- LowULRANUEThptRatio

targetCondition:

type: string

enum:

- Is_less_than

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowULRANUEThptContext"

LowULRANUEThptContext:

type: object

properties:

contextAttribute:

type: string

enum:

- LowULRANUEThptThreshold

contextCondition:

type: string

enum:

- Is_less_than

contextValueRange:

type: number

LowDLRANUEThptRatioTarget:

type: object

properties:

targetName:

type: string

enum:

- LowDLRANUEThptRatio

targetCondition:

type: string

enum:

- Is_less_than

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowDLRANUEThptContext"

LowDLRANUEThptContext:

type: object

properties:

contextAttribute:

type: string

enum:

- LowDLRANUEThptThreshold

contextCondition:

type: string

enum:

- Is_less_than

contextValueRange:

type: number

DlThptPerUETarget:

type: object

properties:

targetName:

type: string

enum:

- DlThptPerUE

targetCondition:

type: string

enum:

- Is_greater_than

targetValueRange:

$ref: "sliceNrm.yaml#/components/schemas/DlThptPerUE”

UlThptPerUETarget:

type: object

properties:

targetName:

type: string

enum:

- UlThptPerUE

targetCondition:

type: string

enum:

- Is_greater_than

targetValueRange:

$ref: "sliceNrm.yaml#/components/schemas/UlThptPerUE”

DlLatencyTarget:

type: object

properties:

targetName:

type: string

enum:

- DlLatency

targetCondition:

type: string

enum:

- Is_less_than

targetValueRange:

$ref: "sliceNrm.yaml#/components/schemas/DlLatency”

UlLatencyTarget:

type: object

properties:

targetName:

type: string

enum:

- UlLatency

targetCondition:

type: string

enum:

- Is_less_than

targetValueRange:

$ref: "sliceNrm.yaml#/components/schemas/UlLatency”

MaxNumberofUEsTarget:

type:object

properties:

targetAttribute:

type: string

enum:

- maxNumberofUEs

targetCondition:

type: string

enum:

- Is_less _than

targetValueRange:

type: array

items:

$ref: ”sliceNrm.yaml#/components/schemas/MaxNumberofUEs”

ActivityFactorTarget:

type:object

properties:

targetAttribute:

type: string

enum:

- activityFactor

targetCondition:

type: string

enum:

- Is_equal _than

targetValueRange:

type: array

items:

$ref: ”sliceNrm.yaml #/components/schemas/ActivityFactor”

UESpeedTarget:

type:object

properties:

targetAttribute:

type: string

enum:

- uESpeed

targetCondition:

type: string

enum:

- Is_less _than

targetValueRange:

type: array

items:

$ref: ”sliceNrm.yaml #/components/schemas/UESpeed”

#-------Definition of the concrete ExpectationTarget  dataType----------#

#-------Definition of the concrete ObjectTarget dataType----------------#

CoverageAreaPolygonContext:

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaPolygon

contextCondition:

type: string

enum:

- Is_within_the_range

contextValueRange:

type: array

items:

$ref: "#/components/schemas/CoverageArea"

CoverageArea:

type: string

CoverageTACContext:

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaTac

contextCondition:

type: string

enum:

- Is_within_the_range

contextValueRange:

type: array

items:

$ref: "nrNrm.yaml#/components/schemas/NrTac"

PLMNContext:

type: object

properties:

contextAttribute:

type: string

enum:

- PLMN

contextCondition:

type: string

enum:

- Is_within_the_range

contextValueRange:

type: array

items:

$ref: "nrNrm.yaml#/components/schemas/PlmnId"

NRFqBandContext:

type: object

properties:

contextAttribute:

type: string

enum:

- NRFqBand

contextCondition:

type: string

enum:

- Is_within_the_range

contextValueRange:

type: array

items:

type: string

RATContext:

type: object

properties:

contextAttribute:

type: string

enum:

- RAT

contextCondition:

type: string

enum:

- Is_within_the_range

contextValueRange:

type: array

items:

type: string

enum:

- UTRAN

- EUTRAN

- NR

EdgeIdenfiticationIdContext:

type:object

properties:

contextAttribute:

type:string

enum:

- edgeIdentificationId

contextCondition:

type: string

enum:

- Is_equal_than

contextValueRange:

type: array

items:

$ref: "edgeNrm.yaml#/components/schemas/EDNidentifier"

EdgeIdenfiticationLocContext:

type:object

properties:

contextAttribute:

type:string

enum:

- edgeIdentificationTarget

contextCondition:

type: string

enum:

- Is_equal_than

contextValueRange:

type: array

items:

$ref: "edgeNrm.yaml#/components/schemas/GeoLoc "

CoverageAreaTAContext:

type:object

properties:

contextAttribute:

type:string

enum:

- coverageAreaTA

contextCondition:

type: string

enum:

- Is_within_the_range

contextValueRange:

type: array

items:

$ref: ”sliceNrm.yaml#/components/schemas/CoverageAreaTAList”

#-------Definition of the concrete ObjectTarget dataType----------------#

#-------Definition of the concrete ExpectionContext dataType----------------#

ServiceStartTimeContext

type:object

properties:

contextAttribute:

type:string

enum:

- serviceStartTime

contextCondition:

type: string

enum:

- Is_equal_than

contextValueRange:

type: array

items:

$ref: "#/components/schemas/startTimeStamp"

ServiceEndTimeContext

type:object

properties:

contextAttribute:

type:string

enum:

- serviceEndTime

contextCondition:

type: string

enum:

- Is_equal_than

contextValueRange:

type: array

items:

$ref: "#/components/schemas/endTimeStamp"

UEMobilityLevelContext:

type:object

properties:

contextAttribute:

type:string

enum:

- ueMobilityLevel

contextCondition:

type: string

enum:

- Is_within_the_range

contextValueRange:

type: array

items:

$ref: ”sliceNrm.yaml#/components/schemas/UEMobilityLevel”

ResourceSharingLevelContext:

type:object

properties:

contextAttribute:

type:string

enum:

- resourceSharingLevel

contextCondition:

type: string

enum:

- Is_within_the_range

contextValueRange:

type: array

items:

$ref: ”sliceNrm.yaml#/components/schemas/UEMobilityLevel”

#-------Definition of the concrete ExpectionContext dataType----------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

Intent-Multiple:

type: array

items:

$ref: '#/components/schemas/Intent-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

resources-intentNrm:

oneOf:

- $ref: '#/components/schemas/Intent-Single'

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

End of Changes