3GPP TSG-SA5 Meeting #143-e	S5-223514

# Online, , 9th May 2022 - 17th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0217 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.532 Clarify usage of notifyMOIChanges for NRMs defined in YANG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | REST_SS, TEI17 |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The notification "notifyMOIChanges" needs some clarifications when used together with a YANG defined NRM to enable interoperability.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications for "notifyMOIChanges" when used with a YANG defined NRM are provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different implementations are possible resulting in interoperability issuers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1.4.1a.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

###### 12.1.1.4.1a.4	Type MoiChange

Table 12.1.1.4.1a.4 -1: Definition of type MoiChange

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| path | Uri | URI specifying the created, deleted or updated resource | M |
| operation | Operation | Operation associated to the reported change ("CREATE", "DELETE", "REPLACE") | M |
| value | oneOf(AttributeNameValuePairSet, AttributeValueChangeSet) | or reporting resource creation or deletion, the optional resource representation (MOC attributes only). In this case, the data type of value is "AttributeNameValuePairSet". For reporting attribute value changes, the (mandatory) new values and (optional) old values. In this case, the data type of value is "AttributeValueChangeSet". | M |

For a "CREATE" or "DELETE" operation only the host and path components are present in the URI carried by the "path" attribute of "MoiChange". They identify the created or deleted resource. The "value" attribute of "MoiChange" may optionally carry the MOC attribute name value pairs of the created or deleted resource in the format of a map. The keys of the map are equal to the MOC attribute names, and the values are equal to the MOC attribute values.

For a "REPLACE" operation, two cases need to be distinguished.

In the first case, one or more value changes of complete MOC attributes are reported. Only the host and path components are present in the URI carried by the "path" attribute of "MoiChanges". They identify the resource, where attribute value changes occured. The "value" attribute is an array of minimum one and maximum two items. If only one array item is present, it carries the MOC attribute names that changed value and their new values. If the optional second array item is present as well, it carries the MOC attribute names that changed value and their old values. The order of items in the array carries semantics and shall therefore not be reversed.

In the second case, a single value change of an attribute part (sub-attribute) is reported. Here the URI needs to carry, besides the host and path components, also the fragment component. The host and path components identify the resource, where the attribute part value change occured. The fragment component identifies the attribute part inside the resource. The URI fragment is specified using JSON pointer in the URI fragment identifier representation as defined in clause 6 of of RFC 6901 [48]. The context for JSON pointer is the updated resource. The "value" is an array of minimum one and maximum two items. If only one item is present, it carries the name of the attribute part that changed value and its new value. If the optional second array item is present as well, it carries the name of the attribute part that changed value and its old value. Hence also in this case, the order of items in the array carries semantics and shall not be reversed.

For example, the following instance of a "moiChanges" array item reports an object creation

notificationId: 123456789

path: 'https://example.com/3GPP/ClassA=1'

operation: CREATE

value:

attrA:

subAttrA1: ABC

subAttrA2: 56

attrB: XYZ

attrC: 123

or, when omitting the optional attribute name vale pairs of the created object, the instance looks like

notificationId: 123456789

path: 'https://nokia.com/3GPP/ClassA=1'

operation: CREATE

The following instance reports a change of the attributes "attrA" and " attrC" with new and old values

notificationId: 123456789

path: 'https://example.com/3GPP/ClassA=1'

operation: REPLACE

value:

- attrA:

subAttrA1: ABC

subAttrA2: 56

attrC: 123

- attrA:

subAttrA1: DEF

subAttrA2: 67

attrC: 345

and the following with new values only

notificationId: 123456789

path: 'https://example.com/3GPP/ClassA=1'

operation: REPLACE

value:

- attrA:

subAttrA1: ABC

subAttrA2: 56

attrC: 123

When a change of the attribute part "attrA:subAttrA1" shall be reported, the instance looks like

notificationId: 123456789

path: 'https://example.com/3GPP/ClassA=1?attributes/attrA/subAttrA1'

operation: REPLACE

value:

- subAttrA1: ABC

- subAttrA1: DEF

or, with the new value only, like

notificationId: 123456789

path: 'https://example.com/3GPP/ClassA=1?attributes/attrA/subAttrA1'

operation: REPLACE

value:

- subAttrA1: ABC

When NRM updates from a YANG defined NRM are reported the following rules apply:

- The RESTCONF resource identifier syntax (RFC 8040 [??], section 3.5.3) shall be used to construct the "href" and “path” values instead of the 3GPP JSON Patch syntax (3GPP TS 32.158 [??], clause 6.4.3).

- The value of "href" shall start with the resource identifier of the root resource of the NRM.

- The value of "value" shall follow the JSON encoding of YANG defined in to RFC 7951 [x].

The difference between the RESTCONF resource identifier and the 3GPP JSON Patch syntax is the following

- The module name is prepended to the node name where the module is defined.

- Multi-value attribute elements are not identified based on their index (loaction). Multi-value attribute elements of scalar types are identified using the element value. Multi-value attribute elements of complex types are identified using keys, that are specified in the YANG definition of the NRM. If no key is dedined, a Multi-value attribute element cannot be identified.

For example, the following example reports the creation of a resource

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /moduleA:ClassA=1

value:

id: 1,

objectClass: moduleA:ClassA,

attributes:

attrA: 123

attrB:

subAttrB1: ABC

subAttrB2: 56

Assume "attrD" is a multi-value attribute. The elements are of type integer with the current value [11, 21,31]. The following notification reports the replacement of the element "21" with "22".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /moduleA:ClassA=1#/attributes/attrD=21

value: 22

Starting from [11, 22, 31] now, this notification reports the addition of element "12" before element "22".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /moduleA:ClassA=1#/attributes/attrD=22

value: 12

The result is [11, 12, 22, 31].

Now assume "attrD" is a multi-value attribute. The elements are of complex type with the current value

[{"subAttrD1": 11, "subAttrD2": "ABC"}, {"subAttrD1": 21, "subAttrD2": "DEF"}, {"subAttrD1": 31, "subAttrD2": "GHI"}

The key is "subAttrD1". Then the following notification reports the element identified by the key "21" changed value; the value of "subAttrD2" was replaced with "AABC"

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /moduleA:ClassA=1#/attributes/attrD=21

value:

subAttrD1: 21

subAttrD2: AABC

End of modifications