| 3GPP TR 28.828 V0.2.0 (2022-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Study on charging aspects for enhanced support of non-public networks (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	8

4.1	General	8

4.2	Non-public networks functionality and architecture	9

4.2.1	Stand-alone Non-Public Network (SNPN)	9

4.2.2	Public Network Integrated NPN (PNI-NPN)	11

4.3	Charging modes	11

5	Charging scenarios and key issues for SNPN	12

5.1	Topic 1: Converged charging for data connectivity in SNPN	12

5.1.1	Use cases	12

5.1.1.1	Use case #1a: End users access the SNPN service directly	12

5.1.1.2	Use case #1b: End users access the SNPN via PLMN	12

5.2.1.1	Use case #1c: Access the SNPN services via PLMN	12

5.1.2	Potential charging requirements	12

5.1.3	Key issues	12

5.1.4	Possible solutions	13

5.1.5	Evaluation	13

5.1.6	Conclusion	13

5.2	Topic 2: Converged charging for access connection in SNPN	13

5.2.1	Use cases	13

5.2.1.1	Use case #2a: Network access control	13

5.2.2	Potential charging requirements	13

5.2.3	Key issues	13

5.2.4	Possible solutions	14

5.2.5	Evaluation	14

5.2.6	Conclusion	14

6	Charging scenarios and key issues for PNI-NPN	14

6.1	Topic 1: Converged charging for access connection in PNI-NPN	14

6.1.1	Use cases	14

6.1.1.1	Use case #1a: Network access control	14

6.1.2	Potential charging requirements	15

6.1.3	Key issues	15

6.1.4	Possible solutions	15

6.1.5	Evaluation	15

6.1.6	Conclusion	15

7	Conclusions and recommendations	15

Annex <A>: Change history	16

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document studies the charging aspects for enhanced support of non-public networks on the TS 23.501 [2], TS 23.502 [3], TS 23.503 [4], TS 23.228 [5] and TS 23.167 [6] incorporating conclusions from 3GPP TR 23.734 [7] and TR 23.700-07 [8].

The following is studied:

- Charging scenarios and potential charging requirements for Non-Public Networks (i.e. Stand-alone Non-Public Network (SNPN) or Public Network Integrated NPN (PNI-NPN)).

- Possible charging solutions for Non-Public Networks with potential impact on charging architecture, charging functions and charging procedures.

- End user charging and Inter-Provider charging by considering the different business roles.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[7]	3GPP TR 23.734: "Study on enhancement of 5G System (5GS) for vertical and Local Area Network (LAN) services".

[8]	3GPP TR 23.700-07: "Study on enhanced support of Non-Public Networks (NPN)".

[9]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

4.1	General

In SA1, Non-Public Networks service requirements are documented in TS 22.261[9]. In SA2, there are two study items and two work items which are related to Non-Public Networks. The corresponding studies are documented in TR 23.734 [7] and TR 23.700-07 [8], respectively and the normative work is documented in TS 23.501 [2], TS 23.502 [3], TS 23.503 [4], TS 23.228 [5] and TS 23.167 [6].

A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [9]. As described in TS 23.501 [2], an NPN is either:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN, or

-	a Public Network Integrated NPN (PNI-NPN), i.e. a non-public network deployed with the support of a PLMN.

Functionality and architecture of Stand-alone NPN are described in clause 4.2.1 and functionality and architecture of Public Network Integrated NPNs are described in clause 4.2.2.

4.2	Non-public networks functionality and architecture

4.2.1	Stand-alone Non-Public Network (SNPN)

SNPN 5GS deployments are based on the architecture depicted in clause 4.2.3 TS 23.501[2], the architecture for 5GC with untrusted non-3GPP access (Figure 4.2.1-1) for access to SNPN services via a PLMN (and vice versa).

Figure 4.2.1-1: Non-roaming architecture for 5G Core Network with untrusted non-3GPP access

To access SNPN services, a UE that has successfully registered with a PLMN over 3GPP access may perform another registration via the PLMN User Plane with an SNPN (using the credentials of that SNPN). In order to obtain access to Non-Public Network services when the UE is camping in NG-RAN of a PLMN, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the Stand-alone Non-Public Network. The architecture for access to SNPN services via PLMN is depicted in Figure 4.2.1-2.

Figure 4.2.1-2: Access to Stand-alone Non-Public Network services via PLMN

To access PLMN services, a UE in SNPN access mode that has successfully registered with an SNPN may perform another registration via the SNPN User Plane with a PLMN (using the credentials of that PLMN). In order to obtain access to PLMN services when the UE is camping in NG-RAN of Stand-alone Non-Public Network, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the PLMN. The architecture for access to PLMN services via SNPN is depicted in Figure 4.2.1-3.

Figure 4.2.1-3: Access to PLMN services via Stand-alone Non-Public Network

4.2.2	Public Network Integrated NPN (PNI-NPN)

Public Network Integrated NPNs are NPNs made available via PLMNs e.g. by means of dedicated DNNs, or by one (or more) Network Slice instances allocated for the NPN. When a PNI-NPN is made available via a PLMN, then the UE shall have a subscription for the PLMN in order to access PNI-NPN.

A Closed Access Group (CAG) identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG. CAG is used for the PNI-NPNs to prevent UE(s), which are not allowed to access the NPN via the associated cell(s), from automatically selecting and accessing the associated CAG cell(s).

4.3	Charging modes

As per TS 23.501 [2], in the context of Non-Public Networks, responsibilities regarding operations have to be clearly defined and assigned to roles. NPN charging can be built upon the roles related to NPN management specified in the clause 4.2 of TS 28.557 [X] without the consideration on the network equipment and infrastructure. Non-public networks in 5G environment involves the services or capabilities provided by multiple service providers in the form of following business roles:

- NPN Operator (NPN-OP): a Network Operator (NOP) is limited to 5G networks for non-public use, i.e. NPNs.

- NPN Service Provider (NPN-SP): a Communication Service Provider (CSP) which provides communication services for non-public use, e.g. vertical, tenant, NOP.

- NPN Service Customer (NPN-SC): a Communication Service Customer (CSC) which consumes communication services for non-public use, e.g. end user, vertical, tenant.

Considering the different business models such as B2B, B2C, and B2B2X, charging modes for non-public networks include (but not limited to):

-	End user charging;

-	Inter-provider charging.

Depending on the scenarios an organisation can play one or several roles simultaneously, and apply charging models based on corresponding business relationship, e.g.:

- Charging models for NPN-SP to charge NPN-SC for using NPN services.

- Charging models for NPN-OP to charge NPN-SP for using their networks and related network services.

- When NPN-OP and NPN-SP role are provided by the same enterprise, to charge NPN-SC role for using their networks and NPN services.

In deployments, there could be business scenarios where one or more of the business roles are supported by a single enterprise. The present document does not impose any restrictions to the possible deployment scenarios.

5	Charging scenarios and key issues for SNPN

5.1	Topic 1: Converged charging for data connectivity in SNPN

5.1.1	Use cases

5.1.1.1	Use case #1a: End users access the SNPN service directly

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501.

Charged Party: An End user (UE) who accessed SNPN services.

Charging Party: SNPN operator who charges the End user for using SNPN services.

5.1.1.2	Use case #1b: End users access the SNPN via PLMN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501, the architecture for 5GC with untrusted non-3GPP access for access to SNPN services via a PLMN (and vice versa).

Charged Party: An End user (UE) who has a subscription with the PLMN which allows access of the SNPN service.

Charging Party:

-	SNPN operator who charges the End user for using SNPN services

-	PLMN operator who charges the 5G data connectivity for the End user.

5.2.1.1	Use case #1c: Access the SNPN services via PLMN

The scenario description is the same with the use case #1b.

Charging Party: PLMN operator

5.1.2	Potential charging requirements

REQ-CH_SNPN_E-01: The SNPN should support collecting charging information of SNPN services usage per UE.

REQ-CH_SNPN_E-02: The 5G system should support collecting charging information based on the 5G data connectivity usage per UE for SNPN services

REQ-CH_SNPN_E-03: The 5G system should support collecting charging information based on the 5G data connectivity usage per SNPN services.

5.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: the charging information collection and reporting in SNPN for services usage per End User.

-	Key Issue #1b: the charging information collection and reporting in 5G system for 5G data connectivity usage for SNPN services per end user.

-	Key Issue #1c: the charging information collection and reporting in PLMN for 5G data connectivity usage per SNPN service.

5.1.4	Possible solutions

5.1.5	Evaluation

5.1.6	Conclusion

5.2	Topic 2: Converged charging for access connection in SNPN

5.2.1	Use cases

5.2.1.1	Use case #2a: Network access control

The combination of a PLMN ID and Network identifier (NID) identifies an SNPN. If a UE performs the registration or service request procedure in an SNPN identified by a PLMN ID and a NID and there is no subscription for the UE, then AMF rejects the UE with a cause code to prevent the UE from automatically selecting and registering with the same SNPN as described in TS 23.501 [2].

The NPN-OP provides the SNPN (e.g. vertical industry customers) to NPN-SP, and NPN-SP provides NPN services to NPN-SC. It is important for NPN-OP to collect usage for SNPN resources associated to access connection for the purpose of charging or statistics.

The potential charging requirements for this UC are: REQ-eNPN_CH_SNPN_NAC-01, REQ-eNPN_CH_SNPN_NAC-02.

5.2.2	Potential charging requirements

REQ-eNPN_CH_SNPN_NAC-01: The 5G system should support converged charging per UE for SNPN network access and usage.

REQ-eNPN_CH_SNPN_NAC-02: The 5G system should support converged charging per vertical industry consumer for SNPN network access and usage.

5.2.3	Key issues

This key issue is for investigating how to support converged charging for access connection in SNPN considering REQ-eNPN_CH_SNPN_NAC-01 and REQ-eNPN_CH_SNPN_NAC-02. This investigation covers the following:

-	Key Issue #2a: Identification of the charging information for converged charging for SNPN network access and usage;

-	Key Issue #2b: Identification of the main interactions with the NFs to obtain the charging information;

-	Key Issue #2c: The charging mechanism supporting converged inter-provider charging for SNPN network access and usage.

5.2.4	Possible solutions

TBD

5.2.5	Evaluation

TBD

5.2.6	Conclusion

TBD

6	Charging scenarios and key issues for PNI-NPN

6.1	Topic 1: Converged charging for access connection in PNI-NPN

6.1.1	Use cases

6.1.1.1	Use case #1a: Network access control

Relying on PLMN network, PNI-NPN provides logically independent 5G non-public network for vertical industry customers through network slice instances/ dedicated DNNs, and supporting of CAG feature. CAG is used for access control to prevent unauthorized UEs from trying to access a PNI-NPN.

The NPN-OP provides the PNI-NPN (e.g. vertical industry customers) to NPN-SP, and NPN-SP provides NPN services to NPN-SC. It is important for NPN-OP to collect usage for PNI-NPN resources associated to access connection for the purpose of charging or statistics.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_NAC-01, REQ-eNPN_CH_PNI-NPN_NAC-02.

6.1.2	Potential charging requirements

REQ-eNPN_CH_PNI-NPN_NAC-01: The 5G system should support converged charging per UE for PNI-NPN network access and usage.

REQ-eNPN_CH_PNI-NPN_NAC-02: The 5G system should support converged charging per vertical industry consumer for PNI-NPN network access and usage.

6.1.3	Key issues

This key issue is for investigating how to support converged charging for access connection in PNI-NPN considering REQ-eNPN_CH_PNI-NPN_NAC-01 and REQ-eNPN_CH_PNI-NPN_NAC-02. This investigation covers the following:

-	Key Issue #1a: Identification of the charging information for converged charging for PNI-NPN network access and usage;

-	Key Issue #1b: Identification of the main interactions with the NFs to obtain the charging information;

-	Key Issue #1c: The charging mechanism supporting converged inter-provider charging for PNI-NPN network access and usage.

6.1.4	Possible solutions

TBD

6.1.5	Evaluation

TBD

6.1.6	Conclusion

TBD

7	Conclusions and recommendations

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222381 |  |  |  | Initial skeleton | 0.0.0 |
| 2022-04 | SA5#142e | S5-222395 S5-222392 S5-222394 |  |  |  | Skeleton update Add scope Add reference | 0.1.0 |
| 2022-05 | SA5#143e | S5-223082 S5-223083 S5-223709 S5-223706 S5-223708 S5-223710 |  |  |  | Introduction of general background Introduction of non-public networks functionality and architecture Introduction of charging modes Add the use case for SNPN Add charging scenarios and key issues for SNPN on converged charging for access connection Add charging scenarios and key issues for PNI-NPN on converged charging for access connection | 0.2.0 |