| 3GPP TR 28.833 V0.2.0 (2022-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Study on Management Aspect of 5GLAN; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4 	Concepts and background	7

5	Use cases and potential requirements	8

5. 1	Use case 1: 5G VN group data management	8

5.1.1	Description	8

5.1.2	Potential requirements	8

5.2	Use case 2: PDU Session management	8

5.2.1	Description	8

5.2.2	Potential requirements	9

5.3	Use case 3: for 5G VN group performance measurement management	9

5.3.1	Description	9

5.3.2	Potential requirements	9

5.4	Use case for 5G VN group end-to-end network KPIS m anagement	9

5.4.1	Description	9

5.4.2	Potential requirements	9

6	Key Issues	9

6.X	Key Issue X	10

6.X.1	Description	10

7	Potential Solutions	10

7.Y	Potential Solution Y	10

7.Y.1  	Description	10

8	Conclusion and recommendation	10

Annex <X> (informative): Change history	10

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document studies the potential use cases, requirements, and enhancement forof the management of system (e.g. performance measurement and related new KPIs in VN group level ) to support 5G-LAN capabilities defined by TS 23.501 [2]. The document provides conclusions and recommendations on the next steps in standardization.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]             SP-220324 " New Study on Management Aspects of 5GLAN "

[2]             3GPP TS 23.501 "System architecture for the 5G System (5GS)".

[3]             3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[4]             3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4 	Concepts and background

## 4.1	General description

As defined in TS 23.501 [2], A 5G Virtual Network (VN) group consists of a set of UEs using private communication for 5G LAN-type services. The service requirements for 5G LAN-type service are specified in TS 22.261 [3].

5G VN Group is a set of UEs using private communication for 5G LAN-type service.

## 4.2	User plane architecture to support 5G LAN-type Services

There are two types of user plane architecture to allow 5G LAN-type services:

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G VN group. Figure 4.2-2 depicts the non-roaming user plane architecture to support 5G LAN-type service using local switch.

Figure 4.2-1: Local-switch based user plane architecture in non-roaming scenario

-	N19-based, where the UL/DL traffic for the 5G VN group communication is forwarded between PSA UPFs of different PDU sessions via N19. N19 is a reference point between two UPFs for direct routing of traffic between different PDU Sessions without using N6. It has a per 5G VN group granularity. Figure 4.2-2 depicts the non-roaming user plane architecture to support 5G LAN-type service using N19 tunnel.

Figure 4.2-2: N19-based user plane architecture in non-roaming scenario

## 4.3	5G VN group management

5G System supports management of 5G VN Group identification and membership (i.e. definition of 5G VN group identifiers and membership) and 5G VN Group data (i.e. definition of 5G VN group data). The 5G VN Group management can be configured by a network administrator or can be managed dynamically by AF.

A 5G VN group is characterized by the following:

-	5G VN group identities: External Group ID and Internal Group ID are used to identify the 5G VN group.

-	5G VN group membership: The 5G VN group members are uniquely identified by GPSI. The group as described in clause 5.2.3.3.1 of TS 23.502 [4] is applicable to 5G LAN-type services.

-	5G VN group data. The 5G VN group data may include the following parameters: PDU session type, DNN, S-NSSAI and Application descriptor, Information related with secondary authentication / authorization (e.g. to enable IP address assignment by the DN-AAA).

The Information related with secondary authentication / authorization corresponds to the procedures described in clause 5.6.6; it allows e.g. the AF to provide DN-AAA server addressing information and possibly to request the SMF to get the UE IP address from the DN-AAA server.

In order to support dynamic management of 5G VN Group identification and membership, the NEF exposes a set of services to manage (e.g. add/delete/modify) 5G VN groups and 5G VN members. The NEF also exposes services to dynamically manage 5G VN group data.

The support of 5G VN group management configured by a network administrator (OAM-based 5G VN group management) is described in clause 5.25.2 of TS 23.501 [6].

5	Use cases and potential requirements

5. 1	Use case 1: 5G VN group data management

5.1.1	Description

5G LAN-type services are provided for the customers who request services with similar functionalities to Local Area Networks (LANs) and VPN’s but improved with 5G capabilities (e.g. high performance, long distance access, mobility and security). To support 5G LAN-type services, 3GPP management system may need to support the management of 5G VN group (e.g. configuration of service area where 5G VN group communication is applicable to the UEs).

As defined in TS 23.501 [8], the 5G VN group configuration is either provided by OAM or provided by an AF to the NEF. It means the 5G VN group parameters about a UE may be configured in subscription data of the UE and delivered together with other subscription data by the UDM for supporting 5G LAN service. For the configuration for 5G VN group data management, the 3GPP management system can support the 5G VN group data management,

including:

- The configuration for 5G VN group data creation based on External Group ID.

- The configuration for 5G VN group data modification based on External Group ID.

- The configuration for 5G VN group data deletion based on External Group ID.

5.1.2	Potential requirements

REQ-LAN-GDM-01 The 3GPP management system should have the capability to manage the 5G VN group data creation.

REQ-LAN-GDM-02 The 3GPP management system should have the capability to manage the 5G VN group data modification.

REQ-LAN-GDM-03 The 3GPP management system should have the capability to manage the 5G VN group data deletion.

5.2	Use case 2: PDU Session management

5.2.1	Description

The session management is applicable to 5G LAN-type services with clarification and enhancement. For example, SMF may support PDU Sessions for a 5G VN group, which offers a virtual data network capable of supporting 5G LAN-type service over the 5G system. The whole PDU sessions belong to the 5G VN group need to select the same SMF based on network configuration. But how to configure to select the same SMF based on network configuration has not defined yet. The 3GPP management system should have the capability to manage the PDU Session for providing 5G LAN-type services, including:

-	A UE gets access to 5G LAN-type services via a PDU Session of IP PDU Session type or Ethernet PDU Session type.

-	A PDU Session provides access to one and only one 5G VN group.

-	A dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G VN group.

-	A PDU Session provides unicast, broadcast and multicast communication for the DNN and S-NSSAI associated to a 5G VN group.

5.2.2	Potential requirements

REQ-LAN-PDU-01 The 3GPP management system should have the capability to manage the PDU Session for providing 5G LAN-type services.

5.3	Use case 3: for 5G VN group performance measurement management

5.3.1	Description

The performance of 5G LAN-type services need to be monitored in 5G VN group level by the operator since it is relevant to whether the end users can use the service of 5G LAN and scale up/down a 5G VN based on capacity for efficient consumption of network resources.

The performance measurement of 5G LAN-type services may include measurement of 5G VN group status, the number of subscribers in 5G VN groups, the duration of 5G VN group communication which information can be gathered from NEF/UDM/SMF/UPF/PCF supporting 5G LAN-type services.

Therefore, the 3GPP management system needs to support the request for 5G VN group performance measurement management as it impacts the end user experience, and the optimization may be required according to the performance.

5.3.2	Potential requirements

REQ-LAN-PMM-01 The 3GPP management system shall have the capability to measure 5G LAN-type services performance (e.g. UE throughput, Performance measurements for NEF/UDM/SMF/UPF/PCF) to be assured in 5G VN group level.

5.4	Use case 4: for 5G VN group end-to-end network KPIS m anagement

5.4.1	Description

It is necessary to evaluate the end-to-end network KPIS in 5G VN group level to evaluate the performance of 5G LAN-type services for provided consistence of group UE experience.

The KPIs as followed can be used to describe some performance requirement for 5G LAN-type services, for examples:

- The 5G VN group status can be calculated by group member registration/de-registration success rate of 5G VN group.

- The duration of 5G VN group communication can be counted by the sum of duration of individual PDU sessions within the 5G VN group communication.

- The 5G VN group communication could be identified by DNN/S-NSSAI by the SMF.

5.4.2	Potential requirements

REQ-LAN-KPIs-01 The 3GPP management system should have the capability to measure end-to-end network KPIS in 5G VN group level to evaluate the performance of 5G LAN-type services.

# 6	Key Issues

# Editor's note: this clause will be used to document the key issues based on the use cased which defined in clause 5.

### 6.X	Key Issue X

### 6.X.1	Description

7	Potential Solutions

Editor's note: this clause will be used to document the potential enhancement of 5G network management to support the key issues which defined in clause 6.

7.Y	Potential Solution Y

Editor's Note:	This clause describes briefly the potential solution at a high-level.

7.Y.1  	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

8	Conclusion and recommendation

Editor's note: this clause will be used to document the conclusions and recommendation of the study.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 05-2022 | SA5#143e | S5-223594 S5-223595 S5-223596 S5-223597 S5-223598 S5-223599 |  |  |  | Update to implement the agreed pCRs in SA5#143e: - S5-223594 pCR TR 28.833 Change TR structure - S5-223595 Add use case1 for 5G VN group data management of FS_5GLAN_Mgt - S5-223596 Add use case2 for PDU Session management of FS_5GLAN_Mgt - S5-223597 Add use case3 for user plane handing of FS_5GLAN_Mgt - S5-223598 Add use case4 for 5G VN performance measurement management of FS_5GLAN_Mgt - S5-223599 Add use case5 for 5G VN group end-to-end network KPIS management of FS_5GLAN_Mgt | 0.2.0 |