| 3GPP TS 28.105 V1.2.0 (2022-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	 Concepts and overview	8

4.1	Overview	8

5	 AI/ML management functionality and service framework	9

5.1	Functionality and service framework for AI/ML training	9

5.1	Functionality and service framework for AI/ML training	9

6	AI/ML management use cases and requirements	10

6.1	General	10

6.2	AI/ML training	10

6.2.1	Description	10

6.2.2	Use cases	11

6.2.2.1	AI/ML training requested by consumer	11

6.2.2.2	AI/ML training initiated by producer	11

6.2.2.3	Selecting AI/ML models and AI/ML-enabled Functions	12

6.2.2.4	Managing AI/ML Training Processes	12

6.2.2.5	Handling errors in data and ML decisions	12

6.2.3	Requirements for AI/ML training	13

7	Information model definitions for AI/ML management	15

7.1	Imported and associated information entities	15

7.1.1	Imported information entities and local labels	15

7.1.2	Associated information entities and local labels	15

7.2	Class diagram	15

7.2.1	Relationships	15

7.2.2	Inheritance	16

7.3	Class definitions	16

7.3.1	AIMLTrainingFunction	16

7.3.1.1	Definition	16

7.3.1.2	Attributes	16

7.3.1.3	Attribute constraints	16

7.3.1.4	Notifications	16

7.3.2	AIMLTrainingRequest	17

7.3.2.1	Definition	17

7.3.2.2	Attributes	18

7.3.2.3	Attribute constraints	18

7.3.2.4	Notifications	18

7.3.4	AIMLTrainingReport	18

7.3.4.1	Definition	18

7.3.4.2	Attributes	18

7.3.4.3	Attribute constraints	19

7.3.4.4	Notifications	19

7.3.5	AIMLTrainingProcess	19

7.3.5.1	Definition	19

7.3.5.3	Attribute constraints	20

7.3.5.4	Notifications	20

7.4	Data type definitions	21

7.4.1	ModelPerformance <<dataType>>	21

7.4.1.1	Definition	21

7.4.1.2	Attributes	21

7.4.1.3	Attribute constraints	21

7.4.1.4	Notifications	21

7.4.2	AIMLEntity <<dataType>>	21

7.4.2.1	Definition	21

7.4.2.2	Attributes	21

7.4.3.3	Attribute constraints	21

7.4.3.4	Notifications	22

7.4.3	AIMLContext <<dataType>>	22

7.4.3.1	Definition	22

7.4.3.2	Attributes	22

7.4.3.3	Attribute constraints	22

7.4.1.4	Notifications	22

7.5	Attribute definitions	22

7.5.1	Attribute properties	22

7.5.2	Constraints	26

7.6	Common notifications	26

7.6.1	Configuration notifications	26

8	Service components	27

8.1	Service components for AI/ML model training MnS	27

9	Solution Set (SS)	27

Annex A (informative):  PlantUML source code for NRM class diagrams	27

A.1	General	27

A.2	PlantUML code for Figure 7.2.1-1: NRM fragment for AI/ML model training	27

A.3	PlantUML code for Figure 7.2.2-1: Inheritance Hierarchy for AI/ML model training related NRMs	28

Annex B (normative): OpenAPI definition of the AI/ML NRM	28

B.1	General	28

B.2	Solution Set (SS) definitions	29

B.2.1	OpenAPI document "AiMlNrm.yaml"	29

Annex X (informative): Change history	33

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document specifies the Artificial Intelligence / Machine Learning (AI/ML) management capabilities and services for 5GS where AI/ML is used, including management and orchestration (e.g., MDA, see TS 28.104 [2]) and 5G networks (e.g., NWDAF, see TS 23.288 [3]).

This document also describes the functionality and service framework for AI/ML management.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[6]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[8]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[9]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[10]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire".

[14]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

AI/ML model: a mathematical algorithm that can be “trained” by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

AI/ML entity: any entity that is either an AI/ML model or contains an AI/ML model and that can be managed as a single composite entity.

AI/ML model training: the capabilities of an AI/ML Training Function to take data, run it through an AI/ML model, derive the associated loss and adjust the parameterization of that AI/ML model based on the computed loss.

AI/ML training: the capabilities and associated end-to-end processes to enable an AI/ML Training Function to train its constituent AI/ML model, e.g., to interact with external parties to collect and format the data required for training the AI/ML model.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SBMA	Service Based Management Architecture

# 4		Concepts and overview

## 4.1	Overview

The AI/ML techniques and relevant applications are being increasingly adopted by the wider industries and proved to be successful. These are now being applied to telecommunication industry including mobile networks.

Although AI/ML techniques in general are quite mature nowadays, some of the relevant aspects of the technology are still evolving while  new complementary techniques are frequently emerging.

The AI/ML techniques can be generally characterized from different perspectives including the followings:

-	Learning methods

The learning methods include supervised learning, unsupervised learning and reinforcement learning. Each learning method fits one or more specific category of inference (e.g., prediction), and requires specific type of training data. A brief comparison of these learning methods is provided in table 4.2-1.

Table 4.2-1: Comparison of Learning methods

|  | Supervised learning | Semi-supervised learning | Unsupervised learning | Reinforcement learning |
| --- | --- | --- | --- | --- |
| Category of inference | Regression (numeric), classification | Regression (numeric), classification | Association, Clustering | Reward-based behaviour |
| Type of training data | Labelled data (Note 1) | Labelled data (Note 1), and unlabelled data | Unlabelled data | Not pre-defined |
| NOTE 1: The labelled data means the input and output parameters are explicitly labelled for each training data example. |  |  |  |  |

-	Learning complexity

As per the learning complexity, there are Machine Learning (i.e., basic learning) and Deep Learning.

-	Learning architecture

Based on the topology and location where the learning tasks take place, the AI/ML can be categorized to centralized learning, distributed learning and federated learning.

-	Learning continuity

From learning continuity perspective, the AI/ML can be offline learning or continual learning.

Artificial Intelligence/Machine Learning (AI/ML) capabilities are used in various domains in 5GS, including management and orchestration (e.g., MDA, see TS 28.104 [2]) and 5G networks (e.g., NWDAF, see 23.288 [3]).

The AI/ML-enabled function in the 5GS uses the AI/ML model for inference.

Each AI/ML technique, depending on the adopted specific characteristics as mentioned above, may be suitable for supporting certain type/category of use case(s) in 5GS.

To enable and facilitate the AI/ML capabilities with the suitable AI/ML techniques in 5GS, the AI/ML model and AI/ML-enabled function (i.e., inference function) need to be managed.

The present document specifies the AI/ML management related capabilities and services, which include the followings:

-	AL/ML training.

# 5		AI/ML management functionality and service framework

## 5.1	Functionality and service framework for AI/ML training

## 5.1	Functionality and service framework for AI/ML training

An AI/ML training Function playing the role of AI/ML training MnS producer, may consume various data for AI/ML training purpose.

As illustrated in Figure 5.1-1 the AI/ML training capability is provided via AI/ML training MnS in the context of SBMA to the authorized consumer(s) by AI/ML training MnS producer.

Figure 5.1-1: Functional overview and service framework for AI/ML model training

The internal business logic of AI/ML training leverages the current and historical relevant data, including those listed below to monitor the networks and/or services where relevant to the AI/ML model, prepare the data, trigger and conduct the training:

-	Performance Measurements (PM) as per TS 28.552 [4], TS 32.425 [5] and Key Performance Indicators (KPIs) as per TS 28.554 [6].

-	Trace/MDT/RLF/RCEF data, as per TS 32.422 [7] and TS 32.423 [8].

-	QoE and service experience data as per TS 28.405 [9] and TS 28.406 [10].

-	Analytics data offered by NWDAF as per TS 23.288 [3].

-	Alarm information and notifications as per TS 28.532 [11].

-	CM information and notifications.

-	MDA reports from MDA MnS producers as per TS 28.104 [2].

-	Management data from non-3GPP systems.

-	Other data that can be used for training.

# 6	AI/ML management use cases and requirements

## 6.1	General

The use cases and requirements for AI/ML management are specified in the following sub-clauses.

## 6.2	AI/ML training

### 6.2.1	Description

In operational environment before the AI/ML Entity (i.e., Inference Function) is deployed to conduct inference, it needs to be trained (e.g., by a separate or an external entity to the Inference function).

The AI/ML Entity is trained by the AI/ML training (AIMLT) MnS producer, and the training can be triggered by request(s) from one or more AIMLT MnS consumer(s), or initiated by the AIMLT MnS producer (e.g., as result of model evaluation).

### 6.2.2	Use cases

#### 6.2.2.1	AI/ML training requested by consumer

The AI/ML training capabilities are provided by an AIMLT MnS producer to one or more consumer(s).

Figure 6.6.2.1-1: AI/ML training requested by AIMLT MnS consumer

The AI/ML training may be triggered by the request(s) from one or more AIMLT MnS consumer(s). To trigger an AI/ML training, the AIMLT MnS consumer requests the AIMLT MnS producer to train the AI/ML model or AI/ML enabled function. In the AI/ML training request, the consumer should specify the inference type which indicates the function or purpose of the AI/ML Entity, e.g., CoverageProblemAnalysis. The AIMLT MnS producer can perform the training according to the designated inference type. The consumer may provide the data source(s) that contain(s) the training data which are considered as inputs candidates for training. To obtain the valid training outcomes, consumers may also designate their requirements for model performance (e.g., accuracy, etc) in the training request.

The AIMLT MnS producer provides a response to the consumer indicating whether the request was accepted.

If the request is accepted, the AIMLT MnS producer decides when to start the AI/ML training with consideration of the request(s) from the consumer(s). Once the training is decided, the producer performs the followings,

-	selects the training data, with consideration of the consumer provided candidate training data. Since the training data directly influences the algorithm and performance of the trained AI/ML Entity, the AIMLT MnS producer may examine the consumer’s provided training data and decide to select none, some or all of them. In addition, the AIMLT MnS producer may select some other training data that are available.

-	trains the AI/ML Entity using the selected training data, and

-	provides the training results (including the location of the trained AI/ML Entity, etc.) to the AIMLT MnS consumer(s).

#### 6.2.2.2	AI/ML training initiated by producer

The AI/ML training may be initiated by the AIMLT MnS producer, for instance as a result of  performance evaluation of the AI/ML model, based on feedback or new training data received from the consumer, or when new training data which are not from the consumer describing the new network status/events become available.

When the AIMLT MnS producer decides to start the AI/ML training, the producer performs the followings,

-	selects the training data,

-	trains the AI/ML Entity using the selected training data, and

-	provides the training results (including the location of the trained AI/ML Entity, etc) to the AIMLT MnS consumer(s) who have subscribed to receive the AI/ML training results.

#### 6.2.2.3	Selecting AI/ML models and AI/ML-enabled Functions

For a given machine learning-based use case, different entities that apply the respective ML model or AI/ML enabled function may have different inference requirements and capabilities. For example, one consumer with specific responsibility and wish to have an AI/ML enabled function trained for city central business district where mobile users move at speeds not exceeding 30 km/hr. On the other hand, another consumer for the same use case may support a rural environment and as such wish to have a model fitting that environment. The different consumers need to know the available versions of AI/ML enabled functions and to select the appropriate AI/ML enabled function for their respective conditions.

Besides there is no guarantee that the available AI/ML enabled functions have been trained according to the characteristics that the consumers expect. As such the consumers need to know the conditions for which the models or AI/ML enabled functions have been trained to then enable the consumers to select the models that are best fitted to their conditions.

The models that have been trained may differ in terms of complexity and performance. For example, a generic comprehensive and complex model may have been trained in a cloud-like environment but when such a model cannot be used in the gNB and instead, a less complex model, trained as a derivative of this generic model, could be a better candidate. Moreover, multiple less complex models could be trained with different level of complexity and performance which would then allow different relevant  models to be delivered to different network functions depending on operating conditions and performance requirements. The network functions need to know the alternative models available and interactively request and replace them when needed and depending on the observed inference-related constraints and performance.

#### 6.2.2.4	Managing AI/ML Training Processes

This machine learning capability relates to means for managing and controlling AI/ML training processes.

To achieve the desired outcomes of any machine learning relevant use-case, the AI/ML Model applied for such analytics and decision making, needs to be trained with the appropriate network data. The training may be undertaken in managed function or in a management function.

In either case, the network (or the OAM system thereof) not only needs to have the required training capabilities but needs to also have the means to manage the training of the AI/ML models and or AI/ML-enabled functions. The consumers need to be able to interact with the training process, e.g., to suspend or restart the process; and also need to manage and control the requests related to any such training process.

#### 6.2.2.5	Handling errors in data and ML decisions

Traditionally, the machine-learning-enabled Functions (e.g., AIML Entity 1 and AIML Entity) are trained on good quality data, i.e., data that was collected when the network was working correctly, to represent the expected context in which the AIML Entity is meant to operate. Good quality data is void of errors, such as:

-	Imprecise measurements, with added noise (such as RSRP, SINR, or QoE estimations).

-	Missing values or entire records, e.g., because of communication link failures.

-	Records which are communicated with a significant delay (in case of online measurements).

Without errors, an AIML Entity can depend on a few precise inputs, and don’t need to exploit the redundancy present in the training data. However, during inference, the AIML Entity is very likely to come across these inconsistencies. When this happens, the AIML Entity shows high error in the inference outputs, even if redundant and uncorrupted data is available from other sources.

AIML Entity1

AIML Entity2

Network Resources

ML Consumer

p

KPIs

Error

p

Figure 6.2.2.5-1: The propagation of erroneous information

As such the system needs to account for errors and inconsistencies in the input data and how the consumers of ML decisions should deal with decisions that are made based on such erroneous and inconsistent data. The system should: 1) enable functions to undertake the training in a way that prepares the AIML Entity s to deal with the errors, i.e., to identify the errors in the data during training; and 2) enable the ML consumers to account for the possibility of erroneous input data into the ML decision makers.

### 6.2.3	Requirements for AI/ML training

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_TRAIN-FUN-01 | The AIMLT MnS producer shall have a capability allowing the consumer to request AI/ML training. | AI/ML training requested by consumer (clause 6.2.2.1) |
| REQ- AIML_TRAIN-FUN-02 | The AIMLT MnS producer shall have a capability allowing the consumer to specify the data sources containing the candidate training data for AI/ML training. | AI/ML training requested by consumer (clause 6.2.2.1) |
| REQ- AIML_TRAIN-FUN-03 | The AIMLT MnS producer shall have a capability allowing the consumer to specify the inference type of the AI/ML Entity to be trained. | AI/ML training requested by consumer (clause 6.2.2.1) |
| REQ- AIML_TRAIN-FUN-04 | The AIMLT MnS producer shall have a capability to provide the training result (including the location of the trained AI/ML Entity) to the consumer. | AI/ML training requested by consumer (clause 6.2.2.1), and AI/ML training initiated by producer (clause 6.2.2.2) |
| REQ-AIML_SELECT-01 | 3GPP management system shall have the capability for authorized consumer to discover the characteristics of available models including the contexts under which each of the models was trained. | Selecting AI/ML models and AI/ML-enabled Functions (clause 6.2.2.3) |
| REQ-AIML_SELECT-02 | 3GPP management system shall have the capability to enable an authorized consumer to select an AI/ML model. | Selecting AI/ML models and AI/ML-enabled Functions (clause 6.2.2.3) |
| REQ-AIML_SELECT-03 | 3GPP management system shall have the capability to enable an authorized consumer to request for a model to be trained to satisfy the consumer's expectations. | Selecting AI/ML models and AI/ML-enabled Functions (clause 6.2.2.3) |
| REQ-AIML_SELECT-04 | 3GPP management system shall have the capability to enable an authorized consumer to request for information and be informed about the available alternative models of differing complexity and performance. | Selecting AI/ML models and AI/ML-enabled Functions (clause 6.2.2.3) |
| REQ-AIML_SELECT- -05 | 3GPP management system shall have the capability to enable an authorized consumer to request one of the known or available alternative models of differing complexity and performance to be used for inference. | Selecting AI/ML models and AI/ML-enabled Functions (clause 6.2.2.3) |
| REQ-AIML_SELECT-06 | The 3GPP management system shall have a capability to provide a selected AI/ML enabled function to the consumer. | Selecting AI/ML models and AI/ML-enabled Functions (clause 6.2.2.3) |
| REQ-AIML_TRAIN- MGT_01 | 3GPP management system shall have the capability to enable an authorized consumer to manage and configure one or more requests for the training of specific AI/ML models or AI/ML enabled functions, e.g., to modify the characteristics of the request or to delete a request. | Managing AI/ML Training Processes (clause 6.2.2.4) |
| REQ-AIML_TRAIN- MGT_02 | 3GPP management system shall have the capability to enable an authorized consumer to manage and configure one or more training processes, e.g., to start, suspend or restart the training; or to adjust the training conditions and/or characteristics. | Managing AI/ML Training Processes (clause 6.2.2.4) |
| REQ-AIML_TRAIN- MGT_03 | 3GPP management system shall have the capability to enable an authorized consumer (e.g., the function/entity different from the function that generated a request for AI/ML enabled function training) to request for a report on the outcomes of a specific training instance. | Managing AI/ML Training Processes (clause 6.2.2.4) |
| REQ-AIML_TRAIN- MGT_04 | 3GPP management system shall have the capability to enable an authorized consumer to define the reporting characteristics related to a specific training request or training instance. | Managing AI/ML Training Processes (clause 6.2.2.4) |
| REQ-AIML_TRAIN- MGT_05 | 3GPP management system shall have the capability to enable the AI/ML Training function to report to any authorized consumer about specific ML Training process and/or report about the outcomes of any such ML Training process. | Managing AI/ML Training Processes (clause 6.2.2.4) |
| REQ-ML_ERROR_01 | The 3GPP management system shall enable an authorized consumer of data services (e.g., an ML-enabled function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR_02 | The 3GPP management system shall enable an authorized consumer of ML decisions (e.g., a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the ML-based function. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR_03 | The 3GPP management system shall enable a producer of data services (e.g., a gNB) to provide to an authorized consumer (e.g., an ML-enabled function) a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR_04 | The 3GPP management system shall enable a producer of ML decisions (e.g., an ML-enabled function) to provide to an authorized consumer of ML decisions (e.g., a controller) an ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the ML-based function. | Handling errors in data and ML decisions (clause 6.2.2.5) |

# 7	Information model definitions for AI/ML management

## 7.1	Imported and associated information entities

### 7.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [12], IOC, Top | Top |
| TS 28.622 [12], IOC, SubNetwork | SubNetwork |
| TS 28.622 [12], IOC, ManagedElement | ManagedElement |
| TS 28.622 [12], IOC, ManagedFunction | ManagedFunction |

### 7.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |

## 7.2	Class diagram

### 7.2.1	Relationships

This clause depicts the set of classes (e.g., IOCs) that encapsulates the information relevant to AI/ML model training. For the UML semantics, see 3GPP TS 32.156 [13].

Figure 7.2.1-1: NRM fragment for AI/ML model training

### 7.2.2	Inheritance

Figure 7.2.2-1: Inheritance Hierarchy for AI/ML model training related NRMs

## 7.3	Class definitions

### 7.3.1	AIMLTrainingFunction

#### 7.3.1.1	Definition

The IOC AIMLTrainingFunction represents the entity that undertakes AI/ML training and is also the container of the AIMLTrainingRequest IOC(s).

The entity represented by AIMLTrainingFunction MOI supports training of one or more AIMLEntity(s).

#### 7.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLEntityList | M | T | F | F | F |

#### 7.3.1.3	Attribute constraints

None.

#### 7.3.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

### 7.3.2	AIMLTrainingRequest

#### 7.3.2.1	Definition

The IOC AIMLTrainingRequest represents the AI/ML model training request that is created by the AI/ML training MnS consumer.

The AIMLTrainingRequest MOI is contained under one AIMLTrainingFunction MOI. Each AIMLTrainingRequest is associated to at least one AIMLEntity.

The AIMLTrainingRequest may have a source to identify where it is coming from, and which may be used to prioritize the training resources for different sources. The sources may be for example the network functions, operator roles, or other functional differentiations.

Each AIMLTrainingRequest may indicate the expectedRunTimeContext that describes the specific conditions for which the AIMLEntity (either AIML Model or AIML-enabled function) should be trained for.

In case the request is accepted, the AI/ML training MnS producer decides when to start the AI/ML training. Once the MnS producer decides to start the training based on the request, the AI/ML training MnS producer instantiates one or more AI/MLTrainingProcess MOI(s) that are responsible to perform the followings:

-	collects (more) data for training, if the training data are not available or the data are available but not sufficient for the training,

-	prepares and selects the training data, with consideration of the consumer provided candidate training data if any. The AI/ML training MnS producer may examine the consumer’s provided candidate training data and select none, some or all of them for training. In addition, the AI/ML training MnS producer may select some other training data that are available,

-	trains the AIMLEntity using the selected and prepared training data.

The AIMLTrainingRequest may have a requestStatus field to represent the status of the specific AIMLTrainingRequest:

-	The attribute values are "NOT_STARTED", "TRAINING_IN_PROGRESS", "SUSPENDED", "FINISHED", and "CANCELLED".

-	When value turns to "TrainingInProcess", the AI/ML training MnS producer instantiates one or more AIMLTrainingProcess MOI(s) representing the training process(es) being performed per the request and notifies the MnS consumer(s) who subscribed to the notification.

When all of the training process associated to this request are completed, the value turns to "FINISHED.

#### 7.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLEntityId | M | T | T | F | T |
| candidateTraingDataSource | O | T | T | F | T |
| traingDataQualityScore | O | T | T | F | T |
| trainingRequestSource | M | T | T | F | T |
| requestStatus | M | T | T | F | T |
| expectedRuntimeContext | O | T | T | F | T |
| performanceRequirements | M | T | T | F | T |
| cancelRequest | O | T | T | F | T |
| suspendRequest | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |

#### 7.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |

#### 7.3.2.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

### 7.3.4	AIMLTrainingReport

#### 7.3.4.1	Definition

The IOC AIMLTrainingReport represents the AI/ML model training report that is provided by the training MnS producer.

The AIMLTrainingReport MOI is contained under one AIMLTrainingFunction MOI.

#### 7.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLEntityId | M | T | F | F | T |
| areConsumerTrainingDataUsed | M | T | F | F | T |
| usedConsumerTrainingData | CM | T | F | F | T |
| confidenceIndication | O | T | F | F | T |
| modelPerformanceTraining | CM | T | F | F | T |
| areNewTrainingDataUsed | CO | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingProcessRef | M | T | F | F | T |
| lastTrainingRef | CM | T | F | F | T |

#### 7.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| usedConsumerTrainingData Support Qualifier | Condition: The value of areConsumerTrainingDataUsed attribute is ALL or PARTIALLY. |
| trainingRequestRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was requested by the MnS consumer (via AIMLTrainingRequest MOI). |
| lastTrainingRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was not initial training (i.e., the model has been trained before). |
| modelPerformanceTraining | The condition is use case " AI/ML training initiated by consumer" is supported. |
| areNewTrainingDataUsed | The condition is use case " AI/ML training initiated by producer" is supported. |

#### 7.3.4.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

### 7.3.5	AIMLTrainingProcess

#### 7.3.5.1	Definition

The IOC AIMLTrainingProcess represents the AI/ML training process.

One AIMLTrainingProcess MOI may be instantiated for each AIMLTrainingRequest MOI or a set of AIMLTrainingRequest MOIs.

For each AIMLEntity under training, a AIMLTrainingProcess is instantiated, i.e., an AIMLTrainingProcess is associated with exactly one AIMLEntity. The AIMLTrainingProcess may be associated with one or more AIMLTrainingRequest MOI.

The AIMLTrainingProcess does not have to correspond to a specific AIMLTrainingRequest, i.e., a AIMLTrainingRequest does not have to be associated to a specific AIMLTrainingProcess. The AIMLTrainingProcess may be managed separately from the AIMLTrainingRequest MOIs, e.g., the AIMLTrainingRequest MOI may come from consumers which are network functions while the operator may wish to manage the AIMLTrainingProcess that is instantiated following the requests. Thus, the AIMLTrainingProcess may be associated to either one or more AIMLTrainingRequest MOI.

Each AIMLTrainingProcess instance needs to be managed differently from the related AIMLEntity, although the AIMLTrainingProcess may be associated to only one AIMLEntity. For example, the AIMLTrainingProcess may be triggered to start with a specific version of the AIMLEntity and multiple AIMLTrainingProcess instances may be triggered for different versions of the AIMLEntity. In either case the AIMLTrainingProcesse instances are still associated with the same AIMLEntity but are managed separately from the AIMLEntity

Each AIMLTrainingProcess has a priority that may be used to prioritize the execution of different AIMLTrainingProcesse instances. By default, the priority of the AIMLTrainingProcess may be related in a 1:1 manner with the priority of the AIMLTrainingRequest for which the AIMLTrainingProcess is instantiated.

Each AIMLTrainingProcess may have one or more termination conditions used to define the points at which the AIMLTrainingProcess may terminate.

The "ProgressStatus" attribute represents the status of the AI/ML model training and includes information the MnS consumer can use to monitor the progress and results. The data type of this attribute is "ProcessMonitor" (see TS 28.622 [11]). The following specialisations are provided for this data type for the AI/ML training process:

The "ProgressStatus" attribute represents the status of the AI/ML model training and includes information the AI/ML training MnS consumer can use to monitor the progress and results. The data type of this attribute is "ProcessMonitor" (see TS 28.622 [12]). The following specialisations are provided for this data type for the AI/ML training process:

-	The "status" attribute values are "RUNNING", "CANCELLING", "SUSPENDED", "FINISHED", and "CANCELLED". The other values are not used;

-	The "timer" attribute is not used;

-	When the "status" is equal to "RUNNING" the "progressStateInfo" attribute shall indicate one of the following states: "COLLECTING_DATA", "PREPARING_TRAINING_DATA", "TRAINING".

-	No specifications are provided for the "resultStateInfo" attribute. Vendor specific information may be provided though.

When the training is completed with "status" equal to "FINISHED", the MnS producer provides the training report, by creating an AIMLTrainingReport MOI, to the MnS consumer.

7.3.5.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLTrainingProcessId | M | T | T | F | T |
| priority | M | T | T | F | T |
| terminationConditions | M | T | T | F | T |
| progressStatus | M | T | F | F | T |
| cancelProcess | O | T | T | F | T |
| suspendProcess | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingReportRef | M | T | F | F | T |

#### 7.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| trainingRequestRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was requested by the training MnS consumer (via AIMLTrainingRequest MOI). |

#### 7.3.5.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

## 7.4	Data type definitions

### 7.4.1	ModelPerformance <<dataType>>

#### 7.4.1.1	Definition

This data type specifies the performance of an AI/ML entity when performing inference. The performance score is provided for each inference output.

#### 7.4.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceOutputName | M | T | F | F | T |
| performanceScore | M | T | F | F | T |
| performanceMetric | M | T | F | F | T |
| decisionConfidenceScore | O | T | F | F | T |

#### 7.4.1.3	Attribute constraints

None.

#### 7.4.1.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

### 7.4.2	AIMLEntity <<dataType>>

#### 7.4.2.1	Definition

This data type represents the properties of an AI/ML entity which could be either an AI/ML model or AI/ML-enabled function containing the AI/ML model. AIML training may be requested for either an AI/ML model or AI/ML-enabled function. The algorithm of AI/ML model or AI/ML-enabled function is not to be standardized.

For each AIMLEntity under training, one or more AIMLTrainingProcess are instantiated.

The AIMLEntity may contain 3 types of contexts - TrainingContext which is the context under which the AIMLEntity has been trained, the ExpectedRunTimeContext which is the context where an AIMLEntity is expected to be applied or/and the RunTimeContext which is the context where the model is being applied.

#### 7.4.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLEntityId | M | T | F | F | T |
| inferenceType | M | T | F | F | T |
| aIMLEntityVersion | M | T | F | F | T |
| expectedRunTimeContext | O | T | T | F | T |
| trainingContext | CM | T | F | F | T |
| runTimeContext | O | T | F | F | T |

#### 7.4.3.3	Attribute constraints

None.

| Name | Definition |
| --- | --- |
| trainingContext Support Qualifier | Condition: The trainingContext represents the status and conditions related to training and should be added when training is completed |

#### 7.4.3.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

### 7.4.3	AIMLContext <<dataType>>

#### 7.4.3.1	Definition

The AIMLContext represents the status and conditions related to the AIMLEntity. Specially it may be one of three types of context - the ExpectedRunTimeContext, the  TrainingContext and  the RunTimeContext.

#### 7.4.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| managedEntityRef | M | T | F | F | F |
| dataProviderRef | M | T | F | F | F |

#### 7.4.3.3	Attribute constraints

None.

#### 7.4.1.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aIMLEntityId | It identifies the AI/ML entity. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTraingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the AI/ML model supports. allowedValues: the values of the MDA type (see TS 28.104 [2]), Analytics ID(s) of NWDAF (see TS 23.288 [3]), types of inference for RAN-intelligence, and vendor’s specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the AI/ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the AI/ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related AIMLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingReportRef | It is the DN of the AIMLTrainingReport MOI that represents the reports of the AI/ML training. allowedValues: DN. | type: DN (see TS 32.156 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the AIMLTrainingReport MOI that represents the reports for the last training of the AI/ML model. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication | It indicates the confidence (in unit of percentage) that the AI/ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLEntityList | It describes the list of AIMLEntity. | type: AIMLEntity multiplicity: * isOrdered: False isUnique: N/True defaultValue: None isNullable: False |
| trainingRequestSource | It describes the entity that requested to instantiate the AIMLTrainingRequest MOI. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| requestStatus | It describes the status of a particular AI/ML training request. T. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | It indicates the priority of the training process. The priority may be used by the AI/ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the AIMLTraining to terminate a specific training process. Editor's Note: The specific nature of the termination conditions is FFS allowedValues: FFS. | type: FFS multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the AI/ML training process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLEntityVersion | It indicates the version number of the AI/ML entity. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained AI/ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| performanceTraining | It indicates the performance score of the AI/ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialisation for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "AIMLTrainingProcess". When the AI/ML training is in progress, and the "status" is equal to " RUNNING" it provides the more detailed progress information. allowedValues for "status" = "RUNNING": - COLLECTING_DATA - PREPARING_TRAINING_DATA - TRAINING The allowed values for "status" = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an AI/ML entity. allowedValues: the name of the MDA output IEs (see TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor’s specific extensions. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an AI/ML entity, e.g., “accuracy”, “precision“, “F1 score”, etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an AI/ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of AI/ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the “F1 score”. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cancelRequest | It indicates whether the AI/ML training MnS consumer cancels the AI/ML training request. Setting this attribute to "TRUE" cancels the AI/ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendRequest | It indicates whether the AI/ML training MnS consumer suspends the AI/ML training request. Setting this attribute to "TRUE" suspends the AI/ML training request. Suspension is possible when the requestStatus is the not “FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| cancelProcess | It indicates whether the AI/ML training MnS consumer cancels the AI/ML training process. Setting this attribute to "TRUE" cancels the AI/ML training request. Cancellation is possible when the progressStateInfo is the not “FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendProcess | It indicates whether the AI/ML training MnS consumer suspends the AI/ML training process. Setting this attribute to "TRUE" suspends the AI/ML training request. Suspension is possible when the progressStateInfo is the not “FINISHED", “CANCELLING” or “CANCELLED” state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| managedEntityRef | It describes the entities that the AI/ML entity is responsible for managing or optimizing. | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the AI/ML entity say for training or inference | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the AI/ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e., that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | the numerical value that represents the dependability/quality of a given decision generated by the ML-based function. The lowest value indicates the lowest level of dependability of the decisions, i.e., that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE: when the performanceScore is to indicate the performance score for AI/ML training, the data set is the training data set. |  |  |

### 7.5.2	Constraints

| Name | Affected attribute(s) | Definition |
| --- | --- | --- |

## 7.6	Common notifications

### 7.6.1	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [11], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

# 8	Service components

## 8.1	Service components for AI/ML model training MnS

The components for AI/ML model training MnS are listed in table 8.1-1:

Table 8.1-1: Components for AI/ML model training MnS

| Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- |
| The operations and notifications for generic provisioning management service (see clause 11.1.1 of TS 28.532[11]). | AIMLTrainingFunction IOC; AIMLTrainingRequest IOC; AIMLTrainingReport IOC; AIMLTrainingProcess IOC. | N/A |

# 9	Solution Set (SS)

The present document defines the following NRM Solution Set definitions for AI/ML management:

-	YAML based Solution Set (Annex B).

Annex A (informative): 
PlantUML source code for NRM class diagrams

# A.1	General

This annex contains the PlantUML source code for the NRM diagrams defined in clause 7.2 of the present document.

# A.2	PlantUML code for Figure 7.2.1-1: NRM fragment for AI/ML model training

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class AIMLEntity <<dataType>>

class AIMLTrainingFunction <<InformationObjectClass>>

class AIMLTrainingRequest <<InformationObjectClass>>

class AIMLTrainingReport <<InformationObjectClass>>

class AIMLTrainingProcess <<InformationObjectClass>>

ManagedEntity "1" *-- "*" AIMLTrainingFunction: <<names>>

AIMLTrainingFunction "1" -d-> "*" AIMLEntity

AIMLTrainingFunction "1" *-- "*" AIMLTrainingProcess: <<names>>

AIMLTrainingFunction "1" *-- "*" AIMLTrainingRequest: <<names>>

AIMLTrainingFunction "1" *-- "*" AIMLTrainingReport: <<names>>

AIMLTrainingProcess "1" <-r-> "1" AIMLTrainingReport

AIMLTrainingReport "1" --> "1" AIMLTrainingReport

AIMLTrainingRequest "*" -l-> "1" AIMLEntity

AIMLTrainingRequest "*" -r-> "*" AIMLTrainingProcess

note left of ManagedEntity

Represents the following IOCs:

Subnetwork or

ManagedFunction or

ManagementFunction

end note

@enduml

# A.3	PlantUML code for Figure 7.2.2-1: Inheritance Hierarchy for AI/ML model training related NRMs

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class ManagedFunction <<InformationObjectClass>>

class AIMLTrainingFunction <<InformationObjectClass>>

class AIMLTrainingRequest <<InformationObjectClass>>

class AIMLTrainingProcess <<InformationObjectClass>>

class AIMLTrainingReport <<InformationObjectClass>>

ManagedFunction <|-- AIMLTrainingFunction

Top <|-- AIMLTrainingRequest

Top <|-- AIMLTrainingProcess

Top <|-- AIMLTrainingReport

@enduml

Annex B (normative):
OpenAPI definition of the AI/ML NRM

# B.1	General

This annex contains the OpenAPI definition of the AI/ML NRM in YAML format.

The information models of the AI/ML NRM are defined in clause 7.

Mapping rules to produce the OpenAPI definition based on the information model are defined in TS 32.160 [14].

# B.2	Solution Set (SS) definitions

## B.2.1	OpenAPI document "AiMlNrm.yaml"

openapi: 3.0.1

info:

title: AI/ML NRM

version: 17.0.0

description: >-

OAS 3.0.1 specification of the AI/ML NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.105; AI/ML Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.105/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

AIMLEntityList:

type: array

items:

$ref: '#/components/schemas/AIMLEntity'

AIMLEntity:

type: object

properties:

aIMLEntityId:

type: string

inferenceType:

type: string

aIMLEntityVersion:

type: string

expectedRunTimeContext:

type: string

trainingContext:

type: string

runTimeContext:

type: string

RequestStatus:

type: string

enum:

- NOT_STARTED

- TRAINING_IN_PROGRESS

- SUSPENDED

- FINISHED

- CANCELLED

PerformanceRequirements:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

ModelPerformance:

type: object

properties:

inferenceOutputName:

type: string

performanceMetric:

type: string

performanceScore:

type: number

format: float

decisionConfidenceScore:

type: number

format: float

TrainingProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type defined in “genericNrm.yaml” with specialisations for usage in the "AIMLTrainingProcess".

type: object

properties:

aIMLTrainingProcessId:

type: string

status:

type: string

enum:

- RUNNING

- CANCELLING

- CANCELLED

- SUSPENDED

- FINSHED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

enum:

- COLLECTING_DATA

- PREPARING_TRAINING_DATA

- TRAINING

resultStateInfo:

type: string

#-------- Definition of abstract IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

AIMLTrainingFunction:

$ref: '#/components/schemas/AIMLTrainingFunction-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

AIMLTrainingFunction:

$ref: '#/components/schemas/AIMLTrainingFunction-Multiple'

AIMLTrainingFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

aIMLEntityList:

$ref: '#/components/schemas/AIMLEntityList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

AIMLTrainingRequest:

$ref: '#/components/schemas/AIMLTrainingRequest-Multiple'

AIMLTrainingProcess:

$ref: '#/components/schemas/AIMLTrainingProcess-Multiple'

AIMLTrainingReport:

$ref: '#/components/schemas/AIMLTrainingReport-Multiple'

AIMLTrainingRequest-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLEntityId:

type: string

candidateTraingDataSource:

type: array

items:

type: string

traingDataQualityScore:

type: number

format: float

trainingRequestSource:

type: string

requestStatus:

$ref: '#/components/schemas/RequestStatus'

expectedRuntimeContext:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

performanceRequirements:

$ref: '#/components/schemas/PerformanceRequirements'

cancelRequest:

type: boolean

suspendRequest:

type: boolean

AIMLTrainingProcess-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLTrainingProcessId:

type: string

priority:

type: integer

terminationConditions:

type: string

progressStatus:

$ref: '#/components/schemas/TrainingProcessMonitor'

cancelProcess:

type: boolean

suspendProcess:

type: boolean

trainingRequestRef:

$ref: 'comDefs.yaml#/components/schemas/DnList'

trainingReportRef:

$ref: 'comDefs.yaml#/components/schemas/Dn'

AIMLTrainingReport-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLEntityId:

type: string

areConsumerTrainingDataUsed:

type: boolean

usedConsumerTrainingData:

type: array

items:

type: string

confidenceIndication:

type: integer

modelPerformanceTraining:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

areNewTrainingDataUsed:

type: boolean

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

AIMLTrainingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingFunction-Single'

AIMLTrainingRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingRequest-Single'

AIMLTrainingProcess-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingProcess-Single'

AIMLTrainingReport-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingReport-Single'

#-------- Definitions in TS 28.104 for TS 28.532 ---------------------------------

resources-AiMlNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/AIMLTrainingFunction-Single'

- $ref: '#/components/schemas/AIMLTrainingRequest-Single'

- $ref: '#/components/schemas/AIMLTrainingProcess-Single'

- $ref: '#/components/schemas/AIMLTrainingReport-Single'

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-01 | SA5#141e | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2022-01 | SA5#141e | S5-221616 | - | - | - | Add scope | 0.1.0 |
| 2022-01 | SA5#141e | S5-221617 | - | - | - | Add overview | 0.1.0 |
| 2022-01 | SA5#141e | S5-221618 | - | - | - | Add service framework for AI-ML model training | 0.1.0 |
| 2022-01 | SA5#141e | S5-221335 | - | - | - | Move in ML model training part from TS 28.104 | 0.1.0 |
| 2022-01 | SA5#141e | S5-221614 | - | - | - | Add the requirements for ML model training for MDA | 0.1.0 |
| 2022-02 | SA5#141e | S5-221620 | - | - | - | Add NRMs for AI-ML model training | 0.1.0 |
| 2022-03 | SA#95e | SP-220128 |  |  |  | Presented for information | 1.0.0 |
| 2022-04 | SA5#142e | S5-222688 | - | - | - | Add requirements for AI/ML selection | 1.1.0 |
| 2022-04 | SA5#142e | S5-222689 | - | - | - | Add requirements for control of AI/ML Training | 1.1.0 |
| 2022-04 | SA5#142e | S5-222690 | - | - | - | Add NRM fragments for AI/ML Training | 1.1.0 |
| 2022-04 | SA5#142e | S5-222691 | - | - | - | Support for Training of ML-based functions | 1.1.0 |
| 2022-04 | SA5#142e | S5-222696 | - | - | - | Add AI-ML background | 1.1.0 |
| 2022-05 | SA5#143e | S5-223646 | - | - | - | Add AIML training attribute in case of AIML training initiated by producer | 1.2.0 |
| 2022-05 | SA5#143e | S5-223383 | - | - | - | Correct the attribute definitions of NRMs for AI-ML model training | 1.2.0 |
| 2022-05 | SA5#143e | S5-223474 | - | - | - | Update AI-ML NRM | 1.2.0 |
| 2022-05 | SA5#143e | S5-223487 | - | - | - | Updates to AI/ML NRM | 1.2.0 |
| 2022-05 | SA5#143e | S5-223650 | - | - | - | Rapporteur clean-up | 1.2.0 |
| 2022-05 | SA5#143e | S5-223652 | - | - | - | Add requirements for handling erroneous data & decisions | 1.2.0 |
| 2022-05 | SA5#143e | S5-223745 | - | - | - | Update the definition of performanceScore | 1.2.0 |
| 2022-05 | SA5#143e | S5-223647 | - | - | - | Add stage 3 solution sets for AI-ML NRM | 1.2.0 |