3GPP TSG-SA5 Meeting #143-e 	S5-223364

# e-meeting, 9 - 17 May 2022

# Source:	China Mobile, HUAWEI

# Title:	TS 28.317 Usecases and requirements for ARCF data handling

# Document for:	Approval

Agenda Item:	6.4.1.1

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	SP-211431 New WID on Self-Configuration of RAN Nes

[2]           S5-222726  TS 28.317 v0.1.0

3	Rationale

This document is going to provide proposals on usecases and requirements for ARCF data handling.

4	Detailed proposal

This document proposes the following updates for TS 28.317.

1st Change

- Management capabilities

5.1	ARCF data handling

5.1.1	Use cases

The goal of the ARCF data handling is to transfer the ARCF data to the 3GPP management system supporting self-configuration and ensure that it is valid when it is used during self-configuration.

The ARCF data needs to be ready in MnS consumer who provides the ARCF data. How to prepare the ARCF data in MnS consumer is out of scope of the present document.

The ARCF data is used by 3GPP management system to perform the RAN NE self-configuration, so the MnS consumer needs to transfer the ARCF data to 3GPP management system or indicates 3GPP management system supporting self-configuration where the ARCF data is available and 3GPP management system can retrieve the data from there.

Before MnS consumer transfer the ARCF data to 3GPP management system, the 3GPP management system may indicate need for ARCF data to the MnS consumer.

When 3GPP management system receive the ARCF data, may perform ARCF data validation to validate the syntax and semantics of ARCF data.

5. 1. 2	Requirements

REQ-ARCF -1: 3GPP management system shall have the capability to allow its authorized consumer to transfer the ARCF data to 3GPP management system or indicate 3GPP management system where the ARCF data is available for downloading.

REQ-ARCF -2: 3GPP management system shall have the capability to allow its authorized consumer to request to validate the previously downloaded ARCF data.

End of change