3GPP TSG-SA5 Meeting #144e	S5-224158

e-meeting 27 June - 1 July 2022

# Source:	China Unicom

# Title:	Add solution for URLLC performance measurements related to reliability

# Document for:	Approval

Agenda Item:	6.8.3 Study on Management Aspects of URLLC

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.832 v0.2.0: “Management Aspects of URLLC”

3	Rationale

It was approved in SP-220146 to study the management aspects of URLLC and one of the objectives is to investigate performance measurements related to URLLC. In order to achieve the objective mentioned above, some performance measurements related to URLLC is proposed in this contribution.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5	Key Issues Investigation and Potential Solutions

5.X	Key Issue #X: Performance measurements related to URLLC

5.X.1	Description

5.X.2	Potential solutions

In order to satisfy the requirement for performance management of network providing URLLC service, new measurements should be investigated and defined to evaluate the reliability and latency performance of network providing URLLC service.

#### 5.X.2.1     Downlink reliability in RAN

a)	This measurement prvides the reliability for the radio network providing URLLC service on downlink transmission containing the air-interface between gNB and UE. It is a measure of the DL packet delivery success including any packet successfully transmitted within a time constraint in the air interface and in the gNB. The measurement is calculated per PLMN ID and per 5QI level and per supported S-NSSAI.

b)	SI.

c)	This measurement is obtained as: 1) calculating the DL delay for a PDCP SDU packets from reception of a PDCP SDU packet until the time when the last part of RLC SDU packet related to the corresponding PDCP SDU was sent to the UE which was consequently confirmed by reception of HARQ ACK for UM mode or the time when the last part of an RLC SDU packet was sent to the UE which was consequently confirmed by reception of RLC ACK for AM mode; 2) counting the number of PDCP SDU packets whose DL delay is within the time constaint TC as numerator; 3) counting the total  number of DL PDCP SDU packets as denominator; 4) dividing the numerator by the denominator is the result for the measurement. The measurement is performed per PLMN ID and per 5QI and per supported S-NSSAI.

d)	Each measurement is a real value representing the reliability. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of S-NSSAIs.

e)	DRB.ReliabilityDlgNBUu.TC_Filter,

where filter is a combination of PLMN ID and QoS level and S-NSSAI.

Where TC indicates the time constraint.

NOTE: TC can be set according to AN PDB of 5QI, or canbe obtained according to attribute of SLA of a slice, or can be pre-configured by OAM.

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance management of the network providing URLLC service on reliability area.

#### 5.X.2.2	Uplink reliability in RAN

a)	This measurement prvides the reliability for the radio network providing URLLC service on uplink transmission containing the air-interface between gNB and UE. It is a measure of the UL packet delivery success including any packet successfully received within a time constraint in the air interface and in the gNB. The measurement is calculated per PLMN ID and per 5QI level and per supported S-NSSAI.

b)	SI.

c)	This measurement is obtained as: 1) calculating the UL delay for PDCP SDU packets from the time when an UL PDCP SDU is scheduled in UE as per the scheduling grant provided until the time when the corresponding UL PDCP SDU is successfully sent to the core network from gNB; 2) counting the number of PDCP SDU packets whose UL delay is within the time constaint TC as numerator; 3) counting the total  number of UL PDCP SDU packets as denominator; 4) dividing the numerator by the denominator is the result for the measurement. The measurement is performed per PLMN ID and per 5QI and per supported S-NSSAI.

d)	Each measurement is a real value representing the reliability. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of S-NSSAIs.

e)	DRB.ReliabilityUlgNBUu.TC_Filter,

where filter is a combination of PLMN ID and QoS level and S-NSSAI.

Where TC indicates the time constraint.

NOTE: TC canbe set according to AN PDB of 5QI, or canbe obtained according to attribute of SLA of a slice, or can be pre-configured by OAM.

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance management of the network providing URLLC service on reliability area.

End of changes