3GPP TSG-SA5 Meeting #144-e 	S5-224371

# e-meeting, 27 June - 1 July 2022

# Source: 	Huawei, Deutsche Telekom

# Title:	pCR TR 28.908 Add use case on AI/ML update

# Document for:	Approval

Agenda Item:	6.7.5.4

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

References

[1]		SP-211443 New Study on AI/ ML management

[2]		3GPP TR 28.908-020 Management and orchestration; Study on Artificial Intelligence/Machine Learning (AI/ML) management

[3]		3GPP TS 28.105 Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management

3	Rationale

The approved new SI [1] proposed to study the AI/ML management capabilities and management services to support/coordinate AI/ML in 5GS (3GPP management system, 5GC and NG-RAN). During the running period, the previously deployed AI/ML entity may no longer be applicable to the current network due to the changed network state. It is necessary to update the AI/ML entity (i.e., replace the running model with a new one) to ensure the performance of inference and analysis. This contribution proposes to add a use case on AI/ML update.

4	Detailed proposal

It is proposed to add the following chapter in TR 28.908 [2].

1st modified section

5.X	AI/ML update management

5.X.1	Description

Due to the complexity and time-varying nature of network, the AI/ML entities previously deployed may no longer be applicable to the current network after running for a period of time. Typically, the performance of a trained model may degrades over time. The AI/ML entitie needs to be updated timely to ensure the performance of inference and analysis.

5.X.2	Use cases

5.X.2.2	AI/ML entities updating initiated by producer

The AI/ML entity updating may be initiated by the AI/ML MnS producer. In order to keep the model at a requested level, the AI/ML MnS producer may periodically conduct AI/ML retraining with new available training data. Once a new version AI/ML entity is obtained after the training is finished, it can be used to update the current AI/ML entity with this new version. In another condition, the AI/ML MnS producer may initiate AI/ML entity updating based on the running model performance. For example, if the performance of the running AI/ML model is decreased under a predefined threshold, the AI/ML MnS producer may decide to start ratraining and then update the AI/ML entity to a new version which performs better.

5.X.3	Potential requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_UPD-CON-3 | The AI/ML MnS producer should have a capability to update the AI/ML entities and inform an authorized consumer about the update status. | AI/ML entities updating initiated by producer (clause 5.X.2.2) |

5.X.4	Possible solutions

TBD

End of modified section