| 3GPP TR 28.830 V0.2.0 (2022-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Study on Fault Supervision Evolution; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background and concepts	7

4.1	Background	7

4.2	Concepts	7

5	Key Issues and potential solutions	7

5.1	Key Issue 1: Missing definitions	7

5.1.1	Description	7

5.1.2	Potential solutions	7

5.1.3	Conclusion - Impact on normative work	9

5.X	Key Issue #X: XXX	9

5.X.1	Description	9

5.X.2	Potential solutions	9

5.X.2.a	Potential solution #<a>: <Potential Solution a Title>	9

5.X.2.a.1	Introduction	9

5.X.2.a.2	Description	10

5.X.3	Conclusion - Impact on normative work	10

6	Relation and interaction with eMDAS and eCOSLA	10

7	Conclusions and recommendations	10

Annex <X> (informative): Change history	11

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

TBD

1	Scope

The present document investigates potential use cases, requirements and solutions (from the perspective of service based management architecture) for the fault supervision evolution, its relation with performance management and fault supervision, relation and interaction with eMDAS and eCOSLA. The document provides conclusions and recommendations on the normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background and concepts

4.1	Background

4.2	Concepts

5	Key Issues and potential solutions

5.1	Key Issue 1: Missing definitions

5.1.1	Description

TS 28.532, clause 11.2, which defines the Fault Supervision MnS, does not provide the necessary definitions and descriptions required to understand the current state of art as to alarm management. This is because much of the material specified and available for the IRP Framework was not moved to SBMA.

5.1.2	Potential solutions

TS 28.532, clause 11.2 should provide all necessary definitions. This clause provides an (non exhaustive) overview on available definitions.

Event:

SA5 IRP: Network occurrence which has significance for the management of an NE. Events do not have state.

Event notification:

SA5 IRP: Notification used to inform the recipient about the occurrence of an event.

Note TS 28.532 specifies the "notifyEvent" notification, whose definition is: "This notification notifies the MnS consumer, who has a subscription receiving this type of notification, that certain network events have occurred with potential service impact, for example, system restart and system redundancy shift (backup)."

Error:

ITU-T X.733: A deviation of a system from normal operation.

Merriam-Webster: A deficiency or imperfection in structure or function.

RFC 8632: Term not used, or definition not provided.

SA5 IRP: Term not used, or definition not provided.

Failure:

Merriam-Webster: A state of inability to perform a normal function.

Fault:

ITU-T X.733: The physical or algorithmic cause of a malfunction. Faults manifest themselves as errors.

Merriam-Webster: A physical or intellectual imperfection or impairment.

RFC 8632: A fault is the underlying cause of an undesired behavior. There is no trivial one-to-one mapping between faults and alarms. One fault may result in several alarms in case the system lacks root-cause and correlation capabilities. An alarm might not have an underlying fault as a cause. For example, imagine a bad Mean Opinion Score (MOS) alarm from a Voice over IP (VOIP) probe and the cause being non-optimal QoS configuration.

SA5 IRP: A deviation of a system from normal operation, which may result in the loss of operational capabilities of the element or the loss of redundancy in case of a redundant configuration.

Discussion: The SA5 definition for fault deviates from the other definitions. The other definitions distinguish clearly between the real reason or underlying cause for a malfunction (which they call fault) and the malfunction itself (which they call error). Malfunction means a deviation of a system from normal operation.

SA5 needs to discuss the old definition of the term fault.

Alarm:

ITU-T X.733: A notification, of the form defined by this function, of a specific event. An alarm may or may not represent an error.

Merriam-Webster: A signal (such as a loud noise or flashing light) that warns or alerts.

RFC 8632: An alarm signifies an undesirable state in a resource that requires corrective action.

SA5 IRP: An alarm signifies an undesired condition of a resource (e.g. network element, link) for which an operator action is required. It emphasizes a key requirement that operators (above Itf-N) should not be informed about an undesired condition unless it requires operator action. Use of this emphasis does not exclude this case: In certain context, it is not possible for alarm reporters (below Itf-N) to know whether a particular undesired condition requires operator action or not. In such context, the NM may receive alarms that do not require operator action.

Discussion: Common to all definitions is that an alarm requires operator attention. Except for that there are differences in the definitions. ITU-T X.733 and Merriam-Webster state that an alarm is something that brings some event to the operator attention. RFC 8632 and SA5 IRP state that an alarm signifies an undesired condition. This does not distinguish between the thing that is being reported and the way how the thing is being reported. An alarm, though, should always refer to how something is reported, for example with a notification or a flashing light or a loud noise.

Alarm correlation:

RFC 8632: Dependencies between alarms; several alarms can be grouped as relating to each other, for example, a streaming media alarm relating to a high-jitter alarm.

Root cause analysis:

RFC 8632: An alarm can indicate candidate root-cause resources, for example, a database issue alarm referring to a full-disk partition.

SA5 IRP: Root Cause Analysis is a process that can determine and identify the network condition (e.g. fault, mis configuration) causing the alarms.

Discussion: The SA5 definition is not compliant to what is done in the interface definition. The interface definition clearly refers to alarms as the result of the root cause analysis and not to network conditions.

SA5 needs to discuss the old definition of the term root cause analysis.

5.1.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

5.X	Key Issue #X: XXX

5.X.1	Description

Editor’s note: This clause provides a description of the key issue.

5.X.2	Potential solutions

5.X.2.a	Potential solution #<a>: <Potential Solution a Title>

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

5.X.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

6	Relation and interaction with eMDAS and eCOSLA

7	Conclusions and recommendations

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222371 | - | - | - | Initial skeleton | 0.0.0 |
| 2022-04 | SA5#142e | S5-222625 S5-222526 | - | - | - | 1. Add TR structure 2. Add scope | 0.1.0 |
| 2022-07 | SA5#144e | S5-224383 | - | - | - | Add key issue on definition of terms | 0.2.0 |