| 3GPP TR 28.910 V0.3.0 (2022-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on enhancement of autonomous network levels; (Release 18 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope		7

2	References		7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms		8

3.2	Symbols		8

3.3	Abbreviations		8

4	Introduction on relevant study items and work items in 3GPP	8

5	Issue investigations and potential solutions for scenarios listed in TS 28.100	8

5.1	Key Issue# 5.1: Enhancement of generic autonomous network level for network optimization	9

5.1.1	Description	9

5.1.1.1 	Issue description	9

5.1.1.2 	Potential requirements	9

5.1a	Key Issue# 5.1a: Enhancement of autonomous network level for radio network coverage optimization	9

5.1a.1	Description	9

5.1a.2	 Potential solutions	9

6	Issue investigations and potential solutions for new scenarios	11

6.1	Key Issue# 6-1: Autonomous network level for RAN energy saving use case	11

6.1.1	Description	11

6.1.1.1	Use case	11

6.1.1.2	Workflow	11

6.1.1.3	 Classification of autonomous network level	12

6.2	Key Issue# 6-x: Autonomous network level for 5GC NF deployment	14

6.2.1	Description	14

6.2.1.1	Use case	14

6.2.1.2	Workflow	14

6.2.1.3	Classification of autonomous network level	14

6.2.2	Potential solutions	16

7	Conclusion and recommendation	16

Annex <X> (informative): Change history	17

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document studies on enhancement of autonomous network levels. It introduces the relevant study items and work items in 3GPP, identifies the additional generic MnS requirements of generic autonomous network level for the scenarios defined in Rel-17 and documents potential solutions for the identified generic MnS requirements. It also identifies the enhanced autonomy capabilities corresponding to different autonomous network levels for additional management use cases which is not defined in Rel-17 and documents potential solutions for the enhanced autonomy capabilities. Based on the investigation and studies, it provides recommendations for the further normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G"

[3]	3GPP TR 28.813: "Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G"

[4]	3GPP TS 28.100: "Management and orchestration; Levels of autonomous network"

[5]	3GPP TS 28.312:" Management and orchestration; Intent driven management services for mobile networks"

[6]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics"

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[8]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[9]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[10]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[11]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[12]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[13]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[14]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Introduction on relevant study items and work items in 3GPP

In 3GPP TS 28.100 [4], categorizations of the tasks in a workflow are specified in clause 4.3.4 including intent handling, awareness, analysis, decision and execution, and these task categories are used in the framework approach for evaluating autonomous network levels (ANL) in clause 5 of TS 28.100.

In clause 4.2.1 of TS 28.535 [13], management control loop including monitor, analytic, decision, execution is described. And in clause 6.1 of  TS 28.104 [6], management loop including observation, analytics, decision, execution is specified as well. The management control loop and management loop defined in TS 28.535 [13] and TS 28.104 [6] are similar, and both of them have the similar consideration with the workflow specified in TS 28.100 [4], i.e. awareness, analysis, decision and execution. MDA role in the management loop is within the scope of analytics (see TS 28.100 [4]).

In addition to the above four items included in management control loop or management loop, intent handling is also introduced in TS 28.100 [4] which means certain autonomous network levels are required to support intent driven management control loop autonomously. And in clause 4.2.2 of TS 28.312 [5], intent driven MnS is defined with the capabilities including intent translation and intent fulfilment result/information evaluating, which are the main tasks included in the categorization of intent handling defined in clause 4.3.4 of TS 28.100 [4].

- And if artificial intelligence / machine learning (AI/ML) technology is utilized in autonomous networks, AI/ML model training MnS specified in TS 28.105 [14], can be used to enable analytics or decision autonomy capabilities and thus to achieve certain autonomous network levels.

5	Issue investigations and potential solutions for scenarios listed in TS 28.100

- Editor's note: this clause will contain the description and potential solutions for additional generic MnS requirements of generic autonomous network level for network optimization, RAN NE deployment and fault management defined in Rel-17, especially those missing requirements to support autonomous network level 4 and 5.

5.1	Key Issue# 5.1: Enhancement of generic autonomous network level for network optimization

5.1.1	Description

5.1.1.1 	Issue description

The generic autonomous network level for network optimization is defined in Clause 7.1 in TS 28.100 [4], which includes generic workflow, generic classification of autonomous network level, generic autonomy capability description for management system, generic MnS requirements and solutions for generic MnS requirements.

Based on current definition, the generic autonomy capability description for management system for level 4 is documented in clause 7.1.3 in TS 28.100 [4]. However, the additional MnS requirements for level 4 are not specified in clause 7.1.4 in TS 28.100 [4].

5.1.1.2 	Potential requirements

Following additional MnS requirements for level 4 needs to be specified to support generic autonomy capability description for management system for level 4.

REQ-ANL-NetOpt-Level_4-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network optimization intent.

REQ-ANL-NetOpt-Level_4-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the fulfilment information of the network optimization intent.

5.1.2		Potential solutions

Following solutions for MnS requirements for level 4 needs to be added in TS 28.100 [4] Table 7.1.5-1: Solutions for generic MnS requirements of autonomous network level for network optimization.

| Level4 | REQ-ANL-NetOpt-Level_4-MnS-1 | This can be implemented by using generic provisioning MnS (e.g, createMOI) defined in TS 28.532 [4] to specify the network optimization intent defined in TS 28.312 [5]. |
| --- | --- | --- |
|  | REQ-ANL-NetOpt-Level_4-MnS-2 | This can be implemented by using generic provisioning MnS (e.g, getMOIAttribbutes) defined in TS 28.532 [7] to obtain network optimization fulfilment information defined in TS 28.312 [5]. |

5.1a	Key Issue# 5.1a: Enhancement of autonomous network level for radio network coverage optimization

5.1a.1	Description

The solution for MnS requirements for radio network coverage optimization is described in A.1.3, Based on current definition, following aspects needs to be further enhanced:

-	The MnS component type A (operation and notification), MnS component type B (Information model) and MnS component type C (management data) to support the MnS requirements for radio network coverage optimization haven't been explicitly described.

5.1a.2		Potential solutions

Based on the solutions for MnS requirements of autonomous network level for radio network coverage optimization defined in clause A.1.3 in TS 28.100[2], following are the enhanced solutions description for generic MnS requirements of autonomous network level for radio network coverage optimization.

Note: the solutions below are not used to evaluate the autonomous network level, which are MnS solutions to support MnS requirements for 3gpp management system derived from autonomy capability of each level

Table 5.1a.2-1: Solutions for generic MnS requirements of autonomous network level for radio network coverage optimization

| ANL | Requirements | Corresponding solutions |  |  |
| --- | --- | --- | --- | --- |
|  |  | MnS Component type A | MnS Component type B | MnS Component type C |
| Level1 | REQ-ANL-NetOpt-Level_1-MnS-1 | createMOI of provisioning MnS defined in TS 28.532[7] | CommonBeamformingFunction IOC in NR NRM defined in TS 28.541[8] | Not applicable |
|  | REQ-ANL-NetOpt-Level_1-MnS-2 | createMOI of provisioning MnS defined in TS 28.532[7] | PerfMetricJob IOC and TraceJob IOC defined in TS 28.622[9] | Not applicable |
|  | REQ-ANL-NetOpt-Level_1-MnS-3 | operations of file data reporting MnS and streaming data report MnS defined in TS 28.532[7] | NR NRM (e.g. NRCellCU, Beam and NRCellRelation IOC) defined in TS 28.541[8] | 1. RSRP measurements (e.g. SS-RSRP), RSRQ measurements (e.g. SS-RSRQ) and SINR measurements (e.g. SS-SINR) defined in TS 28.552 [10]. 2. RSRPs of the serving cell and neighbour cells, and UE location in MDT reports [11] |
| Level2 | REQ-ANL-NetOpt-Level_2-MnS-1 | createMOI of provisioning MnS defined in TS 28.532[7] | MDARequest IOC defined in MDA information model in TS 28.104[6]. | Not applicable |
|  | REQ-ANL-NetOpt-Level_2-MnS-2 | Operations of file data reporting MnS and streaming data report MnS defined in TS 28.532[7] | MDARequest IOC defined in MDA information model in TS 28.104[6] | "CoverageProblemId" of analytics output for coverage problem analysis in TS 28.104[6]. |
|  | REQ-ANL-NetOpt-Level_2-MnS-3 | createMOI of provisioning MnS defined in TS 28.532[7] | MDARequest IOC defined in MDA information model in TS 28.104[6]. | Not applicable |
|  | REQ-ANL-NetOpt-Level_2-MnS-4 | Operations of file data reporting MnS and streaming data report MnS defined in TS 28.532[7] | MDARequest IOC defined in MDA information model in TS 28.104[6] | "CoverageProblemType" and "CoverageProblemAreas" of analytics output for coverage problem analysis in TS 28.104[6]. |
|  | REQ-ANL-NetOpt-Level_2-MnS-5 | createMOI of provisioning MnS defined in TS 28.532[7] | MDARequest IOC defined in MDA information model in TS 28.104[6] | Not applicable |
| Level3 | REQ-ANL-NetOpt-Level_3-MnS-1 | createMOI of provisioning MnS defined in TS 28.532[7] | CCOFunction IOC defined in TS 28.541 [8] and TS 28.313 [12] as control information for CCO function. | Not applicable |
|  | REQ-ANL-NetOpt-Level_3-MnS-2 | createMOI of provisioning MnS defined in TS 28.532[7] | CCOFunction IOC defined in TS 28.541 [8] and TS 28.313 [12] as control information for CCO function. | Not applicable |
| Level 4 | REQ-ANL-NetOpt-Level_4-MnS-1 | createMOI of provisioning MnS defined in TS 28.532[7] | "weakRSRPRatioTarget"and "lowSINRRatioTarget" of RadioNetworkExpectation in intent information model in TS 28.312[5] as expectation targets for radio nework coverage assurance. | Not applicable |
|  | REQ-ANL-NetOpt-Level_4-MnS-2 | getMOIAttributes of provisioning MnS defined in TS 28.532[7] | "targetfulfillmeInfo" for "weakRSRPRatioTarget"and "lowSINRRatioTarget" of RadioNtworkExpectation in intent information model in TS 28.312[5]. | Not applicable |

Editor's Note: above solutions may need to be revisited based on further investigation.

6	Issue investigations and potential solutions for new scenarios

Editor's note: this clause will contain the description and potential solutions for enhanced autonomy capabilities corresponding to different autonomous network levels for additional management use cases which is not defined in Rel-17, including but not limited to energy saving and service provisioning.

6.1	Key Issue# 6-1: Autonomous network level for RAN energy saving use case

6.1.1	Description

6.1.1.1	Use case

The massive deployment of mobile network has brought about a rapid increase in energy consumption. The energy cost and carbon emission also become great challenges for the NOP. Operators are aiming at reducing power consumption in 5G networks to lower their OPEX with energy saving solutions. RAN energy saving represent the energy saving aspects (including capacity booster cell - gNB is fully or partially overlaid by the candidate cell(s) in TS 28.310[2], area based energy saving in TR 28.813[3], and other energy saving aspects) for RAN. So, introducing the autonomous network level for energy saving will benefit for operator to achieve the full autonomy goal step by step and have clear view on which typical issues can be addressed by telecom system in corresponding steps. The requirements for each autonomous level for energy saving are different. So it is important to introduce the autonomous network level definition for energy saving.

The concrete energy saving solutions should not be defined in this document. This key issue only focus on autonomous network level defined for energy saving.

6.1.1.2	Workflow

Intent handling:

-	Task A: RAN energy saving control information generation and determination. The tasks of generating and determining the RAN energy saving related control information (e.g., threshold value of high energy consumption issue analysis) and RAN energy saving actions generation) based on received RAN energy saving intent (e.g. expectation on energy saving targets in the specified areas with considering network performance (e.g. RAN UE throughput) assurance, as well as the frequencies and RATs to be considered).

-	Task B: RAN energy saving intent fulfilment evaluation. The tasks of evaluating RAN energy saving intent fulfilment information (e.g. energy saving targets (e.g., target RAN energy consumption and RAN UE throughput in the specified area are satisfied or not).

Awareness:

-	Task C: RAN energy saving related information collection. The tasks of collecting energy efficiency measurements (e.g., Data Volume measurement, PEE (Power, Energy and Environmental) measurements, network performance data (e.g., RAN UE throughput), network configuration data (e.g. energy saving state), environment data (e.g. electronic map, site location) and alarm data.

Analysis:

-	Task D: RAN energy saving issues identification. The tasks of identifying energy saving issues (including energy efficiency issues (e.g., high energy consumption, low energy efficiency) as TS 28.104 [6] described and performance issue (e.g. low RAN UE throughput) which may be caused by energy saving actions.

-	Task E: RAN traffic and performance prediction. The tasks of analysing current and historical RAN traffic load (e.g., PRB utilization rate, RRC connection number, etc.) and network performance (e.g. RAN UE throughput) to predict the traffic load trend and network performance trend over a period of time which could be used as references for energy saving solutions, analysis evaluation and determination.

-	Task F: RAN energy saving issue demarcation. The tasks of analysing the RAN energy saving issues and determine the RAN energy efficiency issue categories (e.g., high energy consumption, low energy efficiency) and corresponding area (which can be identified by geographical area, RAN NEs or cells), as well as determining the performance issue is caused by energy saving actions.

-	Task G: RAN energy saving issue root cause analysis. The tasks of analysing the root cause of the RAN energy saving issue, including RAN energy efficiency issues and performance issues caused by energy saving actions).

-	Task H: RAN energy saving solutions analysis. The tasks of generating the recommended energy saving solution (e.g. activate or deactivate energy saving state for the energy saving objects, adjust the carrier configuration or transmit power, adjust the energy saving thresholds for different service types.) which can address the identified energy saving issues.

Decision:

-	Task I: RAN energy saving solutions evaluation and determination. The tasks of evaluating the energy saving gains, as well as evaluating the RAN performance if such energy saving actions to be executed, and determining the optimal energy saving solutions (i.e. a set of energy saving actions) to be executed.

Execution:

-	Task J: RAN energy saving solutions execution. The tasks of adjusting the RAN energy saving actions (e.g., activate or deactivate energy saving state for the energy saving objects, adjust the carrier configuration or transmit power, adjust the energy saving thresholds for different service types.) which can address the identified energy saving issues).

6.1.1.3	 Classification of autonomous network level

Level 0:

-	All the tasks in the RAN energy saving workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G, Task H, Task I, Task J) are accomplished by human.

Level 1:

-	Telecom system executes the part of RAN energy saving solutions execution tasks (e.g. activate or deactivate energy saving state for the energy saving cell) based on the specified RAN energy saving solutions (Task J). Telecom system also can execute the tasks of collecting part of energy saving related information (including energy efficiency related information (including Data Volume measurement, PEE (Power, Energy and Environmental) measurements etc.), network performance data, network configuration data and alarm data) based on the specified collection control information (Task C). At this level, telecom system can assist human to improve the execution and awareness efficiency for RAN energy saving.

-	All the other tasks in the RAN energy saving workflow (Task A, Task B, Task D, Task E, Task F, Task G, Task H, Task I) are accomplished by human.

Level 2:

-	Compared to Level 1, telecom system additionally executes the tasks of RAN energy saving issues identification, part of RAN energy saving demarcation and RAN energy saving issue root cause analysis for the energy efficiency issues based on the specified RAN energy saving issue analysis control information (Task D, Task F, Task G). In this level, telecom system also can execute the task of collecting RAN energy saving related information (including energy efficiency data, performance data, network configuration data, environment data, alarm data) based on the specified collection control information (Task C), The tasks of RAN energy saving solutions execution (Task J) are fully accomplished by telecom system. At this level, telecom system can assist human to achieve the closed loop for RAN energy saving based on human defined control information.

-	All the other tasks in the RAN energy saving workflow (Task A, Task B, Task E, Task H, Task I) are accomplished by human.

Level 3:

-	Compared to Level 2, telecom system additionally executes the tasks of RAN energy saving solutions analysis (Task H) and RAN energy saving solutions evaluation and determination (Task I) based on the specified RAN energy saving solution analysis and decision control information. In this level, telecom system also can execute the tasks of RAN traffic load and performance prediction (Task E) based on the specified RAN traffic load and performance prediction control information. The tasks of RAN energy saving related information collection (Task C), RAN energy saving issues identification (Task D) and RAN energy saving demarcation (including RAN energy efficiency issues demarcation and RAN performance issues caused by energy saving actions) (Task F) are fully accomplished by telecom system. At this level, the telecom system can achieve the closed loop automation for RAN energy saving based on the human defined control information.

-	All the other tasks in the RAN energy saving workflow (Task A, Task B) are accomplished by human.

Level 4:

-	Compared to Level 3, the telecom system additionally executes the tasks of RAN energy saving control information generation and determination (Task A), and RAN energy saving intent fulfilment evaluation (Task B) based on received RAN energy saving intent and intent handling control information. The tasks of RAN energy saving issue root cause analysis (including root cause analysis for energy efficiency issues and performance issues caused by energy saving actions) (Task G), RAN energy saving solutions analysis (Task H), RAN energy saving solutions evaluation and determination (Task I), and RAN traffic load and performance prediction (Task E) are fully accomplished by telecom system. At this level, telecom system can achieve the intent driven closed loop automation for RAN energy saving based on human defined intent handling control information.

-	The intent handling control information maybe pre-defined and specified by human to assist the telecom system.

Level 5:

-	Telecom system can autonomously execute the entire workflow of RAN energy saving for all scenarios, which means the telecom system can achieve the full autonomy for RAN energy saving for full scenarios.

Figure 6.1.2.2-1 Autonomous network level for RAN energy saving scenario

6.1.2		Potential solutions

TBD

6.2	Key Issue# 6-x: Autonomous network level for 5GC NF deployment

6.2.1	Description

6.2.1.1	Use case

5GC NF deployment use case refers to the entire workflow of deploying a 5GC NF, full autonomy of 5GC NF deployment can help the network operator to reduce OPEX by reducing manual involvement in such tasks. However, full autonomy of 5GC NF deployment is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing network autonomy mechanisms in corresponding steps. The requirements for each autonomous level for 5GC NF deployment are different.

6.2.1.2	Workflow

Following are the entire workflow for 5GC NF deployment:

Intent handling:

-	Task A: 5GC NF deployment policy generation based on received 5GC NF deployment intent （e.g. type of the 5GC NF, capability of the 5GC NF）. The tasks of generating and determining the 5GC NF deployment related policies (e.g. 5GC NF configuration data generation policies, dialing test policies and 5GC NF virtualised resource feasibility polices).

-	Task B: 5GC NF deployment intent evaluation, the tasks of evaluating 5GC NF deployment fulfilment information (e.g. satisfied or not).

Awareness:

-	Task C: 5GC NF information collection, this task includes collecting the network environment data before deployment of 5GC NF information and collecting the 5GC NF and related network data when the 5GC NF is deployed (e.g. PM, FM and virtualised resource information).

Analysis:

-	Task D: 5GC NF deployment data analysis, the tasks of analysing and generating the recommended configuration data and required virtualised resource information (e.g. VNFD, flavour ID) for deployment of the 5GC NF.

-	Task E: 5GC NF virtualised resource feasibility check analysis, the tasks of analysing there is enough virtualised resource is available for deployment of the 5GC NF.

Task F: 5GC NF commissioning test and dialing test, the tasks of performing the 5GC NF commissioning test and dialing test before putting 5GC NF into service (e.g. ensure the configuration and network connection (including 5GC NF configuration and transport network configuration) is correct).

Decision:

- Task G: 5GC NF configuration data and virtualised resource information determination, the tasks of evaluating and determining the 5GC configuration data and virtualised resource information (e.g. VNF package) to be used for deploying the 5GC NF.

Execution:

-	Task H: 5GC NF deployment, the tasks of deploying the 5GC NF based on determined VNF package, flavour ID and configuring the 5GC NF based on determined data.

6.2.1.3	Classification of autonomous network level

Level 0:

-	All the tasks in the 5GC NF deployment workflow (Task A, Task B, Task C, Task D, Task E, Task F, Task G, Task H) are accomplished by human.

Level 1:

-	Telecom system executes tasks of deploying the 5GC NF based on onboarded VNF package and activating configuration data prepared by human (Task H) and tasks of collecting part of 5GC NF information (inlcuding virtualised resource information) based on specified rules(Task C).

-	All the other tasks in the 5GC NF deployment workflow (Task A, Task B, Task D, Task E, Task F, Task G) are accomplished by human.

Level 2:

-	Compared to Level 1, telecom system additionally executes the tasks of collecting 5GC information (inlcuding virtualised resource information) (Task C). Telecom system also can execute the tasks of analysing the deployment data for the 5GC based on the network function generation rules (e.g. configuration template) specified by human (Task D). The tasks of 5GC NF deployment (Task H) are fully accomplished by telecom system.

-	All the other tasks in the 5GC NF deployment workflow (Task A, Task B, Task E, Task F, Task G) are accomplished by human.

Level 3:

-	Compared to Level 2, the telecom system additionally executes tasks of analysing and determining the deployment data for the 5GC NF based on the network configuration data generation policies specified by human (Task D). Telecom system also can executes tasks of performing 5GC virtualised resource feasibility check and commissioning/dialing test by human (Task E and Task F). The tasks of 5GC NF information collection (Task C) are fully accomplished by telecom system.

-	All the other tasks in the5GC deployment workflow (Task A, Task B) are accomplished by human.

Level 4:

-	Compared to Level 3, the telecom system can additionally execute the tasks of 5GC NF deployment policy generation (Task A) and 5GC NF deployment intent evaluation (Task B) based on intent handling control information specified by human. The tasks of 5GC deployment data analysis (Task D), 5GC virtualised resource feasibility check (Task E), 5GC NF commissioning and dialing test (Task F), 5GC NF configuration data and virtualised resource information determination (Task G) are fully accomplished by telecom system.

-	The intent handling control information maybe pre-defined and specified by human.

Level 5:

-	Telecom system can autonomously execute the entire workflow of 5GC NF deployment for all scenarios,

Figure 6.2.1.3-1 Classification of autonomous network level for 5GC NF deployment

6.2.2	Potential solutions

TBD

7	Conclusion and recommendation

Editor's note: this clause will be used to document the conclusions and recommendation of the study.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222126 | - | - | - | Initial skeleton | 0.0.0 |
| 2022-04 | SA5#142e | S5-222615 S5-222616 S5-222614 | - | - | - | - Add TR structure - Add scope - Add autonomous network level for RAN energy saving | 0.1.0 |
| 2022-05 | SA5#143e | S5-223565 S5-223566 S5-223567 | - | - | - | - Add key issue for enhancement of ANL for network - Add key issue for autonomous network level for 5GC NF deployment - Update the key issue of autonomous network level for RAN energy saving scenario | 0.2.0 |
| 2022-08 | SA5#145e | S5-225679 | - | - | - | 1. Add introduction on relevant SI/WI in 3GPP. 2. Align TR front page title with SA WG5 official title. | 0.3.0 |