3GPP TSG-SA5 Meeting #145-e 	S5-225285

# e-meeting, 15 - 24 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0419 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QBC triggering for LBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 29.571 there should be a statement in the specification for the DNN wherever it uses DNN Network Identifier, or a full DNN with both the Network Identifier and Operator Identifier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding that the default is only the Network Identifier, but that it may optionally also include Operator Identifier, this to keep it backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DNN will be undefined which may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.6.1	General

This subclause specifies the application data model supported by the API.

The Nchf_ConvergedCharging Service API allows the NF consumer to consume the converged charging service from the CHF as defined in 3GPP TS 32.290 [58].

Table 6.1.6.1-1 specifies the data types defined for the ConvergedCharging service based interface protocol.

Table 6.1.6.1-1: Nchf_ ConvergedCharging specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ChargingDataRequest |  | 6.1.6.2.1.1 6.1.6.2.2.1 |  | Describes the attributes of Charging Data Request to CHF for initial, update and termination of the charging session. |  |  |  |
| ChargingDataResponse |  | 6.1.6.2.1.2 6.1.6.2.2.2 |  | Describes the attributes of Charging Data Response from CHF on charging session initial, update and termination. |  |  |  |
| ChargingNotifyRequest |  | 6.1.6.2.1.3 |  | Describes Notifications about events that occurred in request message. |  |  |  |
| ChargingNotifyResponse |  | 6.1.6.2.1.16 |  | Describes the response of notification. |  |  |  |

Table 6.1.6.1-2 specifies data types re-used by the Nchf_ConvergedCharging service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nchf_ConvergedCharging service based interface.

Table 6.1.6.1-2: Nchf_ConvergedCharging re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Supi |  | 3GPP TS 29.571 [371] |  | The identification of the user (i.e. IMSI, NAI, GLI, GCI). (NOTE 1) |  |  |  |
| Uint32 |  | 3GPP TS 29.571 [371] |  | Unsigned 32-bit integers |  |  |  |
| Uint64 |  | 3GPP TS 29.571 [371] |  | Unsigned 64-bit integers |  |  |  |
| PduSessionId |  | 3GPP TS 29.571 [371] |  | The identification of the PDU session. |  |  |  |
| PduSessionType |  | 3GPP TS 29.571 [371] |  | the type of a PDU session |  |  |  |
| Uri |  | 3GPP TS 29.571 [371] |  | String providing an URI |  |  |  |
| AccessType |  | 3GPP TS 29.571 [371] |  | The identification of the type of access network. |  |  |  |
| DateTime |  | 3GPP TS 29.571 [371] |  | The time. |  |  |  |
| ChargingId |  | 3GPP TS 29.571 [371] |  | Charging identifier allowing correlation of charging information |  |  |  |
| RatType |  | 3GPP TS 29.571 [371] |  | The identification of the RAT type. |  |  |  |
| RatingGroup |  | 3GPP TS 29.571 [371] |  | The identification of the rating group |  |  |  |
| Ipv4Addr |  | 3GPP TS 29.571 [371] |  | Ipv4 address. |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [371] |  | The Ipv6 prefix allocated for the user. |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [371] |  | Ipv6 Address. |  |  |  |
| Pei |  | 3GPP TS 29.571 [371] |  | The Identification of a Permanent Equipment. |  |  |  |
| TimeZone |  | 3GPP TS 29.571 [371] |  | Time zone information |  |  |  |
| NfInstanceId |  | 3GPP TS 29.571 [371] |  | String uniquely identifying a NF instance. |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [371] |  | String identifying a Gpsi |  |  |  |
| DefaultQosInformation |  | 3GPP TS 29.571 [371] |  | Identifies the information of the default QoS. |  |  |  |
| SubscribedDefaultQos |  | 3GPP TS 29.571 [371] |  | subscribed default QoS. |  |  |  |
| AuthorizedDefaultQos |  | 3GPP TS 29.512 [302] |  | Authorized default QoS. |  |  |  |
| Ambr |  | 3GPP TS 29.571 [371] |  | Aggregate Maximum Bit rate |  |  |  |
| QosData |  | 3GPP TS 29.512 [302] |  | Contains QoS parameters |  |  |  |
| UserLocation |  | 3GPP TS 29.571 [371] |  | User location information |  |  |  |
| PlmnId |  | 3GPP TS 29.571 [371] |  | PLMN id |  |  |  |
| Guami |  | 3GPP TS 29.571 [371] |  | Globally Unique AMF Identifier |  |  |  |
| DurationSec |  | 3GPP TS 29.571 [371] |  | Identifies a period of time in units of seconds. |  |  |  |
| Snssai |  | 3GPP TS 29.571 [371] |  | SNSSAI |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [371] |  | additional details of the error |  |  |  |
| ServiceId |  | 3GPP TS 29.571 [371] |  | Identifier of service |  |  |  |
| SscMode |  | 3GPP TS 29.571 [371] |  | SSC Mode type |  |  |  |
| PresenceInfo |  | 3GPP TS 29.571 [371] |  | PRA information including PRAId, PRA element list and PRA status |  |  |  |
| Qfi |  | 3GPP TS 29.571 [371] |  | QoS flow identifier designated as "Qfi". |  |  |  |
| AmfId |  | 3GPP TS 29.571 [371] |  | AMF identifier |  |  |  |
| Dnn |  | 3GPP TS 29.571 [371] |  | Data Network Name including Network Identifier, and optionally Operator Identifier |  |  |  |
| GroupId |  | 3GPP TS 29.571 [371] |  | Network internal Identifier for a group of IMSIs |  |  |  |
| ExternalGroupId |  | 3GPP TS 29.571 [371] |  | External Group Identifier for one or more subscriptions associated to a group of IMSIs |  |  |  |
| Bytes |  | 3GPP TS 29.571 [371] |  | String with format "byte" |  |  |  |
| Tai |  | 3GPP TS 29.571 [371] |  | Tracking Area Identifier |  |  |  |
| Area |  | 3GPP TS 29.571 [371] |  | List of TACs or Operator specific codes |  |  |  |
| CoreNetworkType |  | 3GPP TS 29.571 [371] |  | 5GC or EPC |  |  |  |
| ServiceAreaRestriction |  | 3GPP TS 29.571 [371] |  | Service Area restriction |  |  |  |
| GlobalRanNodeId |  | 3GPP TS 29.571 [371] |  | Global RAN Node Id |  |  |  |
| QosCharacteristics |  | 3GPP TS 29.512 [302] |  | Map of QoS characteristics for non standard 5QIs and non-preconfigured 5QIs. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [371] |  | See 3GPP TS 29.500 [299] clause 6.6 |  |  |  |
| NsiLoadLevelInfo |  | 3GPP TS 29.520 [306] |  | Represents the load level information for an S-NSSAI and the associated network slice instance |  |  |  |
| ServiceExperienceInfo |  | 3GPP TS 29.520 [306] |  | ServiceExperience |  |  |  |
| ApplicationChargingId |  | 3GPP TS 29.571 [371] |  | Application provided charging identifier allowing correlation of charging information. |  | AF_Charging_Identifier |  |
| SharingLevel |  | 3GPP TS 28.541 [254] |  | Ressources sharing level |  |  |  |
| MobilityLevel |  | 3GPP TS 28.541 [254] |  | UE mobility Level |  |  |  |
| SsT |  | 3GPP TS 28.541 [254] |  | Slice Service type (SST) |  |  |  |
| Support |  | 3GPP TS 28.541 [254] |  | Supported, Not Supported indicator |  |  |  |
| Float |  | 3GPP TS 29.571 [371] |  | Number with format "float" |  |  |  |
| MaPduIndication |  | 3GPP TS 29.512 [302] |  | MA PDU session indication |  | ATSSS |  |
| AtsssCapability |  | 3GPP TS 29.571 [371] |  | ATSSS capabilities |  | ATSSS |  |
| SteeringFunctionality |  | 3GPP TS 29.571 [371] |  | Steering functionalities for MA PDU session |  | ATSSS |  |
| SteeringMode |  | 3GPP TS 29.512 [302] |  | Steering mode for MA PDU session |  | ATSSS |  |
| OperationalState |  | 3GPP TS 28.623 [257] |  | Operational state |  |  |  |
| AdministrativeState |  | 3GPP TS 28.623 [257] |  | Administrative state |  |  |  |
| RanNasRelCause |  | 3GPP TS 29.512 [302] |  | Indicates the RAN or NAS release cause code information. |  | EnhancedDiagnostics |  |
| Ecgi |  | 3GPP TS 29.571 [371] |  | E-UTRA Cell Id |  |  |  |
| Ncgi |  | 3GPP TS 29.571 [371] |  | NR Cell Id |  |  |  |
| NOTE 1: A SUPI containing GLI or GCI is used to support 5G-RG and FN-RG in scenarios of wireline network. |  |  |  |  |  |  |  |

End of changes