3GPP TSG-SA5 Meeting #146 	S5-226111

Toulouse, France, 14 -18 November 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0801 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to multiplicity of relation between NetworkSlice IOC and NetworkSliceSubnet IOC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 28.530 defines that NetworkSlice instance represents service view of a network slice which exposes the root NetworkSliceSubnet instance. The non-root NetworkSliceSubnet instances are not directly associated with the NetworkSlice object instance. In Figure 6.2.1-1, the relation between NetworkSlice IOC and NetworkSliceSubnet IOC with 1:1 relationship implies that : - Every NetworkSlice MOI must be associated with one NetworkSliceSubnet MOI, and - Every NetworkSliceSubnet MOI must be associated with one NetworkSlice MOI. The NetworkSliceSubnet MOI which does not represent root/top NetworkSliceSubnet does not have a direct relation with NetworkSlice. Hence, the multiplicity of relation between NetworkSlice IOC and NetworkSliceSubnet IOC represented Figure 6.2.1-1 in is incorrect. Further, NetworkSliceSubnets are associated to zero or one NetworkSlice - Zero: allows to have NetworkSliceSubnets associated to each other, without a NetworkSlice - One: Allows a NetworkSliceSubnet to be associated to exactly one NetworkSlice Any NetworkSlice is associated to exactly one NetworkSliceSubnet. Hence a correction is needed to the multiplicity of relationship between NetworkSlice IOC and NetworkSliceSubnet IOC represented Figure 6.2.1-1 needs to be corrected from 1:1 to 0..1:1 to allow the non-root NetworkSliceSubnet instances to exist without a direct relation with the NetworkSlice object instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multiplicity of relation between NetworkSlice IOC and NetworkSliceSubnet IOC is corrected from 1:1 to 0..1:1 in Figure 6.2.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect standards leads to confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Rel-16 related correction is in CR#0801 with tDoc S5-226111 and Rel-17 related correction is in CR#0802 with tDoc S5-226112. Since there are additional changes in Rel-17, the Rel-17 CR is a correction CR and not a mirror. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-225394 is a revision of S5-223451 - S5-226111 is a revision of S5-225394 Note: S5-225394 was submitted for Rel-18. Since the correction is applicable for Rel-16 and Rel-17 too, the correction is applied for Rel-16 Rel-16 in S5-226111 and for Rel-17 in S5-226112. |  |  |  |  |  |  |  |  |

1st Change

### 6.2.1	Relationships

Figure 6.2.1-1: Network slice NRM fragment relationship

NOTE 1:	The <<OpenModelClass>> NetworkService and <<OpenModelClass>> VNF are defined in [40].

NOTE 2:	The target Network Service (NS) instance represents a group of VNFs and PNFs that are supporting the source network slice subnet instance.

NOTE 3:	The instance tree of this NRM fragment would not contain the instances of NetworkService and VNF. However, the NetworkSliceSubNet instances would have an attribute holding the identifiers of NetworkService instances and the ManagedFunction instance would have an attribute holding identifiers of VNF instances.

Figure 6.2.1-2: Transport EP NRM fragment relationship

Figure 6.2.1-3: containment relationship for network slice fragment

End of change