3GPP TSG-SA5 Meeting #146 	S5-226185

Toulouse, France, 14 -18 November 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0386 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Renaming MRO counters from Intra/Inter System to Intra/Inter RAT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the definition of the MRO counters in 38.300, the core networks are considered for intra and inter system counters. With the current standards, an inter-RAT failure from NG-RAN to E-UTRAN will be pegged as intra-system HO failure even though both the NG-RAN and E-UTRAN are served by the same core. This does not reflect the nature of the failure event correctly. The measurement objects used in SA5 for MRO failures considers the source and target cell and not the source and target core network. E.g. NRCellRelation or EutranCellRelation. So it would be better to define MRO counters in SA5 based on RAT instead of core network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inter/Intra System counters have been replaced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-RAT handovers from same core will be incorrectly counterd as inter-system handovers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.658 CR 0062 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of first change

#### 5.1.1.25	Measurements related to MRO

##### 5.1.1.25.1	Handover failures related to MRO for intra-RAT mobility

a)	This measurement provides the number of handover failure events related to MRO detected during the intra-RAT mobility within NG-RAN. The measurement includes separate counters for various handover failure types, classified as "Intra-RAT too early handover", "Intra-RAT too late handover" and "Intra-RAT handover to wrong cell".

b)	CC.

c)	The measurements of too early handovers, too late handovers and handover to wrong cell events are obtained respectively by accumulating the number of failure events detected by gNB during the intra-RAT mobility within NG-RAN.

d)	Each measurement is an integer value.

e)	HO. IntraRAT.TooEarly
 HO. IntraRAT.TooLate

HO. IntraRAT.ToWrongCell

f)	 NRCellCU
 NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.2	Handover failures related to MRO for inter-RAT mobility

a)	This measurement provides the number of handover failure events delated to MRO detected during the inter-RAT mobility between NG-RAN and E-UTRAN. The measurement includes separate counters for various handover failure types, classified as "Inter-system too early handover"(inter-RAT mobility from E-UTRAN to NG-RAN) and "Inter-RAT too late handover" (inter-system mobility from NG-RAN to E-UTRAN).

b)	CC.

c)	The measurements of too early inter-RAT handover events are obtained by accumulating the number of failure events detected during the inter-RAT mobility from E-UTRAN to NG-RAN. The measurements of too late inter-RAT handover events are obtained by accumulating the number of failure events detected during the inter-RAT mobility from NG-RAN to E-UTRAN.

d)	Each measurement is an integer value.

e)	HO. InterRAT.TooEarly

HO. InterRAT.TooLate

f)	NRCellCU

NRCellRelation

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.3	Unnecessary handovers for inter-RAT mobility

a)	This measurement provides the number of unnecessary handover events detected during the inter-RAT mobility from NG-RAN to E-UTRAN. An example of unnecessary handover occurred when a UE handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient.

b)	CC.

c)	The measurement of unnecessary inter-RAT handovers is obtained by accumulating the number of inter-system unnecessary handover events detected during the inter-RAT mobility from NG-RAN to E-UTRAN.

d)	Each measurement is an integer value.

e)	HO. InterRAT.Unnecessary

f)	NRCellCU

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.4	Handover ping-pong for inter-RAT mobility

a)	This measurement provides the number of handover ping-pong events detected during the inter-RAT mobility between NG-RAN and E-UTRAN, see TS 38.300 [49] clause 15.5.2.4. An example of handover ping-pong occurred when a UE is handed over from a cell in a source RAT (e.g. NG-RAN) to a cell in a target RAT different from the source RAT (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source RAT, while the coverage of the source RAT was sufficient for the service used by the UE.

b)	CC.

c)	The measurement of handover ping-pong events is obtained by accumulating the number of failure events detected during the inter-RAT mobility between NG-RAN and E-UTRAN.

d)	Each measurement is an integer value.

e)	HO. InterRAT.PingPong

f)	NRCellCU

NRCellRelation

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.5	Handover failures per beam-cell pair related to MRO for intra-RAT mobility

a)	This measurement provides the number of handover failure events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) related to MRO detected during the intra-RAT mobility within NG-RAN. The measurement includes separate counters for various handover failure types, classified as "Intra-RAT too early handover per beam”, "Intra-RAT too late handover per beam " and "Intra-RAT handover to wrong cell per beam ". The handovers considered are inter-cell handovers.

b)	CC.

c)	The measurements of too early handovers for the beam per adjacent cell, too late handovers for the beam per adjacent cell and handover to wrong cell for the beam per adjacent cell events are obtained respectively by accumulating the number of failure events detected by gNB during the intra-RAT mobility within NG-RAN, where adjacent cells are identified by their NR Cell Identity (NCI).

d)	Each measurement is an integer value.

e)	HO. IntraRAT.bTooEarly.NCI
HO. IntraRAT.bTooLate.NCI
HO. IntraRAT.bToWrongCell.NCI

f)	Beam

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.6	Handover failures per beam-cell pair related to MRO for inter-RAT mobility

a)	This measurement provides the number of handover failure events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) related to MRO detected during the inter-RAT mobility from 5GS to EPS. The measurement counter is classified as handover failure type "Inter-RAT too late handover".

b)	CC.

c)	The measurements of too late handovers for the beam per adjacent cell events are obtained respectively by accumulating the number of failure events detected by gNB during the inter-RAT mobility from NG-RAN to E-UTRAN, where adjacent cells are identified by their E-UTRAN Cell Global Identifier (ECGI).

d)	Each measurement is an integer value.

e)	HO. InterRAT.bTooLate.ECGI

f)	Beam

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.7	Unnecessary handovers per beam-cell pair for inter-RAT mobility

a)	This measurement provides the number of unnecessary handover events per beam-cell pair (source beam, i.e., the last beam before handover, and target cell) detected during the inter-RAT mobility from NG-RAN to E-UTRAN. An example of unnecessary handover occurred when a UE handed over from NG-RAN to other RAT (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient.

b)	CC.

c)	The measurement of unnecessary handovers for the beam per adjacent cell is obtained by accumulating the number of inter-RAT unnecessary handover reports detected by gNB during the inter-system mobility from NG-RAN to E-UTRAN, where adjacent cells are identified by their E-UTRAN Cell Global Identifier (ECGI).

d)	Each measurement is an integer value.

e)	HO. InterRAT.bUnnecessary.ECGI

f)	Beam

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.8	Handover ping-pong per beam-cell pair for inter-RAT mobility

a)	This measurement provides the number of handover ping-pong events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) detected during the inter-RAT mobility from NG-RAN to E-UTRAN. An example of handover ping-pong occurred when a UE is handed over from a cell in a source RAT (e.g. NG-RAN) to a cell in a target RAT different from the source RAT (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source RAT, while the coverage of the source RAT was sufficient for the service used by the UE.

b)	CC.

c)	The measurement of handover ping-pong events for the beam per adjacent cell is obtained by accumulating the number of failure events detected by gNB during the inter-RAT mobility from NG-RAN to E-UTRAN, where adjacent cells are identified by their E-UTRAN Cell Global Identifier (ECGI).

d)	Each measurement is an integer value.

e)	HO. InterRAT.bPingPong.NCI

f)	Beam

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

End of changes