3GPP TSG-SA5 Meeting #146 	S5-226218

Toulouse, France, 14 -18 November 2022

# Source:	Huawei, China Mobile

# Title:	pCR TR 28.910 Add key issue for enhancement of network optimization

# Document for:	approval

Agenda Item:	6.7.1.1

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP draft TR 28.910: “Management and orchestration; Study on enhancement of autonomous network levels v0.3.0”.

[2]	3GPP TS 28.100: "Management and orchestration; Levels of autonomous network"

3	Rationale

In TS 28.100 [2], the workflow and classification of autonomous network level for RAN UE throughput optimization is defined, however the solution for generic MnS requirements of autonomous network level for RAN UE optimization is missing. Following the same pattern as Key Issue# 5.1a: Enhancement of autonomous network level for radio network coverage optimization in TR 28.910[1], this contribution proposes to analyse the solutions for corresponding MnS requirements

4	Detailed proposal

It proposes to make the following changes to TR 28.910[1].

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G"

[3]	3GPP TR 28.813: "Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G"

[4]	3GPP TS 28.100: "Management and orchestration; Levels of autonomous network"

[5]	3GPP TS 28.312:" Management and orchestration; Intent driven management services for mobile networks"

[6]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics"

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[8]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[9]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[10]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[11]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[12]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[X]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

2nd Change

5.1b	Key Issue# 5.1b: Analysis on the solution for MnS requirements of autonomous network level for RAN UE throughput optimization

5.1b.1	Description

Autonomous network level for RAN UE throughput optimization is documented in clause A.2 in TS 28.100[4]. In TS 28.100[4], corresponding workflow and classification of autonomous network level for RAN UE throughput optimization is defined, however it is not clear whether the solution for generic MnS requirements of autonomous network level for RAN UE optimization is well defined. So it is necessary to analyse the solutions for corresponding MnS requirements.

5.1b.2		Analysis

Based on the existing generic MnS requirements of Level 1-Level 3 for the generic network optimization in TS 28.100 [4] and additional MnS requirements for Level 4 for the generic network optimization in clause 5.1, following are the analysis on solutions which can be used to satisfy MnS requirements of autonomous network level for RAN UE throughput optimization.

- Regarding the MnS requirements of Level 1, NR NRM (e.g. NRCellCU, NRCellRelation, NRCellDU) defined in TS 28.541[8] are used to represent network adjustment solution. The UE throughput measurements (e.g. Average DL UE throughput in gNB, Distribution of DL UE throughput in gNB) defined in TS 28.552 [10] and RAN UE Throughput KPIs (e.g. DL RAN UE throughput for a sub-network, DL RAN UE throughput for a NRCellDU) defined in TS 28.554 [X] are used to represent the network related information.

- Regarding the MnS requirements of Level 4, the attribute "aveULRANUEThptTarget", "aveDLRANUEthptTarget","lowULRANUEThptRatioTarget" and "lowDLRANUEThptRatioTarget" of RadioNetworkExpectation in intent information model in TS 28.312[5] as expectation targets for RAN UE throughput assurance.

Following are the gap analyse for the solutions to Level 1-Level 4 MnS requirements for RAN UE throughput optimization.

- No existing solutions can be used to support Additional MnS requirements to support autonomous network level 2. No mechanism to specify concrete RAN UE throughput issue identification, demarcation and root cause analysis control information and No mechanism to allow MnS consumer to obtain the RAN UE throughput issue identification, demarcation and root cause analysis result.

- No existing solutions can be used to support Additional MnS requirements to support autonomous network level 3. No mechanism to specify concrete network adjustment analytic control information and network adjustment decision control information to resolve the RAN UE throughput issues.

5.1b.3		Potential solutions

FFS

End of Changes