3GPP TSG-SA5 Meeting #146	S5-226261

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0233 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.532 Add missing definition of the JSON Patch document |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, REST_SS |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The JSON Patch document misses a REST SS and OpenAPI definition. It is described only by text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The REST SS and OpenAPI definitions of a Patch document is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing REST SS and OpenAPI definitions may result in interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1.1.4.2, 12.1.1.4.1, 12.1.1.4.1a.9 (new), 12.1.1.4.4.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Begin of modifications

###### 12.1.1.1.4.2	Mapping to HTTP PATCH

HTTP PATCH is used to create, update or delete one or multiple resources.

Table 12.1.1.1.4.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | …/{className}={id} | className: string id: string | M |
| scope | n/a | n/a | n/a | n/a |
| filter | n/a | n/a | n/a | n/a |
| modificationList | request body | n/a | Resource, or array(PatchItem) | M |

Four patch media types are available for the request message body. They are listed below together with their request body data types:

-	"application/merge-patch+json" (RFC 7396 [37]),				request body type: Resource

-	"application/3gpp-merge-patch+json" (TS 32.158 [15]),			request body type: Resource

-	"application/json-patch+json" (RFC 6902 [36]),					request body type: array(PatchItem)

-	"application/3gpp-json-patch+json" (TS 32.158 [15]),			request body type: array(PatchItem)

The IS parameters "scope" and "filter" have no SS equivalents in the present document.

Table 12.1.1.1.4.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | n/a | Resource | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for modification of one or multiple resources is as follows:

1.	The MnS consumer sends a HTTP PATCH request to the MnS producer.

-	The path component of the target URI identifies the base resource.

-	The message body shall contain the patch document.

2.	The MnS producer sends a HTTP PATCH response to the MnS consumer.

-	On success, "200 OK" or "204 No Content" shall be returned. When "200 OK" is returned the message body shall include a representation of the updated resources constructed according to the hierarchical response construction method (TS 32.158 [15]).

-	On failure, an appropriate error code shall be returned. The response message body may provide additional error information

Note 1: Void.

Next modification

#### 12.1.1.4	Data type definitions

##### 12.1.1.4.1	General

This clause defines the data types used by the Provisioning MnS. Table 12.1.1.4.1-1 specifies the data types defined in the present document and Table table 12.1.1.4.1-2 the data types imported.

Table 12.1.1.4.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| Scope | 12.1.1.4.1a.2 | Used in the query part of HTTP GET and HTTP DELETE to extend the set of targeted resources beyond the base resource identified with the authority and path component of the URI |
| ScopeType | 12.1.1.4.4.5 | Scope type of a scope |
| CmNotificationTypes | 12.1.1.4.4.3 | Notification type (notifyMOICreation, etc.) |
| SourceIndicator | 12.1.1.4.4.4 | Indicates the source of the operation that led to the generation of the notification. |
| ScopeType | 12.1.1.4.4.5 | Scope type of a scope |
| CorrelatedNotification | 12.1.1.4.1a.3 | Describes the correlated notifications of a single source |
| Resource | 12.1.1.4.1a.1 | Used for resource representations |
| Operation | 12.1.1.4.4.6 | Enum with "create", "delete" and "replace" |
| PatchOperation | 12.1.1.4.4.7 | Enum with "add", "replace", "remove", "copy", "move" and "test" |
| Resource | 12.1.1.4.1a.1 | Used for resource representations |
| Scope | 12.1.1.4.1a.2 | Used in the query part of HTTP GET and HTTP DELETE to extend the set of targeted resources beyond the base resource identified with the authority and path component of the URI |
| CorrelatedNotification | 12.1.1.4.1a.3 | Describes the correlated notifications of a single source |
| MoiChange | 12.1.1.4.1a.4 | Single MOI change reported by notifyMOIChanges |
| NotifyMOICreation | 12.1.1.4.1a.5 | Used in the request body of HTTP POST for the notification type notifyMOICreation |
| NotifyMOIDeletion | 12.1.1.4.1a.6 | Used in the request body of HTTP POST for the notification type notifyMOIDeletion |
| NotifyMOIAttributeValueChanges | 12.1.1.4.1a.7 | Used in the request body of HTTP POST for the notification type notifyMOIAttributeValueChanges |
| NotifyMOIChanges | 12.1.1.4.1a.8 | Used in the request body of HTTP POST for the notification type notifyMOIChanges |
| PatchItem | 12.1.1.4.1a.9 | Specifies a patch item of a patch document |

Table 12.1.1.4.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |
| DateTime | TS 28.623 [44] | Date and time |
| Dn | TS 28.623 [44] | DN type |
| SystemDN | TS 28.623 [44] | systemDN type |
| Uri | TS 28.623 [44] | URI type |
| AttributeNameValuePairSet | TS 28.623 [44] | Set of attribute name/value pairs |
| AttributeValueChangeSet | TS 28.623 [44] | Set of attribute names with their old and new values |
| Filter | TS 28.623 [44] | Filter type |
| NotificationId | TS 28.623 [44] | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| NotificationType | TS 28.623 [44] | Notification type |
| NotificationHeader | TS 28.623 [44] | Notification header |
| ErrorResponse | TS 28.623 [44] | Used in the response body of multiple HTTP methods in case of error |

Next modification

##### 12.1.1.4.1a	Structured data types

###### 12.1.1.4.1a.1	Type Resource

Table 12.1.1.4.1a.1 -1: Definition of type Resource

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| id | string | Identifier of the resource object | M |
| objectClass | string | Object class of the resource object | O |
| objectInstance | Dn | Object instance of the resource object | O |
| attributes | object | "attributes" (JSON) object whose members are the IOC attributes (except for "id”, "objectClass” and "objectInstance"). | M |
| n/a | map(array(object)) | Name contained objects | M |

This definition of "Resource" does not specify any attributes or name contained objects. Resource representations with specific attributes and name contained objects are contained in the NRM definitions. These definitions should be used in implementations of the Provisioning MnS instead of this generic definition.

###### 12.1.1.4.1a.2	Type Scope

Table 12.1.1.4.1a.2-1: Definition of type Scope

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| scopeType | ScopeType | Used in the query component of HTTP GET and HTTP DELETE together with scopeLevel to extend the set of targeted resources beyond the base resource identified with the authority and path component of the URI | M |
| scopeLevel | integer | Used in the query component of HTTP GET and HTTP DELETE together with scopeType to extend the set of targeted resources beyond the base resource identified with the path component of the URI | M |

###### 12.1.1.4.1a.3	Type CorrelatedNotification

Table 12.1.1.4.1a.3 -1: Definition of type CorrelatedNotification

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| source | Dn | Source of the correlated notifications | M |
| notificationIds | array(NotificationId) | Notification identifiers of correlated notifications of that source | M |

###### 12.1.1.4.1a.4	Type MoiChange

Table 12.1.1.4.1a.4 -1: Definition of type MoiChange

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| op | Operation | Operation associated to the reported change ("add", "remove , "replace"). | M |
| path | string | URI path component segments specifying when appended to "href" the created, deleted or updated resource or secondary resource | M |
| value | any type | New value of the created or updated resource or secondary resource. Optional old value of the deleted resource or secondary resource | M |
| oldValue | any type | Old value of the updated secondary resource | O |

The properties "op", "path" and "value" shall use the 3GPP JSON Patch format (3GPP TS 32.158 [15]) for reporting NRM changes. The "merge" operation specified by 3GPP JSON Patch is not supported in "notifyMOIChanges". The "move", "copy" and "test" operations specified by JSON Patch are not supported either.

The "oldValue" is an optional extension for "notifyMOIChanges" allowing to report also the old value before the replacemement of the value by the new value contained in "value".

The following example notification (where JSON is expressed in YAML notation) reports an object creation

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1

value:

id: 1,

objectClass: ClassA,

attributes:

attrA: 123

attrB:

subAttrB1: ABC

subAttrB2: 56

The following example reports the deletion of an object.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1

The following example reports the addition of a new attribute "attrC".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrC

value: XYZ

The following example reports the deletion of the attribute "attrC".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1#/attributes/attrC

The following example reports a value change for the simple attribute "attrA".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

When the old value is reported as well, the notification looks like.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

oldValue: 123

The following example reports a value change for the complex attribute "attrB".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: DEF

subAttrB2: 78

The previous two notifications can be combined into a single notification as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: DEF

subAttrB2: 78

Note the operation "replace" has replace semantics and not merge semantics. The following notification reports the value change of the attribute field "attrB:subAttrB1" and the deletion of the attribute field"attrB:subAttrB2".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: DEF

The value change of the attribute field "attrA:subAttrB1" is reported as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA/subAttrB1

value: DEF

Assume "attrD" is a JSON array with simple elements, then the creation of this multi-value attribute is reported as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrD

value:

- 1

- 2

- 3

Its deletion is reported by the following notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1#/attributes/attrD

The complete replacement of the array is reported by the following notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrD

value:

- 11

- 21

- 31

The following example reports the second item in the array changed to "22".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrD/1

value: 22

Note the array index of the second item is "1".

Assume now "attrE" is a JSON array with complex array items, for example.

[{subItemE1: 11, subItemD2: ABC}, {subItemE1: 21, subItemE2: DEF}, {subItemE1: 31, subItemE2": GHI}.

A value change to

[{subItemE1: 11, subItemE2: ABC}, {subItemE1: 21, subItemE2: XYZ}, {subItemE1: 31, subItemE2": GHI}.

is reported by

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrE/1/subItemE2

value: XYZ

When "subItemE2" is defined as array item key at stage 2, then "attrE" should contain a JSON map.

attrE:

11:

subItemE2: ABC

21:

subItemE2: DEF

31:

subItemE2: GHI

The same change as above is now reported by the notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrE/21/subItemD2

value: XYZ

###### 12.1.1.4.1a.5	Type NotifyMoiCreation

Table 12.1.1.4.1a.5 -1: Definition of type NotifyMoiCreation

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyMOICreation) | M |
| eventTime | DateTime | Event (MOI creation) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeList | AttributeNameValuePairSet | The attributes (name/value pairs) of the created MOI. | O |

###### 12.1.1.4.1a.6	Type NotifyMoiDeletion

Table 12.1.1.4.1a.6 -1: Definition of type NotifyMoiDeletion

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyMOIDeletion) | M |
| eventTime | DateTime | Event (MOI creation) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeList | AttributeNameValuePairSet | Attributes (name/value pairs) of the deleted MOI. | O |

###### 12.1.1.4.1a.7	Type NotifyMoiAttributeValueChanges

Table 12.1.1.4.1a.7 -1: Definition of type NotifyMoiAttributeValueChanges

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyMOIAttributeValueChanges) | M |
| eventTime | DateTime | Event (MOI creation) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeListValueChanges | AttributeValueChangeSet | List with names of changed attributes, together with new value and optionally old value | M |

###### 12.1.1.4.1a.8	Type NotifyMoiChanges

Table 12.1.1.4.1a.8 -1: Definition of type NotifyMoiChanges

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of a common ancestor resource (object) of the resources for which changes are reported. A MnS producer may set this attribute always to the parent of the root resource in the MIB. | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4]. | M |
| notificationType | NotificationType | Notification type (notifyMOIChanges) | M |
| eventTime | DateTime | Event (NRM updates) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| moiChanges | array(MoiChange) | MOI changes to be reported | M |

###### 12.1.1.4.1a.9	Type PatchItem

Table 12.1.1.4.1a.9 -1: Definition of type PatchItem

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| op | PatchOperation | Patch operation. | M |
| from | string | Present only for "copy" and "move" operations, identifies the value to be copied or moved to the location specified by path. | M |
| path | string | Path specifying the patched value. | M |
| value | any type | New value for the resource identified by "path". | M |

Next modification

##### 12.1.1.4.4	Simple data types and enumerations

12.1.1.4.4.1	General

This clause defines simple data types and enumerations that are used by the data structures defined in the previous clauses.

12.1.1.4.4.2	Simple data types

Table 12.1.1.4.3.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| n/a | n/a | n/a |

12.1.1.4.4.3	Enumeration CmNotificationTypes

Table 12.1.1.4.4.3-1: Enumeration CmNotificationTypes

| Enumeration value | Description |
| --- | --- |
| notifyMOICreation | Notification type is notifyMOICreation |
| notifyMOIDeletion | Notification type is notifyMOIDeletion |
| notifyMOIAttributeValueChanges | Notification type is notifyMOIAttributeValueChange |
| noitifyMOIChanges | Notification type is notifyMOIChanges |

12.1.1.4.4.4	Enumeration SourceIndicator

Table 12.1.1.4.4.4-1: Enumeration SourceIndicator

| Enumeration value | Description |
| --- | --- |
| RESOURCE_OPERATION | The notification was generated in response to an internal operation of the resource. |
| MANAGEMENT_OPERATION | The notification was generated in response to a management operation applied across the managed object boundary external to the managed object |
| SON_OPERATION | The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . |
| UNKNOWN | It is not possible to determine the source of the operation. |

12.1.1.4.4.5	Enumeration ScopeType

Table 12.1.1.4.4.4.1-1: Enumeration ScopeType

| Enumeration value | Description |
| --- | --- |
| BASE_ONLY | Selects only the base resource. The "scopeLevel" parameter shall be absent or ignored if present. |
| BASE_ALL | Selects the base resource and all of its subordinate resources (incl. the leaf resources). The "scopeLevel" parameter shall be absent or ignored if present. |
| BASE_NTH_LEVEL | Selects all resources on the level, which is indicated by the "scopeLevel" parameter, below the base resource. The base resource is at "scopeLevel" zero. |
| BASE_SUBTREE | Selects the base resource and all of its subordinate resources down to and including the resources on the level indicated by the "scopeLevel" parameter. The base resource is at "scopeLevel" zero. |

12.1.1.4.4.6	Enumeration Operation

Table 12.1.1.4.4.4.6-1: Enumeration Operation

| Enumeration value | Description |
| --- | --- |
| add | Create operation |
| remove | Delete operation |
| replace | Replace operation |

12.1.1.4.4.7	Enumeration PatchOperation

Table 12.1.1.4.4.4.7-1: Enumeration PatchOperation

| Enumeration value | Description |
| --- | --- |
| add | Add operation |
| replace | Replace operation |
| remove | Remove operation |
| copy | Copy operation |
| move | Move operation |
| test | Test operation |

Next modification

## A.1.1	OpenAPI document "TS28532_ProvMnS.yaml"

openapi: 3.0.1

info:

title: Provisioning MnS

version: 17.1.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

URI-LDN-first-part:

description: See clause 4.4.2 of TS 32.158

default: ''

paths:

'/{className}={id}':

parameters:

- name: className

in: path

required: true

schema:

type: string

- name: id

in: path

required: true

schema:

type: string

put:

summary: Replaces a complete single resource or creates it if it does not exist

description: >-

With HTTP PUT a complete resource is replaced or created if it does not

exist. The target resource is identified by the target URI.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

responses:

'200':

description: >-

Success case ("200 OK").

This status code shall be returned when the resource is replaced, and

when the replaced resource representation is not identical to the resource

representation in the request.

This status code may be retourned when the resource is updated and when the

updated resource representation is identical to the resource representation

in the request.

The representation of the updated resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'201':

description: >-

Success case ("201 Created").

This status code shall be returned when the resource is created.

The representation of the created resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code may be returned only when the replaced resource

representation is identical to the representation in the request.

The response has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

callbacks:

notifyMOICreation:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiCreation'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIDeletion:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiDeletion'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIAttributeValueChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiAttributeValueChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

application/yang-data+json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'              responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

get:

summary: Reads one or multiple resources

description: >-

With HTTP GET resources are read. The resources to be retrieved are

identified with the target URI. The attributes and fields parameter

of the query components allow to select the resource properties to be returned.

parameters:

- name: scope

in: query

description: >-

This parameter extends the set of targeted resources beyond the base

resource identified with the path component of the URI. No scoping

mechanism is specified in the present document.

required: false

schema:

$ref: '#/components/schemas/Scope'

style: form

explode: true

- name: filter

in: query

description: >-

This parameter reduces the targeted set of resources by applying a

filter to the scoped set of resource representations. Only resource

representations for which the filter construct evaluates to "true"

are targeted. No filter language is specified in the present

document.

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Filter'

- name: attributes

in: query

description: >-

This parameter specifies the attributes of the scoped resources that

are returned.

required: true

schema:

type: array

items:

type: string

style: form

explode: false

- name: fields

in: query

description: >-

This parameter specifies the attribute field of the scoped resources

that are returned.

required: false

schema:

type: array

items:

type: string

style: form

explode: false

responses:

'200':

description: >-

Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body. In case the attributes or fields query

parameters are used, only the selected attributes or sub-attributes are

returned. The response message body is constructed according to the

hierarchical response construction method (TS 32.158 [15]).

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

patch:

summary: Patches one or multiple resources

description: >-

With HTTP PATCH resources are created, updated or deleted. The resources

to be modified are identified with the target URI (base resource) and

the patch document included in the request message body.

requestBody:

description: >-

The request body describes changes to be made to the target resources.

The following patch media types are available

- "application/merge-patch+json" (RFC 7396)

- "application/3gpp-merge-patch+json" (TS 32.158)

- "application/json-patch+json" (RFC 6902)

- "application/3gpp-json-patch+json" (TS 32.158)

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/3gpp-merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

application/3gpp-json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned when the updated the resource representations

shall be returned for some reason.

The resource representations are returned in the response message body. The

response message body is constructed according to the hierarchical response

construction method (TS 32.158 [15])

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code is returned when there is no need to return the updated

resource representations.

The response message body is empty.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

delete:

summary: Deletes one or multiple resources

description: >-

With HTTP DELETE resources are deleted. The resources to be deleted are

identified with the target URI.

parameters:

- name: scope

in: query

description: >-

This parameter extends the set of targeted resources beyond the base

resource identified with the path component of the URI. No scoping

mechanism is specified in the present document.

required: false

schema:

$ref: '#/components/schemas/Scope'

style: form

explode: true

- name: filter

in: query

description: >-

This parameter reduces the targeted set of resources by applying a

filter to the scoped set of resource representations. Only resources

representations for which the filter construct evaluates to "true"

are returned. No filter language is specified in the present

document.

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Filter'

responses:

'200':

description: >-

Success case ("200 OK").

This status code shall be returned, when query parameters are present in

the request and one or multiple resources are deleted.

The URIs of the deleted resources are returned in the response message body.

'204':

description: >-

Success case ("204 No Content").

This status code shall be returned, when no query parameters are present in

the request and only one resource is deleted.

The message body is empty.

content:

application/json:

schema:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

CmNotificationTypes:

type: string

enum:

- notifyMOICreation

- notifyMOIDeletion

- notifyMOIAttributeValueChanges

- notifyMOIChanges

SourceIndicator:

type: string

enum:

- RESOURCE_OPERATION

- MANAGEMENT_OPERATION

- SON_OPERATION

- UNKNOWN

ScopeType:

type: string

enum:

- BASE_ONLY

- BASE_NTH_LEVEL

- BASE_SUBTREE

- BASE_ALL

Operation:

type: string

enum:

- add

- remove

- replace

ScopeType:

type: string

enum:

- BASE_ONLY

- BASE_NTH_LEVEL

- BASE_SUBTREE

- BASE_ALL

PatchOperation:

type: string

enum:

- add

- replace

- remove

- copy

- move

- test

Resource:

oneOf:

- type: object

properties:

id:

objectClass:

type: string

objectInstance:

$ref: 'comDefs.yaml#/components/schemas/Dn'

type: string

attributes:

type: object

additionalProperties:

type: array

items:

type: object

required:

- id

- anyOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/resources-genericNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/resources-nrNrm'

- $ref: ' TS28541_5GcNrm.yaml#/components/schemas/resources-5gcNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/resources-sliceNrm'

- $ref: 'intentNrm.yaml#/components/schemas/resources-intentNrm'

Scope:

type: object

properties:

scopeType:

$ref: '#/components/schemas/ScopeType'

scopeLevel:

type: integer

CorrelatedNotification:

type: object

properties:

source:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- source

- notificationIds

MoiChange:

type: object

properties:

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

op:

$ref: '#/components/schemas/Operation'

path:

type: string

value:{}

oldValue: {}

required:

- notificationId

- op

- path

NotifyMoiCreation:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiDeletion:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiAttributeValueChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeListValueChanges:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

required:

- attributeListValueChanges

NotifyMoiChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

moiChanges:

type: array

items:

$ref: '#/components/schemas/MoiChange'

required:

- moiChanges

PatchItem:

type: object

properties:

op:

$ref: '#/components/schemas/PatchOperation'

from:

type: string

path:

type: string

value: {}

End of modifications