3GPP TSG-SA5 Meeting #146 	S5-226284

Toulouse, France, 14 -18 November 2022

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Rel-18 pCR 28.xxx Add some common data type definitions

# Document for:	Approval, Information, Discussion

Agenda Item:	6.5.2.3 - AdNRM_ph2_WoP#3

# 1	Decision/action requested

It is proposed to discuss and approve this pCR.

# 2	References

None.

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TR 28.xxx[1].

Begin of modifications

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[f]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[a]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[b]	IETF RFC 1166: "Internet Numbers".

[d]	IETF RFC 8200: "Internet Protocol, Version 6 (IPv6)".

[c]	IETF RFC 5952: "A Recommendation for IPv6 Address Text Representation".

[e]	IETF RFC 1034: "Domain Names – Concepts and Facilities".

[x]	IETF RFC 3339: "Date and Time on the Internet: Timestamps"

Next modification

# 5	General data types

### 5.1	Simple data types

Table 5.1-1: Data types imported

| Data type |  | Reference | Description |  |
| --- | --- | --- | --- | --- |
| DateTime |  | TS 32.156 [10], clause 5.4.3.1 | Date and time |  |
| Real | TS 32.156 [10], clause 5.4.3.1 | Real number |
| String | TS 32.156 [10], clause 5.4.3.1 | String |
| Integer | TS 32.156 [10], clause 5.4.3.1 | Integer |
| Boolean | TS 32.156 [10], clause 5.4.3.1 | Boolean |
| DN | TS 32.156 [10], clause 5.4.3.1 | Distinguished Name (TS 32.300 [13]) |

Table 5.1-2: Data types defined in the present document

| Type name |  | Type definition | Description |
| --- | --- | --- | --- |
| Float | Real | Floating-point number. Float and Real are used synonymously. |
| FullDate | String | Date as define by "full-time" in RFC3339 [x] |
| FullTime | String | Time as define by "full-time" in RFC3339 [x] |
| Latitude |  | Float | Latitude based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. Minimum: -90.0000, Maximum: +90.0000 |
| Longitude |  | Float | Longitude based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. Maximum: -180.0000, Minimum: +180.0000 |
| Dn | DN | Distinguished Name (TS 32.300 [13]), provided to align with data type naming conventions. |
| NotificationId | Integer | Notification identifier. |
| SystemDn | Dn | System DN |
| Filter | String | Filter. The filter format is protocol specific. |
| DomainName | String |  | Domain Name in textual dotted notation as defined in clause 3.5 of IETF RFC 1034 [e]. The domain name should be fully qualified. |
| Fqdn | String |  | Editor's note: This is ffs if the term Fqdn should be replaced by DomainName. |
| Ipv4Addr | String | Type of an IPv4 address in dotted decimal text representation format of four decimal integers, ranging from 0 to 255 each, as defined in IETF RFC 1166 [b]. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])$' |
| Ipv6Addr | String | Type of an Ipv6 address (IETF RFC 4260 [d]) in the canonical text representation format defined in clause 4 of IETF RFC 5952 [c]. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$' |
| IpAddr | CHOICE(Ipv4Addr, Ipv6Addr) | Type of an IP address, either an IPv4 address or an IPv6 address. |
| Host | CHOICE(Ipv4Addr, Ipv6Addr, DomainName) | Editor's note: This is ffs since host names have a stricter format than domain names. |
| Port | Integer | Port number, as defined e.g. in clause 3.2.3 of IETF RFC 3986 [a] Minimum: 0, Maximum: 65535 |
| Uri | String | Type of a URI as defined in in IETF RFC 3986 [a] |

### 5.2	Enumerations

None.

### 5.3	Structured data types

#### 5.3.1	GeoCoordinate

##### 5.3.1.1	Definition

This data type defines a geographical location on earth.

##### 5.3.1.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| latitude | M | T | T | F | T |
| longitude | M | T | T | F | T |

##### 5.3.1.3	Attribute constraints

##### None.

##### 5.3.1.4	Attribute properties

| Attribute name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| latitude | Latitude of the geographical location | type: Latitude multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | Longitude of the geographical location | type: Longitude multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

## 6	Mobile network specific data types

### 6.1	Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| Mcc | String | Type of a Mobile Country Code, consisting of three decimal digits, as defined in clause 2.2 of TS 23.003 [5] Pattern: '^[0-9]{3}$' |
| Mnc | String | Type of a Mobile Network Code, consisting of two or three decimal digits, as defined in clause 2.2 of TS 23.003 [5] Pattern: '^[0-9]{2,3}$' |
| Nid | String | Network Identifier, which together with a PLMN ID is used to identify an SNPN (see TS 23.003 [5] and TS 23.501 [22], clause 5.30.2.1). Pattern: '^[A-Fa-f0-9]{11}$' |
| Tac | String | 3-octet string identifying a tracking area code (TAC) as specified in clause 9.3.3.10 of TS 38.413 [f], in hexadecimal representation. Pattern: '(^[A-Fa-f0-9]{4}$)¦(^[A-Fa-f0-9]{6}$)' |

### 6.2	Enumerations

None.

## 6.3	Structured data types

### 6.3.1	PlmnId

### 6.3.1.1	Definition

Type of a Public Land Mobile Network (PLMN) identifier (PLMN-Id), as defined in clause 12.1 and clause 2.2 of TS 23.003 [5].

#### 6.3.1.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mcc | M | T | T | F | T |
| mnc | M | T | T | F | T |

#### 6.3.1.3	Attribute constraints

None.

#### 6.3.1.4	Attribute properties

| Attribute name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mcc | Mobile Country Code | type: Mcc multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnc | Mobile Network Code | type: Mnc multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modifications