3GPP TSG-SA5 Meeting #146 	S5-226310

Toulouse, France, 14 -18 November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0027 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add MDA capability for prediction of utilization related performance KPIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MDA has no capability to assist in performance KPIs analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MDA capability for utilization related performance KPIs prediction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MDA capability to analyse one or multiple predicted utilization related performance KPI(s) is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 8.4.X	MDA assisted performance KPIs analytics

### 8.4.X.1	 MDA capability for utilization related performance KPIs

#### 8.4.X.1.1	MDA type

This MDA capability is to analysis for one or multiple predicted utilization related performance KPI(s) for a network slice.

The MDA type for utilization related performance KPIs analysis is: MDAAssistedUtilizationKPIPrediction.

#### 8.4.X.1.2	Enabling data

The enabling data for MDAAssistedUtilizationKPIPrediction MDA type are provided in table 8.4.X.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.X.1.2-1: Enabling data for prediction of utilization related performance KPIs

| Data category | Description | References |
| --- | --- | --- |
| Key Performance Indicators (KPIs) | Mean number of PDU sessions of network and network Slice Instance | Clause 6.4.1 of TS 28.554 [5] |
|  | Virtualised Resource Utilization of Network Slice Instance | Clause 6.4.2 of TS 28.554 [5] |
|  | PDU session establishment time of network slice | Clause 6.4.3 of TS 28.554 [5] |
|  | Mean number of successful periodic registration updates of Single Network Slice | Clause 6.4.4 of TS 28.554 [5] |
|  | Maximum number of PDU sessions of network slice | Clause 6.4.5 of TS 28.554 [5] |

#### 8.4.X.1.3	Analytics output

The specific information elements of the analytics output for prediction of utilization related performance KPIs, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.X.1.3-1.

Table 8.4.X.1.3-1: Analytics output for prediction of utilization related performance KPIs

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| PDUSesMeanNbrPrediction | The prediction of the mean number of PDU sessions that are successfully established in a network slice. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| VirtualResUtilizaitonPrediction | The prediction of utilization of virtualised resource (e.g. processor, memory, disk) that are allocated to a network slice. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PDUEstTimePrediction | The prediction of the time of successful PDU session establishment which related to one single network slice and is used to evaluate utilization provided by the end-to-end network slice and network performance. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RegUpdMeanNbrPrediction | The prediction of the mean number of successfully periodic registration updates in a network slice at the AMF. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PDUSesMaxNbrPrediction | The prediction of the maximum number of PDU sessions that are successfully established in a network slice. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of Change