3GPP TSG-SA5 Meeting #146 	S5-226320

# Toulouse, France, 14 -18 November 2022

# Source:	Huawei

# Title:	Add use case for NF creation as cloud native VNF

# Document for:	Approval

Agenda Item:	6.8.5.1

# 1	Decision/action requested

For approval

# 2	References

[1]	3GPP TR 28.834 V0.3.0 Study on Management of Cloud Native Virtualized Network Functions

# 3	Rationale

UC#4 (scaling of cloud-native VNF) only mentions interface between EM and VNFM (IFA008), the interface between 3GPP management system to NFVO (IFA013) also needs to be considered in scaling use case.

# 4	Detailed proposal

It is proposed to update scaling use case to support IFA013 interface in clause 5.4 of TR 28.834 V0.3.0 [1].

1st change

## 5.4	Use cases# 4: Scaling of cloud-native VNF

### 5.4.1	Description

As a pre-condition to this use case, the network is running normally. The 3GPP management system has subscribed to VNF instance lifecycle notifications from VNFM.

The use case begins when the 3GPP management system decides to scale a VNF.

The 3GPP management system requests ETSI NFV MANO to scale the VNF (as defined in ETSI GR NFV-IFA 008[8] and ETSI NFV-IFA013 V4.3.1 [9]). ETSI NFV MANO inspects the relevant VNF Descriptor (as defined in ETSI GR NFV-IFA 011[7]) to find the VNF components (VNFCs).

ETSI NFV MANO will add or remove VNFC instances as needed for the desired scaling level of the VNF. When the VNFCs have been added or removed, ETSI NFV MANO sends a notification to the 3GPP management system to notify that the VNF has been scaled. This notification includes information on the changes to VNFCs.

### 5.4.2		Issues

TS 28.526 [10] describes the procedures to be used by the 3GPP management system to request scaling. However, TS 28.526 [10] is written in terms (for example Network Manager and Element Manager) which are not compatible with the 5G architecture as described in TS 28.533 [11] and do not support scaling for containerized VNF.

### 5.4.3		Requirements

REQ-NF-Scaling-1: The 3GPP management system shall have a capability to interact with ETSI NFV MANO for scaling a cloud-native VNF.

End of changes