3GPP TSG-SA5 Meeting #146 	S5-226325

# Toulouse (FR), 14-18 November 2022

# Source:	Huawei, Deutsche Telekom

# Title:	Add Reference Architecture

# Document for:	Approval

Agenda Item:	6.9.6.4

# 1	Decision/action requested

Include the proposed text in TR 28.824

# 2	References

[1]		3GPP TR 28.824: "Study on network slice management capability exposure"

# 3	Rationale

In TR 28.824 [1], diagrams illustrating interactions between stakeholders during network slice ordering, provisioning and assurance exist in various places:

- Clause 4.1.1.4.2 (Exposure scenarios)

- Clause 4.1.3.2 (Use case No.1: simple case)

- Clause 4.1.3.3 (Use case No.2: partners involved)

- Clause 5.5 (Exposure of network slice as a product)

  - 5.5.1.1 (Sub-use case 1: NSP and NOP played by the same organization)

  - 5.5.1.2 (Sub-use case 2: NOP role played simultaneously by different organizations)

- Clause 5.6 (Exposure of network slice as a service)

  - 5.6.1.1 (Sub-use case 1: NSP and NOP play by the same organization)

  - 5.6.1.2 (Sub-use case 2: NOP role played simultaneously by different organizations)

From these use case specific architectures, it is possible to derive a reference architecture valid for all use cases, by merging the use case specific architectures.

This pCR proposes to introduce this reference architecture into TR 28.824 [1] clause 8 (Conclusion and Recommendation).

# 4	Detailed proposal

First change

# 8	Conclusion and Recommendation

## 8.X	Reference Architecture for network slice ordering, provisioning and assurance

### 8.X.1	Conclusion

From the use case specific architectures depicted in:

# Clause 4.1.1.4.2 (Exposure scenarios)

# Clause 4.1.3.2 (Use case No.1: simple case)

# Clause 4.1.3.3 (Use case No.2: partners involved)

# Clause 5.5 (Exposure of network slice as a product)

- 5.5.1.1 (Sub-use case 1: NSP and NOP played by the same organization)

- 5.5.1.2 (Sub-use case 2: NOP role played simultaneously by different organizations)

# Clause 5.6 (Exposure of network slice as a service)

- 5.6.1.1 (Sub-use case 1: NSP and NOP play by the same organization)

- 5.6.1.2 (Sub-use case 2: NOP role played simultaneously by different organizations)

, it is possible to derive a reference architecture valid for all use cases, by merging the use case specific architectures.

Figure 8.X.1-1: Reference architecture for network slice ordering, provisioning and assurance.

The interfaces present in this reference architecture are:

- Inter-organization interfaces (represented East-West):

- Between the Customer and the Service Provider:

- Interface ITF-1 to:

- create a product (e.g. network slice) order,

- retrieve information concerning a product (e.g. network slice) order,

- update a product (e.g. network slice) order,

- delete a product (e.g. network slice) order;

Candidate APIs for Interface ITF-1 include, but are not limited to, TMF API 622 (Product ordering), in which case entities managed via this interface include ProductOrder, ProductOrderItem, etc. defined in TMF 622 [2].

- Interface ITF-2 to:

- enable the Service Provider to expose, from its Service Management Layer, network slice management capabilities to the Customer;

Candidate APIs for Interface ITF-2 include 3GPP TS 28.531, TS 28.532, TS 28.545, TS 28.550.

Entities managed via the Interface ITF-2 include NetworkSlice (cf. TS 28.541 [9]).

At the Service Provider’s side of Interface ITF-2, the Exposure Governance Management Function (EGMF) governs the network slice management capabilities exposure.

- Between the Service Provider and its Partner(s):

- Interface ITF-5 to:

- create a product order,

- retrieve information concerning a product order,

- update a product order,

- delete a product order;

Candidate APIs for Interface ITF-5 include, but are not limited to, TMF API 622 (Product ordering), in which case entities managed via this interface include ProductOrder, ProductOrderItem, etc. defined in TMF 622 [2].

- Interface ITF-6 to:

- enable the Partner to expose, from its Service Management Layer, network slice management capabilities to the Service Provider;

Candidate APIs for Interface ITF-6 include 3GPP TS 28.531, TS 28.532, TS 28.545, TS 28.550.

Entities managed via the Interface ITF-6 include NetworkSlice (cf. TS 28.541 [9]).

At the Service Provider’s side of Interface ITF-6, the Exposure Governance Management Function (EGMF) governs the network slice management capabilities exposure.

- Intra-organization interfaces (represented North-South), internal to the Service Provider:

- Between its BSS and its OSS/SML

- Interface ITF-3 to:

- create a service order,

- retrieve information concerning a service order,

- update a service order,

- delete a service order;

Candidate APIs for Interface ITF-3 include, but are not limited to, TMF API 641 (Service ordering), in which case entities managed via this interface include ServiceOrder, ServiceOrderOrderItem, etc. defined in TMF 641 [3].

- Between its OSS/SML and its OSS/NML

- Interface ITF-4 to:

- create a network slice / network slice subnet,

- retrieve information concerning a network slice / network slice subnet,

- update a network slice / network slice subnet,

- delete a network slice / network slice subnet,

- collect PM and FM data concerning a network slice / network slice subnet;

Candidate APIs for Interface ITF-4 include, 3GPP TS 28.531, TS 28.532, TS 28.545, TS 28.550.

Entities managed via the Interface ITF-4 include NetworkSlice, NetworkSliceSubnet, etc. (cf. TS 28.541 [9]).

NOTE: in order not to create confusion, especially for external readers, it has been proposed in Figure 8.X.1-1 to rename interfaces, since interfaces Type-A and Type-B, used elsewhere in this TR, could be mixed up with MnS Component Type-A, Type B, Type C, defined in TS 28.533; similarly, interfaces Type-1 and Type-2, used elsewhere in this TR, could be mixed up with interfaces type 1, 2, etc. defined in TS 32.101.

### 8.X.2	Recommendation

It is recommended that the architecture depicted in clause 8.X.1 be included in a normative TS, as the Rel-18 reference architecture for network slice ordering, provisioning and assurance. The above list of candidate interfaces / APIs is not exhaustive. Other organizations / open source projects / etc. may use this reference architecture with different interfaces / APIs.

End of changes