3GPP TSG-SA5 Meeting #146	S5-226350

# Toulouse, France, November 14 – 18, 2022				                   s5-22abcd

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0028 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of energy saving analysis use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Energy saving analysis use case has a requirement to identify the cell/NF with energy efficiency issue of high energy consumption. So, the VNF energy consumption data is required to monitor the energy consumption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR includes the following changes: - Clarify the use case and requirement to identify the cell/NF with energy efficiency issue of high energy consumption. - Add the enabling data and analytic output. - Add an annex to clarify how analytic output is generated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Energy efficiency issue cannot be detected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.2.4.1.2, 7.2.4.1.3, 8.4.4.1.2, 8.4.4.1.3, 8.5.x (new), Annex C (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void

[3]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "Management and orchestration;5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication managemen; Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[15]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[16]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

[18]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire".

[19]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[20]	3GPP TS 28.511: "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Procedures".

[21]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[22]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[23]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[24]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/Machine Learning (AI/ML) management".

[25]	3GPP TS 32.160: "Management and orchestration; Management service template".

[26]	ETSI GS NFV-IFA 011 (V3.3.1): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; VNF Descriptor and Packaging Specification".

[27]	Recommendation ITU-T X.733: "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[x]	ETSI ES 202 336-12 “Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model”.

[y]	ETSI GS NFV 003: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV”,

[z]	3GPP TS 28.500: "Management concept, architecture and requirements for mobile networks that include virtualized network functions”

Next Modified Section

### 7.2.4	MDA assisted Energy Saving

#### 7.2.4.1	Energy saving analysis

##### 7.2.4.1.1	Description

This MDA capability is for the energy saving analysis.

##### 7.2.4.1.2	Use cases

Operators are aiming at decreasing power consumption in 5G networks to lower their operational expense with energy saving management solutions. Energy saving is achieved by activating the energy saving mode of the NR capacity booster cell or 5GC NFs (e.g. UPF etc.). The energy saving decision making is typically based on the load information of the related cells/UPFs, the energy saving policies set by operators and the energy saving recommendations provided by MDAS producer. To achieve an optimized balance between the energy consumption and the network performance, MDA can be used to assist the MDAS consumer to make energy saving decisions.

To make the energy saving decision, it is necessary for MDAS consumer to determine where the energy efficiency issues (e.g. high energy consumption, low energy efficiency) exist, and the cause of the energy efficiency issues. Therefore, it is desirable for MDA to correlate and analyze the energy saving related performance measurements (e.g. PDCP data volume of cells, power consumption, etc.) and the network analysis data (e.g. observed service experience related network data analytics) to provide the analytics results which indicate current network energy efficiency. In some low-traffic scenarios, MDA MnS consumers may expect to reduce energy consumption to save energy. In this case, the MDA MnS consumer may request the MDAS producer to report only high energy consumption issue related analytics results.  When the consumer expects to improve energy efficiency, although it may lead to high energy consumption in network or in certain parts of network, then the related issue is the low energy efficiency one. In that case, the consumer may request analytics results related to low energy efficiency issue. So, the target could be to enhance the performance of NF for a given energy consumption. This will result in higher Energy Efficiency of network.

To make the energy saving decision, it is necessary for MDAS consumer to determine which Energy Efficiency (EE) KPI related factor(s) (e.g. traffic load, end-to-end latency, active UE numbers, etc.) are affected or potentially affected. The MDAS producer can utilize historical data to predict the EE KPI related factors (e.g. load variation of cells at some future time, etc.). The prediction result of these information can then be used by operators to make energy-saving decision to guarantee the service experience.

The MDAS producer may also provide energy saving related recommendation with the energy saving state to the MDAS consumer. Under the energy saving state, the required network performance and network experience should be guaranteed. Therefore, it is important to formulate appropriate energy saving policies (start time, dynamic threshold setting, base station parameter configuration, etc.). The MDAS consumer may take the recommendations with the energy saving state into account for making analysis or making energy saving decisions. After the recommendations have been executed, the MDA producer may start evaluating and further analyzing network management data to optimize the recommendations.

As described above, energy saving analysis needs to determine when it is necessary to take the energy saving actions for NR and 5GC NFs that can be triggered by energy efficiency related measurements or KPIs. For example, when the energy efficiency is below certain threshold, it is the time for energy saving analysis to analyze the cause and determine the mitigation actions. The energy efficiency for NR and 5GC NFs are tightly coupled to the VNF energy consumption data. Therefore, it is necessary for energy saving analysis to predict the energy consumption for VNF instance(s), based on virtual compute usage, virtual memory usage, and virtual disk usage measurements (see clause 5.7.1.1 in TS 28.552 [4]).

##### 7.2.4.1.3	Requirements

Table 7.2.4.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ES_MDA-01 | MDA capability for energy saving analysis shall be able to identify the energy efficiency issue (including high energy consumption, low energy efficiency), and identify the cell/NFs or location area of where the indicated energy efficiency issue exists. | Energy saving analysis |
| REQ-ES_MDA-02 | MDA capability for energy saving analysis shall be able to identify the root cause of the energy efficiency issue when necessary. | Energy saving analysis |
| REQ-ES_MDA-03 | MDA capability for energy saving analysis shall be able to utilize the network status analysis and predictions information of the energy efficiency KPI factors (including, traffic load trends) to assist achieving energy saving. | Energy saving analysis |
| REQ-ES_MDA-04 | MDA capability for energy saving analysis shall be able to provide the energy saving recommendation, including policies and configuration actions to guarantee the network performance and end user service experience. | Energy saving analysis |
| REQ-ES_MDA-05 | MDA capability for energy saving analysis shall be able to predict the energy consumption for VNF instance(s). | Energy saving analysis |

Next Modified Section

## 8.4	Data definitions per MDA capability

### 8.4.4	MDA assisted energy saving

#### 8.4.4.1	Energy saving analysis

##### 8.4.4.1.1	MDA type

The MDA type for energy saving analysis is: MDAAssistedEnergySaving.EnergySavingAnalysis.

##### 8.4.4.1.2	Enabling data

The enabling data for MDAAssistedEnergySaving.EnergySavingAnalysis MDA type are provided in table 8.4.4.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.4.1.2-1: Enabling data for energy saving analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | PNF Power Consumption: power consumed over the measurement period | Clause 5.1.1.19.2 of TS 28.552 [4]. |
|  | PNF Energy consumption: energy consumed | Clause 5.1.1.19.3 of TS 28.552 [4]. |
|  | SS-RSRP distribution per SSB (beam) of serving NR cell | Clause 5.1.1.22.1 of TS 28.552 [4]. |
|  | SS-RSRP distribution per SSB (beam) of neighbor NR cell | Clause 5.1.1.22.1 of TS 28.552 [4]. |
|  | PDCP Data Volume of NR cells: PDCP data volume delivered in the downlink and uplink | Clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4] |
|  | Traffic load variation: - PRB utilization rate; - RRC connection number; - etc. | Clause 5.1.1.2 and 5.1.1.4 of TS 28.552 [4]. |
|  | UE throughput: - UE throughput in downlink and uplink | Clause 5.1.1.3 of TS 28.552 [4]. |
|  | Delay related measurements of UPF | Clause 5.4 of TS 28.552 [4]. |
|  | Data volume of UPF | Clause 5.4 of TS 28.552 [4]. |
|  | Virtual resource usage of NF: The virtual CPU usage, virtual memory usage, virtual disk usage of virtual network functions | Clause 5.7.1 of TS 28.552 [4]. |
| MDT reports | The RSRPs of UE measurements | RSRPs of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
|  | The RSRQs of UE measurements | RSRQs of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
|  | The UE location information | UE location of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
| QoE Data | The measurements that are collected are DASH and MTSI measurements | TS 28.406 [9]. |
| Configuration data | MOIs of the cells, UPFs and SMFs | TS 28.541 [15]. |
| Network analytics data | The control plane analysis result from the NWDAF, e.g. observed service experience related network data analytics | TS 23.288 [10]. |
| Virtualized NE energy consumption data | The measurement provides the energy consumption of the virtualized NE. | Clause 5.1.1.19.8 in TS 28.552 |

##### 8.4.4.1.3	Analytics output

The specific information elements of the analytics output for energy saving analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.4.1.3-1.

Table 8.4.4.1.3-1: Analytics output for energy saving analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| energyEfficiencyProblematicObject | Indication of NR cells or NFs where the energy efficiency issues occurred or potentially occur. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energyEfficiencyProblemType | Indication of type of the energy efficiency issues. The allowed value is one of the enumerated values: HighEnergyConsumption, LowEenergyEfficiency, Other, Unknown. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trafficLoadTrends | The predictions of the trends of traffic load in a certain time period. The predictions include the traffic load of the issue cell(s) and neighboring cell(s). | M | type:TrafficLoadTrend multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySavingRecommendations | The recommendation shall contain the energy saving policy. For ES on NR cells. It may contain a set of: - Recommended NR Cell (ES-Cell) to enter energySaving state. - Recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - The time to enter and terminate the energy saving state. - The load threshold to enter and terminate the energy saving state for the ES-Cell. For ES on UPFs. It contains a set of: - Recommended UPF (ES-UPF) to conduct energy saving. - Recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. - The time to conduct energy saving for the ES-UPF. | M | type: EsRecommendation multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| statisticsOfCellsEsState | The statistic result of current energy saving state of the cells at a certain time, which can be used by consumers to make analysis (e.g. observed service experience analysis made by NWDAF) or to make decision (e.g. enter/exit the energy saving state based on the current energy saving state). | O | type: StatisticOfCellEsState multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predictedVnfEnergyConsumption | It provides the predicted energy consumption for VNF instance(s). It could be generated, based on the input data of virtual compute usage, virtual memory usage, virtual disk usage (NOTE). |  | type: PredictedVnfEC multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE: Refer Annex C for the inference conducted by MDAF, using ML model. |  |  |  |

Next Modified Section

### 8.5.x	PredictedVnfEC <<dataType>>

#### 8.5.x.1	Definition

This data type specifies the type of predicted VNF energy consumption.

#### 8.5.x.2	Information elements

Table 8.5.x.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| mfDN | It indicates the DN of the ManagedFunction MOI of the VNF instance. | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startTime | It indicates the start time of the analytic window of the energy consumption prediction. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| endTime | It indicates the end time of the analytic window of the energy consumption prediction. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| predictedEC | It provides the value of the VNF energy consumption in kilowatt-hours (kWh). | M | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

Next Modified Section

# Annex C (Informative): VNF energy consumption prediction

# C.1	General

An MDA function may be deployed as an AI/ML inference function using the relevant ML model to predict the VNF energy consumption. Figure C-1 shows that the AI/ML inference function utilizes the ML model to predict the VNF #1 and VNF #2 energy consumption data, based on virtual Compute/memory/disk usage data for VNF #1 and VNF #2, and virtualized NE energy consumption data.

Figure C-1: MDA inference function for VNF energy consumption

Figure C-2 shows an example describing the training of the ML model used by the inference function to predict the VNF energy consumption. It shows that the virtualized NE (see TS 28.500 [z]) contains the NFVI (see ETSI GS NFV 003 [y]) in which two VNF instances (i.e., VNF #1 and VNF #2) are deployed. PEE sensors as defined in ETSI ES 202 336-12 [x] are used to collect the energy consumption of the virtualized NE. Figure C-2 also shows the samples of virtualized NE energy consumption data and the virtual Compute/memory/disk usage data for VNF #1 and VNF #2 (see clause 5.7.1.1.1-3 in TS 28.552 [4]) fluctuate continuously over time. The virtual Compute/memory/disk usage data are time synchronized with the virtualized NE energy consumption data, so they can be used as data samples and data labels for model training.

The model training function will use huge volume of dataset, including the data samples of virtual Compute/memory/disk usage data for VNF #1 and VNF #2, and data labels of the virtualized NE energy consumption data to compute the parameters of the ML model that is used to predict the energy consumption for VNF instance(s).

Figure C-2: VNF energy consumption model training

End of Modified Sections