3GPP TSG-SA5 Meeting #146	S5-226365

# Toulouse, France, 14 - 18 November 2022

# Source:	AsiaInfo

# Title:	pCR 28.834 Add Use Case on performance management of the cloud-native VNF using generic OAM functions

# Document for:	Approval

Agenda Item:	6.8.5.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.834-030 “Study on Management of Cloud Native Virtualized Network Funciton”.

[2]	ETSI GS NFV-IFA 049 V0.0.3 “Network Functions Virtualisation (NFV) Release 4; Architectural Framework”.

3	Rationale

This contribution proposes to add the use case and potential requirements on performance management of the cloud-native VNF using generic OAM functions.

4	Detailed proposal

It proposes to make the following changes to TR 28.834.

1nd Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- NGMN Cloud Native Enabling Future Telco Platforms v5.2. 17. May 2021

- ETSI GR NFV-IFA029: “Report on the Enhancements of the NFV architecture towards cloud-native and Paas”

- ETSI GR NFV-EVE019: “Report on VNF generic OAM functions”

- ETSI GR NFV-IFA037: “Report on further NFV support for 5G”

- ETSI GS NFV-IFA040: “Requirements for service interfaces and object models for OS container management and orchestration specification”

- ETSI GS NFV-EVE 011 “Specification of the Classification of Cloud Native VNF implementations”

- ETSI GR NFV-IFA 008: "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

- ETSI GS NFV-IFA013 V4.3.1 (2022-06) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

- 3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

- 3GPP TS 28.533: "Management and orchestration; Architecture framework".

- [X]		ETSI GS NFV-IFA 049 V0.0.3 “Network Functions Virtualisation (NFV) Release 4; Architectural Framework”.

2nd Change

5	Potential use cases and requirements

Editor's note: this clause will contain the use cases and potential requirements.

5.X	Use case# Num: Performance management of the cloud-native VNF using generic OAM functions

5.X.1	Description

This use case is about the performance management of cloud-native VNFs using the " VNF metrics aggregator function and VNF metrics analyser function", which belongs to generic OAM functions proposed in [X]. The VNF metrics aggregator function can collects the metrics from the VNF/VNFC/NFV-MANO, the VNF metrics analyser function can analyses the metrics provided by the VNF metrics aggregator and can be configured to send notifications based on e.g. statistical processing, abnormal behaviour detection, or threshold crossing.

The 3GPP management system sends a performance management request to the VNF metrics analyser , then the VNF metrics analyser retrieves data from the VNF metrics aggregator.

The VNF metrics analyser processes and evaluates the data collected (e.g analysis and indentifies passibile issues), the 3GPP management system will receive the management resulte from VNF metrics analyser function.

5.X.2	Requirements

REQ-CVNF_TM_CON-1 The 3GPP management system shall be able to send a performance management request for a cloud-native VNF to the VNF metrics analyser function.

REQ-CVNF_TM_CON-2 The 3GPP management system shall be able to receive a returned result from VNF metrics  analyser function about the performance management of cloud-native VNFs.

End of Changes