3GPP TSG-SA5 Meeting #146 	S5-226372

Toulouse, France, 14 -18 November 2022

# Source: 	Huawei, Nokia

# Title:	pCR TR 28.908 Correct the illustration of AI/ML inference history request and control

# Document for:	Approval

Agenda Item:	6.7.5.2

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

References

[1]		3GPP TR 28.908 Study on Artificial Intelligence / Machine Learning (AI/ML) management

3	Rationale

In TR 28.908 [1], Section 5.6 discussed the use case on AI/ML inference history. Figure 5.4.2.2-1 illustrated the example use and control of MLInferenceHistory Request and reporting. In the figure, the operator and consumer are separated. However, the operator is also a kind of consumer when it consumes the AI/ML inference history MnS. Therefore, this contribution is proposed to correct the illustration to avoid misunderstanding.

4	Detailed proposal

It is proposed to update the following chapter in TR 28.908 [1].

1st modified section

5.6	AI/ML Inference History

5.6.1	Description

For different automation requirements, network and management automation functions (e.g. gNB, MDAS, SON) may apply Machine Learning functionality to make the appropriate inferences in different contexts. Depending on the contexts, the AI/MLEntity may take different decisions at inference with different outcomes. The history of such inference decisions and the context within which they are taken may be of interest to different consumers.

5.6.2	Use cases

5.6.2.1	Tracking AI/ML inference decision and context

The AI/ML-enabled function may take different decisions at inference in different contexts and with different outcomes. The selected decisions may need to be tracked for future reference, e.g. to evaluate the appropriateness/ effectiveness of the decisions for those contexts or to evaluate degradations in the AI/MLEntity's decision-making capability. For this, the network not only needs to have the required inference capabilities but needs also to have the means to track and enable usage of the history of the inferences made by the ML applications.

Figure 5.6.2.1-1: Example use and control of MLInferenceHistory Request and reporting in the management plane

5.6.3	Potential requirements

REQ-MLHIST-1 The producer of ML inference history should have a capability allowing an authorized consumer to request the inference history of a specific AI/MLEntity.

REQ-MLHIST-2  The producer of ML inference history should support a capability for an authorized consumer (e.g. the function/entity that generated the Request for ML inference history) to define the reporting characteristics related to a specific instance of ML inference history or the reporting thereof.

End of modified section