3GPP TSG-SA5 Meeting #146 	S5-226373

Toulouse, France, 14 -18 November 2022

# Source: 	Huawei, Nokia

# Title:	pCR TR 28.908 Correct the illustration of AI/ML entity testing and control

# Document for:	Approval

Agenda Item:	6.7.5.2

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

References

[1]		3GPP TR 28.908 Study on Artificial Intelligence / Machine Learning (AI/ML) management

3	Rationale

In TR 28.908 [1], Section 5.4 discussed the use case on AI/ML entity testing. According to the description, an authorized consumer can request or control the AI/ML entity testing. Figure 5.4.2.2-1 illustrated the AI/ML entity testing and control. In the figure, the operator and consumer are separated. However, the operator is also a kind of consumer when it consumes the AI/ML entity testing MnS. Therefore, this contribution is proposed to correct the illustration to avoid misunderstanding.

4	Detailed proposal

It is proposed to update the following chapter in TR 28.908 [1].

1st modified section

5.4	AI/ML entity testing

5.4.1	Description

After an AI/ML Entity is trained, validation is done to ensure the training process is completed successfully. However, even when validation is conducted successfully during development, it is necessary to test and check if the AI/ML Entity is working correctly under certain runtime contexts or using certain testing data set. Testing may involve interaction with third parties (besides the developer of the AI/ML training function) , e.g. the operators may use the AI/ML training function or third-party systems/functions that may rely on the results computed by the AI/ML entity for testing.

After completing the AI/ML entity training, and when the performance of the trained AI/ML entity meets the expectations on both training and validation data, the AI/ML entity is made available to the consumer(s) via the AI/ML training report (see AIMLTrainingReport IOC in TS 28.105 [4]). Before applying the AI/ML entity to the target inference function or AI/ML-enabled function, the AI/ML training MnS producer may need to allow the consumer to evaluate the performance of the AI/ML entity via the AI/ML testing process using the consumer’s provided testing data. The testing data have the same pattern as the input part of the training data.

For these reasons, provision of AI/ML testing and its control need to be standardized to enable the multi-vendor interaction among the different systems. If the testing performance is not acceptable or does not meet the pre-defined requirements, the consumer may request the AI/ML training producer to re-train the AI/ML entity with specific training data and/or performance requirements.

5.4.2	Use cases

5.4.2.1	Consumer-requested AI/ML entity testing

After receiving an AI/ML training report about a trained AI/ML entity from the AI/ML Training MnS producer, the consumer may request the testing MnS producer to test the AI/ML entity before applying it to the target inference function. In the AI/ML testing request, the consumer provides the testing data which have the same pattern as the input part of the training data.

Any AI/ML entity needs to be tested with specific inputs and features that are applicable to the use case and the applicable deployment environment.

The AI/ML testing MnS producer performs the AI/ML testing using the consumer’s provided testing data. The AI/ML testing is to conduct inference on the tested AI/ML entity using the testing data as the inference inputs and produce the inference output for each testing dataset example.

The AI/ML testing MnS producer may be the same as or different from the AI/ML Training MnS producer.

After completing the AI/ML testing, the AI/ML testing MnS producer provides the testing report indicating the success or failure of the AI/ML testing to the consumer. For a successful AI/ML testing, the testing report contains the testing results, i.e., the inference output for each testing dataset example.

The AI/ML testing MnS producer needs to have the capabilities and provide the services needed to enable the consumer to request testing and receive results on the testing of a specific AI/ML entity or of an application or function that contains an AI/ML entity.

To achieve the desired outcomes, any AI/ML entity needs to be tested with the appropriate testing data, which can reflect the current status of the network where the AI/ML entity is expected to be deployed. Correspondingly, the AI/ML testing MnS producer needs to support the required management services to test the AI/ML entities.

5.4.2.2	Control of AI/ML entity testing

Given a testing capability as provided by a given AI/ML testing MnS producer, a consumer (e.g., an operator) may wish to control and manage that testing process capability. For example, the operator may wish to define policies on how frequent testing for a given AI/ML entity may be executed. Correspondingly, the 3GPP management system needs to provide the capability to allow the AI/ML entity testing to be configured.

Figure 5.4.2.2-1: AI/ML entity testing and control

End of modified section