3GPP TSG-SA5 Meeting #146 	S5-226375

Toulouse, France, 14 -18 November 2022

# Source: 	Huawei

# Title:	pCR TR 28.908 Update use case on AIML model performance indicators

# Document for:	Approval

Agenda Item:	6.7.5.2

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

References

[1]		3GPP TR 28.908 Study on Artificial Intelligence / Machine Learning (AI/ML) management

3	Rationale

TR 28.908 [1] discussed the use cases of AI/ML model performance management. Section 5.1.2.1 mentioned some AI/ML model performance indicators that needs to be defined. However, the current decription about AI/ML model performance indicators is not very clear. This contribution proposes to update the use case on AI/ML model performance indicators.

4	Detailed proposal

It is proposed to update the following chapter in TR 28.908 [1].

1st modified section

5.1	AI/ML model performance management

5.1.1	Description

During AI/ML model training, test deployment and inference, the AI/ML model performance evaluation and management is needed. The related performance indicators need to be collected and analyzed. The purpose of AI/ML performance management is to find the problem, figure out what the problem is, and fix it in time to make sure the model can be trained, tested, deployed and inferred healthy.

5.1.2	Use cases

5.1.2.1	AI/ML model related performance indicators

The AI/ML model related performance indicators involving the AI/ML model training, test, deployment and inference need to be considered and defined. The indicators mainly include three aspects:

Model training resource performance indicators:  the performance indicators of the system that the model trains. e.g., "training duration" etc.

Model performance indicators: performance indicators of the model itself, e.g., "accuracy", "precision", "F1 score", etc..

Service-related indicators: the running state indicators of the launched model.

Model-applied network performance indicators: the network performance which is affected by the AI/ML model application. For example, handover success rate in TS 28.552 and TS 28.554 may be affected by the output decision of a model used for energy saving.

These indicators need to be precisely defined. For different service, some indicators can be selected for evaluation. For example, resource-related indicators and model-related indicators may be selected in the training phase, while model-related indicators, model- applied network performance indicators and service-related indicators may be selected in the inference phase. The AI/ML MnS producer should first determine which indicators are needed and then use these indicators for evaluation.

5.1.2.2	Monitoring and control of AI/ML behavior

In a typical network operation, an operator configures and operates an MLENTITY according to the manual of the MLENTITY. Usually, the operator does not need to know the details of the MLENTITY’s internal-decision making, simply due to “too many”MLEntitys running for the network and, too much detail that is unnecessary information for the operator. as it is in the vendor's interest to hide the internal aspects of the implementation of their automation solutions.

However, the operator still needs to guide the solutions and to configure them to achieve the desired outcomes in an MLEntity-agnostic manner. For example, consider the load balancing automation use case (AutoLB) summarized by Table 1. With an AutoLB MLEntitydecides that how distribute load among networking objects. The MLEntity has specific actions that it can take while the operator also has operational actions that it needs to take to steer the solution, e.g. to switch off the solution, to reconfigure the solution, to change the solutions input. As such, the behavior of the MLEntity in terms of the actions it takes under any given conditions needs to be related to the configuration actions from the MnS consumer (e.g. the operator)

Table 5.1.2.2-1: Operability of the Automated load balancing

| Automation use case | Description | MLEntity's context | Example MLEntity's decisions | Operator's Actions |
| --- | --- | --- | --- | --- |
| load balancing (AutoLB) | Distribute load among different networking objects, e.g. among cells. | - Amount of traffic, no of users, etc | - select CIO values | - set the maximum CIO values, - Deactivate AutoLB |

So, it is important that even without knowing the details of the MLEntity, the operator needs to have the understanding of the MLENTITY’s overall behaviour. And, if a part of the MLEntity’s decisions/actions are not what is preferred by the operator, the operator needs to instruct the MLEntity to behave accordingly. Relatedly, means are needed to enable the operator to associate his actions to the context and to enable the operator to guide the AI/ML-enabled function or solution.

5.1.3	Potential requirements

REQ-MODEL_PERF-CON-1		The 3GPP management system should have a capability to select some performance indicators of AI/ML model based on the service.

REQ-AI/ML_BEH-1 	The 3GPP management system shall have a capability to inform authorized AI/ML MnS consumer (e.g. the operator) about the behavior of the MLEntity, in an MLENTITY agnostic manner without the need to expose its internal characteristics

REQ-AI/ML_BEH-2 	The 3GPP management system shall have capability that enables an authorized AI/ML MnS consumer (e.g. the operator) to configure the behavior of the MLEntity, in an MLENTITY agnostic manner that does need to expose its internal characteristics

5.1.4	Possible solutions

TBD

5.1.5	Evaluation

TBD.

End of modified section